The following three tables describe the operation and performance of all types of AIS Station in relation to

- The handling of received messages

- The capability to transmit messages

- The principle parameters of the AIS system as recommended by ITU-R M.1371-3

| # Station type | Class A | Class B “SO” | Class B “CS” | AtoN Type 1 | AtoN Type 2 | AtoN Type 3 | Base Station | Repeater | SART | SAR Aircraft | Limited base Station |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| # Physical layer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dual Channel | C | C | C |  | R | R | C | C | C |  | C |  | C | O |
| Single Channel | N | N | N |  | O | O | N | N | N |  | N |  | N | O |
| Full Frequency Range capability | C | O | O |  | O | O | O | C | O |  | N |  | O | O |
| Rx/Tx | C | C | C |  | N | N | C | C | C |  | N |  | C | C |
| Tx only | N | N | N |  | C | C | N | N | N |  | C |  | N | N |
| DSC Functionality | R | R | R |  | N | N | N | O | N |  | N |  | R | R |
| Operating frequency assignment capability | C | O | O |  | N | N | N | N | N |  | N |  | C | O |
| Tx Power in Watts 12.5/2/1or Manufacturer to Define | 12.5/1 | MTD | 2 |  | MTD | MTD | MTD | MTD | 12.5/2 |  | MTD |  | MTD | 12.5/1 |
| # Link layer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TDMA Synchronisation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UTC Direct | C | C | O |  | C | C | C | C | C |  | C |  | C | C |
| UTC Indirect | C | O | O |  | N | N | N | C | C |  | N |  | C | O |
| Use semaphore synchronisation | C | O | N |  | N | N | N | C | C |  | N |  | C | N |
| Act as semaphore | C | O | N |  | N | N | N | C | N |  | N |  | C | N |
| Timing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Long Transmission packets. No of slots | 3 | 3 | 1 (2) |  | 3 | 3 | 3 | 5 | 3 |  | 1 |  | 3 | 2 |
| Mode of operation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Autonomous | C | C | C | C |  | C | C | C | C |  | C | C |  | C |
| Assigned | C | C | C | N |  | N | N | N | N |  | N | C |  | C |
| Polled | C | C | C | N |  | N | N | C | N |  | N | C |  | C |
| Limitation maximum permissible number of slots per frame. | 20 | 34 | 34 | 20 |  | 20 | 20 | None | None5 |  | 10 | 20 |  | 20 |
| VDL Access schemes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SOTDMA | C | C | N | N |  | N | N | N | N | N |  | C |  | N |
| RATDMA | C | C | N | N |  | N | C | O | C | N |  | C |  | C |
| ITDMA | C | C | N | N |  | N | N | N | N | N |  | C |  | C |
| FATDMA | N | N | N | C |  | C | C | C | O | N |  | N |  | N |
| CSTDMA | N | N | C | N |  | N | O | N | N | N |  | N |  | N |
| FUTDMA | N | N | N | N |  | N | N | O | N | Y |  | N |  | N |
| # Network layer |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Channel operation & management |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Default channel operation AIS 1 & AIS 2 | C | C | C | C |  | C | C | C | C | C |  | C |  | C |
| Channel management msg 22 | C | C | C | N |  | N | N | N | N | N |  | C |  | C |
| DSC telecommand | C | O | O | N |  | N | N | N | N | N |  | C |  | N |
| Manual input | C | N | N | N |  | N | N | N | N | N |  | N |  | N |
| Presentation interface | C | N | N | N |  | N | N | C | N | N |  | C |  | C |
| Reporting Modes A, B or C. | N | N | N | C |  | C | C | N | N | C Mode B only |  | N |  | N |
| Frequency agile | C | O | O | O |  | O | O | C | O | N |  | O |  | O |
| Slot re-use Data link congestion. | C | C | N | N |  | N | C | C | C | N |  | C |  | N |
| Long range | C | N | N | N |  | N | N | N | N | N |  | N |  | N |

4. The limitation is for messages 6, 8, 12 & 14.

5. The development of AIS Repeater Stations is on going within IEC and the limitation on the maximum number of slots per frame is yet to be determined.

C	Compulsory in any configuration

O	Optional

MTD	Manufacturer to determine.

N	Not allowed

| Table of AIS Equipment vs Message Types Transmit behaviour. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Transmit behaviour | Class A | Class B "SO" | Class B "CS" | AtoN Type 1 | AtoN Type 2 | AtoN Type 3 | Base Station | Repeater | SART | SAR Aircraft | Limited base Station |
| Message |  |  |  |  |  |  |  |  |  |  |  |
| 1 | T | P | P | P | P | P | V2 | Ot | T | P | P |
| 2 | T | P | P | P | P | P | V2 | T | P | P | P |
| 3 | T/I | P | P | P | P | P | V2 | T | P | P | P |
| 4 | P | P | P | P | P | P | T/I/V | T | P | P | P |
| 5 | T/I | P | P | P | P | P | V2 | Ot | P | P | P |
| 6 | T | T | Ot1 | Ot | Ot | Ot | T/V | Ot | P | Ot | Ot |
| 7 | T | T | Ot1 | P | P | Ot | T/V | ? | P | T | T |
| 8 | T | T | Ot1 | Ot | Ot | Ot | T/V | Ot | P | T | T |
| 9 | P | P | P | P | P | P | V2 | T? | P | T | P |
| 10 | T | P | P | P | P | P | T/V | T | P | T | P |
| 11 | T | P | P | P | P | P | V2 | T | P | P | P |
| 12 | T | T | P | Ot | Ot | Ot | T/V | T | P | T | T |
| 13 | T | T | Ot | P | P | Ot | T/V | T | P | T | T |
| 14 | T | T | Ot | Ot | Ot | Ot | T/V | T | T | T | T |
| 15 | T | P | P | P | P | P | T/V | T | P | T | T |
| 16 | P | P | P | P | P | P | T/V | T | P | P | P |
| 17 | P | P | P | P | P | P | T/V | P | P | P | P |
| 18 | P | T/I | T/I | P | P | P | V2 | T | P | P | P |
| 19 | P | T/I | I | P | P | P | V2 | T | P | P | P |
| 20 | P | P | P | P | P | P | T/V | T | P | P | P |
| 21 | P | P | P | T | T | T | V2 | T | P | P | T |
| 22 | P | P | P | P | P | P | T/V | T | P | P | P |
| 23 | P | P | P | P | P | P | T/V | T | P | T | P |
| 24A | P | I | I/T | P | P | P | Oi/Ot/V | T | P | I/T | I/T |
| 24B | P | I | I/T | P | P | P | V2 | T | P | I | P |
| 25 | T | P | P | Ot | Ot | Ot | T/V | T | P | T | P |
| 26 | T | T | P | P | P | P | T/V | T | P | T | T |
| 27 | T | P | P | P | P | P | V2 | P | P | P | P |
| Notes |  |  |  |  |  |  |  |  |  |  |  |
| R | Receives and Processes |  |  |  |  |  |  |  |  |  |  |

T		Transmit by own station

V		Transmit if VDM sentence is received via PI

F		Forward to PI

I		Responds if interrogated by msg 15

P		Not allowed/prohibited.

Ot		Optional Tx

Or		Optional Rx

Oi		Optionally if interrogated

- IEC 62287 prohibits MSGs 6, 7 & 8 but this needs to be addressed as binary messages should be allowable for Class B.

- The Base Station is capable of broadcasting all messages via a VDM PI input, however these messages should not be transmitted.

| Table of AIS Equipment vs Message Types Receive Behaviour |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Receive behaviour | Class A | Class B "SO" | Class B "CS" | AtoN Type 1 | AtoN Type 2 | AtoN Type 3 | Base Station | Repeater | SART | SAR Aircraft | Limited base Station |
| Message |  |  |  |  |  |  |  |  |  |  |  |
| 1 | R/F | R/F3 | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 2 | R/F | R/F3 | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 3 | R/F | R/F3 | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 4 | R/F | R/F3 | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 5 | R/F | R/F3 | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 6 | R/F | R/F | Or | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 7 | R/F | R/F | Or | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 8 | R/F | R/F | Or | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 9 | R/F | R/F | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 10 | R/F | NA | NA | NA | NA | NA | R/F | R | NA | NA | R/F |
| 11 | R/F | NA | NA | NA | NA | NA | R/F | R | NA | NA | R/F |
| 12 | R/F | R/F3 | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 13 | R/F | R/F3 | NA | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 14 | R/F | R/F | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 15 | R/F | R/F | R/F | NA | NA | NA | R/F | R | NA | R/F | R/F |
| 16 | R/F | R/F | NA | NA | NA | NA | NA | R | NA | R/F | R/F |
| 17 | R/F | R/F | Or | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 18 | R/F | R/F | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 19 | R/F | R/F | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 20 | R/F | R/F | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 21 | R/F | R/F | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 22 | R/F | R/F | R/F3 | NA | NA | NA | NA | R | NA | R/F | R/F |
| 23 | R/F | R/F | R/F | NA | NA | NA | NA | R | NA | R/F | R/F |
| 24A | R/F | R/F | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 24B | R/F | R/F | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 25 | R/F | R/F | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 26 | R/F | R/F | R/F3 | NA | NA | R/F | R/F | R | NA | R/F | R/F |
| 27 | NA | NA | NA | NA | NA | NA | Or | NA | NA | NA | NA |
| Notes |  |  |  |  |  |  |  |  |  |  |  |
| R | Receives and Process |  |  |  |  |  |  |  |  |  |  |

F		Forward to PI

Or		Optional Rx

NA		Not applicable

3. Only if the Class B has a presentation port.