Report

An e-Navigation Workshop

Develop Assistant Services from Shore Facilities

Minato-Ku, Tokyo, Japan

29 October - 02 November, 2007

Executive Summary

An e-Navigation workshop to discuss the Development of Assistant Services from Shore Facilities was held from 29th October to 02nd November, 2007 at Minato-Ku, in Tokyo, Japan.

The workshop was characterised by a set of discussions where a panel of invited experts exchanged views with officers of Japan Coast Guard (JCG) on the first and third day. The topics discussed included the e-Navigation concept, strategy for its implementation and ECDIS & charting issues.  Positioning systems and communications, along with AIS and VTS issues were also discussed.

Approximately 170 persons attended a public symposium on the second day (30 Oct), where the invited experts made a series of presentations.

The workshop identified six actions items for JCG and two for IALA.

Table of Contents

Introduction	4

Session 1 - Opening of the workshop	4

1.2	 Introduction of participants	4

1.3	Programme for the week	4

Session 2: Study on e-Navigation in Japan	5

2.1	Development of e-Navigation in Japan	5

2.2	Technical Study for e-Navigation within JCG	5

Session 3: Symposium on e-Navigation	6

3.1 The history of the development of e-Navigation and the effort for evolution	6

3.2 Robust radio navigation and board band communication for e-Navigation era	7

3.3 Advanced application of AIS and results of study committee in Japan	7

3.4 Operational Aspect of e-Navigation Activities and Future Expectation of IALA	8

3.5 Electronic Charting and e-Navigation: Challenges and Opportunities	8

Session ４: Information Exchanges on e-Navigation	9

Closing Of The Workshop	10

Annex 1 – Workshop Programme	11

Annex 2 – Summary of discussions held with invited experts	13

Actions Agreed To By The Workshop	21

# Introduction

An e-Navigation workshop to develop assistant services from shore facilities was convened at the Mita-Kaigisho, Tokyo, from October 29th to November 2nd, 2007.  Japan Coast Guard (JCG) coordinated the organisation and conduct of the workshop, which was sponsored by the Ocean Policy Research Foundation (OPRF), Japan.

Sixteen delegates representing four countries attended the workshop.

The five (5) day event witnessed wide representation from JCG officials and the maritime industry in Japan.

# Session 1 - Opening of the workshop

The Chairman, Mr Ryozo SHIGEMATSU, Director for Maritime Traffic Coordination opened the meeting at 1400 hrs on Monday, 29th October, 2007 and welcomed all participants to Tokyo, Japan.  Mr Tadayoshi IMAI, Director of Aids to Navigation Engineering Division, Maritime Traffic Department, also welcomed all participants.  He said JCG recognized that the development of e-Navigation for the safety and efficiency of marine navigation required support from shore facilities such as AIS, VTS radar and DGPS.  He stressed that such work should be carried out in close cooperation with IALA, IMO, IHO and other organisations and countries involved.

Mr IMAI added that the Maritime Traffic Department in JCG had just started to study the capability of providing information from shore based facilities, as a part of e-Navigation, not only for SOLAS vessels, but also for non-SOLAS craft navigating in Japan’s coastal waters. He explained that the purpose of the workshop was to exchange views with and learn about current developments from the four expert participants (IALA, US and Scotland) on issues relating to ECDIS and ECS, position fixing systems and also the provision of information to ships from shore-based facilities. He added that JCG had provided the outcome of their studies and research on Electronic Navigation Support System (ENSS) to some IALA Committees.

Mr. IMAI concluded by stating that he hoped there would be an open and free discussion, and wished the experts an enjoyable stay in Japan.

## 1.2	 Introduction of participants

The Chairperson requested all participants to introduce themselves.

## 1.3	Programme for the week

Mr Seiko FUKUSATO, Senior Officer, Policy, Planning and Research Office, Administration and Planning Division, outlined the schedule for the week, including daily working hours.  He provided information on the e-Navigation Symposium, to be held on October 30th and the technical tour to the Tokyo Wan VTS Centre planned for November 1.  He also wished the participants a comfortable stay in Japan.

# Session 2: Study on e-Navigation in Japan

## 2.1	Development of e-Navigation in Japan

Dr. Junji FUKUTO, Head of Navigation Systems and Human Factor Research Group, Navigation and System Engineering Department, National Maritime Research Institute, presented information on his study titled “Progress of Development of e-Navigation Strategies and Shipboard Systems in Japan”. He said that after co-sponsoring the proposal to IMO MSC 81 (May 2006), Japan Ship Technology Research Association (JASTRA) initiated a research project for taking action on the development of an e-Navigation strategy and on research for navigational support.

Studies had been undertaken on standards for navigational support, in order to contribute to the safety of navigation and the development of the maritime industry.

Dr FUKUTO first introduced following the project activities.

- Proposal and research report to NAV

"An approach to E-Navigation" IMO NAV 52/17/4 (2006)

"A method for identifying essential functions of shipboard system under the E-Navigation strategy" IMO NAV 53/13/1 (2007)

- Capturing user needs

- Making a roadmap for realizing the demanded supports

- Consideration of usability test for marine systems

He then explained the following on-going research topics

- INT-NAV (Integrated Navigational Information System on Seascape Image).

- Collision alert system for small craft

- Navigational intention exchange support system

Dr. FUKUTO's presentation received acclaim from the experts.  Their view was that INT-NAV was a real practical research for supporting active mariners with a display correlating the visual image and digital data. They also commented positively on the navigational intention exchange support system, saying that it provided effective decision making support.  Mr. ALIMCHANDANI asked about how user needs for e-Navigation were captured by the project. He also requested Dr. FUKUTO to submit the user needs using the IALA template to IALA and to the IMO e-Navigation Correspondence Group.  Dr. FUKUTO explained that the user needs were obtained by brain storming with small working group of active mariners and manufacturers, etc.  He also said that he had already been requested to submit it by IMO, and would do so by the middle of November.

## 2.2	Technical Study for e-Navigation within JCG

Mr. Hitoshi HATAKEYAMA, Senior Officer, VTS Office, AtoN Engineering Division, Maritime Traffic Department, made a presentation titled “The Current Technical Study for e-Navigation in JCG”. In his presentation, Mr. HATAKEYAMA provided information on the timetable for the establishment of AIS shore stations in Japan.  He also discussed the provision of effective information using AIS, a study on the effective use of AIS binary messages, the practical use of AtoN AIS, Electronic Navigation Support System (ENSS), and AIS tests in the Tokyo Bay area. In addition, Mr. HATAKEYAMA explained that he had provided this information to the e-NAV3 and ANM10 Committees of IALA.  This information had received positive feedback at the meetings.  Many delegates stated that they wished to see further reports (at future IALA Committee meetings) on the studies and developmental work being done by Japan on ENSS.

In response to a question on AIS AtoN from Prof. ALEXANDER, Mr. HATAKEYAMA explained the difference between physical, virtual and pseudo AIS AtoN.   As the meeting was unsure of the differences, Mr. ALIMCHANDANI explained the differences in more detail, drawing on the explanation given in IALA Recommendation A – 126 (Use of AIS as an Marine Aid to Navigation).

# Session 3: Symposium on e-Navigation

In his opening remarks as Chairman of the e-Navigation Symposium, Mr. Seiko FUKUSATO, Senior Officer of Policy Planning and Research Office, Administration and Planning Division, Maritime Traffic Department stated that JCG had organised the symposium in order to get a better understanding of the global developments in e-Navigation.   JCG was keen to develop e-Navigation by utilising existing systems and equipment and introducing new shore based systems and services.

Mr. FUKUSATO then introduced the four invited e-Navigation experts. They were Mr. ALIMCHANDANI from IALA, Mr. DOUGLAS from the Northern Lighthouse Board, Scotland, Mr. CAIRNS from the USCG, and Dr. ALEXANDER from the University of New Hampshire, USA.   He also stated that the Ocean Policy Research Foundation (OPRF) had sponsored the Symposium.

An audience of approximately 170 persons attended the symposium on the day.

## 3.1 The history of the development of e-Navigation and the effort for evolution

(Presented by Mr. Mahesh ALIMCHANDANI, IALA)

Mr. ALIMCHANDANI made a presentation titled “e-Navigation - A New Paradigm”.  Firstly, he outlined the history of the development of e-Navigation.  He stated that prior to the submission to IMO, IALA was already working on the concept of displaying real-time information on aids to navigation on board ships.  This was known as the e-ANSI project (electronic Aids to Navigation Service Information).

Mr ALIMCHANDANI explained the key findings of the newly-formed e-Navigation Committees and IMO NAV to date.  He added that the task now was to:

(1)	Identify all potential users,

(2)	Develop user requirements,

(3)	Develop a strategic vision, and

(4)	Review the need to consult maritime agencies.

Mr ALIMCHANDANI also offered an explanation as to what e-navigation was.  It was, in the main, the joining of the ship’s navigation team with the VTS team, in order to share tactical and planning information.  He concluded by drawing the symposium’s attention to conclusions and recommendations of the IALA e-Navigation Seminar (held at Trinity House, UK, in July 2007) and those of the Ad-Hoc Meeting on Loran/Chayka (held in Haugesund, Norway, in September 2007).  He also said that the IALA e-Navigation Committee had developed a list of Frequently Asked Questions (FAQs) on e-Navigation, which was available on the IALA website.

In response to a question from the floor on the e-Navigation strategy and human error by mariners, Mr. ALIMCHANDANI explained that regardless of any technical solution, procedures, training and above all, good watch-keeping practice were central to navigational safety.

In response to another question on potential interference, e.g. lack of slots, between AIS Class A and B in congested areas, Mr. ALIMCHANDANI explained that the Class B AIS system had a lower priority on the VHF data link and was unable to reserve slots – it only broadcast when it had an empty slot available.

3.2 Robust radio navigation and board band communication for e-Navigation era

(Mr. Peter DOUGLAS, NLB, Scotland)

Mr. DOUGLAS introduced current and future radionavigation systems and communication systems between ships and ship-shore necessary for e-Navigation, and noted that three essential components of e-Navigation were (1) fully redundant position-fixing (2) ENCs for all navigational areas, and (3) communication capability.

Mr. DOUGLAS further stated a vision on future of satellite navigation systems such as GPS, GLONASS, GALILEO, and DGPS using IALA radio beacon. Mr. DOUGLAS explained the vulnerability of GPS to interference and other failure modes.

Mr. DOUGLAS introduced enhanced Loran (eLoran) as an internationally-standardized positioning, navigation and timing (PNT) service for use by multiple modes, and noted that eLoran met the accuracy, availability, integrity and continuity performance requirement for marine harbours entrance and approach manoeuvres.

Finally Mr. DOUGLAS stressed the needs to develop broadband communication between ships and shore-ship in order to facilitate marine safety by providing effective information and data through the communication. As prospective broadband communication system within relatively short range from shore, Mr. DOUGLAS explained that WiFi or WiMAX was considered.  However Mr. DOUGAS added that governments and marine operators would not accept satellite broadband communication unless it was seen to be cost effective and used spectrum efficiently.

In response to a question from the floor on the technical solution of adopting shorter antenna height for Loran, Mr. Peter DOUGLAS explained that the shorter antenna height was attractive, however further investigation was required as broadcast range reduced considerably.  He added that there was a limit to short antenna height when using 100 kHz.

In response to an another question on the coverage of e-Loran as the back up to GNSS, Mr. Peter DOUGLAS stated that e-Loran should be considered as one of systems to be provided by service providers. He added that any back up to GNSS should be developed taking into account risk and user needs. He stated that the Dover Straits was very crowded and therefore a high risk area. Hence the UK had implemented e-Loran.

## 3.3 Advanced application of AIS and results of study committee in Japan

(Mr. Tatsuo SATO, Japan Aids to Navigation Association (JANA))

Mr. Tatsuo SATO introduced the recent activities on the study of AIS applications.

Mr. SATO introduced the result of the study done in the development of AIS applications and ENSS, including the AIS field test off the Irosaki Lighthouse, the Izu Peninsula, Kanagawa Reef and Nagoya Port during 2004-2006.  In the AIS field test, the following three subjects were evaluated and confirmed by navigators.

(1)	Transmission of AIS binary messages between ships and Tokyo Wan VTS Centre,

(2)	Evaluation of AIS binary message format used for meteorological and hydrographical data, pseudo AIS target data, and virtual aids to navigation data, physical existing AtoN data and synthetic AtoN data, and

(3)	Transmission and display of AIS binary message for virtual aids to navigation and others.

In addition, Mr. SATO introduced analysis of causalities in two categories of the range from coastal line and tonnage, and explained that causalities in range within 20 NM and by ships less than 500 tons occurred significantly.

Finally, Mr. SATO stressed three core elements necessary for development of AIS application, such as ENSS to e-navigation;

(a)	ECDIS or Electronic Chart System (ECS),

(b)	Robust Position Fixing System, and

(c)	Maritime Broadband Communication.

In response to questions from the floor on lower AIS communication ratio, effective AIS antenna location and virtual AtoN AIS for small vessels, Mr. SATO stated that the further investigation on effective AIS communication should be continued, that the investigation of the relevance between the ratio and antenna was conducted by some experts and the result would be reported later.  He added that the use of virtual AtoN AIS and AIS radar targets were recognized as effective tools, however further investigation, including legal issues, should be carried out for small vessels that had not installed AIS.  Prof. AKAI requested that the survey on the back up system in which AIS could not function, should be developed continuously when the expansion of AIS for small vessels was taken into account.

3.4 Operational Aspect of e-Navigation Activities and Future Expectation of IALA

(Mr. Bill CAIRNS (USCG) - Chairman, IALA e-NAV Committee)

With quotes on navigation from some famous historical persons, Mr. Bill CAIRNS started by outlining the role of the IALA e-Navigation Committee and the involvement of IALA in the development of e-Navigation.   He stressed the importance of user requirements for e-Navigation and said that there were three main obstacles to the development of e-Navigation – global ENC coverage, a robust, fail-safe, electronic positioning system and agreed communication links between ship and shore. Mr. CAIRNS explained the concept of e-Navigation with a system architecture diagram and a descriptive model.

Mr. CAIRNS continued by stating the IALA vision for e-Navigation based on trends in maritime operation (and the consequences of these trends) and the key goals.

Finally, Mr. CAIRNS added an explanation of the system to be provided by a common shore based architecture.

## 3.5 Electronic Charting and e-Navigation: Challenges and Opportunities

(Dr. Lee ALEXANDER, University of New Hampshire, USA)

Dr. ALEXANDER provided an overview of some of the more important implications for e-Navigation as it relates to the provision and use of navigation-related information.  He stressed that while there are opportunities to display supplemental information on electronic chart-related displays (e.g., Marine Information Overlays or MIOs), there are also significant challenges.  Mariners want to be able to obtain and use supplemental information in addition to that which is required for “safety-of-navigation.”  In particular, this includes information that can improve the efficiency of maritime navigation and marine environmental protection.  However, for maritime community, this type of information is not readily available.  Currently, many organizations continue to produce individual products/services on a sub-component basis.  Hydrographic Offices are primarily focused on providing nautical chart products and services (e.g., paper, RNCs, ENCs), while Coast Guard/Maritime Safety agencies focus on improving AtoN and VTS.  The lack of coordination and reluctance to change usually leads to a fragmented, sub-optimal “system” due the inability to provide maritime users information that is seamless, readily-available and at reasonable cost.  In particular, government agencies must be willing to recognize that navigation-related information can no longer be considered separate product/service in terms of how produced, what and how used, and that being up-to-date is a critical factor.

In addition to displaying examples of the ‘Next Generation ENC” he also showed simulations of the “Electronic Chart of the Future.”  For the “Next Generation” ENC, it will be able to utilize functionalities in current ECDIS/ENC systems, but also combine the best available data for a given area.  This includes high-density bathymetry (e.g., a bathymetric foundation based on a navigational surface), and being capable of incorporating time-varying data.

Dr. ALEXANDER also explained the use of AIS binary messages (such as IMO Meteorology and Hydrology messages in IMO SN/Circ.236 and IALA Guideline No.1208) for providing enhanced VTS services.

In response to a question on selection of information to be needed from the floor, with his understanding on use of ENC for marine environment protection in addition to use as navigation charts, Prof. ALEXANDER stressed that the survey of user needs was very important, and that e-Navigation should be used by which mariners need indeed, not be used as mandatory rules. e-Navigation should be also used like mobile phone.

# Session 4: Information Exchanges on e-Navigation

The Chairperson, Mr Ryozo SHIGEMATSU, stated that JCG would conduct the session by requesting an exchange of views based on the questionnaire that had been e-mailed to the four foreign experts prior to the meeting.  He reminded the workshop that the questions were in five broad categories.  They were:

(1)	Fundamental issues on e-Navigation,

(2)	Issues on Electronic Navigation Charts in e-Navigation,

(3)	Issues on Position Fixing,

(4)	Issues on AIS, and

(5)	Issues on VTS.

Over a day and a half, there was a frank and vigorous exchange of views on:

- the fundamentals of e-Navigation,

- international developments,

- potential users and their requirements,

- SOLAS issues,

- Cost benefit aspects

- the role and involvement of the various international organizations

- the impact of e-Navigation on traditional aids to navigation,

- ECDIS (mandatory carriage requirements ) and ENC

- Commercial aspects of ENC

- Raster and vector charts,

- robust terrestrial position fixing systems such as eLoran and its development

- the future of DGNSS,

- the provision of effective information from shore systems via AIS

Annex 2 details the discussions referred to above.

# CLOSING OF THE WORKSHOP

Mr Kazuyuki ENDO, Director, Policy Planning and Research Office, Administration and Planning Division, Marine Traffic Department, thanked everyone for their active participation, both during the discussion days and the symposium.  He said that JCG had gained much from the workshop.  He expressed his sincere gratitude to the invited experts and said that the workshop would not have been a success without their active participation.

He thanked all the four experts for their excellent presentations at the public symposium.

Mr ENDO said that Dr Alexander’s last slide (in his presentation at the symposium) had reinforced his own view that neither ECDIS nor e-Navigation was replacements for common sense and good seamanship.  He personally found the workshop very useful.

In closing, he said that JCG would like to continue to work with IALA and other countries, so that they could conduct better research.  He wished everyone a safe journey home.

Closing Remarks by Mr Bill CAIRNS

In responding on behalf of IALA and the invited experts, Mr Bill Cairns thanked JCG and OPRF for the opportunity to participate in the workshop.  On Monday morning, Mr SHIGEMATSU and Mr IMAI pointed the e-navigation ship in the right direction, setting course for a very good workshop – and Mr FUKUSATO led the symposium very ably on Tuesday.

Mr CAIRNS said that on both Wednesday and Friday, there was an excellent exchange of ideas, due to active participation from everyone.  There were also more informal exchanges at the social events.  The technical tour to the Tokyo-Wan VTS and Kannon Saki Lt Ho and Yokohama Landmark Tower were fantastic, he said. He thanked the interpreters for their hard work, patience and good humour.  He concluded by thanking JCG for their unequalled hospitality and the organisation of an excellent workshop

# Annex 1 – Workshop Programme

Monday, October 29, 2007

10:00 - 10:30  Courtesy Call to Mr. Shuuichi YONEOKA, Director General, Maritime Traffic Department, JCG at JCG Headquarters

10:30 - 13:30  Movement from JCG HQs to the Venue and lunch

13:30 - 14:30  Session 1 : Opening of the Meeting

Mr. Ryozo SHIGEMATSU, Director for Maritime Traffic Coordination, Administration and Planning Division

Opening Address

Mr. Tadayoshi IMAI, Director, AtoN Engineering Division

Introduction by each participant

Explanation of the meeting itinerary

14:30-16:25  Session 2 : Study on e-Navigation in Japan

14:30-15:20   Progress of development of e-Navigation strategies and shipboard systems in Japan

Mr. Junji FUKUTO, National Maritime Research Institute, Navigation System Group, Ministry of Land, Infrastructure and Transport

15:20-1535    Coffee Break

15:35-16:25   Technical Study for e-Navigation in Japan Coast Guard

Mr. Hitoshi HATAKEYAMA, Senior Officer, VTS System Office, AtoN Engineering Division

19:00-20:30     Welcome Reception

Tuesday, October 30, 2007

10:15-1620     Session 3 : Symposium on e-Navigation

10:15-1030    Opening Announcements

Mr. Seiko FUKUSATO, Senior Officer, Policy Planning and Research Office, Administration and Planning Division

10:30-11:20    The history of the development of e-Navigation and the effort for evolution

Mr. Mahesh ALIMCHANDANI (IALA)

11:20-12:10   Robust radio navigation and broadband communication for e-Navigation era

Mr. Peter DOUGLAS  (NLB, Scotland)

12:10-13:30   Lunch

13:30-14:20   Advanced application of AIS and results of study committee in Japan

Mr. Tatsuo SATO  (Japan Aids to Navigation Association)

14:20-15:10   Operational aspects of e-Navigation activities and future expectation of IALA

Mr. Bill CAIRNS  (USCG)

15:10-15:30   Coffee Break

15:30-16:20   ENC and e-Navigation: Challenges and Opportunites

Dr. Lee ALEXANDER　(University of New Hampshire)

Wednesday, October 31, 2007

10:00-16:45     Session 4 : Information Exchange on e-Navigation

10:00-10:10   Brief explanation on further discussion for e-Navigation

Mr. Ryozo SHIGEMATSU  (JCG)

10:10-11:10   Fundamental issues on e-Navigation

To be provided by participants

11:10-12:10   Issues on Electronic Navigation Charts in e-Navigation

To be provided by Dr. Lee ALEXANDER mainly

12:10-13:30   Lunch

13:30-14:30   Issues on Position Fixing

To be provided by Mr. Mahesh ALIMCHANDANI, Mr. Bill CAIRNS and Mr. Peter DOUGLAS mainly

14:30-15:30   Issues on AIS

To be provided by Mr. Mahesh ALIMCHANDANI, Mr. Bill CAIRNS and Mr. Peter DOUGLAS mainly

15:30-15:45   Coffee Break

15:45-16:45   Issues on VTS

To be provided by Mr. Mahesh ALIMCHANDANI, Mr. Bill CAIRNS and Mr. Peter DOUGLAS mainly

Thursday, November 1, 2007

10:00-17:00   Technical Tour

Visit to Tokyo-Wan VTS Center in Kanagawa Pref.

18:30-20:00   Reception for the 139th Anniversary of National Lighthouse Day

All participants are invited by President of TOKOKAI (Japan Lighthouse Association)

Friday, November 2, 2007

10:00-12:00   Session 4 : Exchange view and Conclusion

Chaired by Mr. Ryozo SHIGEMATSU

10:00-11:00    Overall exchange view on e-Navigation

11:00-11:50    Conclusion

11:50-12:00    Closing Address

Mr. Kazuyuki ENDO, Director, Policy Planning and Research Office, Administration and Planning Division

12:00         Adjourn

# Annex 2 – Summary of discussions held with invited experts

31 October and 02 November 2007

1-1. e-Navigation strategy and the foreseeable schedule for putting it into practice

Alexander – Believes there are three phases:

1) Defining the Concept

2) Establishing Strategy for Implementation

3) Determining what are the appropriate Practices for:

a) SOLAS and non-SOLAS vessels

He recommends that this be communicated by Japan to IMO MSC85 in 2008.

Alimchandani – Encourages a response by Japan to IMO on strategy and user needs.  It would be very good to send it via the IALA e-Navigation Committee.

- There is an IALA template document that has been developed by e-NAV3, which can be used to define user needs.

- Next IALA e-Nav meeting is in Shanghai; IALA would like to make a determination on its input to IMO at this meeting.

- Timeline is tight (for submission to IMO) and much work needs to be done.

Cairns – There are several upcoming meetings of the IALA e-Nav Committee

- 18 Dec 07 at IALA HQ, France – Working Group on Concept and Architecture

- 14-18 Jan 08 [tentatively at IALA HQ] – Strategy and Operations group will meet

- 25-29 Feb 08, in Shanghai – IALA eNav Committee will meet

Chair

Mentioned that marine environmental protection is also a consideration

1-2 Legal issues for e-Navigation in SOLAS

Alexander - Believes that ECDIS will become mandatory carriage for SOLAS vessels

- May start in 2008, with full implementation by 2012.

- IHO has stated that complete, worldwide ENC coverage should be achieved by 2012

Vessels:

- ECDIS is the preferred means to display e-Nav information for SOLAS vessels

- ECS will be better for smaller vessels

Fukusato - asked if IMO will likely require ECDIS and ECS for displaying e-Navigation information?

- Alexander responded that this is likely, but not certain.  The main factor will be the recognition of the benefit of e-Navigation information and that it is a better way to navigate vessels.

Alimchandani – mentioned that IMO has already required ECDIS as mandatory carriage for High Speed Craft.  While the safety case for ECDIS is clear, what is less well known are the (in) efficiencies in cost, distribution and updates of ENC.  There are clear benefits in ‘real-time’ navigation.

Cairns – believes that putting e-Nav into practice will not require another carriage requirement.  But, ECDIS and ENC carriage will be essential.  E-Navigation will provide information that is currently difficult to obtain, but highly useful for both VTS and ships at sea (the concept of “silent” VTS).  There could also be cost savings for VTS, by reducing the number of personnel required.

1-3

[see previous discussion]

1-4 What are the user numbers and cost benefit?

Cairns – USA has approximately 450 SOLAS vessels >300GT; (note: in Japan the number is about 100).

Alexander – Yes, but perhaps 95% of all vessels in USA waters are foreign flagged!  He believes that this situation may be similar for Japan.

Endo – mentioned that about 50,000 vessels over 1,000 tons transited each year in the area of the Kurushima-Kaikyo Strait where was the important strait in Japan, and that of which about 50 % of these vessels were from foreign flag vessels.

Alimchandani – There are already provided to IMO, some cost-benefit studies to support ECDIS carriage requirements.  This may be a good basis for e-Nav as well.

Douglas - believes that the market for e-Nav will be far greater than that for ECDIS, as e-Nav would apply to SOLAS and non-SOLAS vessels.

1-5 Development of e-Navigation information and operational organization

Douglas – In UK, there is a specific GLA R&D organization, which would include responsibility for the development of e-Navigation.  Key agencies include:

UKHO (ENC production)

Office of Communications (Radio licensing issues)

UK Maritime and Coast Guard Agency (MCGA) and

General Lighthouse Authorities (GLAs) (Aids to Navigation) (AtoN)

There are regular strategy meetings between these agencies, coordinated by the UK Department for Transport.

Cairns – In USA, USCG Headquarters is undergoing a reorganisation.  With regard to e-Navigation, the following offices are involved:

USCG Office of Waterways Management (CG-541) in under CG-5, Policy and Planning

USCG Navigation Systems Division (CG-5413), under Waterways Management, has three branches:

1- Navigational rules

2- Visual aids to navigation

3- e-Navigation (includes radionavigation and IMO e-Navigation matters)

Also includes the USCG Navigation Center (NAVCEN)

USCG Acquisition Directorate, CG-9, includes Office of Research and Development (for applied research), including the Research and Development Centre in Groton Connecticut.

1-6 Cost reduction and service revenue of e-Navigation in UK

Douglas – The original UK submission to IMO stated that e-Navigation “had the potential” lead to a reduction in AtoN.  UK is already experiencing a reduction in some AtoN (deep channel buoys, major floating aids, waypoint lighthouses) and this trend is likely to continue under e-Navigation.

GLAs have a fifteen year strategy document (2020 the vision) and a radionavigation plan.  A visual AtoN Plan is currently being developed.

Aids to navigation in UK are funded by a ‘user-pay’ system, based on a vessel’s arrival in port.  Charge is ~85 JPY/ton.  Funding is specifically set aside for AtoN services.  It does not go toward the communications services provided by UK Maritime and Coast Guard Agency.  Vessels transiting the English Channel pay no dues unless they call into UK ports.

Alexander – Mentioned the results of a recent study (for Canadian Pilots) on the “Use of Portable Piloting Units by Maritime Pilots.” Most likely, pilots will be important users of e-Navigation services.

2-1 to 2-4

Please refer to Dr Alexander’s PowerPoint Slides on the subject.

3-1 – 3-7 e-Loran

Douglas – The North West Europe Loran system (NELS) agreement has terminated, however all the NELS stations remain on air.  UK was not an originally involved (due to the influence of Decca).  Now, this has changed and there is a UK Loran transmitter station at Anthorn.  The future of Loran in Europe will be addressed in a future European Radio Navigation Plan.  Loran is not just a maritime system, but multi-modal (aviation and land transport).  He believes that the future of Loran is tied to e-Navigation.  Also feels that there it is unproductive to focus on Loran as a “legacy system.”  The cost of operating Anthorn is less than $ 500,000 / year.

Alexander pointed out that the ECDIS Performance Standard requires a “second independent positioning system of a different type” (see Section 10.5.6).  This could be fulfilled by e-Loran.

Cairns – There are few maritime users for e-Loran.  In the late 1990’s, there was some support for Loran by recreational users.  But, this is no longer appears to be the case, as Loran is primarily serving timing users ashore. Investment decisions on e-Loran cannot be based on maritime safety enhancements.  It should also be noted that there are few Loran equipment manufacturers.

Current status of Loran in the USA:  Loran is operated by USCG.   There is a difficulty in calculating the cost of an individual station.  Remote locations, availability of commercial power, the need for site security and environment can influence the cost.  The USCG spends $44M/yr to operate 24 stations and the associated monitoring and support network.  The Programmatic Environmental Impact Statement documents the liabilities of an action or inaction.  Through this process, the USCG will know what must be done with respect to the environment to:

1. discontinue Loran

2. convert to eLoran, or

3. add new Loran stations

There is a RTCM Special Committee that is currently dealing with eLoran. Mr CAIRNS also pointed out that the IALA e-Navigation Committee established there is a need for a robust, fail safe backup positioning system as a fundamental element of e-Navigation.  Where Loran currently exists, this could be changed to eLoran and fulfil the requirement for e-Navigation. However, where Loran currently does not exist, it would likely not be cost effective to provide Loran coverage.  Not certain what the cost would be for converting to solid state Loran.  The US Government is evaluating the need for a national back-up to GNSS and an e-Loran concept is being considered.

DGNSS

USCG operates a constellation of 48 maritime and 37 inland DGPS sites and there are currently plans to add 45 more inland sites to support navigation in inland rivers and non maritime applications

Alimchandani:   Mentioned that IALA has a Recommendation on the Future of DGNSS.  Mainly, in the short to medium term, DGNSS was seen as the best means of integrity monitoring and meeting the IMO requirements for navigation in restricted waters.

Douglas:   Most DGNSS equipment is old and reaching end of its useful life. US and GLA in UK have different approaches to next generation of equipment.

US – work underway to develop software RSIM (the reference station and integrity monitoring functions are done on a PC).

UK – undertook a study of options available - have decided to recapitalise equipment with a hardware solution.

UK will shortly put out a tender for this equipment and will aim to replace equipment by the end of 2008.  UK would like to see a larger market, leading to more competition.

US - no firm timing for replacement

Japan:    Has 27 DGPS stations, two-thirds of which are the old type of stations. The medium wave transmission equipment is getting old.  Japan is discussing the upgrades required.  Important factor for Japan is the existence of SBAS, which provides the function of differential GPS.  Japan would like to know the view of the experts.

Douglas:   UK originally intended to close down DGPS service in 2008 because a) it was expected that GALILEO would be available by 2008 and b) it would provide its own integrity function.  As Galileo is running late, and it is no longer expected that GALILEO will be able to provide all the necessary functions from space, radiobeacon DGNSS has to be recapitalised.

Japan’s situation is different to Europe.  In Europe SBAS is provided by EGNOS (which is still not fully commissioned).  It is probable that EGNOS will be available soon, but there is long term guarantee of this service.  Therefore, UK is not relying on SBAS.  Europe also has concerns regarding the coverage of SBAS, as it does not perform well in high latitudes, or in port areas with tall buildings.

Alimchandani:  IALA conducted a survey on the recapitalisation of DGSS recently.  It was found that of those countries that responded (admittedly, fewer than expected), most are planning to recapitalise their systems.  These include Australia, Canada, China, Denmark, Germany, Malaysia, Poland, Portugal, Sweden, UK and USA.

Sato:  IALA has sent out the questionnaire again to countries that have not yet replied.  Japan has responded by saying that they have not yet made any decision and is still considering its options.

Masuda:   Asked if DGNSS was part of the WWRNS.  Cairns explained that a Contracting Government could offer its DGPS system as part of WWRNS.  US wanted to offer its DGPS service to be included, but were unable to meet the continuity requirements.

Douglas:  Ten years ago, GALILEO said that all differential services could be provided from space.  Then, approximately three years ago GALILEO stated that not all of the service could be provided from space, and that ground augmentation would be required.  It was hoped that aided by IALA, a group of countries could be encouraged to submit their systems as part of WWRNS. This was part of a longer term strategy to make DGNSS a mandatory carriage requirement on SOLAS ships.  He sees this occurring as part of e-Navigation, when the carriage requirements for e-Navigation are decided.

Douglas:  Said that his Working Group at e-NAV3 had addressed the continuity issue, which IALA has been studying for some time now. There were concerns that IMO Resolution A.953 had very stringent continuity requirements; these were very difficult to comply with, even more difficult than those for aviation.

IALA has drafted a new version of IMO Resolution A.953, which offers a more realistic target for service providers.  This will be presented to the IALA Council in Dec 2007 for approval.  If approved, IALA intends to submit this in time for IMO NAV 54 (July 2008).

Fukusato:  (Comment only).  In Japan, JCG controls the radiobeacon DGPS system.  JCG is under the Ministry of Land, Transport and Infrastructure.  The Japanese version of SBAS, is named MSAS. It is also operated by the same ministry.  MSAS was made operational in September 2007.  Japan is in a situation where the same organisation controls both systems.  This is somewhat problematic. Just a comment, though.

Alimchandani – Mentioned that at the IALA e-Navigation Seminar in London in July 2007, e-Loran was discussed as a backup for GNSS.  Not just for positioning but timing as well.

He also drew attention to the main outcomes of a recent IALA e-Navigation meeting on Loran / CHAYKA.

Conclusions:

- proof of concept trials have shown that eLoran can meet the IMO positioning requirements in restricted waters. USA, UK and France are also conducting trials in more ‘difficult’ waters and have undertaken to provide the results to IALA.

-  any eLoran system should not have any intellectual property rights restrictions - either on the service or receiver equipment, in the past with AIS receivers

- there is no market for stand-alone eLoran receivers

Recommendations:

- IALA should establish a worldwide radionavigation plan

- International Loran Association’s definition of eLoran was accepted

- IALA’s e-Navigation Committee is to commence work on an eLoran standardisation plan

3-8  Class B AIS and e-Nav

4-1 Regulations on AIS carriage for small vessels

Douglas – In UK, there is no carriage requirement.

Cairns – In USA, AIS domestic carriage requirements extend to all commercial vessels >20m, tugs >8m, and most passenger vessels.

Kondo – In Japan, >500 tons.  But, most vessels are <500 tons.

Miyake:  For small craft in Japan, the advisory committee which was established in the Ministry of Internal Affairs and Communications, on revise of regulations related to the radio law is discussing this issue.  Outcome of these results are expected in May 2008. Then, the law will need to be revised, and this will take one year.  Therefore, AIS Class B can be expected in Japan in two years, probably. He is unsure if carriage will be mandatory or not.  In Japan, it is seems difficult to make it mandatory for small craft and fishing vessels.

Equipment manufacturers advise that many small craft owners are purchasing AIS receiver-only equipment.  So, their location is not transmitted.  But such users are doing so to protect themselves.

Sato:  ENSS is quite effective for Class A and B AIS.

Alimchandani: Explained the concept and functionality of Radar Target Broadcasting from VTS.  The VTS could broadcast its VTS image to all AIS vessels.  In this way, those vessels that have a receive-only AIS could see what the VTS was seeing, not just AIS targets.  This could offer an improvement in safety.

Sato:  Radar picture is quite difficult to transmit. But your idea (Alimchandani) can be explored.  Five years ago, Japan was experimenting sending the traffic image by cell phone. The study results were very good.  However, this did not succeed because reception at sea was not guaranteed.  Japan has now embarked upon ENSS and has shown the results of the studies – and now, Japan is continuing further development for the implementation of ENSS.  The value of ENSS was being recognised.  ENSS will need budgetary support next year, in order to continue R & D work.  A lot of effort has to be made to get funding.  Japan will work hard to implement ENSS and will be looking for international support for ENSS.

4-2 Number of Class B receivers?

UK - No plans for mandatory carriage. Currently, Class B is intended for fishing vessels and pleasure craft.

USA – Current regulations do not address/accept Class B.  However, no proposed rule making has been issued. However future regulatory actions will.

Australia – No carriage requirements as yet, but rule making process is underway.

4-3  Any statistics about prevention of marine accidents due to AIS?

USA – No statistical analysis done.

UK – No statistics.  But, it is known that the behaviour of mariner has changed.

Japan – No statistics on incidents, but there is monitoring of vessel movements.

Australia – Torres Strait and Great Barrier Reef is under complete AIS monitoring by AMSA.

4-3-1 How is AIS being used for VTS

USA – Approximately 10 VTS areas have two-way AIS information exchange.  But, AIS is not used as the primary means of VTS communications.  There will be a nation-wide expansion of AIS coverage.  For areas outside of VTS coverage, AIS will be used to contribute to the common operational picture (COP) of vessel identification and movement.  On the St. Lawrence Seaway – Great Lakes (USA and Canada), since 1997 there is an AIS broadcast service that provides weather, water level and lock procession order information to vessels.

Mr Fukuto suggested that AIS installations in some areas had resulted in an increase in VHF voice communications. One possible explanation is that, because there is more information provided by AIS, there may be more questions by VTS operators of vessel intentions.

Japan -  AIS is used with radar to supplement VTS coverage.  This way all vessels (those with and without AIS) can be identified and monitored.  It is primary the risk of grounding and encounter (not collision) that is being monitored.

UK – In the English Channel, the situation is the same as that in Japan.  Legal situation and possible follow-up can be found at the UK MCGA website (www.mcga.gov.uk)

IALA – (In response to a question) , for Class A receivers, the system has the ability to automatically reduce the radio cell size, so that the system is not overloaded with AIS messages.

Alexander – Mentioned that from a mariner’s perspective, AIS provides increased “situational awareness”.  This has become a behavioral change.  As such, some believe that less VTS (e.g., voice communications) are needed.  This has shown to be the case in Puget Sound/Straits of Juan de Fuca, St. Lawrence Seaway – Great Lakes, and for marine pilots in Europe and N. America.

4-4 Providing information by AIS

USA – In addition to the meteorological / hydrological and lock procession order information via AIS binaries on the St Lawrence Seaway and part of the Great Lakes, other applications are being considered and / or on a trial basis.

Alexander – Marine pilots are already using information for decision support that should eventually be provided by AIS (water levels, current flow, situational awareness, etc.).

4-4-3 – Problems with AIS information

Alimchandani – IALA had dealt with the matter of “incorrect AIS data transmission” a few years ago, when AIS was being introduced.  IMO NAV 54 (July 2008) also has this on its agenda.  Discussion on incorrect AIS installation.  IMO has issued guidelines on AIS installation.

Douglas – AIS could be considered an example of “how not to implement a new navigation system” (e.g., lack of early training in its use, misinterpretation of AIS targets, incorrect ship parameters, etc.).  MKD is of almost no use.

Cairns – In USA, ships can be fined for transmitting incorrect AIS information.

Endo – Hopes that AIS will become more “user-friendly.”

Alexander – Mentioned that mariners strongly dislike the MKD.

5 – VTS operations

USA – All VTS are operated under the legislative authority of USCG, and implemented through regulations.  There are standard operating procedures.  VTS staffing and training are done in accordance with IALA Guidelines.

Alimchandani –

5-2 – No concrete evidence at this time, but Bosporus, Dover, and Singapore Straits have benefited from VTS operations.

5-3 – HELCOM (Baltic Sea network) is working well.  See no conflict between Safe Sea Net, MARNIS and e-Navigation.

The IALA Council has recently approved a revised VTS Committee work program for 2006-10, as, with the advent of AIS, LRIT and regional networks, VTS is becoming a global activity.

New tasks for the VTS Committee include:

- Review / update / provide input to IMO on Resolution A.857 (20).

- Define the concept and develop the scope of the VTM.

- Guidelines on the role of VTS in the establishing and the management of limited access or restricted areas

- Vessel traffic management in terms of the role of AIS for global traffic monitoring.

- Update and develop new guidance on the Training and Certification of VTS Personnel

Japan   – VTS follows the IALA guidelines.  Provide advice to vessels to aid in avoiding marine accidents.  Japan would like to know if this is same in other countries.

USA –  In general, VTSs should function primarily as information services.  There are times when the maintenance of good order and predictability on a waterway requires a VTS to offer recommendations or even directions in its dealings with a vessel.  The exercise of this “control” can be seen as a continuum of four basic levels of increasing involvement:

•	Monitor – The VTS monitors activity in its area of responsibility, in order to build a comprehensive traffic image.  This monitoring is continuous and by its nature influences vessel traffic toward order and predictability.

•	Inform – Based on the information collected and analyzed, the VTS may offer information to the mariner so that he or she may navigate more safely or efficiently.  This information may be provided as a general broadcast, to a specific vessel or in response to a request buy a vessel.  Provision of information is done very often by a VTS.

•	Advise/recommend – In certain situations, the VTS may recommend a course of action to a vessel, based on the VTS’s overview of the vessel traffic situation.  This is an infrequent occurrence, usually along the line of .

•	Direct – In rare circumstances the VTS will direct the movement of a vessel.  This only occurs when it is obvious that the vessel is standing into imminent danger or is obviously in violation of regulations.  Directions are normally given in the form of a desired outcome, rather than specific conning orders.

UK – Does not have any authority to give any direction (order) to ships.

IALA – One of the types of VTS is to provide assistance to help decision making onboard.  The result is a navigational assistance service.

************

# ACTIONS AGREED TO BY THE WORKSHOP

Japan will:

- Submit the results of this e-Navigation workshop to IALA, as an input to the e-NAV4 Committee meeting.

- Conduct research on user requirements and report to IALA (deadline for submission is the inter-sessional meeting of the Operations and Strategy Working Group of the e-Navigation Committee, scheduled for January 2008).

- Conduct research on backup systems for timing and position-fixing, as it is aware of the vulnerability of GNSS.

- Conduct research on the use of AIS as an aid to navigation, in framework of e-Navigation

- Continue to report further development necessary for practical use of ENSS, including any studies carried out on the development of electronic charting and ship-shore communications to IALA.

- Bring the results of this workshop to the attention of the IALA Council, at its December 2007 meeting.

IALA:

- IALA is requested to forward the report on the JCG /OPRF workshop, held in Tokyo in Oct-Nov 2007, to the e-NAV4 Committee meeting.

- IALA invites Japan to keep the e-NAV Committee advised on further developments in ENSS

*******************

This template is available on the IALA website (www.iala-aism.org).