Non-GNSS sources of Position, Navigation & Timing for e-Navigation

Introduction

Marine navigation is currently highly dependent on GNSS for position, navigation & timing (PNT) information; however there is significant awareness of the vulnerabilities of such systems to both intentional and non-intentional interference, and to system failures.

As a result, there is interest in identifying alternative forms of supplying PNT information to provide an element of redundancy, and mitigation of GNSS failure scenarios. This paper explores these technologies.

Requirement

There is widely perceived  to be a requirement for a PNT system to back up GNSS, which is capable of meeting the requirements of IALA Recommendation R-129 on GNSS Vulnerability and Mitigation Methods, and particularly:

- output in IEC-61162 format, usable by ECDIS and AIS

- coverage at least within the Coastal phase of navigation, worldwide

- accuracy standards of 100m in the Coastal phase and 10m in the harbour entrance and approach phase, in accordance with the current requirements for these phases of navigation set out in IMO Resolution A.953(23) World-Wide Radio-navigation System.

There is a tight timescale for this activity, as SOLAS Chapter V is due for revision in 2013, and this is effectively the opportunity to define carriage requirements for e-Navigation.

Potential Solutions

Potential means of sourcing PNT information fall into 4 main areas:

- terrestrial radio systems, typically operating in the lower frequency bands i.e. VLF/LF/MF, but also possible at VHF/UHF.

- Radar/laser based systems

- Inertial Navigation Systems

- Echo Sounder based systems

Terrestrial Radio Systems

Lower frequency terrestrial radio systems have been used for navigation since the early 20th century. Measuring the bearing of transmissions received, as in traditional marine radio beacons, is generally considered too inaccurate (~5) for modern standards, and a system using some form of ranging is considered necessary. Potential systems include:

Omega – the world’s first global PNT system, utilising only 8 transmitter sites at a frequency of 10-14 kHz. The infrastructure required at each site is very significant and most of the stations have now been demolished. Accuracies achieved were low (c.4nM).

Loran-C – Currently available in USA/Canada, Northwest Europe and the Far East, operating at 100 kHz. Originally a hyperbolic system with a nominal accuracy of 400m, this can be improved with the implementation of Time-of-transmission controlled transmitters, and all-in-view receivers incorporating Additional Secondary Factor (ASF) databases and differential corrections, to give accuracies of the order of 10 metres (enhanced or eLoran).

Decca – a similar system to Loran, but with lower power hyperbolic chains operating at 70-130 kHz; the system has now been terminated worldwide.

Rana – a hyperbolic system aimed at the fishing sector, previously used in French waters and operating in the MF beacon bands.

Hyperfix - a land-based, short-range, mediumwave, navigation system formerly very popular with offshore oil drilling operators, and also used by a few navies. Frequencies are between about 1600 and 2500 kHz.

Syledis – another short range hyperbolic system used in the surveying industry, operating in the UHF bands and hence line-of-sight coverage; no longer in use.

Toran – another short range hyperbolic MF system used in the surveying and fishing industries; no longer in use.

Datatrak/DGNSS – Other proprietary navigation systems exist, such as the Siemens-Datatrak system, originally a hyperbolic system, but now re-engineered to use pseudoranges. It would be technically feasible to re-engineer the IALA beacon DGNSS to transmit pseudo-range signals in a similar manner. The ranges achieved by the system and the overlapping coverage currently available in significant areas of the Northern hemisphere, make this option attractive. Either of these technologies would require a redesign of the reference station and the incorporation of a precision timing device to function in this manner. The cost of installing high accuracy clocks at individual reference station is likely to be prohibitive, which implies that the timing source would probably be another PNT system or radio time signals.

Mobile Phone – There is a significant development of mobile phone positioning technology to comply with the USA’s E-911 requirement. Although much of this work is based on GNSS, it would be feasible to use the Timing Advance data within GSM ranging data to transmit real time PNT information back to the user; although ranges would be restricted to a maximum of c.35km from the nearest base stations, range accuracy is approximately 500 metres and the timing source may ultimately be reliant on GNSS.

Radar/laser based systems

Radar based navigation has been widely used since the 1940s, but in it’s traditional format does not permit an automatic presentation of position or time.

Radar Map-matching – radar overlays are available to superimpose over an electronic chart display from a number of manufactures. It is technically feasible to correlate the radar display with the known coast to derive an automatic position, where the coastline is within range and is sufficiently well charted and defined. This can be improved by the installation of radar reflectors at known surveyed positions. The system has limited ability to initialise from an unknown position/landfall and timing is not available.

Secondary radar – this technology is widely used to derive information regarding aircraft and warships. Use for navigation would require the ship to carry a scanning transceiver, and a network of transponders ashore to supply position data.

Artemis – a short range radar based system used in the offshore industry to maintain position from a known object, up to a range of 5km. The ship requires a tracking radar and the reference position requires either a tracking radar or a transponder

Laser Scanning – It is also possible to use laser frequency scanners to track a known position. This technology is used in the surveying/dynamic positioning fields under the trade names Fanbeam and Cyscan with an effective range of 800-2000 metres. Enhanced optical targets with a similar function to a radar reflector can be established in known positions.

VTS – It would be feasible for a shore based radar to resolve the vessel position to a high degree of range accuracy and a slightly lesser degree of bearing accuracy, and then to transmit this position to the vessel for onboard use. The most practical way of achieving this would be as an AIS pseudo-target, however this raises further questions regarding the timing source within AIS. This scenario requires significant expenditure on VTS radar equipment, and would still be unlikely to provide universal coverage, due to radar shadowing.

Inertial Navigation Systems

Inertial Navigation Systems can provide smoothing of a PNT system output and can generate position information for a certain period of time after the loss of a PNT input. They have been used successfully for decades in commercial aircraft and submarine navigation, however the anticipated performance of even $100,000 navigation grade units will only provide accuracies of the order of 10 metres for a few minutes. Timing is not available.

Bathymetric (bottom contour) navigation

It is feasible to navigate a vessel by correlating the depth of the water with the charted depth. If the survey is of high enough quality and the bottom varies sufficiently, it will be possible to define a navigation solution once the system has initialised, in much the same way as terrain mapping technology is used in military aircraft and cruise missiles. Timing is not available.

Comparison of potential services

It is necessary to compare the alternative systems against a number of relevant criteria, which may include:

- Current development of technology

- Current status of relevant marine equipment and performance standards

- Licensing issues

- Cost to user

- Cost to service provider

- Integration with ECDIS

- Coverage

- Accuracy

A simplified comparison can be found at appendix A.

Summary

Most of the terrestrial radio systems listed have the advantage of providing coverage to several hundred kilometres, thus limiting the number of shore stations required. They are also capable of providing a timing source, although care should be taken that this is not ultimately sourced from GNSS.

A number of short range systems exist, but these would generally be very expensive to implement on an international scale, due to the very large number of shore stations required. The exception to this rule is mobile phone based solutions, which already have a large coverage potential, and the potential for ranging/triangulating a phone’s transmissions, however current development of mobile phone positioning is largely based on the phone transmitting a GNSS-based position.

Standalone solutions such as Radar map-matching, INS and Bottom Contour profiling are all proven positioning technologies, and have the benefit that no service provider is required, however all have significant limitations.

Recommendation

The IALA e-Navigation Committee is requested to review this paper, make any additions or corrections as it sees fit, and then use this paper to inform urgent and necessary discussions to identify the most practical means of sourcing robust position-fixing and supplying PNT information to provide an element of redundancy, and mitigation of GNSS failure scenarios.

Having identified the most suitable technologies, the Committee should prepare a World Wide Radio Navigation Plan for approval by the IALA Council as a PNT strategy for e-Navigation, identifying the needs of:

- Users

- Service Providers

Within such policy, priority must be given to establishing:

- Recommendations with regard to Carriage Requirements, to be input into IMO n advance of the 2013 SOLAS Revision

- Links with the bodies controlling marine equipment standards (IEC, RTCM) and any necessary radio frequency allocations (ITU-R)

- Performance standards for any service providers, who may or may not be IALA members

References

- IALA Recommendation R-129 on GNSS Vulnerability and Mitigation

- IMO Resolution A.953(23) World-Wide Radio-Navigation System.

Appendix A: Comparison of PNT technologies

| Name of System | Coverage | Cost to user1 | Accuracy2 | Timing function ? | Cost to Service Provider3 | Current Status | Equipment and Performance Standards |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Omega | Worldwide | $ | No | Possible | $ 4 | No longer in service | Yes (dated) |
| Loran-C | Coastal+ | $ | No | Yes | $$ | Partial coverage | Yes (dated) |
| eLoran | Coastal+ | $ | Yes | Yes | $$ | Partial 7 | In hand |
| Decca | Coastal | $ | No | Possible | $$ | No longer in service | Yes (dated) |
| Rana | Coastal | $ | No | ? | $$ | No longer in service | ? |
| Hyperfix | Coastal | $ | No | ? | $$ | No longer in service | ? |
| Syledis | Line-of-sight | $ | Yes | ? | $$$ | No longer in service | ? |
| Toran | Coastal | $ | No | ? | $$ | No longer in service | ? |
| Pseudorange beacons | Coastal | $ | ? | Yes | $$ | Partial 8 | No |
| Mobile Phone | Line-of-sight | $ | No | Yes | - 5 | Partial | Yes (not marine) |
| Radar map-matching | Line-of-sight | $$ | Variable | No | - | Usable | ? |
| Secondary radar | Line-of-sight | $$ | ? | No | $$$ | Limited development | Some |
| Artemis | Short range | $$ | Yes | No | $$$ | Usable | Commercial |
| Laser scanning | Short range | $$ | Yes | No | $$$ | Usable | Commercial |
| VTS (AIS relay) | Line-of-sight | $ | ? | No | $$$ | Partial | Some |
| INS | Worldwide | $$ | Variable | No | - | Usable for limited period | Some |
| Bottom Contour | Worldwide | $$ | Variable | No | - 6 | Limited development | Some |

Notes:

- Cost to user: $ < $5000; $$ > 10 000

- Accuracy: Does it meet standards in IALA Rec.R-129 ?

- Capital Cost to Service Provider per 1000 km of Coastline: $ < 1,000,000$; $$ < 10,000,000$; $$$ > 10,000,000$

- Omega has high station cost, but few stations required

- Mobile phone cost is borne by telecommunications provider

- Contour matching requires no shore infrastructure, but a very high standard of surveying & charting

- eLoran would develop from existing Loran-C infrastructure

- Marine MF beacons are widespread, but little development work has been undertaken on pseudorange solution