e-NAV4 / 07 / 04

Formerly e-NAV3-output-07

- VHF Spectrum Management

- Introduction

Clear and effective VHF communications remains a key factor in Vessel Traffic Services (VTS), AIS and for future e-Navigation operations.  The use of Marine VHF is a key measure in marine operations and is becoming increasingly relied upon by VTS Centres; as will also be by e-Navigation.  This has resulted in more VTS Centres with their associated VTS Sectors being established which in turn is placing a greater demand on Marine VHF channels available for marine safety communications (i.e. VTS, AIS, DSC).  The limited number of channels in the marine VHF band is widely recognised.  VTS and marine service authorities are having to resort to the use of VHF channels that are not dedicated to ship/shore use.  As a result, there is evidence of increasing interference on VHF channel, potentially creating dangerous situations.

- Discussion

Whilst the problem of interference in the Marine VHF band has been recognised for some years there has been a tendency to live with the problem notwithstanding the potential impact on safety.  There are a number of issues that now make it timely to review the management of the Marine VHF band.  These include:

- Greater expectation of VTS in its contribution to risk control

- Greater reliance by ships on VTS communications

- Competing demands from new technologies such as AIS

- Potential future demands for data transfer to meet VTS and e-Navigation initiatives

- Possibilities created by new technologies for more efficient use of the available spectrum

- Challenges to the use of the Marine VHF spectrum from non-marine related use.  (It is understood that this part of the spectrum is not always reserved in the same way in which corresponding aircraft control channels are allocated)

- The need to protect AIS frequencies from interference

There are two clear measures that could be introduced to alleviate these issues: firstly the creation of more channels within the Marine VHF band, and secondly better management of the existing spectrum.

VHF Channel Allocations.  It is understood that new digital technologies are becoming available that will allow the reduction in channel spacing (narrow-band vs. broad-band) for voice circuits whilst data channels may require similar or greater bandwidth.  Clearly it should be possible to make more efficient use of the spectrum.  However, this would require a fundamental review of the entire Marine VHF Channel allocations beyond that proposed in Recommendation ITU R M.1084-4; it would be a long term project involving international agreements and long lead times to implementation.  Whilst the challenges are significant, the potential long term benefits are enormous; there is even the potential for identifying a greater number of channels within a smaller part of the existing spectrum which could benefit both marine and other potential users.

AIS and VTS Frequency Designations.  According to SOLAS Regulation V/12, VTS contribute to safety of life at sea, safety and efficiency of navigation and protection of the marine environment, adjacent shore areas, work sites and offshore installations from possible adverse effects of maritime traffic.  Furthermore, AIS is ‘Safety of Navigation’ equipment required on board most ships —SOLAS V/19— and is relied upon by Administrations for VTS and navigation safety.  Yet, these frequencies are not designated by ITU or clearly identified by IMO as safety frequencies.  As such, they are not afforded the protection that similar frequencies or services are provided, such as Channels 16 and 70 (DSC), 406 MHz EPIRB’s, and aviation bands.  To provide for the evolution of services through e-Navigation it is required that AIS and a VTS information channel—and possibly other future e-Navigation channels—be afforded protection to ensure navigation safety.

Spectrum Management.  Whilst most countries have authorities in place to administer the spectrum, there is little evidence of effective cross-border management of spectrum issues.  Better liaison between national authorities would offer the potential for a reduction in interference even in the short term.

- Recommendation

The e-Nav Committee:

- Recommends the designation of at least one channel usable world-wide for VTS information;

- Recommends that IALA solicits from the membership candidate channel(s) for this designation;

- Recommends that such channel be addressed by IMO for inclusion into SOLAS Regulations V/12; and,

- Recommends that IALA members submit to IMO and ITU for allocation, designation, and protection of AIS and VTS channel(s).

ACTION ITEMS

The Secretariat is requested to present this paper and the survey and cover letter (e-NAV/WG2/WP5) to the next Council meeting for their approval in soliciting the membership for candidate channel(s).

The Secretariat is requested to carry forward this paper as a working paper to e-NAV4.