| IALA eNAV Liaison to ITU-R WP5B (2008) Fact Sheet |  |
| --- | --- |
| Working Party: WP 5B | Document No: IALA eNAV YYY(alternate version) |
| References: - Draft New Recommendation [M.8BL43], “Characteristics of VHF radio system and equipment for the exchange of data and electronic mail in the maritime mobile service Appendix 18 channels” - Document 8B/641, Annex 10, “Working Document: Example of maritime broadband VHF data system” - Report ITU-R M.2122 (EMC assessment of Appendix 18 services) | Date: 17 January 2008 |
| Document Title: Proposal for revision to Draft New Recommendation [M.8BL43], “Characteristics of VHF radio system and equipment for the exchange of data and electronic mail in the maritime mobile service Appendix 18 channels” |  |
| Purpose/Objective: The Draft New Recommendation referenced above was originally proposed in September 2006 as a combination of two systems, a “narrowband” system for single 25 KHz channels and a “broadband” system for a consolidation of nine duplex channels. The June 2007 WP8B meeting approved two “narrowband” systems and postponed the wideband system for further study based on the new Report on the need to assess EMC issues with proposed new systems in Appendix 18. |  |
| Abstract: New Report ITU-R M.2122 was used to evaluate the proposed “broadband” system for EMC with the systems operating within the current 25 KHz channel structure of Appendix 18. The results were used as the basis for this draft revision to the Draft New Recommendation that describes an EMC-compatible method to achieve the data-rate that was proposed for the “broadband” system. |  |

Proposal for revision to Draft New Recommendation [M.8BL43]

“Characteristics of VHF radio system and equipment for the exchange of data and electronic mail in the maritime mobile service Appendix 18 channels”

- Discussion

      - The new Recommendation was approved for 25 kHz “narrowband” service, but an optional “broadband” service (data-rate ≈133 kbps) was carried to the next meeting for further consideration of a method that is EMC-compatible with 25 kHz Appendix 18 channels.

      - The Working Document, Annex 10 to Document 8B/641-E “Example of maritime broadband VHF data system”, describes a system operating duplex on nine combined 25 kHz channels, resulting in a 225 kHz band. Technical details of this system were used to determine that it was not EMC-compatible with the current safety services in Appendix 18.

      - Some administrations do not have all of the proposed nine (9) channels available in the maritime service; this poses an international interoperability problem for the new system.

      - Annex 1 of the approved new Recommendation describes a spectrum-efficient method that provides up to 43.2 kbps in a 25 kHz channel, and RTCM SC123 is considering this for a new marine VHF digital small messaging service.

      - The method described in proposed Annex 3 can be used in a  4-channel multi-carrier system to provide the “broadband” service at a data-rate of 307.2 kbps (compared to 133 kbps in A above) in a manner that is EMC-compatible with the 25 kHz channels and services in Appendix 18.

      - Resolution of the matters referenced in A to C above should consider a revision to the new Recommendation to add Annex 3 to describe the “broadband” (now proposed as  100 kHz, and not 225 kHz as previously proposed) service.

2.	Proposal

This proposal to revise the Draft New Recommendation (Document 8/BL/43) should be considered as a resolution of the outstanding issues (A to C above) concerning the current proposal (Working Document, Annex 10 to the Chairman’s Report, Document 8B/641-E) for a “broadband” VHF data exchange system.  Spectrum efficiency and EMC considerations conclude that the service should be provided by means of the system described by the proposed revision to Annex 1 of Document 8/BL/43.

3.	Refer to the attachments

Attachment 1: The current proposal in the WP8B Chairman’s Report, Document 8B/641-E, Annex 10

Attachment 2: The proposed draft revision to Draft New Recommendation, Document 8/BL/43-E

  - Annex 1 that provides up to 43.2 kbps in a 25 kHz channel,

  - Annex 2 that provides 21.2 kbps in a 25 kHz channel, and

-	Annex 3 that describes a  100 kHz EMC-compatible  307.2 kbps “broadband” system.

Attachment 1

|  | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Annex 10 to Document 8B/641-E |
|  |  | 16 August 2007 |
|  |  | English only |

Source:	Document 8B/TEMP/273

Annex 10 to WP 8B Chairman's Report

Working Document: Example of maritime broadband
VHF data system

Introduction

This document describes a broadband VHF data system operated by Telenor for the exchange of data and electronic mail in the maritime mobile service. The system is currently in use, covering all the 2 400 km of Norwegian coastline.

The main maritime VHF data system in Norway is narrowband, but this broadband system is an option that is offering higher data speed transfer for users with more demanding requirements.

The system is operating on frequencies specified by footnote o) in ITU Radio Regulations Appendix 18.

# 1	General characteristics

1.1	The system is operating duplex on nine combined 25 kHz channels, resulting in a 225 kHz band. Actual bandwidth usage is dependent upon the data exchange transmission speed required.

1.2	The modulation is 2-level GMSK.

[Editors note – Emission class and data rate to be added?]

1.3	The access method is time division multiple access (TDMA).

1.4	The following area coverage techniques are used:

–	Cellular channel reuse

–	Time sharing transmission.

1.5	The following handover techniques are utilized:

–	Uninterrupted handover (channel and base station)

–	Uninterrupted file transfer.

1.6	The equipment is designed so that frequency changes between assigned channels can be carried out in less than 100 ms.

1.7	Emissions are vertically polarized at the source.

1.8	Switching between reception and transmission should not take more than 5 ms.

1.9	The serial communication channels (SCC) on a single radio modem should be:

–	Ethernet

–	RS232 (NMEA)

–	IEC 61162.

1.10	The radio equipment meets the following norms:

– 	Radio parametres:   ETSI EN 300 113-1, ETSI EN 300 908-9

– 	EMC:   ETSI EN 301 489-5, IEC 60945.

# 2	Transmitters

2.1	The frequency tolerance for coast station transmitters should not exceed five parts in 106, and that for ship station transmitters should not exceed ten parts in 106.

2.2	In order to prevent harmful interference to other users of the maritime VHF band, spurious emissions should be in accordance with the provisions of Appendix 3.

2.3	The carrier power for coast station transmitters should not exceed 50 W.

2.4	The carrier power for ship station transmitters should not exceed 25 W.

2.5	The cabinet radiated power should not exceed 25 µW.

# 3	Receivers

3.1	The receiver sensitivity for bit-error rate (BER) 10–3 should be better than –105 dBm.

3.2	The adjacent channel selectivity should be at least 70 dB.

3.3	The spurious response rejection ratio should be at least 70 dB.

3.4	The radio frequency intermodulation rejection ratio should be at least 70 dB.

3.5	The power of any conducted spurious emission at the antenna terminals should not exceed 2.0 nW.

[Editors note – Add sections as per narrowband examples.]

[Editors note – Give this example the same section headings as the narrowband examples.]

[Editors note – Examples of when and how broadband would be used. Shore applications interoperability with ship board narrowband.]

Attachment 2

|  | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8/BL/43-E |
|  |  | 3 October 2007 |
|  |  | English only |

Source:	Document 8/221(Rev. 1)

Radiocommunication Study Group 8

Preliminary Draft REVISION TO Draft New Recommendation ITU-R [M.8BL43]

Characteristics of VHF radio system and equipment for the exchange of data and electronic mail in the maritime mobile service Appendix 18 channels

Scope

In accordance with Resolution 342 (Rev.WRC-2000), the maritime community is asked to consider the use of new technology on maritime VHF frequencies to make it possible to better respond to the emerging demand for new maritime services. It is also noted that WRC-03 has modified Appendix 18, including the addition of Note o), to permit the possible use on a voluntary basis of various channels or bands created by the conversion of some duplex channels to simplex channels, for the initial testing and possible future introduction of new technologies. This Recommendation describes a VHF radio system currently used and a VHF system currently under development in the maritime mobile service as examples for the exchange of data and electronic mail.

Summary

1)	This Recommendation describes two digital communications systems for the exchange of data and electronic mail in the maritime mobile service on VHF Appendix 18 channels. The relevant channels are those to which footnote o) applies. This Recommendation contains examples for channel usage and technical characteristics of such a system.

2)	These examples are within the scope of Resolution 342 (Rev.WRC-2000).

3)	By adopting Resolution 342, WRC-2000 initiated studies on the modification of Appendix 18 to permit maritime VHF frequencies to be used for data transmission, and ITU-R has considered reports from the operation of systems for the exchange of data and electronic mail on VHF channels.

The ITU Radiocommunication Assembly,

considering

a)	that Resolution 342 of the World Radiocommunication Conference (Rev.WRC-2000) invites ITU-R to finalize studies currently ongoing, inter alia:

–	identify the future requirements of the maritime mobile service;

–	identify suitable technical characteristics of the system or interoperable systems;

–	identify necessary modifications to the table of frequencies contained in Appendix 18;

b)	that IMO has stated that the maritime industry has need for safe, fast and inexpensive communications for business and safety. At IMO COMSAR 8 the future need for harmonization of systems using maritime VHF channels were considered, and ITU-R has been informed of the possible future need for worldwide systems for the exchange of data and electronic mail on maritime VHF channels,

recognizing

that in accordance with Appendix 18 channels used for VHF data shall not cause harmful interference to and shall not claim protection from other stations operating in accordance with Article 5. This includes SOLAS applications such as GMDSS on channel 70 and AIS 1 and AIS 2,

recommends

1	that the characteristics for VHF data described in Annexes 1, 2 and 3 to this Recommendation should be considered as examples of such systems;

2	this Recommendation should be used as a guideline for future digital technologies in the maritime mobile service VHF bands;

3	that new VHF data systems introduced should provide characteristics that are compatible with the existing voice and data system, particularly the AIS.

Annex 1

VHF data system example 1

The following characteristics should be indicative of a VHF radio system for the exchange of data and electronic mail in the maritime mobile service.

# 1	General characteristics

1.1	The class of emission should be 16K0F1DDN

1.2	The necessary band should cater for the channels in Appendix 18 designated with footnote o), each with 25 kHz bandwidth.

1.3	The modulation may be either π/4 DQPSK at 28.8 kbit/s or π/8 D8PSK at 43.2 kbit/s, depending on required station-station radio range and channel signal fidelity.

1.4	The access method may be carrier sense time division multiple access (CSTDMA).

1.5	The following area coverage techniques may be used:

–	cellular channel reuse;

–	time sharing transmission.

1.6	The following handover techniques may be utilized:

–	uninterrupted handover (channel and base station);

–	uninterrupted file transfer.

1.7	The equipment should be designed so that frequency changes between assigned channels can be carried out in less than 100 ms.

1.8	Switching between reception and transmission should not take more than 2 ms.

1.9	The serial communication channels (SCC) on a single radio modem may be:

–	Ethernet;

–	RS232 (NMEA).

1.10	The radio equipment should meet the following norms:

–	radio parameters: ETSI EN 300 113-1;

–	EMC: ETSI EN 301 489-5.

# 2	Transmitters

2.1	The frequency tolerance for coast station transmitters should not exceed 5 parts in 106, and that for ship station transmitters should not exceed 10 parts in 106.

2.2	Spurious emissions should be in accordance with the provisions of RR Appendix 3.

2.3	The carrier power for coast station transmitters should not exceed 50 W.

2.4	The carrier power for ship station transmitters should not exceed 25 W.

2.5	The cabinet radiated power should not exceed 25 µW.

2.6	The adjacent channel power ratio (ACPR) should be at least 70 dB (see Fig. 3).

# 3	Receivers

3.1	The receiver sensitivity for bit error rate (BER) 10–3 should be better than –107 dBm.

3.2	The adjacent channel selectivity should be at least 70 dB.

3.3	The spurious response rejection ratio should be at least 70 dB.

3.4	The radio frequency intermodulation rejection ratio should be at least 70 dB.

3.5	The power of any conducted spurious emission at the antenna terminals should not exceed 2.0 nW.

# 4	Sample emissions spectrum based on variations of ETSI TETRA standard modulation

This proposal refers to the work of RTCM Special Committee 123 (RTCM SC123) which evaluated the ETSI TETRA modulation schemes for use in Appendix 18.

Figure 1

RTCM SC123 test results for TETRA-TEDS modulation

Figure 2

RTCM test results for slightly reduced data rates to fit Appendix 18 emissions mask

Figure 3

Adjacent channel power ratio (ACPR) performance

RTCM test results: 28.8 kbit/s π/4-DQPSK and 43.2 kbit/s π/8-D8PSK modulation

# 5	Conclusions on emissions

Emissions spectrum requirements for Appendix 18 based on maritime IEC test standards will allow both π/4 DQPSK at 28.8 kbit/s and π/8 D8PSK at 43.2 kbit/s modulation to be used.

# 6	System interoperability

6.1	Ship-to-shore

In the ship-to-shore direction interoperability is maintained by the internet service provider (ISP) at the internet protocol (IP) level. Typically, a ship will enter an electronic mail, with or without attachments, in the electronic mail system and then click on the “send” button.

6.2	Shore-to-ship

In this system, there are no interoperability concerns on the part of the shore-side user. The shore-based sender of an electronic mail to a ship can merely:

a)	click on the “reply” button, or

b)	address the message to Shipname@xxx.com or callsign@xxx.com.

The electronic mail will be delivered via whatever system the ship is using. If there is a system failure, there will be an automatic re-route via an alternate system. These automated decisions are based on the contents of an extensive database. Consequently, the electronic mail may be delivered via HF or an alternate satellite-based system. If there is an overall system failure, addressing problem or non-delivery for any reason, the system support operators will be alerted and take corrective action. This ensures that shore-based users need not be concerned about what system or network the ship is using. They need only address the electronic mail and click on “send”.

  - Ship-to-ship

The VDL protocol should also provide for direct transmission between ships where possible (within radio propagation range) in the simplex ship-ship mode. The duplex ship-shore-ship mode should be used for extended range (beyond the ship-ship radio propagation range).

  - Efficient use of the VHF data link (VDL)

System interoperability should be achieved for all transmission modes, ship-to-shore, shore-to-ship, and ship-to-ship. Spectrum efficiency and data throughput should also be considered. For example, application of the electronic mail internet protocol (IP) at the network level and not on the VDL would result in an efficiency improvement of 3:1.

Annex 2

VHF data system example 2

Introduction

This Annex describes an existing narrowband VHF data system for the exchange of data and electronic mail in the maritime mobile service. The system is currently in use, operating from base stations ashore and on offshore installations.

# 1	General characteristics

1.1	 The necessary band should cater for the channels in Appendix 18 designated with footnote o), each with 25 kHz bandwidth.

1.2	The class of emission is 16K0F1DDN.

1.3	The modulation is 4-level GMSK. Transmitted bit rate 21.1 kbit/s.

1.4	The access method is time division multiple access (TDMA).

1.5	The following area coverage techniques are used:

–	cellular channel reuse;

–	time sharing transmission.

1.6	The following handover techniques are utilized:

–	uninterrupted handover (channel and base station);

–	uninterrupted file transfer.

1.7	The equipment is designed so that frequency changes between assigned channels can be carried out in less than 100 ms.

1.8	Emissions are vertically polarized at the source.

1.9	Switching between reception and transmission should not take more than 2 ms.

1.10	The serial communication channels (SCC) on a single radio modem should be:

–	Ethernet;

–	RS232 (NMEA);

–	IEC 61162.

1.11	The radio equipment should meet the following norms:

–	radio parameters: ETSI EN 300 113-1;

–	EMC: ETSI EN 301 489-5 and IEC 60945.

# 2	Transmitters

2.1	The frequency tolerance for coast station transmitters should not exceed five parts in 106, and that for ship station transmitters should not exceed ten parts in 106.

2.2	In order to prevent harmful interference to other users of the maritime VHF band, spurious emissions should be in accordance with the provisions of RR Appendix 3.

2.3	The carrier power for coast station transmitters should not exceed 50 W.

2.4	The carrier power for ship station transmitters should not exceed 25 W.

  - The cabinet radiated power should not exceed 25 µW.

  - Adjacent channel power ratio (ACPR) should be at least 70 dB.

# 3	Receivers

3.1	The receiver sensitivity for bit-error rate (BER) 10–3 should be better than –107 dBm.

3.2	The adjacent channel selectivity should be at least 70 dB.

3.3	The spurious response rejection ratio should be at least 70 dB.

3.4	The radio frequency intermodulation rejection ratio should be at least 70 dB.

3.5	The power of any conducted spurious emission at the antenna terminals should not exceed 2.0 nW.

# 4	Possibilities and advantages

4.1	Coverage and stability

The VHF band has very good qualities regarding range and stability. Typical range from a land-based station is up to 70 nautical miles.

4.2	IP – Ethernet

The common used Ethernet protocol that makes connection to local data networks and other 	data services easy.

4.3	Fixed IP address at the radio on board the ship

This makes it possible to send data to the ship without anyone being needed to activate the link. The ship may also have ten local IP addresses.

4.4	Always connected

There is no connection time. This makes the system very effective for real-time applications, e.g. banking terminals.

4.5	Several services in parallel from one radio on the ship

The system is based on packets all the way. From one radio on the ship one may carry out several different services at the same time. The system is therefore frequency efficient.

4.6	Automatic reconnection after disruption

The system will automatically reconnect and continue the tasks again at the right point. This happens both after short breaks as well as long breaks, e.g. outside radio coverage area.

4.7	Integrated data router

The radio is delivered with an integrated router. It means that tasks may be programmed directly into the radio and may be carried out without the use of a PC. For example, the fishing boat positioning and moving report system is programmed into the radio/router. In addition, the router has very large capacity to carry out several tasks, among other things compression and decompression of electronic mail, web applications and weather maps.

4.8	Several inputs to the radio

Ethernet cable may be plugged directly into the radio or the router, enabling easy establishment of a local net on board the ship. Other digital or analogue inputs may be used for GNSS, measuring instruments, etc.

4.9	Connection to local WLAN

The system may be combined with local wireless networks on board the ship.

4.10	External communication carriers

The system may be delivered with possibilities for seamless connection to external networks, e.g. wireless LANs in harbour areas or to satellite communication.

# 5	Applications

Some current and possible future applications of VHF data are listed below:

–	safe SeaNet reporting (ISPS);

–	fishery catch reporting;

–	fishing boat position and movement reporting;

–	weather maps;

–	general electronic mail;

–	messages to the ship’s agent, the pilot or harbour authorities;

–	banking terminals, especially on passenger ships;

–	safety-related information;

–	telemetry information;

–	updating of electronic maps.

# 6	System interoperability

6.1	Ship-to-shore

In the ship-to-shore direction interoperability is maintained by the Internet service provider (ISP) at the internet protocol (IP) level. Typically, a ship will enter an electronic mail, with or without attachments, in the electronic mail system and then click on the “send” button.

6.2	Shore-to-ship

In this system, there are no interoperability concerns on the part of the shore-side user. The shore-based sender of an electronic mail to a ship can merely:

a)	click on the “reply” button, or

b)	address the message to Shipname@xxx.com or callsign@xxx.com.

The electronic mail will be delivered via whatever system the ship is using. If there is a system failure, there will be an automatic re-route via an alternate system. These automated decisions are based on the contents of an extensive database. Consequently, the electronic mail may be delivered via HF or an alternate satellite-based system. If there is an overall system failure, addressing problem or non-delivery for any reason, the system support operators will be alerted and take corrective action. This ensures that shore- based users need not be concerned about what system or network the ship is using. They need only address the electronic mail and click on “send”.

Annex 3

VHF data system 50 kHz “broadband” example

The following characteristics should be indicative of a VHF radio system for the exchange of data and electronic mail in the maritime mobile service.

# 1	General characteristics

1.1	The class of emission should be 50K0F1DDN

1.2	The necessary band should cater for 50 kHz, two adjacent channels in Appendix 18 designated with footnote o), each with 25 kHz bandwidth.

1.3	The system should be comprised of 16 equal-power subcarriers in the 50 kHz bandwidth with a 16-QAM modulation of each subcarrier, as described in ETSI standard EN 300 392-2 v.3.2.1 (2007-09). This provides a data rate (over-the-air) of 153.6 kbps.

1.4	The access method should be carrier sense time division multiple access (CSTDMA).

1.5	The following area coverage techniques may be used:

–	cellular channel reuse;

–	time sharing transmission.

1.6	The following handover techniques may be utilized:

–	uninterrupted handover (channel and base station);

–	uninterrupted file transfer.

1.7	The equipment should be designed so that frequency changes between assigned channels can be carried out in less than 100 ms.

1.8	Switching between reception and transmission should not take more than 2 ms.

1.9	The serial communication channels (SCC) on a single radio modem may be:

–	Ethernet;

–	RS232 (NMEA).

1.10	The radio equipment should meet the following norms:

–	radio parameters: ETSI EN 300 113-1; EN 300 392-2 v.3.2.1 (2007-09);

–	EMC: ETSI EN 301 489-5.

# 2	Transmitters

2.1	The frequency tolerance for coast station transmitters should not exceed 5 parts in 106, and that for ship station transmitters should not exceed 10 parts in 106.

2.2	Spurious emissions should be in accordance with the provisions of RR Appendix 3.

2.3	The carrier power for coast station transmitters should not exceed 50 W.

2.4	The carrier power for ship station transmitters should not exceed 25 W.

2.5	The adjacent channel power (the power in each of the 25 kHz channels immediately above and below the 50 kHz occupied bandwidth) should not exceed -23 dBm.

2.6	The cabinet radiated power should not exceed 25 µW.

# 3	Receivers

3.1	The receiver sensitivity levels should be better than –106 dBm for shore stations and -103 dBm for ship stations, as described in EN 300 392-2 v.3.2.1 (2007-09) §6.7.2.4.

3.2	The adjacent channel selectivity should be at least 70 dB.

3.3	The spurious response rejection ratio should be at least 70 dB.

3.4	The radio frequency intermodulation rejection ratio should be at least 70 dB.

3.5	The power of any conducted spurious emission at the antenna terminals should not exceed 2.0 nW.

# 4	System interoperability

4.1	Ship-to-shore

In the ship-to-shore direction interoperability is maintained by the internet service provider (ISP) at the internet protocol (IP) level. Typically, a ship will enter an electronic mail, with or without attachments, in the electronic mail system and then click on the “send” button.

  - Shore-to-ship

In this system, there are no interoperability concerns on the part of the shore-side user. The shore-based sender of an electronic mail to a ship can merely:

a)	click on the “reply” button, or

b)	address the message to Shipname@xxx.com or callsign@xxx.com.

The electronic mail will be delivered via whatever system the ship is using. If there is a system failure, there will be an automatic re-route via an alternate system. These automated decisions are based on the contents of an extensive database. Consequently, the electronic mail may be delivered via HF or an alternate satellite-based system. If there is an overall system failure, addressing problem or non-delivery for any reason, the system support operators will be alerted and take corrective action. This ensures that shore-based users need not be concerned about what system or network the ship is using. They need only address the electronic mail and click on “send”.

  - Ship-to-ship

The VDL protocol should also provide for direct transmission between ships where possible (within radio propagation range) in the simplex ship-ship mode. The duplex ship-shore-ship mode should be used for extended range (beyond the ship-ship radio propagation range).

  - Efficient use of the VHF data link (VDL)

System interoperability should be achieved for all transmission modes, ship-to-shore, shore-to-ship, and ship-to-ship. Spectrum efficiency and data throughput should also be considered. For example, application of the electronic mail internet protocol (IP) at the network level and not on the VDL would result in an efficiency improvement of 3:1.

____________

Annex 3 (Alternate Version)

VHF data system 100 kHz “broadband” example

The following characteristics should be indicative of a VHF radio system for the exchange of data and electronic mail in the maritime mobile service.

# 1	General characteristics

1.1	The class of emission should be 100K0F1DDN

1.2	The necessary band should cater for 100 kHz, four adjacent channels in Appendix 18 designated with footnote o), each with 25 kHz bandwidth.

1.3	The system should be comprised of 32 equal-power subcarriers in the 100 kHz bandwidth with a 16-QAM modulation of each subcarrier, as described in ETSI standard EN 300 392-2 v.3.2.1 (2007-09). This provides a data rate (over-the-air) of 307.2 kbps.

1.4	The access method should be carrier sense time division multiple access (CSTDMA).

1.5	The following area coverage techniques may be used:

–	cellular channel reuse;

–	time sharing transmission.

1.6	The following handover techniques may be utilized:

–	uninterrupted handover (channel and base station);

–	uninterrupted file transfer.

1.7	The equipment should be designed so that frequency changes between assigned channels can be carried out in less than 100 ms.

1.8	Switching between reception and transmission should not take more than 2 ms.

1.9	The serial communication channels (SCC) on a single radio modem may be:

–	Ethernet;

–	RS232 (NMEA).

1.10	The radio equipment should meet the following norms:

–	radio parameters: ETSI EN 300 113-1; EN 300 392-2 v.3.2.1 (2007-09);

–	EMC: ETSI EN 301 489-5.

# 2	Transmitters

2.1	The frequency tolerance for coast station transmitters should not exceed 5 parts in 106, and that for ship station transmitters should not exceed 10 parts in 106.

2.2	Spurious emissions should be in accordance with the provisions of RR Appendix 3.

2.3	The carrier power for coast station transmitters should not exceed 50 W.

2.4	The carrier power for ship station transmitters should not exceed 25 W.

2.5	The adjacent channel power (the power in each of the 25 kHz channels immediately above and below the 50 kHz occupied bandwidth) should not exceed -23 dBm.

2.6	The cabinet radiated power should not exceed 25 µW.

# 3	Receivers

3.1	The receiver sensitivity levels should be better than –103 dBm for shore stations and -98 dBm for ship stations, as described in EN 300 392-2 v.3.2.1 (2007-09) §6.7.2.4.

3.2	The adjacent channel selectivity should be at least 70 dB.

3.3	The spurious response rejection ratio should be at least 70 dB.

3.4	The radio frequency intermodulation rejection ratio should be at least 70 dB.

3.5	The power of any conducted spurious emission at the antenna terminals should not exceed 2.0 nW.

# 4	System interoperability

4.1	Ship-to-shore

In the ship-to-shore direction interoperability is maintained by the internet service provider (ISP) at the internet protocol (IP) level. Typically, a ship will enter an electronic mail, with or without attachments, in the electronic mail system and then click on the “send” button.

  - Shore-to-ship

In this system, there are no interoperability concerns on the part of the shore-side user. The shore-based sender of an electronic mail to a ship can merely:

a)	click on the “reply” button, or

b)	address the message to Shipname@xxx.com or callsign@xxx.com.

The electronic mail will be delivered via whatever system the ship is using. If there is a system failure, there will be an automatic re-route via an alternate system. These automated decisions are based on the contents of an extensive database. Consequently, the electronic mail may be delivered via HF or an alternate satellite-based system. If there is an overall system failure, addressing problem or non-delivery for any reason, the system support operators will be alerted and take corrective action. This ensures that shore-based users need not be concerned about what system or network the ship is using. They need only address the electronic mail and click on “send”.

  - Ship-to-ship

The VDL protocol should also provide for direct transmission between ships where possible (within radio propagation range) in the simplex ship-ship mode. The duplex ship-shore-ship mode should be used for extended range (beyond the ship-ship radio propagation range).

  - Efficient use of the VHF data link (VDL)

System interoperability should be achieved for all transmission modes, ship-to-shore, shore-to-ship, and ship-to-ship. Spectrum efficiency and data throughput should also be considered. For example, application of the electronic mail internet protocol (IP) at the network level and not on the VDL would result in an efficiency improvement of 3:1.

____________

The RTCM (Radio Technical Commission for Maritime services) Special Committee 123 (SC123)