GNSS and Augmentation for e-Navigation

### Introduction

Marine navigation is highly dependent on GNSS for position, navigation & timing (PNT) information, which provides current information regarding the position of the vessel, and also a direct input into:

- Integrated Bridge Systems

- ECDIS

- ARPA

- GMDSS

- AIS

- VDR

These inputs influence both the onboard decision-making process, and also that on other vessels and ashore by relaying the vessel information by AIS.

The marine infrastructure is also dependent on PNT information to synchronise time slots in AIS, and to synchronise flashing lights.

This paper aims to summarise current and planned developments in PNT technology, noting that robust PNT information is an essential foundation of e-Navigation, and requires three complementary components:

- A Core Global Navigation Satellite System (GNSS)

- Augmentation of GNSS to ensue that GNSS system performance is fit for purpose

- Adequate backup in the event of GNSS system failure

Requirements

Requirements of navigation systems are generally described as their ability to meet five core functions:

- Accuracy

- Availability

- Continuity

- Integrity

- Coverage

These functions are well described in other publications (e.g. the IALA Navguide). Marine requirements standards are summarised in IMO Resolutions A.953(23) World-Wide Radio-Navigation System, and A.915(22) Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System.

A guarantee of service provenance and security, in the form of Signal Authentication, may also be a future requirement.

Current GNSS receiver technology is defined in IEC 61108 and incorporates Receiver Autonomous Integrity Monitoring (RAIM).

It is anticipated that a future ‘e-Navigation’ receiver will incorporate core GNSS, augmentation and backup capability within a single unit.

Current and Planned Evolution of Space-based GNSS Capabilities

GPS

The current GPS constellation is composed of a mix of 30 satellites from Block IIA, IIR and IIR-M. Of these sets of satellites, only the IIR-M support new signals, namely the civil L2C and the military M-code. The block IIR-M satellites will be followed by the Block IIF, of which only 12 are being procured with no intention to procure additional satellites of this type, since the next generation of satellites   GPSIII - is already in its definition phase.

The Block IIF will incorporate the additional civil signal in the L5 band envisaged for safety-of-life applications, e.g. civil aviation. Regarding spacecraft clocks, after a period where only Rubidium clocks were on-board, the Block IIF satellites will incorporate again a Caesium clock. The launch of the first Block IIF satellites is planned for 2008.

When all the Block IIR-M and Block IIF satellites are in orbit, there will be 20 satellites supporting signals in the L2C band and 12 in the L5 band. Therefore, the achievement of the Full Operational Capability (24 satellites) for L2C and L5 depends on the plans for GPSIII. With the first GPS III launch not scheduled before 2013 the FOC for L2C cannot be expected before 2014 and the FOC for L5 not before 2016. On the other hand, the FOC for GPS III is planned for 2021. The GPS III satellites will incorporate the new civil signal in the L1 band (L1C), directional inter-satellite links and spot-beams. GPS III is envisaged to comprise 27-30 satellites distributed in 6 orbital planes at the same altitude and inclination as the present GPS. They are likely to be procured in three consecutive blocks of satellites with a gradual increase of capabilities:

• Block IIIA: satellites with Block IIF capabilities, enhanced anti-jam and transmission in the L1-C bands.

• Block IIIB: satellites with Block IIIA capabilities, directional cross-link antennas and high-speed up and down-links.

• Block IIIC: satellites with Block IIIB capabilities, reprogrammable navigation payload, integrity monitoring, search and rescue Distress Alarm Satellite System (DASS) and space environment detection payloads.

The GPS ground control segment is also undergoing a process of modernization. The network of monitoring stations has been already increased from the original five to now 13 stations and recently a switch to a new master control station has been completed.

Moreover, the GPS performance is well exceeding specifications. Reported horizontal accuracies are below 3 metres (95%) under quiet ionospheric and clear-sky visibility conditions.

WAAS

WAAS is an augmentation to GPS implemented by the US FAA to support the use of the system for general and commercial aviation over the Continental United States, which recently was extended to also cover parts of Mexico and Canada. The WAAS architecture includes presently 38 reference stations, 3 master stations, 4 up-link stations, 2 geostationary satellite links and 2 operational control centres.

WAAS supports non-precision approaches with vertical guidance up to 350 feet above runway surface and aims to provide by September 2008 a Localizer Performance with Vertical guidance (LPV) service with a performance equivalent to the Instrument Landing System (ILS) and supporting precision approaches with vertical guidance up to 300 feet above the runway.

With the implementation of upgrades over the period 2009-2013 the LPV performance will be enhanced down to 200 feet decision height. The upgrades will include acquisition of an additional GEO satellite and tuning of integrity algorithms for the monitoring of the ionosphere. Following the introduction of LPV-200, it is planned to start introducing an operational dual frequency service in the L1/L5 bands in 2014.

To maintain a redundant capability with single and dual-frequency services is one of the drivers of the WAAS programme to ensure a robust safety-of-life service and it is well reflected in the strategy for the definition of the SBAS GEO broadcast in the L1/L5 bands:

• For the GEO L1: to maintain the current L1 message of corrections for L1 users only adding inter-signal corrections allowing to translate L1-only data for use by GPS dual frequency users of L1/L5.

• For the GEO L5: corrections for L1/L5 users, ionosphere corrections to allow single frequency L5 operation in case of loss of L1 (interference), inter-signal corrections to translate the L1/L5 corrections for L5 users only if L1 is lost.

GLONASS

Following Russia’s renewed interest in GLONASS the system is going through a period of rejuvenated development. According to plans announced in 2006, the goal was to reach a constellation of 18 satellites in 2007 and 24 in 2009.

There are currently 20 satellites in orbit, out of which 11 are operational, 3 are in the commissioning phase, 2 under maintenance and 4 are about to be decommissioned. It seems that the original goal of having 18 operational satellites in 2007 will not be reached, but with a triple launch still planned for 2007, that goal could indeed be achieved by mid 2008.

Moreover, a new type of satellite (GLONASS-M) has been introduced, with a design life-time of 7 years and a second civil signal, and there are plans are to start launching a new generation of satellites (GLONASS-K) in 2009 with improved performance, extended life-time to 10 years and a third civil signal. A final decision on whether GLONASS in the future will incorporate a CDMA signal has not yet been taken.

Russia is also considering an augmentation to GLONASS called SDCM (System for Differential Corrections and Monitoring), which appears to be in a preliminary phase. The concept originally diverged substantially from SBAS standards but it seems that there is an interest to converge towards a more interoperable system.

COMPASS

China is currently engaged in the development of a global system complemented by a regional component. The global system is expected to be composed of a constellation of 27 to 30 satellites in Medium Earth Orbit (MEO) with a regional component composed of up to 5 GEO satellites and possibly a number of 3 or more Inclined Geosynchronous Orbit (IGSO) satellites. The development schedule for Compass foresees a full operational capability by 2012.

The first COMPASS MEO satellite was launched in April 2007 and transmitted signals overlapping with the Galileo signals in the E2, E6 and E5b bands. Most notably, power levels are 4-7 dB’s above those of GPS L1/CA.

Regarding the GEO space segment, there are already 4 GEO satellites in orbit using a combination of signals in the L and S-bands. The use of S-band has an advantage for integration with modern mobile telephone terminals since it allows the use of more effective antennas combining communications and navigation. Currently the use of S-band for navigation is only possible in the Asian region.

GAGAN, IRNSS

In India, two regional systems are currently under development: the GPS-Aided Geo Augmented Navigation system (GAGAN) and the Indian Regional Navigation Satellite System (IRNSS).

GAGAN is an SBAS augmentation system of the type similar to WAAS and EGNOS. It is at the end of its Technology Demonstration Phase. GAGAN includes 8 reference stations, 1 mission control centre, 1 up-link station and 1 Geo link through the L1/L5 transponder on the INMARSAT 4-F1 satellite. The full operational capability has been announced for 2010 by when the system would have been extended with an additional master control centre, an additional up-link station, 2 additional GEO links (L1/L5) plus one in-orbit spare and more reference stations.

The Indian Regional Navigational Satellite System (IRNSS) will as an independent navigation system cover the Indian region through a space segment of 3 GEO satellites and 4 IGSO satellites. The inclination of the orbital plane of the IGSO satellites is low, which allows seeing all the satellites of the constellation simultaneously over India. The first IRNSS satellite is planned to be launched in 2009 with a system completion by 2011.

Three services of IRNSS are foreseen:

• Open Service using signals in the L5 and S bands.

• Precise Positioning Service using signals in the L5 and S bands.

• Restricted Access Service using signals in the L5 band only.

The Open and Precise services target dual frequency users but there is also the intention to compute and broadcast ionosphere-corrections to support single frequency users. Because of the reduced extension of the IRNSS network of reference stations the satellites will, apart from the navigation payload, also include a dedicated C-band uplink/down-link ranging payload to support precise satellite orbit determination.

MSAS, QZSS

In Japan there are two regional systems under development: Multi-Satellite Augmentation System (MSAS) and Quasi-Zenith Satellite System (QZSS). MSAS is an SBAS type system similar to EGNOS and WAAS. MSAS is already in its initial operational capability phase with 2 GEO-links using the L1 band via dedicated satellites shared with communications and meteorological missions.

QZSS is based on 3 satellites in slightly eccentric highly inclined IGSO orbits guaranteeing always one satellite in visibility of Japan with a minimum elevation angle of 60 degrees. Each satellite will transmit 6 signals in the L-band: 3 in L1, one in E6, one in L2 and one in L5.

One of the signals aims to provide sub-meter accuracy and integrity while maintaining compatibility with SBAS. The signal in E6 aims to support a commercial service with high data rate (2 Kbps). The other signals are GPS signals including L2C and L1C standards. The QZSS programme considers for the moment the launch of only one satellite for validation purposes by 2009.

GALILEO and EGNOS

The present European GNSS infrastructure is composed of EGNOS, an augmentation to GPS L1 designed for safety-of-life applications (i.e. civil aviation) and which is already providing initial operational services, and Galileo. When Galileo becomes operational, the system will provide a set of global services defined as Open Service (OS), Safety-of-Life Service (SOL), Commercial Service (CS) and Public Regulated Service (PRS).

The OS signals were designed for mass market users, the SOL service performance is mostly driven by civil aviation requirements to support precision approaches, the CS is a service of data dissemination to be used for commercial on-demand applications and the PRS is a service for security government authorized applications. In addition each Galileo satellite will embark a separate payload to support the SAR service. This would be the architecture of the European GNSS by the middle of next decade without considering any evolutions.

The first launch of an operational Galileo satellite is scheduled for 2010 and the Full Operational Capability is planned for 2013. The first generation of Galileo will incorporate a number of features which will not be included in some of the other systems before well into the next decade. This concerns the signal in the E1 band with the advanced Multiplex Binary Offset Carrier (MBOC) modulation also adopted for GPSIII L1C, and the safety-of-life signal in the E5b band with a function similar to the L5 signal to be introduced on the GPS block IIF.

Galileo also incorporates some features which are not yet known to be planned for other systems. This is the case of the large-bandwidth signal in the E5aE5b band, and a global integrity service for Safety-of-Life applications. There are, on the other hand, also features known to be planned for other systems, which are not included in the first generation of Galileo, in particular inter-satellite links and spot beams in GPSIII.

EGNOS is currently in its initial operational phase with discussions on-going to secure the long-term operations of the system. The system provides currently an SBAS service in the L1 band through two GEO transponders on INMARSAT satellites. A third GEO transponder on ARTEMIS is used to support system upgrades. The required replacement of the transponders during the period 2008-2011 must be planned as an integral part of the evolution of EGNOS.

EGNOS is presently fully interoperable with the current generation of WAAS and MSAS. However, as discussed above these systems are already planning an evolution towards a dual-frequency capability in the L1/L5 bands.

Potential Evolution of the GNSS Architecture and Technology Improvements

Evolution of the GNSS infrastructure needs to be considered, because there are several opportunities for fulfilling emerging user needs. As shown in the previous section, the following new services can be identified:

- Provision of augmentation services to all constellations in view to improve the performance obtained with the Galileo-only signals of one or several of the following parameters: accuracy, integrity, availability or robustness.

- Provision of additional channels for supporting the dissemination of augmentation data at regional level both for safety-of-life and mass-market applications and to service users in the civil aviation, maritime and land domains for all latitudes of the European region.

- Provision of additional navigation signals at higher frequencies (S, C-band) for better performance (accuracy, robustness) and better interoperability with the evolutions in spectrum utilization for mobile communications.

#### Augmentation

Requirement

There is a requirement for augmentation of GNSS, to ensure that the  integrity and accuracy requirements defined in IMO Resolutions A.953(23) World-Wide Radio-navigation System, and A.915(22) Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System are met.

These documents summarise the integrity requirement as a means of warning the user of system non-availability or discontinuity within a Time to Alarm of 10s for all application.

Horizontal accuracy requirements vary from 10m in Coastal waters to 100m in Ocean waters in A.953(23), and between 0.1m and 10m in A.915(22), dependent on the application. Currrently the 10m requirement is met by DGNSS and the 100m requirement by ‘raw GNSS. It is anticipated that in future 10m will be met by GNSS, 1 metre by DGNSS, and 0.1metres by High Accuracy solutions such as RTK.

The e-Navigation concept must define how augmentation to a suitable standard is to be achieved, to fulfil the essential requirement for robust radio-navigation.

There is a tight timescale for this activity, as SOLAS Chapter V is due for revision in 2013, and this is effectively the opportunity to define carriage requirements for e-Navigation.

###### Potential Solutions

Potential means of sourcing augmentation information fall into 4 main areas:

- Satellite based systems, either co-located within the GNSS satellites, or provided externally, as discussed previously

- LF/MF terrestrial radio systems

- VHF/UHF line-of-sight radio systems

- Hybrids of these technologies

These systems are described fully in other documents, and were previously compared in IALA Recommendation R-135 on the Future of DGNSS (December 2006).

###### Satellite Based Solutions

The ultimate aim may well be that the GNSS satellites, whether GPS, GLONASS, Galileo or Compass, are capable of assessing their own integrity and transmitting necessary information to ground users. Such developments do not appear to be imminent. The accuracy of GNSSS will undoubtedly continue to increase with the proliferation of satellite numbers, which will improve satellite geometry and reduce dilution of precision (DOP), a better prediction of satellite ephemeredes and with multiple frequency signals. The latter will facilitate correction of the ionospheric transmission delay, which is one of the largest error sources.

Currently, satellite based augmentation consist of geostationary satellites, either provided by government agencies primarily for aviation safety (WAAS, EGNOS, MSAS, Beidou) or by commercial suppliers (Omnistar, Veripos, C-NAV) for the surveying industry. Most satellite based augmentation systems (SBAS) utilise a network of ground based Reference stations to monitor the integrity and accuracy of the GNSS system. A wide-area position solution is then identified and broadcast via the satellite. This generally results in a solution with low spatial decorrelation errors, but is less capable of coping with localised ionospheric effects (see Skone). The coverage of GEO based signals also diminishes with increasing latitude and in built-up port areas.

LF/MF Terrestrial Radio Systems

Augmentation may also be provided at relatively low data rates via LF/MF radio systems. The current IALA beacon system operates in the marine radiobeacon band around 300kHz, and achieves a typical service radius of 100-200nm from each station. It is also possible to transmit corrections within other radiobeacon bands, such as around 515kHz.

A further method is to transmit augmentation systems over the data channel of eLoran, as in the Eurofix message. This has been successfully transmitted from 4 European stations.

Each site requires a minimum of one GNSS Reference Station, together with modulation equipment. In order to confirm the integrity of the broadcast message it is also necessary that a minimum of one Integrity Monitor be provided, which may either be located with the Reference Station, or offsite.

###### VHF/UHF line-of-sight radio systems

Short range VHF/UHF GNSS augmentation is typically used within the surveying industry, usually in conjunction with carrier wave GNSS in a technique known as Real Time Kinematic (RTK) GNSS which gives accuracies of the order of a few millimetres. These systems are also used within the marine industry, typically to assist with docking procedures. These systems function via line-of-site radio, with a typical effective range of 10 km. AIS incorporates a functionality to transmit DGNSS correction messages via Message Type #17.

Each site requires a minimum of one GNSS Reference Station, together with modulation equipment. In order to confirm the integrity of the broadcast message it is also necessary that a minimum of one Integrity Monitor be provided, which may either be located with the Reference Station, or offsite.

Pseudolites, transmitting GNSS signals from ground stations, are capable of supplementing and augmenting GNSS from space.

###### Hybrid systems

Hybrid systems aim to give additional coverage without the expense of Reference Station and Integrity Monitoring equipment at each transmission site. Such systems may feature:

- radiobeacons to rebroadcast SBAS messages, either in their original format (RTCA), or processed to a different format (RTCM), as described in IALA recommendation R-135.

- AIS rebroadcasting either processed SBAS messages or radiobeacon messages via Message Type #17.

- Any radio carrier using a network of offsite Reference Stations to calculate a wide area solution. The correction data may be transmitted to the radio carrier by dedicated communications link or the internet. An example of this approach is the Virtual Reference Station, where corrections are broadcast typically via the local mobile phone network.

High Accuracy Services

Achieving sub-metre or even decimetre level accuracies via GNSS is possible via Real Time Kinematic techniques, broadcast via any carrier of adequate bandwidth. Trials have also been carried out in the United States regarding transmission of compressed RTK data on a low bandwidth beacon, and in Korea regarding high accuracy by improved ionospheric and tropospheric correction data. Decimetre level accuracies are also claimed by commercial SBAS DGNSS service providers, and may also be achievable via pseudolite technology.

###### Comparison of potential services

It is necessary to compare the alternative systems against a number of relevant criteria, which may include:

- Current development of technology

- Current status of relevant marine equipment and performance standards

- Cost to user

- Cost to service provider

- Coverage

- Accuracy

A simplified comparison can be found at appendix A.

Analysis of Maritime User Needs and Opportunities

The maritime sector is presently dominated by the use of GPS.  DGPS services are provided through the IALA DGPS network, which covers a large part of the world’s coastal regions.

Regarding performance, with accuracies and alarm limits of 10 and 25 metres respectively, all phases of maritime navigation are currently met except for the port phase, where the accuracy requirement is 1-2.5 metres. At present the 10 metre requirement is met with the use of the IALA DGPS stations, but these do not support the more demanding requirements of the port phase.

SBAS services for maritime users should not be seen as competitors to the IALA differential stations, but rather as providing a good synergy with them, since SBAS can provide near-global (unapproved) marine services for the leisure market and the IALA stations can broadcast marine standard signals with full coastal coverage for the professional user. However, if marine standards, and adequate Service Level Agreements, do not exist, there is an increasing possibility of any incident leading to litigation.

AIS standards give highest priority to an position data derived from an external differentially corrected GNSS source. Where this not provided, it is acceptable to use internal message Type #17, and it would be practical for administrations with a single port and no offshore hazards to supply this service, assuming that the VHF data link can support this and a viable user base exists. With a relatively high data rate, AIS may be able to supply future High Accuracy services (i.e. RTK sub-metre services), again assuming that the local VHF data link can support this. Alternatively, this information could be broadcast via wideband digital VHF channels; however it should be noted that only vessels fitted with a multi-frequency GNSS receiver and a suitable data receiver, will be able to utilise RTK services.

### Recommendation

The IALA e-Navigation Committee is requested to review this paper, make any additions or corrections as it sees fit, and then use this paper to inform urgent and necessary discussions to identify the most practical means of sourcing robust PNT information.

Having identified the most suitable technologies, the Committee should prepare a World Wide Radio Navigation Plan for approval by the IALA Council as a PNT strategy for e-Navigation, identifying the needs of:

- Users

- Service Providers

Within this Plan, priority must be given to establishing:

- Recommendations with regard to Carriage Requirements, to be input into IMO in advance of the 2013 SOLAS Revision

- Links with the bodies controlling marine equipment standards (IEC, RTCM) and any necessary radio frequency allocations (ITU-R)

- Performance standards for any service providers, who may or may not be IALA members

###### References

- IMO Resolution A.953(23) World-Wide Radio-Navigation System

- IMO Resolution A.915(22) Revised Maritime Policy and Requirements for a Future Global Navigation Satellite System

- The European GNSS Evolution Programme: Preliminary Views on Period 2, European Commission, 2007

- IALA Recommendation R-129 on GNSS Vulnerability and Mitigation

- IALA Recommendation R-135 on the Future of DGNSS (December 2006)

- Performance Evaluation of DGPS versus SBAS/WADGPS for Marine Users, S.Skone, ION-GNSS 2007

- http://www.navcen.uscg.gov/gps/WAAS-DGPS.htm

- IEC 61993-2 Ed.1, Maritime navigation and radiocommunication requirements - automatic identification systems (AIS) - part 2: class a shipborne equipment of the universal automatic identification system (AIS) - operational and performance requirements, methods of test and required test results.

Appendix A: Comparison of Augmentation technologies

| ##### System | Coverage | Cost to user1 | Marine standards | Cost paid by | Cost to Marine Service Provider3 | Approximate Accuracy achievable | Current Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Government SBAS | GEO2 | $ | No4 | Government | - | 1-3 metres | Varies with system; widespread takeup as often incorporated within GNSS receiver |
| Commercial SBAS | GEO2 | $$ | Commercial | User | - | Decimetre | Well established; niche markets in surveying/ offshore/DP markets |
| IALA Beacon | 100-200 nM | $ | Yes | Marine Service Provider | $ | 1-3 metres | Well established service provision; continuing take up by professional users |
| Loran Eurofix | ~500nM | $ | No | Marine Service Provider | $5 | 1-3 metres | 4 Operational sites; minimal user takeup |
| Commercial UHF Real time Kinematic (RTK) | Line-of-sight to 10km | $$ | No | Local port/User | - | <5cm | Limited establishment; niche markets in DP/ automatic docking markets; requires L1/L2 receiver |
| AIS | Line-of-sight | $ | Yes | Marine Service Provider | $$5 | 1-3 metres | Service provision trialled; large number of receivers, but few capable of using DGNSS |
| SBAS/Beacon hybrid | GEO + Coastal | $ | Partial4 | Government/ Marine Service Provider | $ | 1-3 metres | Not established |
| AIS/Beacon hybrid | Line-of-sight | $ | Yes | Marine Service Provider | $$5 | 1-3 metres | Service provision trialled; large number of receivers, but few capable of using DGNSS |
| RTK via beacon | 100-200 nM | ?? | No | Marine Service Provider | $ | 30cm ? | Trialled |
| Pseudolites | Line-of-sight | ? | No | Local port/User | $$5 | Decimetre ? | Not established |

Notes:

- Cost to user: $ < $1000 with no annual fees; $$ > $1 000 with annual fees

- Performance depends on coverage of reference stations, and is limited by latitude and high buildings

- Capital Cost to Marine Service Provider per 1000 km of Coastline: $ < 10,000,000$; $$ > 10,000,000$

- SBAS uses RTCA standard

- System performs functions other than augmentation