PROPOSAL ON THE  DEVELOPMENT OF “IALA GUIDELINES ON PROVISION OF VIRTUAL AtoN AIS” AND

DISPLAY OF “SYMBOLS”

1. Purpose

Virtual AtoN AIS is considered as an imaginary entity having a capability to provide navigation support information effectively utilized by AIS equipped ships. However, there is at present no specific standards on operation of virtual AtoN including such aspects as the regional area where virtual AtoN should be provided, the type into which each of virtual AtoN may be categorized, the standard with which virtual AtoN will be established, etc. In addition, symbols of virtual AtoN AIS to be displayed on AIS user equipment have not defined yet.

In light of this situation, in Japan, a study-and-research work on virtual AIS for aids to navigation was commenced by a study committee organized with professors and experienced engineers including a pilot, a captain of large ship, etc. In the course of studies, the committee conducted a field test at sea to evaluate their findings.

This paper will, based on accomplishments the committee had worked out through a series of the studies, propose to develop guidelines on operation of

virtual AtoN AIS, which should be brought up to an international standard, and to design symbols of virtual AtoN AIS.

2. Notes on Studies

The study committee carried on the study-and-research work about utilization of Virtual AtoN AIS, spending two years from April 2005 to March 2007, as summarized below.

2.1. Categorization of application area

Regional area where virtual AtoN AIS should be applied was divided into three different waters; a traffic congested, a coastal and a narrow channel. Then, application of virtual AtoN AIS was discussed for each of the waters.

2.2. Function of Virtual AtoN AIS

Discussions on the type to be assigned and the function to be designed for each of the virtual AtoN lead to a consensus that the AtoN AIS should be, in terms of an information provider, categorized into two; a permanent and a temporary presentation tool.

2.3. Draft of Symbols

The committee discussed and tried to design the symbols for each type of the virtual AtoN.

2.4. Field test at sea

To verify findings the committee had obtained through studies and discussions, a field test was conducted, deploying several types of virtual AtoN AIS at selected points in a modeled sea area. The user equipment prepared for assessment of the test was located on an AtoN evaluation vessel of Japan Coast Guard which contributed as a mobile station in the field.

With this test arrangement, drafted symbols of virtual AtoN were assessed for their effectiveness and practicability through reproduced images and characters on user equipment onboard.

3. Proposal

3.1. Development of Guidelines on “Provision of Virtual AtoN AIS”

The study committee could confirm effectiveness and practicability of virtual AtoN AIS through a series of study-and-research work. Consequently, it was felt that, in future, an operational guidance will be required as an international standard in order for the Virtual AtoN AIS to be effectively utilized in daily marine life as a navigation support tool of AIS equipped ships.

It is, therefore, proposed that “Guidelines on operation of Virtual AtoN AIS” should be developed as soon as possible, covering all the aspects required for operation of the AIS - namely, region of application, type and function of virtual AtoN, etc.

Aimed to serve as a basic paper for development of the guidelines, a draft “Operation of Virtual AtoN AIS in different applications” is attached herewith as Annex 1/prop.2, which was prepared by the committee.

3.2. Proposal on Symbols of Virtual AtoN AIS

Effective utilization of virtual AIS needs an internationally accepted standard on symbols defined by type so that the user can clearly identify the AtoN of its interest.

To that end, the committee decided to develop draft symbols which were intended to give no confusion to the users and to enhance conveniences of the users in identifying descriptions presented by shapes and/or texts on a map, taking account of the following aspects as the basic criteria.

The drafted shall;

-  comply with the universally established symbols used for charted physical AtoN (real entity), and

-  not contain a symbol depicting a light in the illustration of virtual AtoN.

Noting these fundamental principles as the basis, symbols for each type of virtual AtoN were designed and a preliminary paper “Symbols of Virtual AtoN AIS” was produced as Annex 2/prop.2 shows. Thus, we propose this as a draft of internationally standardized design of the symbols.

For the preliminary symbols, comments on further improvement were brought in about the size and other aspects, as a result of another field test at sea conducted later (see Annex 3/prop.2).

Annex 1/prop.2

DRAFT “GUIDELINES ON PROVISION OF VIRTUAL AtoN AIS”

1. Categorization of Application Area

1.1. Traffic congested areas

In crowded shipping areas, since the number of AIS messages originated from AIS equipped ships increases, transmission of AIS information in less urgency should be kept at a minimum level in quantity in order to avoid overcrowding of the messages.

1.2. Coastal areas and narrow channels

１

In coastal areas and narrow channels, since the number of AIS messages sent out from AIS equipped ships is considered not so many, it is desirable for virtual AtoN AIS to provide, without any restriction, highly effective information according to the features of the surroundings.

2. Function of Virtual AtoN AIS

| Application Mode | Function | Description | Type of Virtual AtoN AIS | Symbol (Ref:Annex2) |
| --- | --- | --- | --- | --- |
| Permanent Marking（Obstacles） | Marking of Shoals and Reefs | Virtual AtoN must be effectively utilized at the place where a physical AtoN is difficult to be built due to big waves, strong winds, etc. A clear marking of the shoals and/or reefs will serve for improvement in safety of navigation. | Isolated Danger Marks | Fig. 9 |
| ２ Permanent Marking (navigation support） | Marking of Fairway Limits | Virtual AtoN must be effectively utilized at the places where lighted buoys cannot be installed due to unsolved problems on, e.g., sharing of the water surface in fairways with fishermen. A clear marking of the fairway limits will serve for orderly flow of marine traffic and improvement in safety of navigation. | Lateral Marks | Fig. 5, 6, 7, 8 |
|  | Marking of Fairways | Virtual AtoN must be effectively utilized at the place where a physical AtoN is difficult to be built due to the water depth, seabed, etc. A clear marking of the fairway will serve for improvement in safety of navigation. | Lateral Marks, & Safe Water Marks | Fig. 5, 6, 7, 8 Fig.10 |
|  |  | Virtual AtoN must be effectively utilized at the point in approach to a harbor entrance where a ship changes its course and a physical AtoN is difficult to be installed. A clear marking of the point of approach will serve for an orderly flow of ships at a harbor entrance and improvement in efficiency of shipping. | Safe Water Marks | Fig.10 |
|  | Marking of Fairways (Recommendable) & Marking of the Limits of Safe Water | Virtual AtoN must be effectively utilized at the area where navigation becomes difficult due to a thick fog, a heavy rain, etc. (This application can also be adoptable as a temporary marking during a limited visibility.) Marking of a recommendable fairway during a limited visibility will serve for improvement in safety of navigation and efficiency of shipping. | Lateral Marks, & Safe Water Marks | Fig. 5, 6, 7, 8 Fig.10 |
| Temporary Marking | Marking of a Navigation Restricted Areas | Virtual AtoN must be effectively utilized when navigation restriction is required due to e.g., marine accidents or when marking of a wreck is needed. A temporary navigation restriction will prevent subsequent disasters. | Cardinal Marks, & Special Marks | Fig. 1, 2, 3, 4 Fig.11 |
| ３ Temporary Marking | Designation of Temporarily Recommendable Fairways | Virtual AtoN AIS must be effectively utilized for indication of fairways when a large scale disaster hit the area. A clear marking of temporarily recommendable fairways will be expected to serve for the relief ships dispatched to the site to support safe and effective relief activities. | Lateral Marks & Safe Water Marks | Fig. 5, 6, 7, 8 Fig.10 |
|  | Marking of Aids to Navigation in Trouble | Virtual AtoN AIS must be effectively utilized when a physical AtoN lost ability to perform regular functions due to a natural disaster. When a physical AtoN lost ability to perform regular functions due to natural disasters, recovery actions are required at an earliest opportunity. Virtual AtoN can respond to the circumstance even if the actions by personnel cannot be achieved due to meteorological and/or hydrographical conditions, and keep the influence of the trouble reach to ships in navigation at a minimum level. | Cardinal Marks, Lateral Marks, Isolated Danger Marks, Safe Water Marks, Special Marks, & Other Position Marks | Fig .1, 2, 3, 4 Fig .5, 6, 7, 8 Fig. 9 Fig.10 Fig.11 Fig.12 |

４

Annex 2/prop.2

Draft “Symbols of Virtual AtoN”

| Fig. No. | Type of AtoN |  | (draft) Symbols of Virtual AtoN |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Floating Aids | Fixed Aids |  |
| 1 | Cardinal Marks | North Cardinal Mark | Virtual AIS | Virtual AIS |  |
| 2 |  | East Cardinal Mark | Virtual AIS | Virtual AIS |  |
| 3 |  | South Cardinal Mark | Virtual AIS | Virtual AIS |  |
| 4 |  | West Cardinal Mark | Virtual AIS | Virtual AIS |  |

Draft “Symbols of Virtual AtoN”

| Fig No. | Type of AtoN |  | (draft) Symbols of Virtual AtoN |  | Remarks |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Floating Aids | Fixed Aids |  |
| 5 | Lateral Marks | Port Hand Mark | Virtual AIS | Virtual AIS |  |
| 6 |  | Starboard Hand Mark | Virtual AIS | Virtual AIS |  |
| 7 |  | Mark for Preferred Channel to Port | Virtual AIS | Virtual AIS |  |
| 8 |  | Mark for Preferred Channel to Starboard | Virtual AIS | Virtual AIS |  |

Draft “Symbols of Virtual AtoN”

| Fig. No. | Type of AtoN | (draft) Symbols of Virtual AtoN |  | Remarks |
| --- | --- | --- | --- | --- |
|  |  | Floating Aids | Fixed Aids |  |
| 9 | Isolated Danger Marks | Virtual AIS | Virtual AIS |  |
| 10 | Safe Water Marks | Virtual AIS |  |  |
| 11 | Special Marks | Virtual AIS | Virtual AIS |  |
| 12 | Lighthouse | Virtual AIS Virtual AIS |  |  |

Note 1: For the symbols of Physical AtoN AIS and Synthetic AIS, the circle of a dashed line in symbols above shall be replaced with the one of a solid line.