| From: IALA | e-NAV 4/output/12 |
| --- | --- |
| To: IEC TC80 AIS WG | February 28, 2008 |

Liaison Note to IEC

## Selection of AIS DGNSS Services in a Class A transponder

# Introduction

IALA is currently revising Recommendation A-124, which covers the implementation of an AIS shore-based infrastructure. One aspect covered in the Recommendation is the provision of a DGNSS service over AIS. It is likely that when an administration wishes to provide such a service, several reference stations will be implemented, and therefore it is likely that mobile stations in the area will receive DGNSS data from multiple sources. It was noted that IEC standards for mobile AIS stations, in particular the Class A IEC standard, IEC 61993-2, does not currently have a test to ensure that the transponder does not switch between reference stations as the messages are received.

# DGNSS Station Selection

The current revision of ITU-R M.1371 states that a station should use the nearest reference station, taking into account the Z-count and the health of the DGNSS station. A reference station selection process already exists in the DGNSS radiobeacon receiver IEC standard, IEC 61108-4, and this is used as a basis for the reference station selection process in AIS mobile stations shown in Figure 1.

The process allows for the closest reference station to be used, unless the reference station is marked unhealthy, or if a set of corrections has not been received for 60 seconds or more. In this case, the next closest reference station should be used. This will ensure that the best reference station for the mobile station is used at all times.

# Conclusion

IALA asks IEC to include the DGNSS reference station selection process in all relevant IEC standards, and tests be added to ensure that this station selection process is followed. The tests should also ensure that when a single-slot Message 17 containing an integrity warning for a particular SV is received, then the mobile station correctly removes it from the position fix solution.

Select closest reference station

Use signal

Check all available reference stations [every minute]

Mark reference station unhealthy

Mark reference station unavailable

Select next closest reference station

START

Y

N

N

Y

N

Y

Any closer (by 10%) station

Is station still healthy?

Age of corrections >60 seconds?

Figure 1 – Proposed DGNSS reference station selection process to be included in IEC61993-2.