Mariner E-Navigation User Requirements

Key Navigation Functions

The primary user requirements are to satisfy the key navigation functions to navigate a ship from berth to berth.

These functions are: 

1. Voyage planning
2. Execution of the voyage
3. Collision avoidance
4. Manoeuvring and conning the vessel
5. Communications - outward and inward
6. Recording of navigational events, and 
7. Voyage reports

Needs and wants of the merchant mariner within the concept of e-navigation for the execution of the above functions are listed below.

1  Voyage Planning

- To comply with SOLAS/statutory requirements for voyage planning

- A standardised user interface and information system and presentation display (e.g. S Mode)

- Up to date reliable and appropriate charts (assumed to be ENCs),

- Supporting hydrographic and environmental information:

- Sailing directions (eg. Pilots)

- Tidal stream and current tables/predictions

- Tide tables/predictions

- Real-time tide and tidal stream information (where necessary)

- Meteorological information relevant to the intended voyage and surrounding areas

- Information about relevant AtoN for the intended voyage

- Information about communications, Electronic Position Fixing Systems (EPFS), Global Maritime Distress and Safety System (GMDSS), pilot services and port operations, Vessel Traffic Management (VTM) and reporting systems and satellite navigation systems.

- Other thematic and miscellaneous information on fisheries, environmental protection, other maritime activities, including Notices to Mariners, ENC updates and navigation warnings.

- Other exceptional information about the status and reliability of supporting information services (including position fixing and hydrographic data)

- Information about fairway and port operations relevant to the voyage (eg. Availability of locks, dredging activity etc)

- Electronic data within system on vessel handling, manoeuvring and dynamics

- Effective identification of ship specific hazards relevant to the voyage

- Sharing and agreement about voyage planning information between ships, between ships and pilots and between ships and shore organisations

- In certain high traffic waterways mariners and shore authorities will need to consider the coordination of passage planning information (e.g. allocated way points or a Marine Electronic Highway) to provide planned separation between ships and so reduce the risk of collision

2  Execution of the voyage plan

- A navigation information management system with a standardised user interface and presentation display (e.g. S Mode) that is designed in accordance with Human Element principles which operates in a way that actively engages the mariner and eliminates single person/point failure

- Continuous and automatic electronic position determination, with an alternative independent position determination capability

- Clear indication of determined position accuracy

- [Capability to manually compare positional information from multiple sources]

- To actively engage the mariner in the process of navigation and provide an alternative method of position determination (e.g. a manual capability to electronically capture and record observed compass bearings and radar range and bearing information).

- Continuous presentation of own ship position in relation to local geography, bathymetry and hazards to navigation with appropriate alarm and alert functions

- Clear indication of the quality of hydrographic data

- Continuous and reliable presentation of future under keel clearance (UKC)

- Supporting hydrographic and environmental information:

- Sailing directions (e.g. Pilots)

- Tidal stream and current information

- Tide prediction information

- Real-time tide and tidal stream information (where necessary)

- Meteorological information relevant to the intended voyage and surrounding areas

- Information about relevant AtoN for the intended voyage

- Information about communications, Electronic Position Fixing Systems (EPFS), Global Maritime Distress and Safety System (GMDSS), pilot services and port operations, Vessel Traffic Management (VTM) and reporting systems and satellite navigation systems.

- Other thematic and miscellaneous information on fisheries, environmental protection, other maritime activities, including Notices to Mariners, ENC updates and navigation warnings.

- A capability to filter received information/data to that which is relevant to the vessel and its current voyage

3  Collision avoidance

- A navigation information management system with a standardised user interface and presentation display (e.g. S Mode) that is designed in accordance with Human Element principles which operates in a way that actively engages the mariner and eliminates single person/point failure

- Automatic high integrity continuous electronic detection and determination of position of other ships and fixed and floating obstacles

- A method of manually engaging the mariner in the assessment of risk of collision (e.g. a manual capability to electronically capture and record observed compass bearings and radar range and bearing information)

- Determination and presentation of risks of collision (e.g. calculation of closest point and time of approach) and the provision of collision avoidance support information (e.g. relative and true vectors for detected contacts, and/or trial manoeuvre information (i.e. modelling)

- In certain busy waterways there may need to be the provision by a shore management authority of an assigned passage plan.  These could be designed and allocated so as to provide lateral separation between ships.

4 Manoeuvring and conning the vessel

If e-navigation extends to manoeuvring of vessels through a device such as a track control within a ship’s e-navigation equipment, mariner’s needs are as follows

- Ready access to all the information listed in 1, 2 and 3 above through a navigation information management system with a standardised user interface and presentation display (e.g. S Mode) that is designed in accordance with Human Element principles which operates in a way that actively engages the mariner and eliminates single person/point failure

- Timely decision support advice (e.g. UKC information)

- A standardised ship control interface including unambiguous switching between automatic and manual modes of operation

- Clear indication of any automation modes

- Clear real-time indication of accuracy of manoeuvre

- Clear vessel/system/control feedback information (e.g. rate of turn, rudder angle, shaft revolutions ahead/astern, log speed, etc)

5  Communications - outward and inward

Mariners need to communicate outward and inward in an effective and reliable way,

- The methods and technology for communications need to be as transparent as possible to mariners.

- Content for communication shall use standard formats wherever possible.

- All communication requirements shall be assessed to ensure maximum efficiency while reducing unnecessary interference or distraction as far as possible

- Critical communication shall have a means of acknowledgment and recording.

- Reporting

- A capability to receive potential incident advice from a shore authority

On board communications systems for both transmission and reception purposes include:

- MF, HF and VHF (including GMDSS compatible Digital Selective 	Calling [DSC] radio) equipment.

- Satellite communications equipment ( Ship Earth Stations);

- Search and Rescue Transponders (SARTS)

- Low powered on board communications equipment (including UHF 	and repeater stations)

- Emergency Position Indicating Radio Beacons

E-navigation should make the operation of all such systems as simple and intuitive as possible, particularly where human input is required. Where standard reports need to be transmitted, these need to be generated autonomously and transmitted without distraction to the navigator.

The alerting of navigators to incoming communications, particularly safety related information, should be standardised and based on sound ergonomic principles.

Language difficulties are likely to remain obstacles to the use of spoken messages. Training will reduce problems.

6. Recording of navigational events

SOLAS Chapter V, Regulation 28 states that:

“1. All ships engaged on international voyages shall keep on board a record of navigational activities and incidents which are of importance to safety of navigation and which must contain sufficient detail to restore a complete record of the voyage, taking into account the recommendations adopted by the Organization. When such information is not maintained in the ship's log-book, it shall be maintained in another form approved by the Administration. 2 Each ship of 500 gross tonnage and above, engaged on international voyages exceeding 48 hours, shall submit a daily report to its company, as defined in regulation IX/1, which shall retain it and all subsequent daily reports for the duration of the voyage. Daily reports may be transmitted by any means, provided that they are transmitted to the company as soon as practicable after determination of the position named in the report. Automated reporting systems may be used, provided that they include a recording function of their transmission and that those functions and interfaces with position-fixing equipment are subjected to regular verification by the ship's master. The report shall contain the following: .1 ship's position; .2 ship's course and speed; and .3 details of any external or internal conditions that are affecting the ship's voyage or the normal safe operation of the ship”.

E-navigation recording must take into account the difficulties itemised above, facilitating manual inputs where required

7   Voyage Reports

Standard “e-navigation” formats for commercial voyage reports should be devised and embedded within the “S-mode” system.

**************************************

Vessel Traffic Service/Management E-Navigation User Requirements

[VTS 25 – WP02 Contains useful information about the development of VTM]

A function of Vessel Traffic Service/Management (VTS/M) organisations in the overall e-navigation concept is to backup the integrity of a ship’s own data systems and the offering of advice and support when relevant.

Currently, VTSs are provided world-wide to improve navigational safety, safety of life at sea and protection of the marine environment.  There are two types of VTS: Port and Coastal, and there is a clear distinction between the two.  A Port (or river) VTS is a service provided when entering or leaving ports or harbours or when sailing along rivers or through waters which restrict the manoeuvring of ships.  A Coastal VTS is mainly concerned with vessel traffic passing through a sea area.  There are IMO Guidelines for the operation of VTS (Resolution A.857 (20)), which offers guidance on design and operation of VTS systems.

All the foregoing shorebased VTS functions are capable of being executed and communicated by electronic means.  A standardised VTS, if not an “S-mode” VTS, may be defined under the e-navigation concept.

Key Vessel Traffic Service/Management Functions

The primary user requirements are to satisfy the key service/management functions support safe navigation of ships.

These functions are:

1. Detect, identify and track targets

2. Communications

3. Risk assessment and management

4. Traffic management

5. Receipt and provision of Maritime Safety Information

6. Information management including data recording

1.  Detect, identify and track targets

- A positive means of detection, identification and tracking of all targets of interest (e.g. from multiple sources)

- A display of an overall image of the area of responsibility, including the traffic image and all relevant regional information (i.e. the Recognised Maritime Picture (RMP) which would normally include the information contained within official nautical charts and other relevant static and dynamic information layers)

2.  Communications

- Sufficient standardised and harmonised means to communicate all needed digital and analogue information between all relevant participants (e.g. ship/ship, ship/shore and shore/shore)

- A positive and reliable means for a VTS/M operator to initiate a process to alert a mariner of potential or developing risks (e.g. collision, grounding), through the provision of advice, the issuing of warnings and the activation of an onboard alert/alarm

- A standardised format be used for the transmission and sharing of VTS/M information

3.  Risk assessment and management

- Capability to feed and monitor internationally recognised risk factor models/tools

- Effective planning, modelling and simulation tools for traffic management and risk analysis

- A capability to automatically determine if a potential incident is developing and a means to alert the operator

4.  Traffic management

- A capability to assimilate, coordinate and prioritise all necessary information so as to effectively manage vessel traffic within the area of responsibility

5.  Receipt and provision of Maritime Safety Information (MSI)

- A capability to receive, formulate and disseminate MSI to vessels within the area of responsibility

6.  Information management including data recording

- A capability for information recording and archiving VTS/M information in accordance with local information archiving requirements, [consideration needs to be give to legal aspects, such as the meeting of any legal or evidentiary requirements]

- A capability to retrieve and replay VTS/M information

A.916(22) IMO Guidelines for the recording of events relating 	to navigation - Contained in Annex 22 - Recording of Navigational Events