- IMU Technologies for Marine Navigation

- Background

Almost all maritime vessels are now using Global Navigation Satellite Systems (GNSS) receiver equipment to determine the ship’s position. In addition to providing the primary navigation data, the GNSS on a modern bridge supplies also position inputs to other systems, including AIS, ECDIS and GMDSS. Loss of GNSS position and time would render AIS and ECDIS unusable.

Therefore it could be essential to use a backup system which is able to continue to output an electronic position with a similar level of accuracy for a specified time period.

Inertial navigation systems are not widely used on maritime vessels. This has been because of their historically high cost and/or insufficient performance.

The following paper will provide a short description of different Inertial Measurement Unit (IMU) technologies their pros and cons and will give an estimation about the expected future performance.

- Comparison of different IMU-Technologies

Inertial navigation is a self-contained navigation technique in which measurements provided by accelerometers and gyroscopes are used to track the position and orientation of an object relative to a known starting point.

IMUs typically contain three orthogonal rate-gyroscopes and three orthogonal accelerometers, measuring angular velocity and linear acceleration respectively.

The performance and costs of the IMU heavily depends on the different technologies which are used for gyros as well as for accelerometers. The most common technologies for gyroscopes and accelerometers as well as their pros and cons are given in Annex 1.

Current Performance of IMU

Once the sensors have been combined to make an IMU, software has to be added to turn the raw rate and the raw acceleration data into a Inertial Navigation System (INS). The performance of an INS is usually rated in terms of its position error growth rate once the INS is navigating in “free inertial” mode (no aiding). A typical classification is given in table 1.

| INS Classification | Position Error Growth | Rate Heading Errors |
| --- | --- | --- |
| Low | > 2nm/hr | >0.2° |
| Medium | 0.5 to 2nm/hr | 0.05 ° to 0.2 ° |
| Precision | <0.5nm/hr | <0.05 ° |

Table 1: INS Classification, [1]

Furthermore it is interesting to compare the performance of a typical high grade IMU (tactical or navigational ) with a typical MEMS based IMU:

| Sensor type | Position drift | Price level |
| --- | --- | --- |
| High grade IMU | ≈100 m < 15 min ≈ 10 m < 5 min | 50-100 T$ |
| MEMS | ≈ 100-200 m < 60 sec | ≈ 5 T$ |

Table 2: Comparison of High grade IMUs against MEMS based IMUs, [1], [2]

It should be noted that the position drift is mainly dominated by the lowest performance gyro installed in the IMU. If the IMU is aided with speed information the roll and pitch gyro drift can be compensated in the measurement to improve performance.

Conclusions

- At the current stage it cannot be expected that even high grade IMUs can be considered as a primary backup to GNSS positioning sensors.

- Current high grade IMUs have the capability to bridge for GNSS outages in order of 
5-15 min, [2].

- MEMS is an enabling technology but MEMS based IMUs can at present not be used as a stand alone navigation systems because of the high bias and drift errors.

- It is necessary to integrate IMUs with other sensors like speed log, magnetic sensors or with GNSS to compensate the biases and drift errors.

- MEMS could  beneficially be used to support an MEMS based INS in a deep or tight coupling method to enhance signal stability and integrity.

M. Hoppe, December 2007

[1] Oliver J. Woodman, An introduction to inertial navigation, August 2007

[2] Evaluation of integrated GNSS and INS for marine navigatuion, e-Nav 2, 2007

[3] Integrated Inertial Positioning Systems, Zupt,LLC, http://www.zupt.com

- ANNEX 1:  Comparison of different sensor technologies

- The most common technologies for gyros are given in table 1.

| Gyroscopes (Angular rate measurements ) |  |  |  |
| --- | --- | --- | --- |
| Technology | Description | Pros | Cons |
| Mechanical | classical gyro, that has a steadily spinning mass with a free moving axis (gimbal) measuring the angular momentum. Dry-tuned gyros (DTG) or dynamically tuned gyros are spinning mass gyros that are designed to cause very small mechanical constraints. | - Wide performance range 0,0001°/h – 100°/h - Very Low noise | - High costs - Moving parts cause friction - Long warm up time - Fragile - Regular maintenance needed - “g”- dependent drift |
| Optical | Optical gyros permit the reflection of a laser ray many times within an enclosure. If the enclosure rotates, the duration between the moment of laser emittance and eventual reception is different. With ring laser gyros (RLG), the laser reflection is achieved with mirrors inside the enclosure. With fiber optic gyros (FOG), the laser reflection is achieved with a coil of optical fiber. | - Fast reaction - Fast turn on (<1s) - No moving parts - Very rugged - No “g”- dependent drift - Good performance (bias stability ≈ 0.004°/h) | - High costs - Performance increases with size - RLG is a high voltage device - FOG is very temperature sensitive - The laser emitter deteriorates over time |
| MEMS (Vibrating) | Vibrating gyros use micro-electro-mechanical system (MEMS) technology and a vibrating, quartz tuning-fork to measure Coriolis force. When rotated, a vibrating element (vibrating resonator) is subjected to the Coriolis Effect, causing secondary vibration orthogonal to the original vibrating direction. By sensing the secondary vibration, the gyro can detect the rate of turn. | - Very small - Very low costs - No moving parts | - Limited performance (bias stability ≈ 10-70°/h) - Only for short time applications |

- Table 1: Overview about gyroscope technologies [1], [3]

- The most common technologies for accelerometers are given in table 2.

| Accelerometers (linear acceleration ) |  |  |  |
| --- | --- | --- | --- |
| Technology | Description | Pros | Cons |
| Mechanical Pendulous/Translational or mass displacement | A mechanical accelerometer consists of a mass, e.g. suspended by springs. The displacement of the mass is measured using a displacement pick-off, giving a signal that is proportional to the force acting on the mass in the direction of the input axis. | - Wide performance range - Low bias error - Well understood error model | - Analog output - Self heating - High power consumption - Costs |
| Resonant Element Frequency | Resonant Element accelerometers consists of a cantilever beam which is resonated at a particular frequency. A mass is attached to one end of the beam which is free to move. The other end is rigidly attached to the case. When an acceleration is applied along the input axis the beam bends. This causes the frequency to change proportionally to the applied strain. By measuring this change in frequency the acceleration can be determined. | - Digital Output - Low power | - Costs - Not good in high shock environment - Detailed calibration required |
| MEMS | MEMS accelerometers can use the same principles as mechanical and resonant element sensors. | - Very small - Very low costs - No moving parts | - they are currently not as accurate as accelerometers manufactured using traditional techniques due to noise and uncorrected bias errors |

- Table 2: Overview about accelerometer technologies, [1]