Annex III

Issues to consider for an e-Navigation strategy

Issues for the IMO e-Navigation Strategy

IALA eNav WG1 Intersessional meeting 14 -18 Jan 08

WORKING PAPER

# Introduction

The IMO Safety of Navigation sub-Committee at its meeting in July 2007 (NAV 53) noted that an e-Navigation Strategy must be finalised at NAV 54 in June 2008 for approval by the Maritime Safety Committee. It was considered that the following should be taken into account: global coverage of ENCs; training, competency and common language skills for all involved in ship operations, both at sea and ashore;

safety and environmental concerns relating to migration from physical to virtual aids to navigation; workload and motivation of watch-keepers; users’ requirements.

User Needs Capture & Analysis

The IMO Safety of Navigation sub-Committee (NAV 53) clearly indicated that the development of e-Navigation be ‘User Need Led’ and that these user needs form the basis for any development strategy. In support of this the IALA e-Navigation Committee (IALA eNav3 – Sept 07) identified a wide range of e-Navigation users and created a user needs capture methodology which was submitted and accepted by the IMO e-Nav CG. The methodology focused on capturing the specific needs in terms of harmonised capture, integration, exchange, presentation and analysis of maritime information.

Feedback on the user needs was received by the IMO CG and IALA from many administrations and maritime organizations. The initial feedback was analysed at the intersessional meeting of the IALA e-Nav WG1 (Strategy & Operations) which met from 14-18 January 2008 including representatives from the IMO, IMO e-Nav CG, IALA and IHO.

The working group recognised that further feedback was needed from other user groups and that an IMO strategy will have to address the development and implementation of e-Navigation in order to meet these stated needs.

The WG decided that the preliminary analysis of the user needs feedback were enough to identify the core high level user needs for an IMO e-Navigation strategy.

User Needs

Based on an assessment of international feedback on e-Navigation using an agreed methodology for capture, the following are deemed to be the core user needs:

Common Maritime Information Data Structure: Mariners expressed a clear need for all maritime information pertaining to the planning and execution of voyages, the assessment of risk, and compliance with regulation to be accessible from a single integrated system. Furthermore, shore users expressed a clear need for all information pertaining to their maritime domain, vessels and their voyages, to be contained in an internationally agreed common maritime information data structure. Such a data structure was also seen as an essential requirement for the sharing of information between shore authorities on an international basis.

Automated and Standardised Reporting Functions: A clear need was expressed for e-Navigation to provide more automated and standardised reporting functions. There is a clear need for increased communication of ship and voyage information to be transmitted ashore, and for safety related information to be sent from shore to all maritime users. These reporting requirements have proved to be highly distracting to all users, and should be automated or pre-prepared as much as possible both in terms of content and communications technology. The group recognised that security, legal and commercial issues will have to be addressed for this need.

Human Centred Presentation Needs: All users expressed a clear need for navigation displays to be designed to clearly indicate risk and to optimise support for decision making. Strong support was given to an integrated ‘alert management system’ such as contained in the present IMO Integrated Navigation System (INS) performance standards (alerts being structured as alarms, warnings and cautions). Further consideration should be given to the use of a decision support system that offers suggested reactions to certain alerts, and the integration of navigation alerts onboard ships within a whole ship alert management system.

There is a need by mariners for standardised presentation and operation functionality to enhance the effectiveness of internationally standardised training, certification and familiarisation. The concept of S-Mode was supported for the ship.

Shore users expressed a clear need for their displays to be fully flexible supporting both a Common Operating Picture (COP) and a User Defined Operating Picture (UDOP). Strong support was given for the use of Geographical Information Systems (GIS) and for layered and/or tabulated displays.

Effective and Robust Ship and Shore communications: A clear need was expressed for there to be an effective and robust means of communications for ship and shore users. Relevant shore authorities need an effective means of communicating with vessels to provide safety and operational information with an automated communications selection system. Communication with vessels and between vessels should be effective, make best use of audio/visual aids and standard phrases, minimise linguistic challenges and distraction to operations.

Human Machine Interface: e-Navigation systems must be designed to engage and motivate the user while managing workload. It was clear that as electronic systems take on a greater role, facilities need to be developed for the capture and presentation of non-electronic data such as visual observations and user knowledge and experience. The presentation of information for all users should be designed to reduce ‘single person errors’ and enhance team operations. On board ship this includes the pilot and lookout. There is a clear need for good ergonomics both in the physical layout of equipment and in the use of light, colours, symbology and language issues.

Data and System Integrity: A clear user need was expressed for e-Navigation systems to be resilient and to take into account issues of data validity, plausibility and integrity, and for the systems to be robust, reliable and dependable. Requirements for redundancy, particularly in relation to position fixing systems should be considered.

Analysis: There is a clear need for an effective use of analysis functions within e-Navigation to improve performance and prevent single person error. Shipboard systems should include analysis functions that support the user in complying with regulations; identifying risks, and avoiding collisions and groundings including the calculation of dynamic Under Keel Clearance (UKC) and air drafts. Shore based systems should support environmental impact analysis, operational planning, hazard/risk assessment, reporting indicators and prevention.

Consideration should also be given to the use of analysis for activities including: response recovery, risk assessment and planning, detection and prevention, mitigation, preparedness, resource management and communication.

e-Navigation analysis should be designed to complement users capabilities while compensating for any limitations.

Strategy

e-Navigation is an International Maritime Organization (IMO) led concept based on the harmonisation of marine navigation systems and supporting shore services driven by user needs.

To develop e-Navigation within the IMO and in order to meet the stated user needs the work programme will have to be comprised of a number of work streams.

Proposed work streams for e-Navigation will be Strategy and Implementation, Architecture, Human Element, Institution and Regulation, Position Fixing, Communication and Information Systems, ENCs, Equipment and Standardisation, and Scalability.

Strategy and Implementation: The strategy and implementation stream will oversee the total project including concept development, capture and analysis of user needs, work program allocation, work program coordination, implementation and review. Public relations and promotion for the e-Navigation concept will be an essential task, as will be the development of methodologies for continual capture of e-Navigation needs. In the future, maritime incidents and the development of new technologies should be subjected to a methodological assessment as to how they can be addressed within an e-Navigation concept. Interchange with relevant maritime projects should be maintained throughout the e-Navigation process (e.g. MarNIS, MEH) in order to benefit from synergy.

Architecture: The overall technical, structural and conceptual architecture will need to be developed and maintained particularly in terms of the needs for data structures, information systems, communications technology and regulations.

Human Element: Training, competency, language skills, workload and motivation are identified as essential. Alarm management, information overload and ergonomics have also been prominent concerns. A group will be needed to take responsibility for these aspects of e-Navigation in coordination with existing IMO Human Element work.

Institution and Regulation: The development and implementation of e-Navigation will inevitably build upon the existing work of the IMO and thus coordination within the contexts of the carriage requirement for SOLAS navigation equipment; the provisions of SOLAS Chapter V Reg 15, standards for ECDIS and INS; Human Element Issues of HMI, Ergonomics, and the implementation of new technology (MSC Circ. 1091), symbols (MSC 192(79)) ; and the STCW Convention. The provision and development of e-Navigation will also have to be managed in accordance with international regulations, national legislation and standards.

Position Fixing: The development and provision of position fixing systems to meet the quality standards of e-Navigation will be a major work stream.

Communications and Information Systems: Communications and information systems will be a major element of e-Navigation and will have to be developed and coordinated internationally and within technical standards for data structure, technology, bandwidth and frequency allocations.

ENCs: At NAV 53 “it was the opinion of the IHO that there would be adequate coverage of consistent ENCs by the time any further mandatory carriage requirements were likely to be adopted by IMO. The Sub-Committee was also of the opinion that the availability of ENCs worldwide was most important and requested IHO and Member Governments to continue their efforts in increasing the coverage. e-Navigation should take into account the added functionality within the developing IHO standard S100.

Equipment and Standardisation: this part of the work will follow the development of performance standards and will involve manufacturers’ groups.

Scalability: IMO Member States have a responsibility for the safety of all classes of vessels and scalability has been recognised as a key requirement for e-Navigation. Extension of the concept to non-SOLAS vessels should be seen as an important task, to be addressed, in the first instance through consultation on user requirements.