# SOME QUESTIONS ON e-LORAN

In the following, a number of questions regarding the eLoran system (or any other suggested backup system), have been put forward. They have to be resolved by the IALA e-Navigation Committee prior the committee suggesting (to IMO) that a system can be considered as a back up to GNSS.

- What is the accuracy of the planned backup system in archipelagic conditions when accounting for a particularly bad earth conductivity as well as seasonal variations in the conductivity? What is the accuracy in waters covered by thick ice (>2 metres)?

A1. It can be assumed that to provide an acceptable alternative to GNSS the system will need to provide comparable performance (10m 95%). The high accuracy of eLoran is achieved by first surveying the spatial variations, due to ground conductivity, over the area within which the IMO standard for “harbour entrances, harbour approaches and coastal waters with a high volume of traffic and/or a significant degree of risk” is required. Then temporal variations are removed by use of a local reference station. There are now records covering several years from such reference stations in the US in areas with strong seasonal variations. These records currently suggest that reference stations cope with the conditions cited. European trials are being planned to include difficult areas, such as archipelagos.

- Could the planned GNSS backup system be made available globally, or cover at least the most of the important shipping lanes of the world, including the southern hemisphere?

Reason: If the eLoran is to be a backup system for Global Navigation Satellite Systems, only a global backup system will considered acceptable.

A2. eLoran already has the potential to cover all major northern hemisphere shipping lanes. Economic considerations may determine the extension of coverage to the southern hemisphere. Few electronic navigation systems, with the recent exception of GNSS, have global coverage. IALA radiobeacon DGPS, for instance, falls well short thereof, but is nonetheless of great value. Currently, there are some 70 Loran stations worldwide that could be upgraded to eLoran standard. They are concentrated where maritime activity is most intense: across the USA and Canada; western Europe; western and eastern Russia; China, Japan and South Korea; India; and Saudi Arabia. They cover a much higher proportion of coastal and harbour navigation than their geographical coverage would suggest.

- If eLoran is to be chosen what messaging system shall be chosen -  the US one or the European one, as regards to transmitting additional information to the user.

A3. eLoran standardisation is being discussed in various fora, notably RTCM. A single world standard may well emerge, but modern receivers have no difficulty interpreting a variety of protocols. It will be quite straightforward to specify receivers capable of handling not only the US 9th pulse standard and Eurofix, but also the Russian SNS.

4.  What can be done to resolve the obvious difference in opinion between Russia and other countries regarding the status and use of eLoran?

Reason: It seems obvious that Russia considers the Chayka system to be a military system. In a military system the emphasis is always on security, not on safety and availability.

A4. Both the existing GNSS (GPS & GLONASS) are dual-purpose, military/civil systems. Recognition of systems through the IMO WWRNS provides assurances on availability and information to users.

5. How can the setting up a global or even a regional eLoran system be fulfilled in practical terms?

Reason: Even if it is anticipated that a proliferation of eLoran will occur once the system is set up somewhere in the world, the cost of even one single station is in the order of 10 mill. €. A large sum sum for almost any single country. As the eLoran coverage in most cases exceeds 1500 km, the coverage probably covers several countries, which obviously should share the cost. Experience shows, however, that there are several severe obstacles. Financial co-operation between adjoining countries is by no means obvious. Also, building extremely large antennas in urban or semi-urban areas has become very difficult and sometimes impossible.

A5. As discussed at the ad hoc meeting, there are solutions available, in particular use of commercial service providers, public-private partnerships and existing regional agreements (e.g. FERNS). The costs of eLoran are relatively small compared with GNSS, but external funding could be explored (e.g. World Bank, cf MEH). All new and modernised stations are likely to operate unattended, greatly reducing costs.

6. Should other financing and multimodal co-operation options for a backup system be looked into?

Reason: It has become obvious that only a small part of future users are in the maritime community (at present almost none). It seems unjust that the cost is carried by maritime users only. It seems that there are a lot more potential users in the road traffic, air traffic, rail traffic and timing communities. To get all these users together to pay their share of the cost is a real, but perhaps impossible challenge.

A6. As in the answer to 5, World Bank funding may be a solution, but the first step would be recognition under regional (e.g. ERNP) and international agreements.  Whilst cost-sharing with other communities is desirable, the potential costs to the maritime community of not implementing e-Navigation and not having a backup to GNSS should be balanced against the cost of eLoran.

7. Should not other probable options for a backup system to GNSS be looked into?

Reason: Opinion that eLoran is the only viable candidate is the only opinions expressed so far. This is based on the fact that infrastructure partly exists, in patches here and there, and that too only in the northern hemisphere. 
No attention has been given the fact that some potential infrastructure already exists, which could accommodate a backup system, providing ranging and messaging facilities - namely the IALA DGPS system and the AIS base station network. Some of the present DGPS stations are even re-using former DECCA infrastructure, providing for a better than average coverage. A mini-Loran system has been discussed but also a completely new system could be designed, that could provide for suitable ranging signals, simultaneously with sending corrections from the different navigation satellite systems.

A7. Yes, this task has been assigned to IALA e-NAV WG2. However, it should be recognised that eLoran is a system which can be introduced relatively quickly – technical and regulatory problems have already been solved. The DGPS and AIS options should be investigated, but neither provides a standalone position-fixing system in its present form.

It should also be borne in mind that comparison of candidate backups to GNSS has been the subject of considerable study in recent years. eLoran was developed as a result of such a study undertaken by the US and led by the Federal Aviation Administration specifically to meet aviation, maritime and timing specifications. Subsequently, a separate study of backup systems for telecommunications timing by the US Naval Observatory (USNO) and the National Institute of Standards and Technology (NIST) came out firmly in favour of eLoran as the outstanding solution. In the recent aviation backup study by the US NGATS Institute, eLoran came out best, and by a significant margin for General Aviation. Above all, the US Government’s high-level Independent Assessment Team, which conducted a study of all known studies on the topic is understood to have favoured eLoran unanimously.

******************