| Source: Document 5B/18 | INTERNATIONAL TELECOMMUNICATION UNION | e-NAV4 / 10 / 06 |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 5B/TEMP/26(Rev.1)-E |
|  |  | 13 February 2008 |
|  |  | English only |

Working Party 5B

LIAISON STATEMENTS TO IMO, IALA and CIRM Preliminary draft new report – improved satellite detection of ais

Subject:			Satellite Detection of AIS
Status:			For information
Contact:		Jon Turban
E-mail:			Jon.V.Turban@USCG.mil

Satellite detection of AIS messages has been studied and documented in Report ITU-R M.2084 (2006) “Satellite detection of automatic identification system messages”. The Report considered the operational and technical characteristics of the shipborne AIS and the requirements and limitations of reception of AIS messages from satellites. Further studies were proposed to deal with the technical limitations that hinder detection of AIS-equipped target ships in high-traffic areas on the designated AIS VDL (VHF Data Link). The technical limitations specifically cited were:

1)	the length of the AIS message in the time slot (insufficient time buffer for the satellite detection range);

2)	the large number of messages in the satellite antenna footprint (excess re-use of the time slots in the VDL as detected by the satellite);

3)	the sharing of AIS frequencies by terrestrial services within the satellite antenna footprint (coverage pattern).

It should also be noted that WRC-07 added a secondary mobile-satellite service allocation on AIS-1 and AIS-2 frequencies to allow satellite reception of AIS on a secondary basis.

This preliminary draft new Report considers:

1)	a new Message 27 (for satellite detection of AIS) to Recommendation ITU-R M.1371-3 that fits the timing requirements of the AIS and the satellite reception; and

2)	separate frequencies (to be shared) for this service that fit within the tuning range of the shipborne AIS, are dedicated exclusively to VHF maritime mobile services, pose minimal conflicts with other current maritime uses, and have minimal impact on Appendix 18;

3)	that this service should be limited to AIS Class A only and not to AIS Class B in order to appropriately limit the number of AIS messages in the satellite antenna footprint.

At the request of IMO, ITU-R WP 5B would initiate steps to implement the “improved satellite detection” prescribed in this preliminary draft new Report through or by:

1)	recommending revision of footnote n) to Appendix 18 (to allow sharing of frequencies) at the next WRC;

2)	revision of Recommendation ITU-R M.1371-3 (to add the new message) is required.

To effect the operation of this improved satellite detection, IMO would need to specify modifications to the shipborne AIS Class A equipment.

_____________

Attachment: Document 5B/TEMP/27 [not attached]