| Source: Document 5B/18 | INTERNATIONAL TELECOMMUNICATION UNION | e-NAV4/ 10 / 07 |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 5B/TEMP/27(Rev.1)-E |
|  |  | 13 February 2008 |
|  |  | English only |

Working Party 5B

Preliminary Draft NEW itu-r REPORT

Improved satellite detection of AIS

# 1	Introduction

Satellite detection of AIS messages has been requested by administrations. In response, studies were made that were documented in Report ITU-R M.2084 (2006) “Satellite detection of automatic identification system messages.” The Report considered the operational and technical characteristics of the shipborne AIS and the requirements and limitations of reception of AIS messages from satellites. Further studies were proposed to deal with the technical limitations that hinder detection of AIS-equipped target ships in high-traffic areas (e.g. the Dover Straits, Singapore, North East United States, and the northern Gulf of Mexico) on the designated AIS VDL (VHF Data Link). The technical limitations specifically cited were:

1)	the length of the AIS message in the time slot (insufficient time buffer for the satellite detection range);

2)	the large number of messages in the satellite antenna footprint (excess re-use of the time slots in the VDL as detected by the satellite);

3)	the sharing of AIS frequencies by terrestrial services within the satellite antenna footprint (coverage pattern).

This Report on further studies shows the need for a special AIS message that is shortened in length and can be transmitted on a special reporting schedule on two other frequencies when those frequencies are designated. Accordingly, a new Recommendation ITU-R M.1371 AIS message (Message 27) and new Appendix 18 frequencies for this service are proposed.

# 2	Solving the problem of overlapping messages (blurred reception)

The differences in propagation delay between vessels and the satellite causes the AIS messages to overlap. Thus, the slot composition must be adjusted to increase the time buffer so that the AIS transmissions can be received by the satellite in separate slots without a time overlap. Table 1 serves as the basis for the proposed new Message 27 slot composition shown in Table 5 based on an allowance for the “propagation time delay difference (bits).” Tables 2-4 show the technical parameters, reporting intervals and data packet structure of the AIS messages that are currently approved.

TABLE 1

Satellite AIS reception technical parameters

| AIS satellite propagation calculations |  |  |  |
| --- | --- | --- | --- |
| Constants: |  |  |  |
| Speed of light (metres/sec) | 299792458 |  |  |
| AIS bit-time @ 9600 bps (milliseconds) | 0.1041667 |  |  |
| Nautical mile (km per nm) | 1.852 |  |  |
| Calculations: | Orbit #1 | Orbit #2 | Orbit #3 |
| Satellite orbital altitude (km/nm) | 600/324 | 948/512 | 1000/540 |
| Slant range to horizon (km/nm) | 2831/1529 | 3604/1946 | 3709/2003 |
| Ground range to horizon (km/nm) | 2664/1438 | 3281/1772 | 3359/1814 |
| Difference in propagation distance (km/nm) | 2231/1205 | 2656/1434 | 2709/1463 |
| Propagation time delay difference (milliseconds) | 7.44 | 8.86 | 9.04 |
| Propagation time delay difference (bits) | 71 | 85 | 87 |
| Typical satellite visibility (minutes) | 10.2 | 13.6 | 14.0 |

TABLE 2

Overview of shipboard AIS technical parameters

| AIS parameters | Values |
| --- | --- |
| Frequencies | 161.975 and 162.025 MHz |
| Channel bandwidth | 25 kHz |
| Platforms | Class A ships, Class B ships, coast stations, navigation aids |
| Power | 12.5 W (Class A); 2 W (Class B) |
| Antenna type(1) | 1/2 λ dipole |
| Antenna gain(1) | 2 dBi with cosine-squared vertical elevation pattern; Minimum gain = –10 dBi |
| Cable loss(1) | 3 dB (estimated) |
| Receiver sensitivity | –107 dBm for 20% packet error rate (PER) (minimum) –109 dBm for 20% PER (typical) |
| Modulation | 9 600 bits per second GMSK |
| Multiple access mode | TDMA (self-organizing, random, fixed and incremental) |
| TDMA frame length | 1 min; 2 250 time-slots |
| TDMA slot length | 26.7 ms; 256 bits (see Table 4) |
| Message types | 26 types |
| Message length | 1 to 5 slots with 1 slot being the dominant type |
| Periodic message interval | 2 s to 6 min transmit intervals (see Table 3) |
| Required D/U protection ratio | 10 dB at PER = 20%(2) |
| (1) Typical parameters not defined in Recommendation ITU-R M.1371. (2) Parameter specified in IEC 61993-2. |  |

TABLE 3

Reporting intervals for AIS messages

| AIS platform | Reporting interval |
| --- | --- |
| Dynamic information: |  |
| Coast station | 3 1/3 to 10 s interval (10 s nominal) |
| Class A ship | 2 s to 3 min interval (approximately 7 s average) (see Table 4) |
| Class B ship | 5 s to 3 min interval (30 s nominal) |
| Search and rescue aircraft | 10 s interval |
| Aid to navigation | 3 min interval |
| Static and voyage information | 6 min interval |
| Safety and administrative messages | As required |
| Data message | As required |

TABLE 4

Default AIS data packet bit structure

| Power ramp up | 8 bits |  |
| --- | --- | --- |
| Training sequence | 24 bits | Necessary for synchronization |
| Start flag | 8 bits |  |
| Data field | 168 bits | Default length |
| Cyclic redundancy code | 16 bits | Necessary for error detection |
| End flag | 8 bits |  |
| Buffer | 24 bits (typically, the last 20-bit positions are empty) | Necessary to accommodate bit stuffing, propagation and repeater delays, and jitter |
| Total | 256 bits |  |

TABLE 5

Modified AIS packet bit structure for satellite reception

| Slot composition | Bits | Notes |
| --- | --- | --- |
| Ramp up | 8 | Standard |
| Training sequence | 24 | Standard |
| Start flag | 8 | Standard |
| Data field | 96 | Data field is 168 bits for other single-slot messages. This field is shortened by 72 bits to support the satellite AIS system buffer. |
| CRC | 16 | Standard |
| End flag | 8 | Standard |
| Satellite AIS system buffer | 96 | Bit stuffing = 4 bits Synch jitter (mobile station) = 3 bits Synch jitter (mobile/satellite) = 1 bit Propagation time delay difference = 87 bits Spare = 1 bit |
| Total | 256 | Standard (NOTE – Only 160 bits are used in the 17 millisecond transmission) |

Thus the proposed data field for Message 27 is shown in Table 6.

TABLE 6

Proposed new data field for AIS satellite detection Message 27

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message (similar to all other messages) |
| Repeat indicator | 2 | Repeat indicator value should be 3 |
| User ID | 30 | MMSI number |
| Position accuracy | 1 | As defined for Message 1 |
| RAIM Flag | 1 | As defined for Message 1 |
| Navigational status | 4 | As defined for Message 1 |
| Longitude | 18 | Longitude in 1/10 min (180º, East = positive, West = negative |
| Latitude | 17 | Latitude in 1/10 min (90º, North = positive, South = negative |
| SOG | 6 | Knots (0-62); 63 = not available = default |
| COG | 9 | Degrees (0-359); 511 = not available = default |
| Status of current GNSS position | 1 | 0 = Position is the current GNSS position; 1 = Reported position is not the current GNSS position = default |
| Spare | 1 | Set to zero, to preserve byte boundaries |
| Total number of bits | 96 |  |

# 3	Solving the problem of the large number of messages in the satellite antenna footprint

The large number of messages in the satellite antenna footprint (excess re-use of the time slots in the VDL as detected by the satellite) is attributable to both the large number of ships and the reporting rate. Studies show that 100% of AIS Class A ships can be detected if:

1)	an appropriate reporting rate for the AIS Class A ships is selected; and

2)	the AIS Class B is eliminated from satellite reception.

## 3.1	Selecting a reporting interval for satellite reception of the AIS Class A

The proposed reporting interval for the proposed new Message 27 is 3 minutes in order to give the best detection probability at a typical observation time of 13.6 minutes as shown in Fig. 1. The reports from administrations on simulation results (for AIS Class A) also support this proposed reporting interval. Note that the use of two frequencies (refer to § 4 for the proposed use of the two channels 75 and 76) will double this capacity and also provide added protection from interference due to possible problems on a single channel.

FIGURE 1

Optimizing the reporting rate for AIS Class A ships

Assumptions:

1)	Uniform ship distribution

2)	Target ship located at 40 deg. latitude

3)	160 bit packet length (256 bits normal packet – 96 bits buffer for Message 27)

4)	Polar orbiting satellite with 13.6 minutes average satellite visibility period.

Conclusions:

1)	A 3-minute reporting interval can achieve 100% detection of AIS Class A ships

2)	AIS Class A already has an available 3-minute schedule (refer to Table 3).

## 3.2	Eliminating the AIS Class B from the satellite reception

Reports from administrations on simulation results indicate that both AIS Class A transmissions at 12.5 Watts and Class B “CS” transmissions at 2 Watts have sufficient signal margins to support satellite AIS reception. However, the satellite footprint is too wide to accommodate the expected combined population of both classes of AIS. Furthermore, some administrations have stated the need, based on simulation results, to request that special frequencies be designated for transmitting the proposed new Message 27. Therefore, this proposal is to add the new proposed Message 27 to the AIS Class A only with a message priority level 4 and on the other frequencies. In that case, the AIS Class A equipment would not be required to receive on those frequencies and the reporting interval could be implemented with RATDMA based solely on the activity of AIS 1 and AIS 2 without the need to monitor the satellite-designated channel to further qualify candidate time slots for transmission.

# 4	Operating frequencies for satellite detection of AIS

Separate frequencies for satellite detection of AIS should be selected from within Appendix 18 because the tuning range of the shipborne AIS Class A is limited to these frequencies. Also, with respect to possible additional AIS frequencies, Report ITU-R M.2084 indicated that the interference environment resulting from the existing services in those bands must be taken into account in determining the feasibility of accommodating satellite AIS in any given band or channel, due to the large satellite antenna footprint that overlaps both land and sea. Examination of Appendix 18 indicates that only 3 frequencies (channels 16, 75 and 76) are exclusively dedicated to maritime use and restricted from terrestrial use on a global basis. Footnote n), referring to channels 75 and 76, currently limits power to 1 Watt for radio transmissions because they are adjacent channels to channel 16 (the distress and calling channel). However, transmission of Message 27 at 12.5 Watts only once every 3 minutes for 17 milliseconds, alternating between channels 75 and 76, would not interfere with voice communications on any of these channels 16, 75 and 76, and it would be detectable over a 1 Watt radio transmission. It should be noted that at least one administration uses channels 75 and 76 for low-power (1 Watt) maritime communications in-port.

## 4.1	Mathematical analysis of impact to Channels 16, 75 and 76

Studies (the JSC report) have shown that AIS transmissions do not significantly degrade voice communications on the adjacent channels due to the shortness of the duration of one AIS time slot compared to the time intervals used by human speech. The JSC report used the metric of “voice articulation score” (VAS) to make this assessment, and it concluded that the VAS on the channel adjacent to the AIS was 90-95% for the normal reporting interval of the shipborne AIS. In this particular case, the proposed new satellite AIS Message 27 is significantly shorter (only 17 milliseconds) than one AIS time slot (27 milliseconds), and the interval of the proposed transmission is significantly less (only once every 3 minutes) than the AIS reporting interval (2-10 seconds). It is unlikely that this short burst will open the squelch of shipborne VHF radios. The burst may possibly be detected during the reception of a voice call from a distant station, but the duration of the proposed burst will not spoil the detect-ability of human speech, based on the VAS metrics used in the JSC report.

For SOLAS ships underway (not in port) it is also important to consider the communications quality based in the possible deterioration of the SINAD (signal, interference, noise and distortion).

The duty cycle (D) of the proposed Message 27 is: D = B/T, where B = burst length (0.017 seconds) and T = time interval (180 seconds).

Thus, D = 0.017/180 = 0.000094.

This equates to a SINAD value for radiocommunications of:

SINAD = 10 log (1/(DxN), where D = duty cycle of the Message 27, and N = the number of ships that are significantly closer than the two ships carrying on the voice communications.

Thus, SINAD = 40.2 dB for N = 1, and SINAD = 20.08 dB for N = 104.

The SINAD value used by IEC for receiver sensitivity testing is 20 dB. For SOLAS ships underway (not in port), it is not feasible to have enough ships (more than N = 104) in the near vicinity (within the ship-ship radio transmission range, approximately 20 nm) to cause the SINAD for a desired voice signal to be reduced below 20 dB.

# 5	Summary

This Report addresses the technical limitations cited in Report ITU-R M.2084 that invited further studies. The possible solutions  reached herein are:

1)	A special short AIS message (proposed Message 27, of only 96 bits) that is tailored for satellite reception would solve the problem of blurred reception.

2)	A special reporting interval (proposed 3 minutes) is needed for the satellite AIS message.

3)	Separate operating frequencies in addition to AIS 1 and AIS 2 are needed that are not subject to terrestrial use.

4)	Frequencies should be taken only from Appendix 18 due to the limited tuning range of the shipborne AIS.

5)	Appendix 18 contains only 3 frequencies (channels 16, 75 and 76) that are exclusively dedicated to maritime use (channels 75 and 76 should be designated to be shared with this service). Accordingly, footnote n) to Appendix 18 should be modified to facilitate this sharing arrangement.

6)	Satellite detection of the shipborne AIS should be limited to the AIS Class A (SOLAS Class) because the AIS Class B population is too large to be included.

7)	Recommendation ITU-R M.1371-3 should be revised to add the proposed Message 27 along with its transmissions on the designated channels 75 and 76, and the AIS Class A equipment should be updated to add this message to facilitate improved satellite AIS detection.

Additional studies on AIS satellite detection are encouraged by administrations.

________________

These satellite visibility periods are for a typical overhead pass of the satellite based on a circular, polar-orbiting satellite for the indicated satellite altitudes and a target ship located at 40º Latitude. Typical visibility periods will vary depending on a number of factors including satellite inclination angle and target ship latitude.

JSC Report (JSC-PR-04-007) “EMC Analysis of Universal Automatic Identification and Public Correspondence Systems in the Maritime VHF Band” primarily addresses the effects of AIS on the VPC service, but the information is useful in assessing this type of co-channel and adjacent channel interference. This document is freely available on the United States Coast Guard Navcen website.