Discussion note between AIS TWG and the Working Group 2

Re: Broadcasting DGNSS corrections using AIS

# Introduction

The ITU Recommendation ITU-R M.1371 allows for a message that broadcasts DGNSS corrections over AIS. As a part of the new revision of IALA Recommendation A-124, it is necessary to recommend best practice with regards to the implementation of such a system.

The recommendation will take guidance from the existing requirements of a DGNSS system, namely those described in IMO Resolution A.953(23) (and summarised in IALA Recommendation R-121. For convenience, these are shown in Table 1.

| Area | Absolute horizontal accuracy (95%) | Availability | Continuity | Time-to-Alarm |
| --- | --- | --- | --- | --- |
| Ocean | ≤ 100 m | >99.8% over 30 days | N/A | N/A |
| Coastal/harbour with low level of risk | ≤ 10 m | >99.5% over 2 years | >99.85% over 3 hours | 10 seconds |
| Coastal/harbour with high level of risk | ≤ 10 m | >99.8% over 2 years | >99.97% over 3 hours | 10 seconds |

Table 1 - Accuracy, availability and continuity requirements for a radionavigation system (e.g. GNSS) in order to be considered to be a part of the Worldwide Radionavigation System by IMO (A953(23), December 2003).

# Issues to be addressed

Suitability of requirements – Are there any reasons why the requirements described above would not apply to an AIS DGNSS service? There is some concern of the lack of an official definition on the availability of an AIS Service. This may be resolved by a paper on this subject submitted to eNav3.

Response:

Yes, the AIS DGNSS system should use the requirements of IMO. The definition of the service availability and continuity may need to be defined by the competent authority until a formal definition of these in an AIS service.

Update interval - One aspect of a DGNSS to be resolved is how often a complete set of corrections needs to be transmitted. One trial in Russia suggested that 30 seconds is the maximum time period between correction sets. The reason for is unclear, however, the IEC standard for GPS receivers recommend a 30 second correction age mask for the DGNSS test. It seems that most manufacturers have used this as the default value for the correction age mask. Is an update rate of 30 seconds sufficient for a DGNSS system, ignoring the time-to-alarm (TTA) requirement?

Response:

It is recommended that a set or subset of the corrections are transmitted approximately every 5 seconds using FATDMA. This will fulfil the TTA requirement, and should reduce the problems of using RATDMA (see below). This clearly has implications on the VDL loading and the FATDMA planning. This is to be discussed further within the AIS TWG.

Time-to-alarm requirement – One suggestion for dealing with the 10-second TTA requirement is to broadcast the alarm using a Message 17 transmitted using RATDMA. This would be in addition to the regular correction set transmitted every 30 seconds or so. Given that it may take 4 seconds for an RATDMA message to be transmitted from the time the message has arrived at the base station, is this an acceptable method of meeting the 10-second TTA requirement?

Response:

WG2 felt that it was necessary to ensure, as much as possible, that the integrity message is transmitted, the RATDMA access scheme should not be used. There is concern with slot clashes and the possibility of the message not being sent with the 4 second RATDMA time period.

Multiple stations – what should happen if the mobile station receives corrections from several reference stations at the same time? Suggest that all corrections are applied since it is unlikely to cause a major problem (errors are within the accuracy requirement). The mobile station may be able to calculate its own corrections, based on multiple correction sets and taking into account the distance between it and the different reference stations.

Response:

The group discussed how might be best to implement a system. It was recommended that A-124 should state that IMO DGNSS requirements should be met. The transmitted Message 17 should be monitored off-air to ensure that the position solution derived from it is correct and within IMO limits.

The issue of multiple stations will be discussed at a later date.

It was noted that AIS DGNSS Service should not be used over a very wide area, due to the potential of high VDL loading. Instead, high-risk areas should be identified, and the AIS concentrated in those areas. The service could be considered as a duplicate service for an existing radiobeacon DGNSS (as required to meet the high-risk IMO requirements).

# Action

AIS TWG to review Annex 16 of A-124, with the above comments taken into consideration.