SPECIFIC GUIDANCE ON RECAPITALISATION OF DGNSS

IALA has recommended that members operating the beacon DGNSS service, which is the international standard for maritime augmentation, should consider modernization to ensure that levels of service can be maintained and future requirements can be met (Ref: R-135). This recommendation took into account the potential alternatives, in particular Satellite Based Augmentation Services (SBAS) and the Automatic Identification System (AIS).

The simplest option may be replacement of existing hardware with similar dedicated Reference Stations and Integrity Monitors (RSIM), but this would limit the potential for development and there are other possibilities, including software RSIM, Virtual Reference Station Networks and integration with SBAS (Ref: GLA Study).

IALA members are currently developing solutions:

- The US has opted to install software RSIM equipment across all USCG and Inland Waterways transmitter sites (Ref: US specification docs.)

- The UK and Ireland will shortly tender for replacement RSIM equipment, which may be fulfilled by either hardware or software solution (Ref: GLA specification docs)

- Germany has developed a concept making use of VRS, based on a separate DGNSS service and transmissions on MF beacon and AIS carriers (Ref: e-NAV4/7/10)

- Poland is recapitalizing RSIM equipment, and plans to transmit VRS data in the future

- Sweden is currently studying options for software solutions using RSIM and options for transmitting data from VRS via the MF beacon system

- Other administrations, including those of Denmark, Canada, Norway, South Africa and Finland are currently studying options for recapitalization

- China is expanding coverage with the construction of three additional stations in 2008

IALA members are encouraged to:

- Recapitalize existing DGNSS systems prior to their obsolescence, noting the advice given in R-135

- Consider the options for replacement systems, with reference to the documents listed below

- Share information regarding studies carried out and specifications of replacement solutions

- Investigate potential future developments compatible with the development of shore based e-Navigation architecture

- Investigate techniques for adding value to transmissions, such as transmission of safety related information or backup positioning capabilities

- Indicate whether they would support an IALA Workshop (or inter-sessional meeting) to exchange information regarding studies and developments