| INTERNATIONAL MARITIME ORGANIZATION | #### IMO | E |
| --- | --- | --- |

Ref.:  T2-OSS/1.4	25 April 2008

LIAISON STATEMENT TO ITU, CIRM, IEC AND IALA

AIS SAFETY RELATED BROADCAST MESSAGES USED FOR DISTRESS PURPOSES

Certain models of class A Automatic Identification Systems (AIS) provided for preconfigured safety related messages, which have been used, without any procedural basis, to convey text messages indicating distress situations.  A similar situation is developing with respect to class B equipment.  Although most class B AIS are not required to provide short range messages, and most do not, those that do must use preconfigured messages.  AIS standards give no guidance as to what predefined messages should be used, in effect leaving it to manufacturers and Administrations to decide.

IMO is of the view that the use of predefined distress text messages by AIS devices should be prohibited and not be considered a part of any distress and safety system at this time.  While the GMDSS is foreseen to be subject to continuing improvements, and perhaps the use of technologies such as AIS may be considered in future, the current infrastructure, together with AIS technological limitations, do not support use of the text messaging facilities as part of the GMDSS.

Shore-based establishments may not be capable of receiving these text-type distress messages and may not take the same responsive action as if an alert had been received over GMDSS equipment.  Reliance on AIS text messaging in distress situations could result in untimely delays and increase the probability of fatalities.

IMO intends to issue a circular to mariners describing the limitations of using AIS predefined distress text messages in distress situations.

____________