| INTERNATIONAL MARITIME ORGANIZATION | #### IMO | E |
| --- | --- | --- |

Ref.:  T2-OSS/1.4	25 April 2008

LIAISON STATEMENT TO ITU, IEC, IALA and CIRM

AIS SEARCH AND RESCUE TRANSMITTER (AIS-SART)

IMO would like to thank ITU-R for the liaison statement (Document 5B/TEMP/25) concerning the proposal for an identifier for AIS-SARTs based on the structure:

917203X4X5Y6Y7Y8Y9

The Sub-Committee on Radiocommunications and Search and Rescue (COMSAR), at its twelfth session (7 to 11 April 2008), noted that this structure does not contain a MID and therefore concurs with the recommendation of the Sub-Committee at its tenth session that there is no need for the identifier transmitted from an AIS-SART to include a country or vessel identifier in order to duplicate the SART function whose transmission similarly has no country nor vessel identifier. The identifier proposed by ITU-R can be permanently encoded into the AIS-SART with no need for change with change of flag of a ship etc. and consequent improvements in the reliability of the device.

The COMSAR Sub-Committee considered whether any additional identification of the AIS-SART would be helpful and concluded that the transmission of a safety related broadcast message (Message 14 of Recommendation ITU-R M.1371) with the text “SART ACTIVE” or “SART TEST” would be helpful on ships which had no graphical indication of AIS targets.

The COMSAR Sub-Committee also considered whether it would be of benefit to transmit a new navigational status in the position report (Message 1 of Recommendation ITU-R M.1371) set to a new code 14 for DISTRESS when the AIS-SART is active and a new code 15 for NOT DEFINED when the AIS-SART is undergoing a self test.  However, given that very few ships will have an AIS installation which is capable of interpreting these new codes and given that the AIS-SART is not a distress alerting device it was concluded that this was not of benefit.

______________