# AIS Sea Trials 9th to 13th June 2008, Oban, Scotland.

- Joint initiative by MCA and the 3 UK GLAs.

- Support from manufacturers through CIRM.

- Two principal objectives;

  - Class B assignment by Message 23.

  - AIS SART optimisation for IEC Performance and Test Standard.

## Class B AIS

- Proved Message 23 assignments worked for the four Class B (two different manufacturers) provided for the trial.

- Having proven the principle in a relatively quiet AIS area the intention is to have further trials in a busy area such as the Solent.

- Trials were completed on Friday 13th June and detailed analysis of the AIS  data is required before a report can be published.

## AIS SART

- Range and coverage trials by deploying in the water sailing away until contact is lost then sailing towards until target is found.

- Simulated a Search and Rescue by deploying the SARTs and then going looking for them.

- Used 8 prototype AIS SARTs and 2 Radar SARTS during trial.

- Mounted on dummy at sea level, at ½ metre and at one metre.

- Detected dummy at 4.3 NM, ½ metre at 6/7 NM and the one metre at 9.7 NM.

- By careful tuning of the radar, which required a certain amount of radar expertise, it was shown that the radar SART almost meet the performance of the AIS SART in terms of range and coverage.

- Options for bursty behaviour and Tx power were tested and this will be used by the IEC AIS WG to complete the test and performance standard for AIS SARTs.

SART MMSIs

970990004 sweetie jar 1 metre ht

970990002 sweetie jar ½  metre ht

970990003 sweetie jar 1 metre ht

970990007 3 balls 1 metre ht

970990001 guy the dummy.

970990008 sweetie jar ½  metre ht

970990005 rib

03 @ 7.7 NM

04 @ 7.5 NM

08 @ 7.3 NM

01 @ 4.4 NM

## Participants

True Heading AB.

Peter Forey

Kongsberg Seatex, Kristain

Jotron, Otto Holm, ANO Norwegian.

L3 Communications, Johnny Schultz.

Pharos marine

Equipment.

True Heading. Class B AIS Mobile, Pro AIS receiver, AIS yacht.

Pharos marine, ATONIS AIS AtoN, ATONIS repeater.

Kongsberg Seatex, BS 500 AIS Base Station.

L3 Communications, Protec AIS Base Station, AtoN AIS Station, AIS SAR Transponder

CNS Systems, AIS SAR Transponder.

Peter Forey Class B AIS, Radar SART.