e-NAV5/9/8

St Johns Output 7

IALA Recommendation A-124

Annex 5

Interfacing model 
of the AIS Service

# Revision

| # | Date | Description | Initials |
| --- | --- | --- | --- |
| V 0.1 | 18. Jan 2008 | Version 0.1 – Initial draft | JHO |
| V 0.2 | May 2008 | First complete draft | JHO+SB |

# Table of Contents

1	Revision	2

2	Table of Contents	3

3	Introduction	4

4	Reference	4

5	Definitions	4

6	Abbreviations	5

7	The AIS Service as a shore-based e-Navigation service	5

7.1	The place of the AIS Service in the e-Navigation architecture	5

7.2	The AIS Service within the shore-based e-Navigation system architecture	7

7.3	The functional interface of the AIS Service to shore-based applications	9

8	Identification of the individual Interfaces of the AIS Service	13

9	Application data encoding and protocol stacks for the Machine-to-Machine interfaces of the AIS Service	18

9.1	Introduction	18

9.2	Interfacing the AIS Service to other services of the shore-based e-Nav system	20

9.3	Internal Interfaces of the AIS Service	22

10	Human Machine Interface design considerations	23

10.1	Introduction	23

10.2	Options for delivering Human Machine Interfaces	23

10.3	Interfacing the AIS Service to Technical Operation Personnel with Human-Machine-Interfaces	24

Attachment A on Application Encoding by IEC 61162-derived sentences	26

A.1	Important differences of IEC 61162-derived encoding compared to “traditional” IEC61162-1	26

A.2	Sentence structure of the IEC 61162-derived application data encoding	27

A.2.1	Characters and Identifiers	27

A.2.2	Data field encoding	28

A.2.3	Resulting Sentence Structure	30

A.3	Bibliography	30

Attachment B on IT protocol stacks recommended	31

Attachment C – Basic AIS Service sentences definition using IEC61162-derived encoding	32

C.1	Basic AIS Service (BAS) Sentences (LSS Sentences external to the AIS Service)	32

C.2	PSS Sentences	32

C.3	LSS Sentences to the AIS Service Management (LSS internal to the AIS Service)	32

C.4	ASM Sentences	32

# Introduction

This Annex expands on the description of the AIS Service regarding the Interfacing Model.

| IALA Recommendation A-124 on the shore-based AIS Service |  |
| --- | --- |
|  | General |
|  | Deliverables of the AIS Service to the shore-based e-Nav system - Annex 1 Basic AIS Services - Annex 2 Data Model of the AIS Service |
|  | Service Model - Annex 3 Distribution Model - Annex 4 Interaction and data storage model of the AIS Service - Annex 5 Interfacing Model of the AIS Service - … - Annex 8 Test Model for the AIS Service |
|  | Design Considerations |
|  | VDL Management |
|  | Implementation, Installation, and Maintenance |
|  | Functional Components of the AIS Service |

This Annex exhibits the following flow of thought, when addressing interfacing issues of the AIS Service:

- The context of the AIS Service as an eNav service within the shore-based eNav system is considered. Interfacing issues resulting from that are discussed. This is the bird’s eye view on the AIS Service from a shore-based eNav system perspective.

- All of the interfaces of the AIS Service are identified, i.e. a complete list of all interfaces of the AIS Service is provided. There are both Machine-Machine-Interfaces (M2M) between components and Human-Machine-Interfaces (HMI) specifically supporting the Technical Operation Personnel. This complete list of interfaces takes into account in particular the contribution of the Distribution Model (Annex 2) of the AIS Service.

- In a next step, the Machine-Machine-Interfaces of the AIS Service are addressed in more detail. Issues like recommended data encoding and recommended protocol stacks are discussed, taking into account latest developments of IT interfacing techniques.

- Finally, the Human-Machine-Interfaces of the AIS Services are discussed in more detail. Information is provided for administrations as to what are best practices when setting up Human-Machine-Interfaces of the AIS Service towards their Technical Operation Personnel.

# Reference

Additional references not included in Main Body of this Recommendation:

--- TBD ---

# Definitions

Additional definitions not included in Main Body of this Recommendation:

--- TBD ---

# Abbreviations

Additional abbreviations not included in Main Body of this Recommendation:

--- TBD ---

# The AIS Service as a shore-based e-Navigation service

## The place of the AIS Service in the e-Navigation architecture

Figure 1 illustrates the place of the AIS Service in the e-Navigation architecture as described by IALA Recommendation on the E-Navigation Architecture – the shore perspective (IALA Recommendation eNAV-101). Figure 1 also applies the general architectural statements of that Recommendation to the AIS technology.

In addition, Figure 1 clearly identifies the co-operative nature of the information exchange between the shore-based AIS Service and the applications connected to it on the shore side and the mobile AIS stations and the mobile applications connected to them on the mobile side. The shore based applications connected to the AIS Service will be further explored in the next section.

It should be noted, that the AIS and the AIS Service share the dependencies of the e-Navigation architecture, i.e. the dependency on GNSS for position fixing and/or timing and the dependency on infrastructure.

Figure 1: Place of the AIS Service within the e-Navigation architecture

IBS

Ship’s

sensors

Shipboard

Applications

AIS VHF

Data Link

(AIS VDL)

AIS Technology

= encapsulated

link technology

Shore-based e-Nav system

Ship technology 
environment

Appli-cation 1

Application 2

Application 3

Application 4

mobile AIS station

INS

AIS Service

Application-to-
application 
(peer-to-peer)

functional
connection

other

ships

other

ships

mariner

Shore-based
users

World Wide Radionavigation System (WWRS) of IMO, including GNSS, GNSS augmentation and terrestrial backup

## The AIS Service within the shore-based e-Navigation system architecture

IALA Recommendation on the common shore-based e-NAV system architecture” (eNAV-201, Edition [2008]) defines the relative place of the AIS Service within that system architecture. It identifies the AIS Service as a shore based e-Navigation service within the so called “Data Collection and Data Transfer services” of the common shore-based e-Navigation system architecture (compare Figure 2).

Figure 2: Place of the AIS Service within the shore-based e-Navigation system architecture

In particular the need to provide a functional interface between the AIS Service and other requesting services within the shore-based eNAV system is identified.

Since the “Data Collection and Data Transfer services” of the shore-based e-Nav system will be requested by other e-Nav services within that system, there can be identified multiple client-server-relationships. Those can be combined to a hierarchy within the shore-based e-Nav system as indicated by Figure 3 (following page).

Figure 3: Client-server relationships of the AIS Service with other shore-based e-NAV services – resulting hierarchy of shore-based e-NAV services and the place of the AIS Service

Figure 3 shows, that the AIS Service can / will be requested by

- the Value-added Data Processing Services: The AIS Service is requested as an information source on traffic objects (mainly ships) for data correlation and combination purposes (“data mining”);

- the User Interaction Service: The AIS Service is requested as an information source on traffic objects (mainly ships). The received data is then displayed to shore-based users by a Human Machine Interface. Also the AIS Service is requested for transparent communications via the AIS VHF Data Link.

- the Gateway Service: The AIS Service is requested as an information source on traffic objects (mainly ships); information is forwarded to shore-based external systems by appropriate Machine Machine Interfaces.

- other Data Collection and Data Transfer Services: The AIS Service is requested as an information source in some cases (e. g. for received Aids-to-Navigation data to be forwarded to the Visual Aids Service) but mainly for transparent communications via AIS (e.g. for setting up synthetic/virtual visual aids-to-navigation on behalf of the Visual Aids Service or for DGNSS correction data transmission via AIS).

Hence, in these cases the AIS Service will act as a requested service (server). The (other) requesting services act as clients to the AIS Service.

There are (few) cases, where the AIS Service will act as a requesting service (client) to other e-Nav services within the shore based e-Nav system, e.g. when initiating DSC channel management via the DSC Service.

Note, that the need for data transmission via e.g. a data transfer network due to the geographical distribution of components of the AIS Service is encapsulated within the AIS Service and therefore “not visible” at this level.

[Note also, that the so called „System or Virtual MMSI“ (00MID9xxx) for addressed information exchange via AIS would allow to address specific services and/or users within the shore-based e-Nav system (refer to Recommendation ITU-R M.585-3 “Assignment and use of maritime mobile service identities”).]

## The functional interface of the AIS Service to shore-based applications

### Introdution

The above figures indicated the functional interfaces between the AIS Service within the shore-based e-Nav system by arrows. The primary concern is with the information exchanged between the AIS Service and other shore-based e-Nav  services. The functional interface thereby facilitates the integration of the AIS Service into shore-based applications.

Since information is abstract, the functional interfaces need first and foremost to be considered in functional terms of information delivery and only later from a technological point of view. The functional interface encapsulates the technical details of both the AIS technology and of the layout and local configuration of AIS Service.

### The AIS Service as a “requested service”

Due to the hierarchy shown in the previous section where the AIS Service has been identified as a requested service (compare Figure 3), it is the AIS Service which provides the functional interface description: the requesting services (clients of the AIS Service) need to comply with the interfaces provided by the AIS Service. They can do so e.g. by using an “AIS Service driver” which is common practise in IT. Thereby they encapsulate the details of their interfacing to the AIS Service and make their own functionality / software independent of smaller changes of the interface of the AIS Service.

### Use case description and Entity-relationship-description

There are at least two relevant angles of perspective regarding the functional interface of the AIS Service:

- the use case model, which allows to seamlessly derive use cases of the AIS Service from user requirements;

- the entity relationship model, which focuses on relevant associations between abstract and/or physical entities within the AIS Service and abstract and/or physical entities outside the AIS Service.

Both views complement each other:

- The use case model answers the question: “What is the AIS Service good for?” and thus directly supports application design once the service use cases of the AIS Service have been identified and described.

- The entity relationship model answers the question: “What aspects does a competent administration need to consider, when setting up and running its AIS Service?” It is particular important for life-cycle management.

### The functional interface of the AIS Service in terms of use cases

The functional interface of the AIS Service in terms of use cases can be determined when correlating the following aspects:

- What fundamental functionalities would be expected from the shore-based AIS Service. This is the top-down requirement derivation, eventually starting at user requirements and business processes. Figure 4 shows application of that the seamless derivation process regarding the AIS Service (compare IALA Recommendation on shore-based e-Navigation architecture – the shore perspective).

Figure 4: Seamless derivation of service use cases and component requirements from system level requirements

- What fundamental functionalities could be delivered by the shore-based AIS Service taking into account the engineering knowledge regarding the capabilities and limitations of the AIS technology. This is the bottom-up approach.

Both point of view meet in the definition of what has been called the external Basic AIS Services (BAS). Hence, he functional interface in terms of service use cases of the AIS Service is defined as a set of external BAS.

### Functional interfaces in terms of entity relationships

Figure 5 (which is Figure 1 of the Main Body) shows highlighted the associations or interactions of the AIS Service with external entities, both abstract and physical. Associations which cross the boundary of the AIS Service require external interfacing of the AIS Service in most cases.

In addition, there are internal interfaces between the functional components of the AIS Service, which eventually will materialize physically within an IT environment.

It should be noted, that the content of the Human Machine Interfaces (HMI) of the AIS Service to the Technical Operation Personnel (TOP) and to the Technical Development Personnel (TDP) is determined by the tasks the AIS Service requires over its full life-cycle from that personnel (compare Annex 13 – “Efficient operation and maintenance of the AIS Service”). Hence, this relationship is highlighted in the Figure 5. The actual physical implementation of the Human Machine Interfaces will be done by the appropriate functional components as shown below.

It should also be noted, that certain abstract models of the AIS Service which were developed previously have a strong bearing on the Interface Model, namely

- the Basic AIS Service definitions and the definitions of other use cases

- the Data Model of the AIS Service

- the Structure Model of the AIS Service

- the Interaction Model and the Data Storage Model of the AIS Service

Figure 5: Entities which require interfacing of the AIS Service (highlighted)

# Identification of the individual Interfaces of the AIS Service

After having determined the place of the AIS Service as a whole within the common shore-based e-Nav system architecture, it is now necessary to turn to the AIS Service and its individual interfaces as such. As a first step, it is helpful to re-consider the Figure 10 of the Main Body of this recommendation. In this Figure, the tasks of the functional components were identified within the AIS Service and in relationship to the external shore-based e-Nav system or the AIS VDL. Figure 6 is now used to highlight the tasks of many functional interfaces due to the tasks of the functional components they connect.

Figure 6: Identification of the tasks of the functional interfaces (highlighted) in relationship to the functional component’s tasks they connect

Note: In Figure 6 above the connection of the AIS VHF Data Link is not highlighted. That is due to the fact, that the “interface” of the AIS VDL is provided externally to the individual AIS Service and even the individual shore-based e-Nav system. The AIS VDL exists without them. Thus, the AIS VDL conception-wise provides the “interface” in the form of a “signal-in-space” description, and every individual AIS Service must adhere to that “interface”.

Note: The Human Machine Interfaces to the Technical Operation Personnel have not been included in Figure 6.

The tasks highlighted in the Figure 6 above have already been taken into account when developing the Structure Model of the AIS Service (compare Main Body of this Recommendation). Figure 7 on the following page now highlights the interfaces between the functional components of the Structure Model.

Figure 7: Structure model of the AIS Service with highlighted interfaces

Technical Operation Personnel (master control)

HMI

Logical Layer -

Logical AIS Shore Station (LSS)

HMI

Net Data

Status

AIS Service

Management Layer

HMI

Physical AIS Shore Station (PSS)

HMI

Fixed AIS Stations Layer (Base stations, repeater stations)

HMI

RF components of the AIS Service

AIS VHF Data Link (VDL)

(Time Division Multiple Access)

Technical Operation Personnel

(there are tasks for each layer in principle)

Configuration

Physical

Layer /

Remote

Shore

Station

* Note: This symbolic representation does not imply any inference as to the amount of required personnel for the technical operation of the AIS service. This symbolic representation aims at indicating that human interaction with the largely automated AIS Service is required as the last resort - and in some cases possibly on a regular basis - on all layers of the AIS Service.

Boundary of shore-based e-Nav system

Based on Figures 6 and Figure 7 above, Figure 8 on the following page goes on to specifically identify all interfaces of the AIS Service, both internal and external, both Machine-to-Machine and Human Machine interfaces. Figure 8 asserts completeness in regard to the various interfaces of the AIS Service. Figure 8 constitutes the core of the Interfacing model of the AIS Service.

Figure 8: Complete identification of functional interfaces of the AIS Service (external/internal; not including interfaces of infrastructure components)

# Application data encoding and protocol stacks for the Machine-to-Machine interfaces of the AIS Service

## Introduction

### Bundle of application data encoding and protocol stacks

This chapter recommends certain concrete application data encoding techniques and protocol stacks for the Machine-to-Machine interfaces of the AIS Service, which were identified above. By the same token, certain other options for application data encoding are discouraged.

The recommendation always comes together as a bundle of at least one application data encoding technique and one IT protocol stack. The reason is, that some application data encoding techniques are fit to certain IT protocol stacks, while this may not be true of different combinations.

### Data Model of the AIS Service and the impact of the UMDM

When designing the application data encoding, the Data Model of the AIS Service has a strong bearing: It is these data objects and data types which need to be encoded so that a physical data exchange may take place between machines. Since, the Data Model of the AIS Service is a subset of the (emerging) Universal Maritime Data Model, the application data encoding technique should provide for abstract data encoding, sufficient flexibility to cater for future changes and in particular for the Universal Data Object Identifier (U.DOI) as one fundamental ingredient.

### Open System Philosophy

Also, the application data encoding and IT protocol stacks recommendations should be in keeping with the open system philosophy of the common shore-based e-Navigation system architecture. Hence, IT protocol stacks compatible with the ISO/Open System Interconnection (OSI) protocol stack should be recommended.

### The persistent role of the – traditional – IEC 61162 encoding technique – slightly modernized

Combining these requirements with the nature of the data flow to and from the AIS Service, it appears that an application data encoding technique would be best suited, which would combine sentence-orientation with abstract and flexible data encoding capabilities. Also it should be based on well established maritime standards.

Already many years ago, the “traditional” IEC 61162 was created to fulfil these requirements. All data objects are represented as abstract data in ASCII characters with field delimiters. However, the “traditional” IEC 61162 sentence structure rigidly implied the meaning of certain fields by order of appearance. This made the exchange of single data objects cumbersome, because “empty” fields would be exchanged as well.

The notion of a binary encoded field, which would contain a AIS message as binary encoded for the purpose of the AIS VHF Data Link taking into account its bandwidth constraints, was incorporated only due to the physical limitations of the Presentation Interface protocol of mobile and fixed AIS stations. This notion was initially considered alien to the original philosophy of the IEC 61162, and therefore rightly caused some concern. Today, a co-existence between the traditional abstract data encoding and the binary encoding within the framework of the IEC 61162 has been achieved.

It should be remembered, however, for what reasons the binary encoded field for the AIS message content was created in the first place. It was created to cater for the limitations of the Presentation Interfaces of the mobile and fixed AIS stations. It was not created for the dissemination of AIS data in highly integrated and sophisticated shore-based IT systems, such as the shore-based e-Nav system. In particular, there are no such (bandwidth) limitations as were encountered at the AIS VDL and at the Presentation Interfaces of AIS stations. However, the proliferation of the intricacies of the AIS VDL would tie those systems to the development of the Recommendation ITU-R M.1371 for all time. From a system engineering point of view, this close connection carries many disadvantages.

This gave rise to the notion to use XML extensively for the AIS Service as well. This application data encoding technique provides maximum flexibility for abstract data, but introduces substantial overhead and would result in bandwidth requirements even unnecessarily high for shore-based systems. Hence, XML is an alternative only for interface functionalities with low data exchange rate.

The best solution which would combine the advantages of the traditional IEC 61162 data encoding technique, i.e. the abstract data encoding, the sentence-oriented structure and its wide recognition in the maritime field, with the current need for even more elaborate abstract data encoding and flexibility, expressed by concepts like the U.DOI. The U.DOI as a leading designator for the data fields of a sentence would provide the flexibility in terms of the concrete meaning of the individual data fields, thus overcoming the rigidity of the traditional approach.  This approach would be compatible with the philosophy of the traditional IEC 61162 while introducing one a little overhead both in terms of transmission bandwidth and parsing compared to XML. This approach is called IEC 61162-derived application data encoding throughout this Annex. The details are developed in Attachment A.

Concluding, the sentence-oriented encoding technique IEC61162-derived, using abstract data encoding in accordance with the traditional IEC 61162 philosophy and in accordance with the concept of the Universal Data Object Identifier (U.DOI) (refer to Attachment A), appears to be best practise for data exchange when considering the nature of the AIS Service while providing sufficient flexibility for life-cycle-management.

### Encoding of Data Container content transparently forwarded by AIS Service

It should be noted, that the Data Model of the AIS Service also accommodates such data items (“data containers”), which are forwarded by the AIS Service transparently between the requesting service and the AIS VHF data link in either direction without changing or recognizing their content in any way. Their encoding is completely and exclusively determined by the requesting service, i.e. the encoding of the content of those data containers is described within the documentation of the requesting service, only. The AIS Service does not need to know this encoding used.

Examples for transparent forwarding of these data containers and for encoding description of those data container’s content hosted exclusively at the requesting service are:

- transparent reception/transmission of application-specific (“binary”) data: any other e-Nav service may hold the encoding description for its application-specific data.

- differential correction data: the encoding description for this data is maintained exclusively in the DGNSS Correction Service of the shore-based e-Nav system architecture.

## Interfacing the AIS Service to other services of the shore-based e-Nav system

### Net data exchange interfacing to requesting e-Nav services

Table 1 (overleaf) shows the recommended application encoding and IT protocol stacks for the Machine-to-Machine interface of the LSS to the requesting services of the shore-based e-Nav system.

Some rationale and explanations to the Table are as follows:

- The rationale for the asymmetry regarding XML recommended only as an output data encoding technique from the LSS to the requesting e-Nav service is as follows: While the XML may be useful for the requesting shore-based e-Nav service and while it is relatively easy to compile XML files as output data, the AIS Service as such does not benefit from XML usage internally but would require a parsing process for incoming XML messages.

- All of the above interfacing techniques can be implemented concurrently at the LSS interface, thus providing a portfolio of data access options to the designer of the requesting service of the shore-based e-Nav system and hence some flexibility over life-cycle.

- Alternatively, only some of the above interfacing techniques can be implemented at the discretion of the authority.

Table 1: Recommended application encoding and IT protocol stacks for Machine-to-Machine interface of the LSS to the requesting services of the shore-based e-Nav system

| External Basic AIS Service | IEC 61162-derived + SMTP | IEC 61162-derived + TELNET/SSH | XML + HTTP/FTP | IEC 61162-derived + SQL | Remarks Refer to Attachments regarding details on the IEC 61162-derived encoding method and on the recommended protocol stacks. |
| --- | --- | --- | --- | --- | --- |
| External BAS Cat. I | X | X | X | - | Push, dynamically: AIS Service to requesting service |
| External BAS Cat. II | X | X | - | - | Push, dynamically: Requesting service to AIS Service |
| External BAS Cat. III | X | X | - | - | Triggered once and autonomously performed by AIS Service for the requested period of time; Requesting service prompts AIS Service. |
| External BAS Cat. IV | X | X | Q: - (1) A: X | X | Triggered once and autonomously performed by AIS Service for the requested period of time; AIS Service prompts Requesting service. |
| Replay | X | X | Q: - (1) A: X | X | Replay of recorded data by AIS Service based on selection of the requesting service; requesting service prompts AIS Service; once initiated replay push by AIS Service. |

Legend:

Q	Query of requesting service of the shore-based e-Nav system

A 	Answer/reply of the AIS Service

X 	Recommended

- 	Discouraged

(1) 	Use either IEC-61162-derived or SQL sentences for query.

### Management interface of AIS Service to requesting e-Nav services

The Management Interface of the AIS Service to the requeting e-Nav services serves to exchange configuration and status data between the AIS Service Management and the Service Management Layer of the requesting service.

This interface is by nature a query and answer interface. Queries are in particular the configurations or re-configurations regarding the Basic AIS Services by the requesting service and requests for specific status information from requesting service. Answers are in particular the acknowledgments or status alerts given by the AIS Service Management to the requesting service.

Hence, the following recommendation appears to be the best practice:

- IEC 61162-derived + SQL + TELNET/SSH and/or

- IEC 61162-derived + SMTP and/or

- IEC 61162-derived + TELNET/SSH

All of the above interfacing techniques can be implemented concurrently at the LSS interface, thus providing a portfolio of configuration and status-access options to the designer of the requesting service of the shore-based e-Nav system and hence some flexibility over life-cycle.

Alternatively, only some of the above interfacing techniques can be implemented at the discretion of the authority. XML encoding is possible theoretically but is discouraged due to the lack of advantage for encoding configuration and status data on one hand and the massive overhead on the other hand when using it.

## Internal Interfaces of the AIS Service

The following Table 2 shows the recommended options for application encoding and IT protocol stack for the internal interfaces of the AIS Service. For descriptions of the application encoding methods and for the IT protocol stacks refer to the Attachments to this Annex.

Note: Only internationally fully described methods for application encoding and IT protocol stacks are recommended in keeping with the open system philosophy.

Table 2: Recommended application encoding and IT protocol stacks for Machine-to-Machine interfaces internal to the AIS Service

| Machine-Machine-Interface (M2M) | Recommended Option | Alternative for individual interface | Remarks |
| --- | --- | --- | --- |
| AIS net data interface PSS to Logical Layer (to IDS and to LSS) | - Application encoding: IEC 61162 derived - IT protocol stack: SMTP | Not recommended | - |
| Status/Configuration interface LSS to ASM-Node | - Application encoding: IEC 61162 derived - IT protocol stack: SMTP | Not recommended | - |
| Status/Configuration interface PSS to ASM-RS | - Application encoding: IEC 61162 derived - IT protocol stack: SMTP | Not recommended | - |
| Status/Configuration interface ASM-RS to ASM-Master | - Application encoding: IEC 61162 derived - IT protocol stack: SMTP | - Application encoding: like recommended option - IT protocol stack: TELNET or SSH | - |
| Status/Configuration interface ASM-Node to ASM-Master | - Application encoding: IEC 61162 derived - IT protocol stack: SMTP | - Application encoding: like recommended option - IT protocol stack: TELNET or SSH | - |
| Interface AIS Base Station to PSS | - Application encoding: IEC 61162 as amended by IEC 62320-1 - IT protocol stack: TCP-Socket | - Application encoding: like recommended option - IT protocol stack: RS-232 or equivalent | - |

# Human Machine Interface design considerations

## Introduction

In-depth consideration should be given to the Human Machine Interface design of the components of the AIS Service. Ergonomic requirements need to be fulfilled by default. Also, the HMIs determine to a large degree the efficiency of the Technical Operation Personnel over the full life-cycle of the AIS Service. Hence comfortable remote control functionalities and decision support should be considered.

## Options for delivering Human Machine Interfaces

Different classes of HMI can be identified as follows.

- HMI direct.

- Local HMI.

- HMI locally dispatched.

- Remote HMI.

The following explanations are given:

1. HMI direct

The „HMI direct” comprises all signalling, displays and input devices which are mounted on the machine and form an integral part of it, i.e. on-/off-switches, reset keys, LED indications, etc. The “HMI direct” is always at the site where the machine is located, i.e. at the Remote site and at the Node site . These interfaces regularly are intended for quick access to the most important information regarding the current status of a machine, but don’t serve for comfortable Technical Operation.

Figure 9: The „HMI direct“

2. Local HMI

Similar to „HMI direct“. The difference is: The „Local HMI“ comprises all separate signalling, displays and input devices which are connected to the machine but which are wholly dependent on that machine for their functionality. Examples are computer peripherials such as mouse, keyboard and display.

Figure 10: The „local HMI“

3. HMI locally dispatched

The „HMI locally dispatched“ is human interaction device at the site of the machine, i.e. at Remote site or Node site, which is connected to the machine by a Local Area Network, i.e. it is on the same premise as the machine it connects to, and which runs independent of the machine as far as the basic human interaction functionalities are concerned, but depends on the machine regarding the information as such.

Figure 11: The „HMI locally dispatched“

4. Remote HMI

The „Remote HMI “ is human interaction device at the Technical Operation Personnel site (either the fixed regular working place or mobile working place), which is connected to the machine by a Wide Area Network, i.e. it is not on the same premise as the machine it connects to, and which runs independent of the machine as far as the basic human interaction functionalities are concerned, but depends on the machine regarding the information as such. The provision of Remote HMI alone allows for centralized Technical Operation Personnel site(s); compare Distribution Model.

Figure 12: The „Remote HMI“

## Interfacing the AIS Service to Technical Operation Personnel with Human-Machine-Interfaces

### Recommended minimum HMIs for functional components

The following functional components of the AIS Service should provide a internet technology based machine-machine-interface, which would support the “HMI locally dispatched” and the “Remote HMI”:

- LSS

- PSS

- Fixed AIS Stations

- ASM Master

- ASM Node

- ASM RS

When the above functional components are materialized as integral components consisting of both hardware and firmware, i.e. as physical black boxes delivering the required functionality at their interfaces, there should be in addition appropriate “HMI direct” and/or “Local HMI”. The Fixed AIS Stations should have in addition an appropriate “HMI direct” and/or “Local HMI” in any case.

### Recommended IT protocols

For “HMI locally dispatched” and the “Remote HMI” the following interfacing setup is recommended:

Session of the Technical Operation Personnel with the machine under consideration:

- to initiate a session on a HMI:

A user enters the IP address of the machine into a standard internet browser. The standard browser connects to the machine via HTML (including use of internationally standardized script languages) and HTTP. The machine acts as a web server to the standard browser and presents a HTML screen and also functionality, which allows to automatically start a separate TELNET or SSH session window.

- during the session: The session is continued by using the TELNET or SSH session window. Note: During the session the data exchanged may be proprietarily encoded.

Requirements for the HTML usage:

The version of the HTML should be selected conservatively in order to allow a large variety of browsers to run the session. No browser specific HTML dialects or functionalities should be used. The HTML script should be checked by an appropriate testing tool for rigidly conforming to the international HTML specification choosen.

The current version of HTML, which fulfils these requirements, is HTML3, --- standard --- .

Requirements for the optional use of script languages:

--- similar statements apply

# Attachment A on Application Encoding by IEC 61162-derived sentences

# Important differences of IEC 61162-derived encoding compared to “traditional” IEC61162-1

The application encoding by IEC61162-derived sentences exhibits the following important differences to the „traditional“ IEC 61162-1:

- All statements of IEC 61162-1 regarding transport-oriented layers of protocol stack („hardware-related“ statements) ignored: This means, the IEC 61162-derived sentences can be transmitted by any internet-based IT protocol stack, such as SMTP, FTP, TELNET, and SSH. Hence, only the fundamental encoding philosophy for IEC 61162 sentences is preserved to a certain degree.

- Library-like, flexible composition of a sentence extensively using the Universal Data Object Identifier (U.DOI): The content of every data field of a sentence is identified by the U.DOI expressively stated. Hence, the order of appearance of data objects is arbitrary. Also, the number of data fields exchanged in one sentence may change without the need to exchange “empty” data fields. 

There is one limitation, though: Within one sentence, only data applicable to the same object referenced by the data may be transmitted. I.e. it is not allowed/possible to transmit data concerning e.g. two different ships in one sentence. 

Also: Certain rules may apply for certain data objects of complex data types, requiring that these data objects always come together in a given order of appearance.

- Context-sensitive Sentence-Identifier: The Sentence-Identifier is defined by the source functional component, and not defined based on sentence content (as with the „traditional“ IEC 61162-1. Since the content of the data field is clearly and unambiguously defined by the U.DOI (see above), the Sentence Identifier now defines the rule base for the recipient of the sentence, when evaluating the data content. I.e. the sentence-identifier explicitly introduces the context of the sentence, thus adding value and clarity. 

Note, that this does not mean that a (protocol-dependent) absolute address of the transmitter of the sentence is introduced (such as an IP address). This is being dealt with at protocol level. The point is, that the recipient of the sentence – by whatever protocol the recipient may have received the sentence – can and need to evaluate the sentence by the context-sensitive rule base given the Sentence-Identifier.

- No destination field: A destination field is not given on application encoding level within the sentence because this is safely and sufficiently done on the transport protocol level, e.g. by proper IP address routing. This means, that the recipient is always implicitly known: Whichever component receives the sentence, it deals with it. Thereby the sentence may be easily treated in store-and-forward systems (compare e.g. replication processes in Service-Wide Data Storage (IDS)) without changing anything within the sentence. Also, this allows for maximum flexibility of the architecture.

- Data fields must not be and need not be empty. Follows from the above flexibility and avoids overhead.

# Sentence structure of the IEC 61162-derived application data encoding

The above rules are used to define the sentence structure by “tweaking” the appropriate statements of the “traditional” IEC 61162-1.

## Characters and Identifiers

The smallest building block of a sentence is a character.

### Character set

In accordance with “traditional” IEC 61162-1, all characters are to be interpreted as ASCII characters. Also, the Most Significant Bit (MSB) of any 8-bit-character is always to be encoded as Zero (D7 = 0).

### Reserved Characters

In accordance with „traditional“ IEC 61162-1, the following character values are reserved for special purposes and may not be used.

|  | ASCII | HEX | DEC |
| --- | --- | --- | --- |
| Beginning of Sentence indicator | $ | 24 | 36 |
| End of Sentence indicator | <CR> <LF> | 0D 0A | 13 10 |
| Field delimiter | , | 2C | 44 |
| U.DOI delimiter | : | 3A | 58 |
| Attribute delimiter | ; | 3B | 59 |
| Delimiter before checksum field | * | 2A | 42 |

Table 1: Reserved Characters („traditional” IEC 61162-1)

In addition, for the IEC 61162-derived sentence format specified herein, the following reserved characters are assigned a special meaning as given in Table 2.

|  | ASCII | HEX | DEC |
| --- | --- | --- | --- |
| U.DOI delimiter | : | 3A | 58 |
| Attribute delimiter | ; | 3B | 59 |

Table 2: Additional Reserved Characters (for the IEC 61162-derived encoding)

### Valid characters for data encoding

In accordance with IEC 61162-1, excluding additionally the reserved characters „ : “ and „ ; “, see Table 2.

### Undefined Characters

ASCII characters which are neither reserved nor valid must not be used for data encoding. If their use is required, however, the rules of IEC 61162-1 for this case apply.

### Symbol characters

In accordance with IEC 61162-1.

### Functional component identifier = functional source ID

The Functional Component Identifier  (FCI) allows to uniquely identify the class of the functional component throughout the whole shore-based e-Nav system, which issued the sentence under consideration. The Functional Component Identifier also serves as a functional source ID of the sentence.

The Functional Component Identifier always comprises 6 charaters which are subdivided into two blocks of three characters each as follows:

- The first three characters identify the shore-based e-Nav service from which the sentence originated. The first three characters are identical to the service abbreviations assigned in the IALA Recommendation on the common shore-based e-Nav system architecture (IALA Recommendation e-Nav 201). 

For sentences originated in the AIS Service, the first block of the FCI is always: “AIS”

- The last three characters characterize the class of functional component of the AIS Service, from which the sentence originated. They are defined as follows:

  - “LSS”: Logical Shore Station

  - “PSS”: Physical Shore Station

  - “SMN”: AIS Service Management – Node site agent (ASM-Node)

  - “SMM”: AIS Service Management – Master (ASM-Master)

  - “SMR”: AIS Service Management – Remote site agent (ASM-Remote Site)

- Examples for the FCI of sentences originating in the AIS Service:

- FCI = “AISLSS” = Source of the sentence received is a LSS of the AIS Service

- FCI = “AISPSS” = Source of the sentence received is a PSS of the AIS Service.

## Data field encoding

A data field contains a sequence of valid characters.

Every data field is between two field delimiter characters, compare Table 1.

Different to IEC 61162-1, every data field, which is not a mandatory data field as described in the following sections,  always contains an U.DOI delimiter and an Attribute delimiter, as defined in Table 2.

### First mandatory data field: Functional Source Identifier field

The Functional Source Identifier field is the first data field immediately after Beginning of Sentence indicator. It is mandatory. It must not be empty. The Functional Source Identifier field consists only of the Functional Component Identifier (FCI) as described above.

It is this mandatory data field, which defines the rule base for the recipient of the sentence, when evaluating the data content. I.e. the Functional Source Identifier field explicitly introduces the context of the sentence, thus adding value and clarity.

### Second mandatory data field:  Instance-ID

The ID of the Instance of the Functional Source Component or short: Instance-ID is the second mandatory data field. It uniquely identifies the instance of the functional component, which originated the sentence, in the instance realm. It is a unique running number.

The number is assigned manually by the Technical Operation Personnel or automatically AIS Service Management during initialization and remains unchanged during the life cycle of the component. The Instance-ID also serves to identify historical instances, such as instances of LSS, which have expired.

Note, that the Instance-ID should be not be confused with a (protocol-dependent) absolute address of the physical transmitter of the sentence (such as an IP address). This is being dealt with at protocol level.

### Third mandatory data field: Sentence ID

The Sentence ID is the third mandatory data field. It is a unique running number created by the individual instance of the functional source component, which originated the sentence.

Together with the Functional Source Identifier field (1st mandatory field), the  with the Functional Source Identifier field, serves to uniquely identify a sentence within the whole of the shore-based e-Nav system.

By using the Sentence ID, an acknowledgment to a sentence can easily and uniquely be correlated with the sentence referenced by that acknowledgement.

### Fourth mandatory data field: Time of Sentence Generation

The Time of Sentence Generation data field is the fourth mandatory data field. It designates the UTC-time at which the sentence was generated.

The Time of Sentence Generation should not be confused with any other time stamp contained in other data fields. The object, to which this time stamp, i.e. the Time of Sentence Generation, belongs, is the sentence under consideration.

The Time of Sentence Generation data field always consists of two attributes, namely date and time. The two attributes are separated by the Attribute delimiter character.

- The date always is in the format: Day (2 digits) – Month (2 digits) – Year (4 digits), hence: DDMMYYYY. There is no character allowed between Day, Month and Year.

- The time always is in the format: Hour (2 digits) – Minute (2 digits) – Seconds (2 digits), hence: HHMMSS. There is no character allowed between Hour, Minute and Second.

- An example for this data field is as follows:

30122004;123659	means: 30th December 2004, 12:36:59 UTC

### Fifth mandatory data field: at least one net data field

After the Time of Sentence Generation there is at least one net data field. Should there be more than one net data field, they are separated by the Field delimiter character.

Every Data field contains exactly one (1) data object, which may have one or more attributes.

Every data field always consists of the U.DOI, followed by the U.DOI Delimiter character, followed by at least one Attribute as defined in the data model for that data object identified by the U.DOI:

<U.DOI><U.DOI Delimiter><Attribute 1>[<Attributs-Delimiter><Additional Attributes>]

Different Attributes are separated by the Attribute Delimiter.

The order of appearance of the attributes of the data object is defined in the definition of that data object in the data model. All attributes must always be included, empty attributes are not allowed.

Binary data field content should be encoded in accordance with the rules stated at section “undefined characters”.

### Last and mandatory field: Checksum Field

In accordance with  IEC 61162-1, must not be omitted.

## Resulting Sentence Structure

The resulting sentence structure is as follows:

| ASCII | HEX | Description |
| --- | --- | --- |
| $ | 24 | Start of sentence |
| < Functional Source Identifier field > |  | Identifier for e-Nav service of origin of sentence and identifier for functional component class |
| “,“ <Instance-ID > |  | Unique instance identifier for the instance of the functional component |
| “,“ <Sentence ID > |  | Running number serves as ID of this sentence from the above instance of the above functional component |
| “,“ <Time of Sentence Generation> |  | Date and Time (UTC), when sentence was generated by source component |
| “,“ <data field> |  | A net data field |
| [“,“ <data field>] |  | Possibly additional data fields, but no empty fields allowed |
| “*“ <checksum field> |  | Check sum |
| <CR><LF> | 0D 0A | End of sentence |

Table: Mandatory Structure of a Sentence

Example sentence:

$AISPSS,023,02,02122004;114022,4023:50;35.34*0A<CR><LF>

Meaning of example sentence:

- Source: a AIS PSS with the AIS PSS instance ID 023

- Sentence-ID: 02, which means that this sentence was originated early in the life cycle of this AIS PSS instance 023

- Date and Time of Sentence Generation: 02 Dec 2004, 11:40:22 UTC

- Data field: U.DOI = 4023 (whatever this may designate)

    - Value of first attribute of that data object: 		50

    - Value of second attribute of that data object: 	35.34

(whatever this may mean)

# Bibliography

IEC 61162-1: Maritime navigation and radiocommunication equipment and systems. Digital interfaces. Documentation and test requirements. Single Talker and Multiple Listener

# Attachment B on IT protocol stacks recommended

SMTP-, HTTP-, FTP-, TELNET-, and SSH-Protocol Stacks

Figure B.1: IT protocol stacks for the interfaces of the AIS Service

Recommended versions of these Protocols and relevant international standards:

- SMTP: [TBD]

- HTTP: [TBD]

- FTP: [TBD]

- TELNET: [TBD]

- SSH: [TBD]

# Attachment C – Basic AIS Service sentences definition using IEC61162-derived encoding

In due course, this Attachment will contain the explicit and detailed description of the sentences used of the AIS Service employing the IEC61162-derived encoding.

It is the intention that this explicit and detailed description will be considered normative by IALA national members and by industry.

This description will be derived concurrently from the following Annexes above:

- from Annex 1: definition of which data objects contained and of recommended order of appearance

- from Annex 2: the data object and data type definitions (being compatible with the UMDM)

- from this Annex 5: the application encoding scheme IEC-61162-derived as described in Attachment A.

# Basic AIS Service (BAS) Sentences (LSS Sentences external to the AIS Service)

These sentences are used to exchange BAS data between the AIS Service and other shore-based e-Nav services.

[These sentences will be contained here in a future edition of this Annex.]

# PSS Sentences

These sentences are used to exchange data between the PSS and any other functional component of the AIS Service.

[These sentences will be contained here in a future edition of this Annex.]

# LSS Sentences to the AIS Service Management (LSS internal to the AIS Service)

These sentences are used to exchange data between a LSS instance and the ASM (agent).

[These sentences will be contained here in a future edition of this Annex.]

# ASM Sentences

These sentences are used

- to exchange configuration and status data between the AIS Service and other e-Nav services of the shore-based e-Nav system;

- to exchange data between the ASM agents and the ASM master, i.e. between ASM-RS and ASM-Master and between ASM-Node and ASM-Master.

[These sentences will be contained here in a future edition of this Annex.]

It should be noted, that the AIS Service as such does not provide a direct user interface; refer to main body of this Recommendation.

This is a state-of-the-art engineering principle. It protects application software from unnecessary changes due to configuration or technology improvements within the AIS Service.

Note: In Figure 5 the relationship to the AIS VHF Data Link is not highlighted. That is due to the fact, that the “interface” of the AIS VDL is provided externally to the individual AIS Service and even the individual shore-based e-Nav system. The AIS VDL exists without them. Thus, the AIS VDL conception-wise provides the “interface” in the form of a “signal-in-space” description, and every individual AIS Service must adhere to that “interface”.

Cautionary Note: The use of the IP address of the functional component as the Instance-ID is strongly discouraged because IP addresses may be dynamically re-assigned during the life-cycle of the same instance of a functional component and may be re-used for a different instance of the same functional component after its life-cycle has expired.