e-Navigation Committee						e-NAV 5/13/04

5th Session								15-19 Sept 2008

Agenda Item 13

Proposed Revision of IMO Resolution A.953(23)

World Wide Radio Navigation System

At e-NAV 3 and e-NAV 4 a draft submission to 54th Session of the IMO sub-Committee on Safety of Navigation (NAV 54) was developed, proposing revision of IMO Resolution A.953(23), with the primary purpose of making the continuity requirements more realistic and achievable. The intention was that the input would be made by a number of member administrations (UK, USA, Netherlands & Sweden). In March 2008, advice was received from the Chairman of the sub-Committee that the matter could not be considered at NAV 54, because it was not a specific Work Item. A submission would be required to MSC to obtain approval for its consideration at NAV.

The case for the revision of A.953 remains and the attached submission is closely based on the previously proposed input to NAV 54. The Committee is invited to consider the submission and to request its members to encourage their respective administrations to sponsor and support it at MSC 85.

MARITIME SAFETY COMMITTEE	MSC 85/xx/xx

85th Session	26-28 Nov. 2008

# Original: ENGLISH

Agenda item xx

World-Wide Radio-navigation System (WWRNS)

New Work Item - Proposed Revision of IMO Resolution A.953(23)

Submitted by the Netherlands, Sweden, United Kingdom and United States

| SUMMARY |  |  |
| --- | --- | --- |
| Executive summary: |  | This document proposes that amendment of Resolution A.953(23) be approved as a Work Item for the Safety of Navigation sub-Committee, in order that more Administrations might be encouraged to submit suitable radionavigation services to IMO as a component of the World-wide Radio-navigation System (WWRNS). |
| Strategic direction: |  | 5.2 – Enhancing technical, operational and safety management standards. |
| High-level action: |  | 5.2.4 – Keep under review measures to improve navigational safety, including enavigation, ships’ routeing, ships’ reporting systems, vessel traffic services, requirements and standards for shipborne navigational aids and systems. |
| Planned output: |  | 5.2.4.3 – Update and development of worldwide radionavigation systems (GPS, GLONASS and GALILEO) (MSC) |
| Action to be taken: |  | Paragraph 7 |
| Related documents: |  | A.953(23), A.915(22) |

Introduction

1.	At its 23rd session the IMO Assembly adopted Resolution A.953(23),  World-wide Radio-navigation System (WWRNS), detailing the Organization’s policy for the recognition and acceptance of suitable radio-navigation systems intended for international use.  The Maritime Safety Committee was tasked with keeping the aforementioned resolution under review for adjustment as necessary.

2.	Whilst the recognition of the need for a world-wide radio-navigation system to provide ships with navigational position-fixing throughout the world was welcomed, it is now considered that certain operational requirements act as a disincentive to Administrations who may wish to submit WWRNS services to the Organization.

3.	Further to paragraph 2, proposed amendments to the ‘Revised Report on the Study of a World-wide Radio-navigation System’ included in Resolution A.953(23) are detailed in the Annex to this document.

Continuity requirement

4.	On examination of the Resolution it was considered that the existing service reliability requirement of 99.97% over a 3 hour period is unrealistic and is significantly in excess of that required by the aviation industry.  Marine users are unlikely to be undertaking activities requiring this level of system performance without reference to visual and / or radar inputs.  Activities requiring this level of system performance would also require users to undertake precautions such as predicted constellation limitations / DOP over the duration of the planned evolution.

5.	It is therefore proposed that a more realistic requirement would be for a continuity level of 99.97% over 15 minutes (based on the critical phase of a manoeuvre such as an approach to a harbour mouth, and equivalent to 99.64% over 3 hours) in all harbours, harbour approaches and other restricted waters.

Further considerations

6.	On further examination of the Resolution, it was also noted that:

- the different service level requirements should be re-organised with the least demanding (i.e. Ocean) phase first, in keeping with the format of similar documents;

- specific reference to augmentation should be removed, as the means of providing a service is not relevant to this document;

- there was confusion between the update rate of data supplied by the WWRNS and the user’s ability to process that data;

- the update rate of data supplied by the WWRNS should now be considered to be not more than 2 seconds in all cases;

- the reference in the Resolution to vessels capable of exceeding 30 knots was not required.

Action requested of the Committee

7.	The Committee is invited to note the above observations and to consider approving ‘Revised Report on the Study of a World-wide Radio-navigation System’ as a new Work Item for the Safety of Navigation sub-Committee at its 55th Session.

ANNEX

REVISED REPORT ON THE STUDY OF A WORLD-WIDE RADIONAVIGATION SYSTEM

1 	INTRODUCTION

1.1 	Studies on a world-wide radio-navigation system have been taking place since 1983. These studies have provided a basis on which chapter V of the 1974 SOLAS Convention has been amended to include a requirement for ships to carry means of receiving transmissions from suitable radio-navigation systems throughout their intended voyage.

1.2 	The operational requirements for world-wide radio-navigation systems are given in the appendix.

1.3 	It is not considered feasible for IMO to fund a world-wide radio-navigation system. Existing and planned systems which are being provided and operated by Governments or organizations have therefore been studied, in order to ascertain the conditions under which such systems might be recognized or accepted by IMO.

2	PROCEDURES AND RESPONSIBILITIES CONCERNING THE RECOGNITION OF SYSTEMS

2.1 	Procedures and functions of IMO

2.1.1 	The recognition by IMO of a radio-navigation system would mean that the Organization recognizes that the system is capable of providing adequate position information within its coverage area and that the carriage of receiving equipment for use with the system satisfies the relevant requirements of the 1974 SOLAS Convention, as amended.

2.1.2 	IMO should not recognize a radio-navigation system without the consent of the Government or organization which has provided and is operating the system.

2.1.3 	In deciding whether or not to recognize a radio-navigation system, IMO should consider whether:

- the Government or organization providing and operating the system has stated formally that the system is operational and available for use by merchant shipping;

- its continued provision is assured;

- it is capable of providing position information within the coverage area declared by the Government or organization operating and providing the system with a performance not less than that given in the appendix;

- adequate arrangements have been made for publication of the characteristics and parameters of the system and of its status, including amendments, as necessary; and

- adequate arrangements have been made to protect the safety of navigation should it be necessary to introduce changes in the characteristics or parameters of the system that could adversely affect the performance of ship-borne receiving equipment.

2.1.4 	In deciding, in the light of any changes to a recognized system, whether the system should continue to be recognized, the criteria listed in paragraph 2.1.3 should be applied.

2.2 	Responsibilities of Governments or organizations

2.2.1 	The provision and operation of a radio-navigation system is the responsibility of the Governments or organizations concerned.

2.2.2 	Governments or organizations willing to have a radio-navigation system recognized by IMO should formally notify IMO that the system is operational and available for use by merchant shipping. The Government or organization should also declare the coverage area of the system and provide as much other information as practicable to assist IMO in its consideration of the factors identified in paragraph 2.1.3.

2.2.3 	Governments or organizations that have a system recognized by IMO should not allow changes to the operational characteristics of the system under which the system was recognized without notifying IMO (see resolution A.577(14)).

3 SHIP-BORNE RECEIVING EQUIPMENT

3.1 	To avoid the necessity of carrying more than one set of receiving equipment on a ship, the ship-borne receiving equipment should be suitable for operating either with a world-wide radio-navigation system, or with radio-navigation systems which cover the area in which the ship trades.

3.2 	Ship-borne receiving equipment should conform to the relevant performance standards not inferior to those adopted by the Organization.

3.3 	Radio-navigation systems should make it possible for ship-borne receiving equipment automatically to select the appropriate stations for determining the ship’s position with the required performance.

3.4 	Ship-borne receiving equipment should be provided with at least one output from which position information can be supplied in a standard form to other equipment.

APPENDIX

OPERATIONAL REQUIREMENTS

1 	INTRODUCTION

1.1 	The operational requirements for a world-wide radio-navigation system should be general in nature and capable of being met by a number of systems. All systems should be capable of being used by an unlimited number of ships.

1.2 	The requirements may be met by individual radio-navigation systems or by a combination of such systems.

2 	NAVIGATION IN OCEAN WATERS

2.1 	Where a radio-navigation system is used to assist in the navigation of ships in ocean waters, the system should provide positional information with an error not greater than 100 m with a probability of 95%. This degree of accuracy is suitable for purposes of general navigation and provision of position information in the GMDSS.

2.2 	In view of the fact that merchant fleets operate world-wide, the information provided by a radio-navigation system must be suitable for use for general navigation by ships engaged on international voyages in any ocean waters.

2.3 	Taking into account the radio frequency environment, the coverage of the system should be adequate to provide position-fixing throughout this phase of navigation.

2.4 	The radio-navigation system should permit an update rate of the computed position data not less than once every 2 s.

2.5 	Signal availability should exceed 99.8% calculated over a 30-day period.

2.6 	A warning of system non-availability or discontinuity should be provided to users as soon as practicable by Maritime Safety Information (MSI) systems.

3 	NAVIGATION IN THOSE HARBOUR ENTRANCES, HARBOUR APPROACHES AND COASTAL WATERS WITH A LOW VOLUME OF TRAFFIC AND/OR A LESS SIGNIFICANT DEGREE OF RISK

3.1 	Where a radio-navigation system is used to assist in the navigation of ships in such waters, the system should provide positional information with an error not greater than 10 m with a probability of 95%.

3.2 	Taking into account the radio frequency environment, the coverage of the system should be adequate to provide position-fixing throughout this phase of navigation.

3.3 	The radio-navigation system should permit an update rate of the computed position data not less than once every 2 s.

3.4  	Signal availability should exceed 99.5%, calculated over a 2-year period.

3.5 	When the system is available, the service continuity should be ≥99.97% over 15 minutes.

3.6 	A warning of system non-availability or discontinuity should be provided to users within 10 s.

4 	NAVIGATION IN THOSE HARBOUR ENTRANCES, HARBOUR APPROACHES AND COASTAL WATERS WITH A HIGH VOLUME OF TRAFFIC AND/OR SIGNIFICANT DEGREE OF RISK***

4.1 	Where a radio-navigation system is used to assist in the navigation of ships in all such waters, the system, should provide positional information with an error not greater than 10 m with a probability of 95%.

4.2 	Taking into account the radio frequency environment, the coverage of the system should be adequate to provide position-fixing throughout this phase of navigation.

4.3 	The radio-navigation system should permit an update rate of the computed position data not less than once every 2 s..

4.4 	Signal availability should exceed 99.8%, calculated over a 2-year period.

4.5 	When the system is available, the service continuity should be ≥99.97% over 15 minutes.

4.6 	A warning of system non-availability or discontinuity should be provided to users within 10 s.

 IEC publication 61162

 SOLAS regulation V/13 requires each contracting Government to provide, as it deems practical and necessary either individually or in co-operation with other contracting Governments, such aids to navigation as the volume of traffic justifies and the degree of risk requires.

 This applies to the computed and displayed position data, but not to the update rate of correction data, which remains valid for approximately 30 s.

 Calculated in accordance with guidance contained in IALA Recommendation R-121 on the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5-325 kHz.



