IALA Recommendation A-124

Annex 16

DGNSS Broadcasts from an AIS Service

# Revision

| # | Date | Description | Initials |
| --- | --- | --- | --- |
| V 0.1 | 2007-05-29 | Version 0.1 | JKJ |
| V 0.1.1 | 2007-05-30 | Added comments, and cleaned up document | AIW |
| V 0.1.2 | 2007-06-01 | Reorganised into newer template | AIW |
| V 0.1.3 | 2007-12-05 | Further editing of the document | AIW |

# Table of Contents

1	Revision	2

2	Table of Contents	3

3	Introduction	4

4	Reference	4

4.1	IMO references	4

4.2	ITU References	4

4.3	IALA References	5

4.4	IEC references	5

5	Definitions	5

6	Abbreviations	5

7	DGNSS Broadcast from an AIS Service	5

Define operating characteristics	6

8	Performance requirements for a DGNSS Service	6

8.1	Accuracy, Availability and Continuity	6

8.2	Integrity	6

8.3	AIS Requirements	7

8.3.1	VDL Loading Requirements	7

9	Functional descriptions	7

9.1	Overall System Description	7

9.1.1	DGNSS Corrections	8

9.1.2	DGNSS Corrections within the Logical Shore Station	8

9.1.3	Service Planning	9

9.1.4	Service Limitations	9

9.2	Interfacing a DGNSS Reference Station and Integrity Monitor	9

9.2.1	Overall Description	9

9.2.2	Correction Sources	10

9.2.3	Processing Multiple Sources of DGNSS Corrections	11

9.2.4	Processing Correction Data	11

9.3	Transmission Schemes	12

9.3.1	Multiple Base Stations	13

9.4	Receiving DGNSS Corrections on Ships	13

9.4.1	Priority of Received DGNSS corrections	13

9.4.2	Receiving Message 17 from Multiple AIS Base Stations	13

9.4.3	Receiving Corrections from Multiple DGNSS Systems	13

# Introduction

The purpose of Differential GNSS (DGNSS) is to increase accuracy and integrity of vessel EPFS. A base station may transmit DGNSS corrections via AIS VDL if it is connected to a DGNSS reference station. This service can be provided automatically and may be considered a back-up service for the radiobeacon DGNSS service.

This annex describes how a base station may transmit DGNSS corrections, and ensure that a consistent approach is taken whilst implementing a DGNSS service over AIS. In particular, this annex covers the use of Message 17 for the broadcast of differential corrections. Annex 14 provides guidance on how to reserve slots for Message 17. This annex does not define how the differential corrections are generated.

[Requires reference to regulations or requirements for DGNSS over AIS here]

- Basic overview of the annex including purpose, justification, and any regulations or requirements addressed by the annex.

- Position of this annex within the overall structure of A124 using the following format:

| A124 |  |
| --- | --- |
|  | General |
|  | Service Model |
|  | Design Considerations |
|  | VDL Management |
|  | Implementation, Installation, and Maintenance |
|  | Functional Components of the AIS Service - Annex 3 Interaction model of the AIS Service - Annex 9 Functional description of the LSS - Annex 10 Functional description of the PSS - Annex 11 Functional description of the ASM - Annex 16 DGNSS Broadcast from an AIS Service |

# Reference

## IMO references

[RNAV] Draft Revision of "IMO Resolution A.815 (19):1995 World wide radio navigation system"; needed for requirements of any DGNSS service provided by AIS stations

Replaced by A.953(23)

## ITU References

[M823] "Recommendation ITU-R M.823-3; Technical characteristics of differential transmissions for Global Navigation Satellite Systems from maritime radio beacons in the frequency band 283.5-315 kHz in Region 1 and 285-325 kHz in Regions 2 and 3"

## IALA References

[IALA R-121] IALA Recommendation R–121; “Recommendation on the Performance and

Monitoring of DGNSS Services in the Frequency Band 283.5 – 325 kHz”; Edition 1.1, Dec 2004

## IEC references

The IEC test standards apply to the Class A and Class B shipborne mobile AIS stations. Due to the co-operative nature of the AIS as a whole, these IEC test standards need to be considered in the present document to the extent that functionality of these mobile stations require certain AIS shore station functionality. Also, the test section of the present document was informed by the IEC test standards for Class A and Class B shipborne mobile AIS stations. In addition, the Presentation Interface message definitions contained in IEC 61162-series have been used and adapted to the needs of AIS shore stations.

- [62320-1] IEC Standard 62320-1 – AIS base stations

- [62320-2] IEC Standard 62320-2 - Aids-to-Navigation AIS Stations

- [62320-3] IEC Standard 62320-3 – Simplex repeater station  (to be developed)

- [62320-4] IEC Standard 62320-4 - General purpose Base station  (to be developed)

- [61993-2] IEC Standard 61993-2 - Class A mobile AIS stations

- [62287-1] IEC Standard 62287-1 - Class B (CSTDMA) mobile AIS stations

- [62287-2] IEC Standard 62287-2 - Class B (SOTDMA) mobile AIS stations (to be developed)

- [61108-4] ??

- [61162-series] "Maritime Navigation and Radiocommunication Equipment and Systems"

- [61162-1] "Part 1: Single talker and single listeners", as revised 2001.

- [61162-2] "Part 2: Single talker and multiple listeners, high-speed transmissions", as revised 2001.

[61162-3] "Part 3: Multiple talker and multiple listeners, high-speed network bus" (future publication).

# Definitions

Additional definitions not included in A124:

# Abbreviations

Additional abbreviations not included in A124:

RSIM – Reference Station and Integrity Monitor

# DGNSS Broadcast from an AIS Service

Body of Annex to be determined by the content however the following should be considered:

Interfaces (logical/physical) to other functional blocks or to other E-Nav services

Functional Description

Requirements to Other Components

Special Considerations

Use of information

Risks

Clearly define any data structures

Use pictorial examples whenever possible

# Define operating characteristics

# Performance requirements for a DGNSS Service

## Accuracy, Availability and Continuity

An AIS-based DGNSS service should comply with the current IMO requirements for the accuracy, availability and continuity for a World-wide Radionavigation System. These are currently described in IMO Resolution A.953(23), and summary of the requirements are given in Table 1. Also in the table is the number of stations needed to meet the IMO requirements. This is based on IALA Recommendation R-121.

| Area | Absolute horizontal accuracy (95%) | Availability | Continuity | Augmentation |
| --- | --- | --- | --- | --- |
| Ocean | ≤ 100 m | >99.8% over 30 days | N/A | None |
| Coastal/harbour with low level of risk | ≤ 10 m | >99.5% over 2 years | >99.85% over 3 hours | Single station |
| Coastal/harbour with high level of risk | ≤ 10 m | >99.8% over 2 years | >99.97% over 3 hours | 2 or more stations |

Table 1 - Accuracy, availability and continuity requirements for GNSS by IMO

At the time of writing, the definition of Availability and Continuity for an AIS service has not been defined. The definition of continuity is currently being discussed at IMO. The competent authority should define these criteria locally and ensure that their AIS DGNSS service complies with the IMO requirements using these definitions.

## Integrity

The competent authority should refer to IALA Recommendation R-121 for recommendations of implementing integrity checks of the DGNSS system. This should include a separate integrity check of the contents of the transmitted Message 17.

It should be noted that, in addition to the requirements highlighted above, the time for broadcasting an integrity warning to the user, from the time the integrity problem is

identified, should not be greater than 10 seconds. When it has been determined that the error condition no longer exists, there may be a delay, similar to that used for initially identifying the error, before the error warning is withdrawn.

## AIS Requirements

DGNSS Corrections should be transmitted over the AIS VDL using Message 17 only. In accordance with ITU-R M.1371, the surveyed position of the DGNSS reference station must be given in Message 17. In accordance with IALA Recommendation R-121, the reference station antenna should be surveyed to a 3-D accuracy of 0.2m. Corrections may be given for GPS, GLONASS, and any other future operational GNSS services (e.g. Galileo, Compass). Recommended RTCM messages to be transmitted in AIS Message 17 are:

- RTCM Messages 1 or 9 for GPS corrections

- RTCM Message 31 or 34 for GLONASS corrections

At the time of writing, it is recommended that no other RTCM Message Types be used as these will use valuable bandwidth. Future RTCM Messages Type may be relevant for transmission, although consideration must be given to legacy mobile users.

Message 17 should preferably be transmitted using FATDMA (Usage category 2 – Annex 14) with a transmission scheme that allows the system to meet the 10-second TTA requirement. It is recommended that RATDMA is not used for Message 17 transmissions due to the possibility of the message not being broadcast in time to meet the TTA requirement. The probability of a slot collision is also increased with the use of RATDMA, particularly in busy areas.

### VDL Loading Requirements

With regular, high volume data being broadcast on the AIS VDL, the potential issues regarding the loading of the VDL must be considered. The slots allocated to the service should comply with the FATDMA Plan (see Annex 14). Through trials, it has been found that most Class A stations will revert to an uncorrected position state after the age of the corrections exceed 30 seconds, and therefore, an new set of corrections should be made available every 30 seconds or less. This should not affect the 10-second TTA requirement.

# Functional descriptions

## Overall System Description

Figure 1 – DGNSS Corrections within the AIS shore-based infrastructure

The AIS shore-based infrastructure allows for DGNSS corrections to be broadcast to AIS users in the vicinity of the physical shore stations. Figure 1 shows how DGNSS corrections enter the AIS shore-based infrastructure, and that the information flows down to the VDL. The majority of the AIS shore-based infrastructure remains identical to the general structure of the AIS Service, with the ASM controlling the configuration of the LSS and PSS, and reporting back the status of the AIS Service back to the External Services.

### DGNSS Corrections

The DGNSS corrections are always provided by an External Service, and are not generated by the AIS service. Section 9.2 consider the options in terms of suitable sources of DGNSS corrections.

### DGNSS Corrections within the Logical Shore Station

The DGNSS corrections being provided to the AIS Service are in a data format defined by RTCM SC-104. The information can also be provided by the DGNSS_COR object defined in this document. The DGNSS correction data from the External Service is encapsulated in an IEC 61162 VDM sentence by the Logical Shore Station (LSS) for processing by the Physical Shore Station (PSS). The LSS ensures that the latest full set of corrections is used, and that they are transmitted at the correct time. The LSS also prioritises the DGNSS corrections so that any new integrity alarms that have been identified by the DGNSS corrections source are transmitted in the next Message 17 slot(s).

### Service Planning

The competent authority should consider the impact of providing an AIS DGNSS service on the VDL. It is not recommended that an AIS DGNSS service be deployed over a wide area (it is better to deploy a radiobeacon DGNSS service for this purpose). However, an AIS DGNSS service could be considered where it is necessary to obtain high accuracy and high integrity position fixes from vessels.

The competent authority should minimise the number of base stations providing the AIS DGNSS service. This may require careful planning using any available terrain information.

### Service Limitations

At the time of writing, a DGNSS service over AIS has several limitations in the service it is capable of providing.

On most vessels, the external GNSS is not capable of switching between an external radiobeacon DGNSS receiver and an output provided by AIS. The implication of this is that an AIS DGNSS service cannot be considered as the backup to an existing radiobeacon service and visa versa. It should be noted that a Class A and B stations has the ability to switch between an external DGNSS receiver and a Message 17 source (priority given to Message 17), although this can only be used to differentially correct the internal GNSS receiver.

There is also concern on the difference between the position being broadcast by the vessel over AIS and the position being used by the vessel’s commander. The IEC standard for Class A and B stations asks that the position being broadcast is prioritised based on the status of the internal and external position fixing systems (EPFS). If the external EPFS is differentially corrected, then this is the position broadcast and used by the vessel’s commander. However, for most vessels, the external EPFS is not differentially corrected (and may not be capable of do so in its current configuration), and so the AIS will broadcast its internal differentially corrected (by Message 17) GNSS position. The vessel’s commander will continue to use the external GNSS, and a situation will occur where other vessels are more confident of a ship’s position than the commander of the vessel.

A competent authority should be satisfied with the benefits of providing a DGNSS corrections service over AIS given the potential limitations above.

## Interfacing a DGNSS Reference Station and Integrity Monitor

### Overall Description

In order to provide the correction data to the AIS base station, a DGNSS Reference Station and Integrity Monitor (RSIM) is required. The purpose of the RSIM is to generate pseudorange corrections for the Satellite Vehicles (SV) (note: SV is usually an abbreviation for Space Vehicle) in view, and ensure that the corrected position is within limits. When the position is out-of-tolerance, the Integrity Monitor raises an alarm for the appropriate SV, and notifies users not to use it in their navigation solutions.

The AIS shore stations should be capable of using such a data stream to provide a DGNSS service over AIS. There are many ways this is possible, some of which are highlighted below. It should be noted that where the RSIM and the AIS transmitting station are physically separated, the integrity and reliability of the intermediate communications structure should be given careful consideration since any failure will cause the DGNSS service to be disrupted.

### Correction Sources

#### Independent RSIM

It is recommended that an AIS DGNSS Service relies on a dedicated RSIM that provides corrections for transmission by AIS only. This provides an independent DGNSS service which can be used as a back-up to other DGNSS services (e.g. radiobeacon DGNSS).

It should be noted that the cost to implement and operate an independent RSIM for an AIS DGNSS Service will be greater than to utilise an existing RSIM as described in the MF radiobeacon option below. It may be possible to use one central RSIM for mutliple AIS base stations, although care should be taken to ensure that spatial decorrolation is considered.

#### MF Radiobeacon

If the AIS base station is in an area covered by an MF radiobeacon service, it may be possible to use this to provide the RTCM data stream required to generate Message 17. A suitable MF receiver will be required. The output from this receiver would be passed through the RTCM interface of the LSS for further processing.

Care should be taken when utilising such a system to provide an DGNSS service over AIS because it is reliant on an external system to provide the differential data. Any failures of the radiobeacon service will be reflected almost immediately in the DGNSS service over AIS. If two or more radiobeacon services operate in the area, it may be possible to utilise them to increase availability of the AIS service. The competent authority should also ensure that the MF signal can be received reliably 24 hours a day (it is affected by skywave propagation during night hours that can cause fading). If the MF radiobeacon receiver is installed at a distance from the transmitting AIS base station, then care should be taken to ensure that spatial decorrolation of the corrections is considered. A much better solution would be a direct datalink to the RSIM at the beacon station location.

#### Data interface Guidance RTCM Input/Output in Binary Format

- The LSS interface for the input of DGNSS corrections should comply with the Data Interface Specification defined in ITU-R M.823. The information enters through a dedicated input port. The competent authority should also refer to RTCM SC-104 XXX GET CORRECT DOC XXX RTCM RECOMMENDED STANDARDS FOR DIFFERENTIAL GNSS (GLOBAL NAVIGATION SATELLITE SYSTEMS) SERVICE  Version 2.3

### Processing Multiple Sources of DGNSS Corrections

In some locations, the system may have access to more than one reference station. In this case, a decision on how to deal with them must be made in order to provide a single set of corrections for each navigation system. The base station may be configured to transmit the data for the nearest reference station. However, a more sophisticated solution would be to receive data from all sources, and interpolate according to distance from “Virtual RSIM” position. This will provide a more accurate set of corrections for vessels near the transmitting AIS base station.

### Processing Correction Data

ITU-R M.1371 provides the standard Message 17 format to be transmitted by the AIS base station. The data for the fields are generated by the LSS and directly from the data stream. Table 2 shows the data source for each field in Message 17. It should be noted that the preamble and parity bits are discarded, and will be replaced by the receiving AIS mobile stations. The header contains the Station Health message. This should NOT be discarded.

| AIS Message 17 |  | Data source |
| --- | --- | --- |
| Static data |  |  |
| Parameter | Number of Bits |  |
| Message ID | 6 | Set by AIS shore-based Service |
| Repeat Indicator | 2 | Set by AIS shore station |
| Source ID | 30 | Set by AIS shore station |
| Spare | 2 | - |
| Longitude | 18 | Set by AIS shore station (refer to remark below) |
| Latitude | 17 | Set by AIS shore station (refer to remark below) |
| Spare | 5 | - |
| Differential Correction data (header) |  |  |
| Parameter | Number of Bits |  |
| Message Type | 6 | RTCM Header, Word 1, d9-d14 |
| Station ID | 10 | RTCM Header, Word 1, d15-d24 |
| Z count | 13 | RTCM Header, Word 2, d1-d13 |
| Sequence number | 3 | RTCM Header, Word 2, d14-d16 |
| N | 5 | RTCM Header, Word 2, (d17-d21) - 2 1) |
| Health | 3 | RTCM Header, Word 2, d22-d24 |
| Differential Correction data |  |  |
| Correction Data | Up to XXX | RTCM Data, Word 3 onwards 2) |

Table 2 - Data source for fields in Message 17. Differential data to be transmitted are Words 3 onwards without the parity bits.

1)The frame length in RTCM header is two more than the number of words (N) following the header

2)16 fill bits are used for RTCM word (N+2) if number of satellites Ni = 2,5,8 or 11

The reference station position fields should be filled with the position of the differential correction source, and not the position of the transmitting AIS base station. This information is either manually configured into the transmitting Base Station, or is determined by the evaluation of the RTCM Message Type 3. If a virtual RSIM has been defined, then its position should be transmitted in Message 17.

## Transmission Schemes

When selecting a transmission scheme to provide a DGNSS service over AIS, it is important to consider the potential loading on the VDL. The FATDMA planning allows the possibility to transmit a full set of corrections (containing up to XXX SV data in a 3-slot message) approximately every15 seconds (see Annex 14). This easily meets the 30-second correction age requirement, but does not meet the 10-second TTA requirement. This requirement is met by reserving a single slot every 3 1/3 seconds (increment of 250 slots per channel) that will enable a message to be sent for a single SV. This message will simply disable the use of the SV in the navigational solution. The transmission scheme is summarised in Figure 1.

Figure 1 - Proposed FATDMA transmission scheme for transmitting DGNSS corrections over AIS for one GNSS system. The diagram shows the sequence for one frame. Not to scale.

The advantage of this scheme is that a number of GNSS systems could be serviced with the minimum additional loading on the VDL. The regular 1-slot reservations could be shared for all the GNSS systems.

It is unlikely that multiple GNSS failures will occur at the same time, and therefore, the regular 1-slot reservations could be used for all the GNSS systems. This practice will substantially reduce the VDL loading when providing a DGNSS service for more than one GNSS system. Table 3 provides a brief summary of the VDL loading for a number of GNSS systems.

| Number of GNSS Systems | Slots per frame required for full constellation data | Slots per frame required for the TTA slots | Total number of slots | VDL Loading (over two channels) |
| --- | --- | --- | --- | --- |
| 1 | 12 | 18 | 30 | 0.67% |
| 2 | 24 | 18 | 42 | 0.93% |
| 3 | 36 | 18 | 54 | 1.20% |

Table 3 - Loading on the VDL when one base station provides DGNSS service for multiple GNSS systems.

### Multiple Base Stations

[It is envisaged that in some areas, the coverage of base stations providing DGNSS over AIS would overlap. From a time-slot reservation point of view, it is better for each base station to provide their own TTA slot reservations, using different time slots. The implication of this is that the VDL loading will increase linearly with the amount of base stations in the local area. It is up to the competent authority to ensure that the loading of the VDL is not adversely affected.

If Base Stations are transmitting different sets of corrections for the same GNSS system, then the competent authority should be aware that vessel positions may shift slightly due to spatial decorrelation as different sets of corrections are applied to the GNSS unit.] Harmonise with Annex 3 and 14