e-NAV5/9/11

St Johns Output 10

IALA Recommendation A-124

Annex 17

Channel management by an AIS Service

# Revision

| # | Date | Description | Initials |
| --- | --- | --- | --- |

# Table of Contents

# Introduction

Basic overview of the annex including purpose, justification, and any regulations or requirements addressed by the annex.

This annex is a part of the description of the VDL usage and VDL management functions allowed by an AIS Service and gives a description of the channel management.

Position of this annex within the overall structure of A124:

| A124 |  |
| --- | --- |
|  | General |
|  | Service Model |
|  | Design Considerations of the AIS Service |
|  | Functional Components of the AIS Service |
|  | Implementation, Installation, and Maintenance |
|  | VDL usage and VDL management - Annex 14 FATDMA planning and operation of an AIS Service - Annex 15 Assigned mode operation of the AIS Service - Annex 16 DGNSS broadcast from the AIS Service - Annex 17 Channel management by an AIS Service - Annex 18 General loading management by an AIS Service |

Every aspect of channel management is not well understood at this stage of the development of the AIS. This chapter provides an introduction to Channel Management for competent authorities. Some recommendations will be derived at the end of the chapter.

A section introducing most fundamental definitions and concepts is presented first, This introduction starts with the mobile AIS station and assumes that it is this station has to operate in accordance with regional operating settings that are different from the default operating settings. It then moves on to the consequential requirements for the shore infrastructure set up by the competent authority.

The justification for this annex is to describe the potential for altering the VDL characteristics of mobile AIS equipment, the implications and the reasons why competent authorities may wish to do so.

# Reference

As defined by A124 and in addition:

# Definitions

As defined by A124 and in addition:

# Abbreviations

As defined by A124 and in addition:

# Channel management

## Fundamental concepts

In response to a request from IMO seeking global channels for AIS, the ITU designated two worldwide channels from the VHF maritime mobile band for this purpose (refer to ITU-R Radio Regulations (RR) Appendix S18).  The channels are AIS1 – No. 2087 (161.975 MHz) and AIS2 – No. 2088 (162.025 MHz) - with 25 kHz bandwidth, and in accordance with Recommendation ITU-R M.1084. Two channels were selected to increase capacity and mitigate Radio Frequency (RF) interference. AIS1 is the “primary channel” and AIS2 the “secondary channel” in “high seas” areas. This distinction will become relevant when considering some details of the transition between regions.

By default every mobile AIS station operates on these two channels, AIS 1 and AIS 2, as defined in Recommendation ITU-R M1371-3. A mobile AIS station is thus capable of receiving two messages, from two different stations concurrently, provided that it does not transmit at the same time. Every mobile AIS station transmits at its “nominal reporting rate”. This nominal reporting rate is given in  ITU-R M.1371-3, §4.2.1. Each of the two channels by default is used to transmit scheduled transmissions, such as autonomous and continuous position reports, at half of the “nominal reporting rate.” E. g. a Class A shipborne mobile AIS station moving at a speed of more than 14 knots is supposed to report its position in intervals of 2 seconds. Therefore the nominal reporting rate would be once per two seconds. This means, that each of the two channels AIS1 and AIS2 will receive a scheduled position report from this mobile AIS station once every four seconds, i. e. at half the rate of the nominal reporting rate. To understand this fact is crucial for the understanding of the impact of channel management on reporting rates. This behaviour is called dual channel operation.

### Reasons for Channel Management

The ITU also provided for administrations to designate “regional frequency channels for AIS” where channels 2087 and 2088 are unavailable and, if necessary, to derive new S18 channels using Recommendation ITU-R M.1084 (simplex use of duplex channels and/or 12.5 kHz narrow-band channels).

In addition channel management may be used to mitigate throughput breakdown caused by (local) RF interference, or blocking of one or both of the default operating channels by locally switching over to alternate operating channels.

A channel management scheme is also required when duplex AIS repeaters are used. The justification for duplex repeating may be derived from local unavailability of the default operating channels.

### Parameters subject to channel management and their default settings

The following operating parameters (of any mobile AIS station) may be changed by channel management. Also their default setting and range of possible settings are given (see Recommendation ITU-R M.1371-3, Annex 2, §4.1):

- frequency / nominal bandwidth of the primary operating channel = channel A (as expressed by channel number in accordance with Recommendation ITU-R M.1084).
Default: channel number 2087 = AIS1, 25 kHz bandwidth
Possible range: all channels of 25kHz or 12.5kHz nominal bandwidth which can be identified by a channel number given in Recommendation ITU-R M.1084. Local possibilities may be depending on regulatory considerations.

- frequency / nominal bandwidth of the secondary operating channel = channel B.
Default: channel number 2088 = AIS2, 25 kHz bandwidth 
Possible range: refer to primary operating channel

- Transmitter power level setting

- Default: high power level setting 
Possible range: low power level setting = 2W; high power level settings = 12.5W

- Transmit/Receive mode:
Default: Dual channel operation (receive on both channels A and B simultaneously; transmit on channels A and B alternately, using half the nominal reporting rate on both channels A and B. Assigned mode may change the reporting rate of a mobile station without affecting the use of dual channels alternately.) = TxA/TxB; RxA/RxB
Possible range:
(TxA/TxB, RxA/RxB); 
(TxA, RxA/RxB) = transmit only on primary operating channel (channel A) and receive on both channels simultaneously (while not transmitting);
(TxB, RxA/RxB) = transmit only on secondary operating channel (channel B) and receive on both channels simultaneously (while not transmitting);

- Narrow-band mode for primary channel A: 
Default: nominal bandwidth as specified by channel number (see above)
Possible range: nominal bandwidth as specified by channel number; reduced to 12.5 kHz (when channel number designates a channel with 25kHz nominal bandwidth)
Note: It should be observed that the optional use of 12.5 kHz bandwidth reduces somewhat the receiver sensitivity and FM discrimination (slot sharing capability), but still gives the required transmission rate of 9600 bits per second.

- Narrow-band mode for secondary channel B: refer to Narrow-band mode for primary channel A

- Transition zone size:
Default: 5 nautical miles
Possible range: 1 to 8 nautical miles in steps of 1 nautical mile

- Addressed mode: A base station can command specific channel management behaviour, using the above parameters, to an individual mobile AIS station.

- Geographical region: Region defining latitudes and longitudes - not only do these values establish the location and size of the region, they are also used to identify the region and the station characteristics data for that region. See below.

Channel management is performed, when mobile AIS stations are switched from their default operating settings to any different operating setting, which may only differ in one parameter.

The whole set of channel management settings and region specification is called “regional operating settings”.

The above list highlights that channel management is not just about changing operating frequencies. Channel management should rather be understood as a walk in a multi-dimensional channel parameter space. The complexity of channel management results from the many inter-dependencies between these parameters.

### The definition of a region and its transitional zone

Depending upon the geographic location of the station and the regional operating settings stored in memory, a mobile AIS station is supposed to automatically change several fundamental operating characteristics from their default values.  This feature is designed into the operation of every mobile AIS station.  This feature allows local authorities to automatically manage mobile AIS stations' use of the VHF marine band.

Channel management data is organised by geographic regions.  This section describes how these geographic regions are defined. In fact, channel management in effect is only possible within precisely defined geographical regions. This means, that any channel management operation applies to the specified region, only. Mobile stations outside all regions, to which channel management settings apply, will either revert to, or continue to use the default settings.

The regional operating areas are designated by a Mercator projection rectangle with two reference points using WGS84 datum. The first reference point is the geographical co-ordinate address of the north-eastern corner (to the nearest tenth of a minute) and the second point is the geographical co-ordinate address of the south-western corner (to the nearest tenth of a minute) of the rectangle. Since it is an area on the curved surface of the earth, a region is not a true rectangle in shape.  The sides of the region follow either constant latitude or longitude lines.  A rectangular representation of this area is shown in Figure 1.

Figure 1 Rectangular representation of a region and its components

Inside every region's boundary there exists a "transition zone."  The width of the transition zone is one of the parameters that is used to define the region. The transition zone for a region is the area between the "Transition Zone Boundary" and "Region Boundary" as shown in Figure 1. The transition zone width is specified in increments of one nautical mile and range of one to eight nautical miles. If no value is given in the region's definition, the default width is five nautical miles. The zone size is the same on the 4 inside borders of a region.

While the transition zone is expressively defined in ITU-R M.1371-3, the portion of the region that is not inside the transition zone is not given a special name.  In order to simplify the discussion below, the term "Inner Region" will be used to refer to this area. This means that the area of a region can be described as being equal to the area of the transition zone plus the area of the inner region.

When the mobile AIS station receives different channel management commands for the same geographical region, the latest information received will be used in accordance with an algorithm described below.

Any Class A shipborne mobile AIS station can internally store information for eight different regions. This gives the possibility to “download” to Class A shipborne mobile AIS stations information for several regions (for example covering inland waterways) from one shore station.

The AIS automatically changes to a transitional mode of operation when it is within the specified transitional zone which surrounds the region boundaries. In this zone ships transmit and receive on one of the channels for the area it is leaving and one of the channels for the area it is entering. Within the transitional zone, the reporting rate will be the nominal reporting rate for both channels (as opposed to just half the nominal reporting rate in each individual channel during default dual channel operation).

Within the transitional zone, Class A shipborne mobile AIS stations will ignore any assignment of higher nominal reporting rates by shore stations. This guarantees that the broadcasts of mobile AIS stations operating in the transition zone will be received at nominal reporting rate for the benefit of other mobile stations in the immediate vicinity of that station.

The precise details of the behaviour of a mobile AIS station within a transition zone are described below.

### A Region's relationship to the high seas (or default) region

The "high seas" or default region has the primary channel of AIS1, and the secondary channel of AIS2 with a bandwidth of 25 kHz on both channels.  The size of the transition zone is 5 nautical miles.  The power level is "high power" and all of the mobile AIS station‘s receivers and the frequency-agile transmitter are used.  Figure 2 shows the relationship of Region X from Figure 1, and the high seas region.

Figure 2 Relationship of region X to the High Seas Region

Figure 3 Breakdown of operating zones for two adjacent regions

The high seas region's regional operating settings are all equal to the default values.  They cannot be changed by channel management. Channel management data can only create additional regions that exist within the high seas region. Wherever a defined region is created, if not adjacent to another defined region, this relationship with the high seas region is automatically created.

### Two Regions' relationship including the High Seas Region

If two regions are defined with either a latitude or longitude boundary exactly the same, to 1/10 minute, the two regions exist adjacent to each other (see the common side of Region X and Region Y in Figure 3).  If the boundary of a region falls inside the boundary of another region, the regions conflict and a decision about which region to use must be made by the mobile AIS station.  Recommendation ITU-R M.137113, Annex 2, § 4.1.8, states, "The most current and applicable commands received should override previous channel management commands." Applying this rule would mean the most current regional definition would apply and the older definition ignored.  This also implies that the data and time that a region's data is received should be retained along with the region's data.

Figure 3 shows two adjacent regions, Region X and Region Y, surrounded by the high seas region.  The Region X transition zone is further broken down into zone 1 and zone 2.  Zone 1 is the portion of the Region X transition zone nearest to the boundary in common with Region Y.  Zone 2 is the portion of the Region X transition zone nearest to the boundary in common with the high seas region.  Recognition, that the transition zone may be sub-divided depending on the relationship it has with adjacent regions, is important to understanding how a mobile AIS station will safely operate as it travels between and among regions that use different radio frequencies, signal bandwidths, power, etc.  In a similar fashion, the Region Y transition zone is broken down into zone 3 and zone 4; and the high seas region's transition zone is broken down into zone 5 and zone 6.

How a mobile AIS station will safely operate as it travels between and among regions will be described in detail below, using the above definitions.

### IMO requires maximum extent of automated channel management

The AIS is designed as an automatic system. Accordingly, even operating conditions that differ from the default, such as channel management should be automated as much as possible. This is in full accordance with the appropriate IMO guidance (refer to IMO/NAV47 “Draft Liaison Statement to ITU-R Working Party 8B”, which is Annex 15 of document NAV47/13, 26 July 2001, paragraph 1.2):

“IMO notes that there may be areas where alternative frequencies are in use but where no base stations exist. This should be an unusual situation, however where it exists, information should be available to all ships sailing in these areas. Therefore, IMO requests that all Administrations notify IMO of these areas for the circulation by the appropriate IMO circulars as well as promulgate this information to shipping in these areas by a suitable means. Also IMO recognises that from the viewpoint of avoiding accidents due to human error, automatic switching should be the normal procedure and manual switching should be limited to specific purposes such as maintenance for the equipment.”

This statement clearly applies to both shipboard and shore side.

### Overview on means for automatic and manual channel management

Automatic switching of regional operating settings for mobile AIS stations can be done by one of the three means described below. Automatic switching is considered a safer way than manual switching. Among the automatic means, there are two ways to switch regional operating settings from the shore by the competent authority and one automated way to switch regional operating settings on board the ship.

A competent authority may set up AIS shore stations utilising the internal BAS “VDL channel management”, that uses Message 22 (“Channel Management message”) and that gives information of region boundaries, channels and other parameters to be used within the region.   The internal BAS “VDL channel management” also provides a service that requests the DSC service to transmit a channel management command via DSC Channel 70.

In addition, a shipborne information system, which may be connected to the AIS, may input regional operating settings to the AIS. This information may be derived automatically from i.e. a database or from a manual input to this shipborne information system.

Manual channel switching by the AIS operator on the ship can be performed via any suitable interface. The Class A shipborne mobile AIS station provides for manual inputs via the Minimum Keyboard and Display.

Manual channel switching should be avoided if possible, in normal operation and should only be based on information issued by the competent authority of that region (see IMO statement above).

The database of the shipborne information system may not fully reflect the current regional operating settings required for the particular region, or the manual input may be flawed. Therefore, regional operating settings received from a shipborne information system or by manual input, will not be accepted by a Class A shipborne mobile AIS station, if a regional operating setting was received for the same region from a shore station recently.

### Channel management as a privilege and as a responsibility for competent authorities

Since the exchange of navigational data between ships and between ship and shore can only be done when both the transmitting and the receiving station use the same channel with compatible operating settings, a wrong operating setting in just one mobile AIS station may result in it being “invisible” as far as the AIS service is concerned. This may be a safety issue. Therefore, ITU only allows competent authorities to undertake channel management.

Since channel management is one of the most complex of the AIS functionalities and since channel management, when done wrongly, can be potentially hazardous to safety, it is strongly recommended that every competent authority should consider the use of channel management carefully before implementing it. As a consequence, careful planning is required before a Competent Authority implements a Channel Management scheme.

## Channel management commands to a Class A shipborne mobile AIS station

All Class A shipborne mobile AIS stations are using the following algorithm to keep the internal eight store memory up to date and to accept new regional operating settings. This algorithm, in general, has three different stages:

- continuous checking of store regional operating settings, and possibly automatic erasure of remote or old settings.

- checking of input before accepting it as new regional operating settings. It should be noted, that this is an exception from one fundamental concept of shipborne equipment design, i. e. that the receiving device normally does not check the data it is receiving. For example, the mobile AIS station does not check the sensor data it receives for reporting.

- operations performed after a new regional operating setting has been accepted.

In detail the Class A shipborne mobile AIS station will perform the following steps (in the following order, if applicable):

- All stored regional operating settings will be time/date-tagged and they will be tagged with information by what input means this regional operating setting was received onboard (via AIS VDL, DSC telecommand, Manual input via Minimum Keyboard and Display (MKD), input via Presentation Interface).

- The Class A shipborne mobile AIS station constantly checks, if the nearest boundary of the region of any stored regional operating setting is more than 500 nautical miles away from its current position, or if any stored regional operating setting was older than five weeks. Any stored regional operating setting which fulfils any one of these conditions will be erased from the memory. This means, that the AIS station automatically “forgets”.

- Any regional operating settings will be handled as a whole, i. e. a change requested for any parameter of the regional operating settings will be interpreted as a new regional operating setting input to the device.

- When the mariner requests to manually input a regional operating setting via the MKD, the regional operating settings in use, which may be the default operating settings, will be presented to the user on the MKD. The mariner will then be allowed to edit these settings partly or in full. The Class A shipborne mobile AIS station will always ensure that a manually input region conforms to the most fundamental rules for regions. After completion of input of an acceptable regional operating settings set, the Class A shipborne AIS station will require the mariner to confirm a second time that the input data shall be stored and possibly used instantaneously.

- Regardless of means of input, automatic or manual, the Class A shipborne AIS station will ignore any new regional operating setting which includes a region, which does not conform to the most fundamental rules for regions. In addition, it will not accept a new regional operating setting, which was input to it via its Presentation Interface, if the area of this new regional operating setting partly or totally overlaps the areas of any of the stored regional operating settings, which were received from a base station via AIS VDL or by DSC telecommand within the last two hours.

- A channel management command or a DSC telecommand addressed to one individual Class A shipborne mobile AIS station will be accepted only if that station is in a region defined by one of the stored regional operating settings. In this case the set of regional operating settings will be composed by combining the received parameters with the stored region in use. A channel management command addressed to an individual Class A shipborne mobile AIS station will not be accepted for the high seas area.

- If the area of the new, accepted regional operating setting partly or totally overlaps the area of one or more older regional operating settings, this or these older regions will be erased instantaneously from the memory (“overlap” rule). The region of the new, accepted regional operating setting may be adjacent and may thus have the same boundaries as older regional operating settings. This will not lead to the erasure of the older regional operating settings.

- Subsequently the Class A shipborne mobile AIS station will store a new, accepted regional operating setting in one free memory location of the eight memories for regional operating settings. If there is no free memory location, the oldest regional operating setting will be replaced by the new, accepted one.

- No means other then defined herein are allowed to clear any or all of the stored regional operating settings of the Class A shipborne AIS station. In particular, it is not possible to solely clear any or all of the stored regional operating settings by a manual input via the MKD or by an input via the Presentation Interface without input of a new regional operating setting.

There is not yet a similar algorithm developed for other classes of mobile equipment, except the most fundamental rules in Recommendation ITU-R M.1371-3. The IEC has begun work on Class B standardisation, however. IALA will begin work on A-to-N AIS stations in 2002.

## Behavior of a shipborne mobile AIS station entering or moving in a channel management scheme

When a mobile AIS station enters a transition zone, it changes operation to the "two-channel transitional operating mode."  How the mobile AIS station operates among zones and regions will now be discussed.  In particular, how the mobile AIS station schedules periodic repeated messages.

"Two-channel transitional operating mode" is entered when there is a change of either operating frequency or bandwidth, or both, from one region to the next.  This mode of operation begins when a transition zone is entered.  There are three exceptions to this rule:

1.	If the regional operating settings in both regions use the same channels and bandwidths, operation in the "two-channel transitional operating mode" is not necessary.

2.	If the primary channel is the same in both regions, the mobile AIS station should use only that single primary channel at nominal reporting rate while operating in the "two-channel transitional operating mode”.

3.	If the primary channel in one region is the same as the secondary channel in the other region and that secondary channel is a simplex channel, the mobile AIS station should use only that channel at the nominal reporting rate while operating in the "two-channel transitional operating mode."

### Description of mobile AIS station operation in the "two-channel transitional operating mode"

The following describes the operation of a mobile AIS station as it moves between Regions X and Y through zones 1 and 3 of Figure 3. The possible movements of the station are shown in Figure 4 with notes describing changes to the station’s operation.

REGION X

Channel A (primary) = Ch-1

Channel B (secondary) = Ch-2

REGION Y

Channel A (primary) = Ch-3

Channel B (secondary) = Ch-4

WHEN STATION EXITS

TRANSITION ZONE IN REGION X

Rate on Ch-1 returns to NRR/2.

Reports on Ch-3 are discontinued

Operation on Ch-2 returns to NRR/2

WHEN STATION EXITS

TRANSITION ZONE IN REGION Y

Rate on Ch-3 returns to NRR/2

Reports on Ch-1 are discontinued

Operation on Ch-4 begins

REGION X

TRANSITION ZONE BOUNDARY

REGION Y

TRANSITION ZONE BOUNDARY

REGION X / Y

BOUNDARY

WHEN STATION ENTERS

TRANSITION ZONE IN REGION X

Report rate on Ch-1 changes to NRR.

Reporting on Ch-2 is discontinued

Operation on Ch-3 changes to NRR.

Two-channel transitional

operating mode

WHEN STATION ENTERS

TRANSITION ZONE IN REGION Y

Report rate on Ch-3 at NRR

Reporting on Ch-4 is discontinued

Operation on Ch-1 at NRR

WHEN STATION CROSSES

REGIONS' BOUNDARIES

The internal designations of

the station's primary and

secondary channels are

switched.

Legend:

NRR

= nominal reporting rate

NRR/2

= half of nominal reporting rate

Figure 4 Expected operation of a mobile AIS station in "two-channel transitional operating mode"

Upon entering the transition zones between two regions, the mobile AIS station should switch to the "two-channel transitional operating mode" by:

- Switching the secondary receiver channel to the primary channel of the adjacent region and begin initialisation phase, while

- Continuing to broadcast and receive on the primary channel (channel A) for the occupied region for one minute, and broadcast on the secondary channel (channel B) for one minute closing out the previously reserved slots.

- Increasing the reporting rate on channel A to the nominal reporting rate, and after the first minute,

- Beginning broadcasts on the adjacent region's primary channel  - at the nominal reporting rate.

Note:	While inside the transition zones, the mobile AIS station's primary channel (channel A) is defined as the primary channel for the region that the mobile AIS station is inside at any given moment.  The mobile AIS station should use this rule to change the primary channel, as needed, while it is in the "two-channel transitional operating mode."  While inside the transition zones, the mobile AIS station's secondary channel (channel B) is defined to be the primary channel (channel A) of the nearest adjacent region.  This rule should be used, as needed, to change the secondary channel while the mobile AIS station is in the "two-channel transitional operating mode."

Upon exit of the transition zones, the mobile AIS station should discontinue "two-channel transitional operating mode" by:

- Switching the secondary receiver channel to the secondary channel of the occupied region and begin initialisation phase,

- Decreasing the reporting rate on the primary channel (channel A) for the occupied region to half of the normal channel reporting rate (this will take about one minute), and broadcast on the secondary channel (channel B) for one minute closing out the previously reserved slots, then

- Beginning broadcast on the secondary channel - at half of the nominal reporting rate.

### Operation of a mobile AIS station moving between and among three regions

The description of mobile AIS station operation during movement between two regions can be used to describe more complex relationships.  As described above, once a mobile AIS station enters the transition zone, it operates at nominal reporting rate on both the primary channel for the region it is in, and the primary channel for the next closest region.  This is the rule used to draw the zones of Figure 5. Figure 5 contains both Figure 3 and a table indicating the proper operating channels for each of the zones in the figure. In this example the third region is the high seas region.

| Location of mobile AIS station | Primary channel | Primary channel reporting rate | Secondary channel | Secondary channel reporting rate |
| --- | --- | --- | --- | --- |
| ZONE 1 | Ch-1 | Double | Ch-3 | Double |
| ZONE 2 | Ch-1 | Double | AIS1 | Double |
| ZONE 3 | Ch-3 | Double | Ch-1 | Double |
| ZONE 4 | Ch-3 | Double | AIS1 | Double |
| ZONE 5 | AIS1 | Double | Ch-1 | Double |
| ZONE 6 | AIS1 | Double | Ch-3 | Double |

Figure 5 Channel selection for transition zone operation

Using the table in Figure 5 and the concepts described in Figure 4, the operation of a mobile AIS station can be described for ane possibls movements between and among the three regions of Figure 5. For example, Table 1 shows the channel selection and reporting rates during mobile AIS station movement between the regions described bey"):

- Start in Region X.

- Enter zone 1

- Cross into zone 3.

- Cross into zone 4.

- Cross into zone 6

- Exit into high seas region

| Step | Movement of mobile AIS station | Primary channel | Primary channel reporting rate | Secondary channel | Secondary channel reporting rate |
| --- | --- | --- | --- | --- | --- |
| 1 | REGION X | Ch-1 | Nominal | Ch-2 | Nominal |
| 2 | ZONE 1 | Ch-1 | Double | Ch-3 | Double |
| 3 | ZONE 3 | Ch-3 | Double | Ch-1 | Double |
| 4 | ZONE 4 | Ch-3 | Double | AIS1 | Double |
| 5 | ZONE 6 | AIS1 | Double | Ch-3 | Double |
| 6 | HIGH SEAS REGION | AIS1 | Nominal | AIS2 | Nominal |

Table 1 Example of channel selection and reporting rates during mobile AIS station movement between regions

Other examples can be similarly constructed and analyzed.  Under all combinations and circumstances, the station must have one channel in common with nearby stations.

### Single-channel operation

The possibility of single-channel AIS operation is briefly addressed under the parameter Tx/Rx Mode.  Little, if any, information is given elsewhere in the ITU recommendations.  Conditions that may warrant single-channel operation include limited, continued operation after a duplex repeater failure, areas with severe spectrum limitations, and remote waterways with little marine traffic.

If the Region Y secondary channel (Ch-4) is not defined, Figure 5 can be used to describe how the mobile AIS station should operate as it moves from any location in Figure 5 into the inner region of Region Y.  For example, Table 2 shows channel selection and reporting rates during mobile AIS station movement between region X and region Y, when region Y secondary channel is not defin"):

- Start in Region X

- Enter zone 1

- Cross into zone 3

- Exit into Region Y's inner region

| Step | Movement of mobile AIS station | Primary channel | Primary channel reporting rate | Secondary channel | Secondary channel reporting rate |
| --- | --- | --- | --- | --- | --- |
| 1 | REGION X | Ch-1 | Nominal | Ch-2 | Nominal |
| 2 | ZONE 1 | Ch-1 | Double | Ch-3 | Double |
| 3 | ZONE 3 | Ch-3 | Double | Ch-1 | Double |
| 4 | REGION Y | Ch-3 | Nominal | None | N/A |

Table 2 Example of channel selection and reporting rates during mobile AIS station movement between region X and region Y, when region Y secondary channel is not defined

## Requirements and recommendations for competent authorities with regard to channel management

After this description on how the mobile stations operate within a given frequency management scheme, it becomes obvious, that the regional operating settings should be set up by the competent authority in such a way that the transition between the different regions is safe. The onus for safe and proper channel management is on the competent authority.

### Fundamental layout rules for when planning regions

In order to fulfil this requirement the competent authority should make sure that the following conditions are met (requirements taken directly from Recommendation ITU-R M.1371-3 or inferred from description above):

1.	For practical reasons, in order to provide safe transitions between regions, the regions should not be smaller than 20 nautical miles but not larger than 200 nautical miles on any boundary side. (from ITU-R M.1371-3)

2.	The boundaries of adjacent regions need to be identical. A gap between adjacent regions, however small, will be interpreted by mobile AIS stations as a combination of three regions in total: two regions with regional operating settings different from default, separated by the “high seas” region. Therefore, the mobile station will enter the transition zone behaviour of three different regions when moving from one region to the other through the “high seas” region.

Region A

Region B

Region A

Region B

Unacceptable regional boundary definition

Acceptable regional boundary definition

Region A

Region B

Region A

Region B

Unacceptable regional boundary definition

Acceptable regional boundary definition

Figure 6

3.	The distance between regions, which are meant to be neighbouring, but not to be strictly adjacent, should be at least the sum of the size of the transition zones of the two regions plus one nautical mile.

Region A

Unacceptable regional boundary definition

Acceptable regional boundary definition

Region B

Region A

Region B

Figure 7

Having more than three adjacent regions at any regional boundary intersection must be avoided. When there are three or more adjacent regions, the minimum distance between the first and the second adjacent corners on one side and any other corner should be at least 8 nautical miles (which is the maximum size of a transition zone). This 8-miles-distance rule will be checked by the Class A shipborne mobile station before accepting a new regional operating setting.

Figure 8

The region rules and general operation of the mobile AIS station are designed to successfully operate under the "worse case" conditions where every channel in every region is different.  Although the AIS technology and methods adequately deal with these possible conditions, the actual application of channel management should recognise that AIS channel management is the safest when the primary channel of the regions is either AIS1 or AIS2.

### Channel management by automatic means, i. e. by base stations

It is strongly recommended – in full accordance with the IMO guidance cited above - that all areas where the default operating settings does not apply should be covered by AIS shore stations utilising  the internal BAS “VDL channel management”, that uses Message 22 (“Channel Management message”), and that gives information of region boundaries, channels and other parameters to be used within the region.   The same BAS also provides a service to transmit a channel management command via DSC Channel 70.

Base Stations that provide service in regional operating areas should use one of the above Basic AIS Services also for all regions surrounding the actual regional areas.

It is recommended that Message 22 should be used by a competent authority for all regions at least every 10 minutes. The actual rate should depend upon the speed of vessels within regional transition zones.

With regard to transmitting a channel management command via DSC Channel 70, similar considerations should take place. It should be noted, however, that the ITU restricts the use of DSC Channel 70 for AIS related purposes, such as channel management, to 0.0375 Erlang in total (refer to Recommendation ITU-R M.1371-3, Annex 3, §1.3). Therefore, DSC may not be adequate for all situations where channel management is required.

### Change of regional operating settings over time

The change of regional operating settings over time may be required in some situations. Although this potential of the AIS needs much more investigation, some fundamental principles can be stated already.

#### Fundamental rules

Since the Class A mobile AIS station stores regional operating settings for up to five weeks, carefully planned overwriting of the older, stored regional operating settings is needed when there are changes required for the same area over time.

Channel management regions should only be changed – for safety reasons - over a longer period of time, i. e. in the order of half-hours instead of minutes.

The steps should be carefully planned beforehand, and the effects of any change should be well understood before implementing it. After one change step the system should be allowed to return to a stationary mode, i. e. all transitional states should have had time to subside. When there is a need to change operating frequencies within a region, there should be a minimum time period of 9 minutes after the first operating frequency has been changed before the second operating frequency is changed (refer to Recommendation ITU-R M.1371-3, Annex 2, §4.1.9).

Base stations should never change two channels (A and B) simultaneously within any region.

#### Preliminary procedures to change regions over time: Changes in region boundaries

Two possible changes of regional boundaries apply:

A region will be deleted, i. e. operation is intended to return to default
If the competent authority determines, that a region should no longer exist as a defined region, and the settings of that region should be changed to the default settings, the procedure as described as follows should be applied for that particular region: A channel management command must be transmitted by a base station to change the first operational frequency and all other parameters except the second operating frequency set to the default values, followed by a second channel management message after at least 9 minutes to change the second operational frequency. The region (geographical area) should be identical in size and location as the region to be deleted. If all AIS stations in that region are using the new operational frequencies, periodical channel management messages are no longer needed if the new operational settings are all equal to the default settings. The regional operating settings, which are stored in the memory of the Class A shipborne mobile AIS stations, will then be deleted in accordance with the algorithm given above, i. e. after five weeks latest.

- A region will be moved or a new region will overlap the current region.

Old region

High seas region

New region

Figure 9

The picture illustrates the possible movement of a region from the old position to the new position. The same applies if a new region overlaps the old region. Any new region overwrites the stored old region as a whole immediately (refer to description of memory algorithm above). Therefore, the change from old to new could be accomplished in one step, i. e. all mobiles, which have been in an old region and are no longer in any specifically region – because the old region has been overwritten – would automatically be in “high seas” and would thus use default settings for all parameters immediately.

While this may be desired with some parameters other than operating frequency or just one operating frequency to be returned to default, this would not be a safe procedure, if more than one operating frequency different from AIS1 and AIS2 would need to return to default. Instead a step-by-step approach should be planned as explained before.

## Duplex Repeaters

A duplex repeater and the use of channel management regions can provide safe passage for ships transiting between adjacent regions where different frequencies are used, since each ship is using at least one of the same frequencies as each other ship in its region or service area plus one of the same frequencies as each other ship within the adjacent region.  It is essential that one channel (channel A, or the primary channel) always be designated as a simplex ship-ship channel in every region.

If duplex repeaters are used, base stations should ensure that the same simplex channel is always selected as the primary channel (channel A) in every regional area.

## Future work to be added at a later date

Further work is needed in particular in the following areas:

- More elaborate planning criteria for reasons to go into channel management, at all. We need to elaborate all the pitfalls with regard to channel management.

- Describe the measures for safe behaviour of other classes of AIS stations (class B, AtoN, SAR, AIS SART, base station) in channel management environments.

- Elaborate procedures for change of regional operating settings over time by competent authorities

- Criteria for real-time decision making for changing operating frequencies due to e. g. RF interference / jamming

- Use and effect of duplex repeating on channel management scenarios.

- Consider the potential to use the concept of a universal geographical grid scheme (as per the FATDMA planning recommendations), for channel management regions. This would eliminate overlap of defined regions, and reduce potential for conflicting channel management commands within the same geographical area. Guidance on “cells” that overlap 2 or more countries’ EEZs for coordination etc.

Note:  There is a slight ambiguity at the corners of the High Seas Transition Zone Boundary.  Figure 2 is drawn using the 5 nautical mile distance from the region X boundary.  Some equipment designers may use lines of constant latitude and longitude that are 5 nautical miles from the region X boundary.  This would result in 90 degree corners rather than constant radius corners.  These optional interpretations do not represent a significant operational safety issue.  Because accurate curved lines are difficult to draw, the remaining diagrams will use "square" corners on the high seas transition zone boundary - with this ambiguity implied.