e-Navigation Committee							e-NAV 5/9/17

5th Session									15 Sept 2008

Definition of AIS AtoN Availability

Introduction

This paper is developed from a submission to IALA e-NAV 3, by the General Lighthouse Authorities and suggests a definition of availability for the AIS AtoN service. Availability should be specified for AtoNs (IALA Navguide 6.1.2 Level of Service), therefore it needs to be defined and a procedure for its measurement agreed.

The Committee is invited to consider whether the definition provided in this document is suitable for inclusion in the revision of Recommendation A-126.

AIS AtoN service availability definition

The proposed AIS AtoN service availability definition is as follows:

The AIS AtoN service shall have a service availability corresponding to IALA Category 1, 2, or 3 (depending on the importance of AtoN) for the intended transmissions1  where the proposed AIS AtoN service definition is:

The transmission of the following correct information (message 21) relating to all AtoNs identified as AIS AtoNs:

- - The type of AtoN.

- - The name of the AtoN.

- - A valid 2D position of the AtoN within the accuracy indicated by the position accuracy indicator.

- - A position accuracy indicator.

- - Type of position fixing device.

- - Off position indicator.

- - Time stamp.

- - Dimensions of the AtoN and reference positions.

- - Virtual AtoN flag.

- - RAIM flag.

1 Note that the service availability shall be calculated as a 3 year rolling average for all AIS AtoN.

The AIS AtoN transmissions containing the AtoN information shall have a signal level of greater than or equal to –[107]dBm when measured at the air-antenna interface of the user’s receiver2 within the following coverage areas:

- 5-10 n.m. of the AIS AtoN for floating AtoN depending on height of AtoN.

- 10-25 n.m. of the AIS AtoN for fixed AtoNs depending of height of AtoN.

2 Expected signal strength can be computed for a suitable measuring point using standard propagation calculations