| e-NAV Committee 5th Session Agenda item 12 | e-NAV5 / 12 / 03 September 15 -19 , 2008 |
| --- | --- |

Proposed revisions to the IALA Navguide and Recommendation R-121

to take account of Galileo

Submitted by Trinity House on behalf of the GARMIS Project on the maritime standardisation of Galileo.

The proposed revisions are contained in the following annexes:

Annex 1. IALA Aids to Navigation Guide (NAVGUIDE)

Annex 2. IALA Recommendation R-121 on the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5 kHz, Edition 1.1, December 2004

ANNEX 1 - NAVGUIDE Comments

| Section & Page Number | Relevance of Galileo |  | Suggested amendment to take account of the introduction of Galileo |
| --- | --- | --- | --- |
| Section 2.1: Navigational Methods p16 |  | Under the radionavigation section it refers to “using radio signals to determine a position or a line of position (e.g. LORAN C, GPS etc.)” | Make reference to GNSS so it includes Galileo. |
| Section 2.3.3: Harbour Approach Methods p19 |  | In this section, the document discusses “The advent of GPS and of DGPS”. | Refer to “GNSS and of DGNSS” respectively. |
| Section 2.3.4: Restricted Waters p20 |  | The following sentence appears “..but as with Harbour Approach navigation, they can be achieved with a combination of DGPS and Electronic Charts Systems.” | Reword to be generally “DGNSS”. |
| Section 2.4.1: Measurement Error p20 |  | The random errors section does not make reference to the use of dual systems in reducing these errors. | Update to state that random errors can be reduced when using a dual system such as Galileo & GPS. |
| Section 2.4.2.2: Navigational Position Fixing Measurements p21 |  | Table 2 of “Accuracy of some position-fixing processes and systems” mentions specifically the accuracy of GPS. | Update this table to indicate accuracy of Galileo. |
| Section 2.5.2.3 Chart Datum Issues p24 & p25 | This section refers to the WGS-84 datum and GPS positions and the issues when transferring between other datums. |  | Update to include converting between the Galileo datum and other chart datums. |
| Section 2.5.2.3 Chart Datum Issues p25 | Figure 4, GPS Notes on Charts, provides examples of the style of note found on some charts. |  | Possibly one Galileo example could be used. |
| Section 3.1.13.5.4 Positioning of Floating Aids p80 | The second paragraph refers to DGPS as the best method to position fix floating aids. |  | Modify to incorporate all DGNSS systems. |
| Section 3.2.1.1 IALA Policy p94 | In the bullet-point on “Satellite Systems”, GPS and GLONASS are specifically mentioned as space-based systems used to derive a position fix. |  | When Galileo becomes operational, it will need to be added to this list of systems. (Will not be in time for next edition) |
| Section 3.3.2 Global Navigation Satellite System (GNSS) p99 | “GPS and GLONASS have been recognised as components of the WWRNS.” Galileo is not currently included. |  | Once Galileo has been accepted as part of the WWRNS, this paragraph will need to be updated. (Will not be in time for next edition) |
| Section 3.3.2.3 Galileo p101 | This section on Galileo concludes that “full operational capacity is scheduled for 2008”. |  | Full operational capacity of Galileo is not expected until 2013. |
| Section 3.3.3 Differential Global navigation SATELLITE System (DGNSS) p101 | The beginning of this section describes the benefits of using a differential GNSS augmentation system, citing one of the results for the user as “almost immediate notification of faulty satellites (compared with up to two hours with GPS).” |  | The differential GNSS augmentation system will provide the same advantages to the Galileo OS, as for GPS. The benefit to users of Galileo in the future could be highlighted here. |
| Section 3.3.3.1 DGNSS Broadcast Stations p101 | The first paragraph states that the current maritime radionavigation band (283.5 to 325 kHz) is used for broadcasting existing DGNSS corrections. |  | It is expected that differential corrections for Galileo will use the same frequencies, and therefore it is unlikely that this section will need modifying. |
| Section 3.3.3.2 “Wide Area” DGNSS systems p102 | This section makes reference to “EGNOS includes corrections and integrity information on GPS and GLONASS”. |  | EGNOS will also supplement Galileo when it becomes fully operational and therefore this sentence should be updated so that it includes Galileo. |
| Section 4.4.4.1.1 Recording of Aids to Navigation Positions p124 | The first bullet-point mentions when aids to navigation positions can be recorded, specifically “where an Authority has operational DGPS stations, a program should be implemented to determine the WGS84 positions”. WGS84 is also mentioned throughout the other bullet-points. |  | This section does not include differential Galileo or its reference frame; therefore, it needs to amended to be more generic. |
| Section 6.1.2.2.1 Level of Service/Operational Performance Statements, p145 | In the visual design 2nd paragraph, “differential GPS corrections” are made reference to in conjunction with LOS. |  | Differential Galileo corrections are likely to introduced, so this paragraph should be amended to “differential GNSS corrections”. |
| Section 6.1.2.5 Mix of Aids to Navigation (Layers of Service) p147 | Table 27, “Indicative accuracies of aids to navigation systems”, provides details of systems with <100m accuracy, specifically GPS and GLONASS. |  | It is expected that Galileo will have an accuracy of <100m, therefore the relevant parts of this table should be updated and Galileo included. |
| Section 6.1.4.3 Definition and Comments on Terms p154 | In the Continuity section, it makes reference to “a DGPS station” and “the DGPS service” which is only specific to GPS. |  | In this example, “DGPS” should be replaced with DGNSS, so that it will incorporate differential Galileo. |
| Section 6.1.4.5 Availability and Continuity of Radionavigation Services, p155 | In this section, it refers to DGNSS and implies this is only DGPS because it is in brackets, “DGNSS (DGPS) services” |  | The word “DGPS” should be removed so that DGNSS also refers to differential Galileo. |
| Annex C, figure 4, p191 | GPS Notes on Charts |  | (see comments above) |

Annex 2 - IALA R-121 Comments

| Section & Page Number | Relevance of Galileo | Suggested amendments to this document to take account of Galileo |
| --- | --- | --- |
| Section 1: General p5 | This section refers to GNSS, specifically mentioning GPS & GLONASS. | To accommodate Galileo, it should include Galileo when referring to GNSS. This section should also include a paragraph on the need for differential corrections for the Open Service (OS) for Galileo. |
| Section 2: Signal Format p5 | The ITU-R Recommendation will need to be revised to accommodate Galileo. | This document should be altered to include Galileo by referring to “GNSS and of DGNSS” respectively. |
| Section 3: Reference Datum p6 | This section highlights the problems with using “a datum that is not consistent with WGS-84 in the case of GPS or PE90 in the case of GLONASS” and using Message Type 4 to indicate the datum. | A similar problem will occur with Galileo and its datum GTRF, therefore this should be included in this section. |
| Section 5: Integrity p10 | This section does not mention inbuilt integrity in a GNSS system. | Integrity will be provided by some of the Galileo services, so this should be referenced in this section. |
| Section 9: Receiver Performance p12 | The first paragraph states that users should operate the relevant GNSS receiver to the IEC 61108 technical specifications, specifically referring to GPS & GLONASS receivers. | When Galileo receivers have been developed, an additional reference should be made, stating that Galileo receivers should confirm to the relevant IEC 61108 document (IEC 61108-3, at CDV stage). |
| Section 11.2: Availability p14 | The website addresses are referred to in this section, which provide availability information for GPS & GLONASS. | The relevant availability information for Galileo, once it becomes fully operational, should be added in this section: http://ec.europa.eu/dgs/energy_transport/galileo/index.en.htm |
| Section 11.2: Availability, Momentary unusable period p15 | In the paragraph on Accuracy tolerance, it states that “the receiver should revert to GPS-only operation when an “unhealthy” state is received”. This is therefore specific to GPS. | This paragraph should be updated, so that it refers to the receiver reverting to GNSS-only operation when an “unhealthy” state is received, so Galileo is included. |
| Section 11.2: Availability, Momentary unusable period p15 | In the Message transmission time, “at 100 bps baud rate, the maximum transmission time of RTCM type 9-3, one of three sets is 2.1s.” | Depending on the baud rate for Galileo, this paragraph may need altering when Galileo becomes fully operational. (Not before the next revision) |
| Section 11.4: Continuity p17 | This section refers generally to the continuity of DGNSS. | Some Galileo services will guarantee continuity of service provision for specific applications; this could be referred to in this section. |
| Section 12: Publication of Information p19 | In the header information section, it provides general information about the differential station. | It may be worth noting that the header provides the Message Type number, which identifies the GNSS for which corrections are being provided. |
| Section 12: Publication of Information p19 | In table 5.1, “template for administrations using Type 7 message”, the datum field shows that ASCII code should be used to state which datum is being used, “WGS-84 for GPS, PE90 for GLONASS”, or Local Datum”. | This table should be modified to include the datum for Galileo (GTRF?) when Galileo becomes operational. (Not before the next revision) |
| Section 12: Publication of Information p19 | In table 5.2, “template for administrations using Type 27 message”, the datum field shows that 0 should be used to indicate WGS-84 datum or 1 should be used to indicate local datum. | A note may be needed to distinguish between non-WGS-84 geocentric reference frames and local datum. GTRF may be considered equivalent to WGS-84 in this context. |
| Appendix 1: DGNSS Broadcast Site Settings p24 | The first paragraph suggests, “service providers should consult the RTCM Recommended Standards for Differential NAVSTAR GPS Reference stations” to gain insight into the functions of the reference station and integrity monitor. | The latest version of the RTCM document should be referred to (currently v 2.3) to ensure that information relevant to Galileo is covered. |
| Appendix 1 DGNSS Broadcast Site Settings p25 | Table 3: DGNSS Broadcast Site Settings does not currently provide scope to use different GNSS systems together, or prioritise the use of a single GNSS system over the use of multiple satellites, from multiple GNSS systems. | This table may need to include settings/thresholds for using multiple GNSS systems, e.g. Galileo & GPS, once Galileo is fully operational. (not before next revision) |
| Notes on Table 3: A – Reference Station p27 | The “A-1: Minimum satellites” section does not state whether there is a minimum number of satellites from each GNSS system, when using multiple GNSS systems to calculate position. | This section does not include differential Galileo or its reference frame; therefore, it should be amended to be more generic. |
| Notes on Table 3: A – Reference Station p28 | The final paragraph in “A-3: Range Rate Correction” references that “Range rate corrections may be set to zero to eliminate the effects of random noise error generation as long as DGPS corrections are broadcast within 30 to 60 seconds.” | This sentence should be modified to “DGNSS corrections”, so that it includes differential Galileo corrections. |
| Notes on Table 3: A – Reference Station p28 | In the first paragraph in “A-4: Integrity Monitor Feedback” it refers to this feedback providing the Reference Station with information on the accuracy of the DGPS broadcast”. The remainder of this section makes other references to DGPS and therefore does not include Galileo. | This first sentence should be modified to “DGNSS broadcast” and the remainder of this section should also replace GPS & DGPS with GNSS & DGNSS, respectively, so that Galileo corrections, receivers, transmitters & data are included. |
| Notes on Table 3: B – Integrity Monitor p29 | In the section “B-1: RTCM correction age” it makes reference to DGPS receivers and “the remaining errors within the GPS signal like ionosphere and troposphere change, slowly with time”. This section therefore does not recognise that these errors are applicable to other GNSS systems. | In the first sentence, the “Integrity Monitor’s DGPS receiver” should be altered to DGNSS receiver so it includes Galileo receivers. Selective Availability will not be an issue with Galileo (or GPS?), however the ionosphere and troposphere will introduce errors into the Galileo signal, and this sentence should be modified accordingly. |
| Notes on Table 3: B – Integrity Monitor p32 | In the section “B-8: Absolute Horizontal Position” it refers to an internal GPS receiver which it uses to correct pseudo ranges to generate a DGPS corrected position. This section therefore does not include the Galileo system. | “GPS” and “DGPS” should be replaced with GNSS and DGNSS, respectively, so that it will include Galileo. |
| Notes on Table 3: B – Integrity Monitor p32 | In the section “B-9: Pseudo-range residual”, it states that “The Integrity Monitor applies pseudo-range corrections received over the data link to individual pseudo ranges generated by its internal GPS receiver”. This section therefore does not include the Galileo system. | “GPS” should be replaced with GNSS, so that it will include Galileo. |
| Appendix 2: Definitions and Glossary of Terms used in GNSS p35 | In this appendix, there is no mention of Galileo and its corresponding reference datum (GTRF?). | Definitions of Galileo and its reference frame (GTRF?) should be added to this glossary of terms once Galileo becomes fully operational. |