Report of the “DGPS re-capitalization meeting” in Gdynia, Poland

May 29-30, 2007

Executive Summary

- Fifteen persons from 8 countries attended the meeting of “DGPS re-capitalization”, which was hosted by Maritime Office Gdynia at Gdynia City Museum in Gdynia, Poland.

- On 29th May work progressed on presenting reports on current status of DGPS services. Developed service providers’ requirements for future DGNSS were presented.

- The development of a strategy for future implementing Local and Regional Augmentation Systems based on National Geodetical Networks was noted.

- The response of representatives present at the meeting indicate that most administrations providing DGNSS services are undertaking recapitalisation or planning such activities based also on co-operation with geodetic networks.

- On 30th May a technical visit to remote site of DGPS Rozewie reference and transmitting station was arranged to present latest RSIM equipment with RTK option.

- Field measurements of local antenna system parameters were performed with the use of sophisticated radio-test equipment. A practical lesson on ATU measuring procedure was given.

- It was proposed to organize periodically (intersessionally) working group to deal with new maritime DGNSS practical technical issues, shore-based system architecture for e-Navigation.   And to exchange experience.

___________________________________________________________________

Report of the “DGPS re-capitalization meeting”   Gdynia, Poland

# General

The meeting of the “DGPS re-capitalization” was held from May 29-30, 2008, hosted by Maritime Office Gdynia at Gdynia City Museum in Gdynia, Poland.

In result of invitation fourteen persons from 7 Baltic countries and Scotland attended the meeting.

Apologies were received from Mr Nick Ward (England) and Mr Roman Czerniajev ( Russia).

Nick Ward, vice chairman of IALA e-Nav Committee was informed and invited as the IALA observer. He wished us to have a fruitful technical workshop.

Tab I List of Participants

| Name | Organisation | Country |
| --- | --- | --- |
| 1. Archie Whyte | Northern Lighthouse Board | Scotland |
| 2. Jarle Hauge | Norwegian Coastal Administration | Norway |
| 3. Szymon Wajda | Head Office of Geodesy and Cartography | Poland |
| 4. Alar Shit | Estonian Maritime Administration | Estonia |
| 5. Johan Winell | Swedish Maritime Administration | Sweden |
| 6. Benny Pattersson | SMA | Sweden |
| 7. Marcin Waraksa | Maritime Office Gdynia, AtoN | Poland |
| 8. Michael Hoppe | German Federal Waterways Administration | Germany |
| 9. Marek Dziewicki | Maritime Office Gdynia, AtoN | Poland |
| 10. Jerzy Zemla | Maritime Office Gdynia, Geodesy | Poland |
| 11. Grażyna Sadlon | Maritime Office Gdynia, Geodesy | Poland |
| 12. Ivan M. Carlsson | Danish Maritime Safety Administration | Denmark |
| 13. Marek Ledóchowski | Maritime Office Gdynia, AtoN | Poland |
| 14. Alexander Sinilnikov | VTS Kaliningrad, Branch FSUE “Rosmorport” | Russian Federation |
| 15 Rolf Zetterberg | Swedish Maritime Addministration | Sweden |

## 2. OPENING

The Chairman Marek Dziewicki opened the meeting, welcomed participants and presented the Agenda (Appendix 1).

The Chairman underlined the need and positive aspects of exchanging information on national policies of DGNSS recapitalization in the light of IALA recommendations.  He asked participants to take note of the conclusions and possible effects for Baltic countries who overlap their navigational services.

# 3. PRESENTATIONS

Three presentations from national administrations were given by:

- Poland: “Do we need  DGNSS?” Marek Dziewicki, MOG  – about current status of Polish DGPS network in the light of IMO and IALA requirements. A system of two marine reference DGPS stations with central control station and remote monitors was presented. Some ideas about future integration with geodetic RAAS system to create VRS were given, finally he suggested the integrated approach to be taken.

- Sweden: “ DGPS re-capitalization” Johan Winell, SMA – provided information on current problems and proposed concepts to develop a national system and overcome recapitalization problems taking also into account SWEPOS.

- Germany: “New Technology Antenna and DGNSS Recapitalisation”

M. Hoppe GFWA, - informed on:

- Present IALA DGNSS Status in Germany, inland stations and far field remote monitors,

- A new type of MF transmitting antenna (NTA),

- Instrumentation for impedance measurements at a MF-Antenna system,

- DGNSS recapitalization plan in Germany by creating RAAS and VRS national network operated from national centre of control. He explained also the new concept of “moving centre of system accuracy”.

A spoken report was offered by Archie Whyte from NLB, Scotland.

The structure of the three General Lighthouse Authorities (GLAs) of the UK and Ireland was outlined. The GLAs comprise the Northern Lighthouse Board (NLB) for Scotland, Trinity House (TH) for England and Wales the Commissioners for Irish Lights (CIL) for the whole of Ireland.

A description of the network was given: NLB as 4 DGPS stations, TH has 7 DGPS stations and CIL has 3 DGPS stations. Each GLA has 2 Far Field Monitors and 1 Control Station. This gives a total of 23 sites.

The GLAs DGPS equipment is approximately 10 years old and is at the end of its designed period. An increase in faults has been experienced and spares are becoming increasingly difficult to obtain. For these reasons together with the availability of enhanced functionality and also with a view to Galileo, it was decided to replace the DGPS equipment.

He also mentioned that Invitation to Tender (ITT) for the DGPS Recapitalization had been placed and responses to tender should be received by 13th June 2008.

The structure of the ITT includes replacement of RSIM equipment, GPS antennas, transmitters and ATUs. Additionally individual site specific work has also been included. For example, the main antenna masts at Point Lynas and Sumburgh Head are to be replaced and it was a condition of the ITT that prospective tenderers had to visit both these site to ensure that they had a clear understanding of requirements. Another feature of the ITT is that the old equipment will be recovered and retained and will be reinstalled in the event of problems occurring with the new equipment which cannot be resolved. This is only a precautionary measure and is not expected to be utilized.

Once the contract has been placed, the tender information will be available to colleagues in associated DGPS administrations.

Final presentation was given about Polish geodetic system ASG-EUPOS established and being operated by Head Office of Geodesy and Cartography.   Szymon Wajda from HOGC gave an overview of a system in Poland consisting  of 86 reference stations linked by broadband IT network with two national control centres in Warsaw and Katowice. He outlined objectives and basic real time services (DGPS, RTK) being offered by system as well as available VRS technology.

- DISCUSSION

Rolf Zetterberg advised that due to SMA statistics only about 25% ships is carrying the differential GNSS receivers only. The group agreed with thesis that the meeting represents service provider point of view and national policy positions are to be decided in advance. These should take care also for hydrography and engineering applications. The future of national DGNSS belongs rather to RAAS ( Regional Area Augmentation Services) and large area networks rather then to local single  RS.

The question of overlapping DGNSS services of neighbouring countries was raised. Some members noted with concern that the main problems of system availability were usually in result of technical or mechanical malfunction of transmitting hardware (amplifiers, ATU, antenna system).

5. TECHNICAL VISIT to RS DGPS Rozewie

Members of the meeting were invited and encouraged to participate in technical visit to DGPS reference station and collocated Lighthouse Rozewie. The foundation of the lighthouse is on a high cliff 60 m above the shore of Baltic. A local museum devoted to steam power generator installed there to provide electricity for lighthouse and air pressure for old marine horn was also opened to the group.

First of all the site of RS DGPS installation  was visited and presented to the expert group. Next interested in radio-measurements participants attended a lesson of measuring ATU and MF antenna parameters. Michael Hoppe brought a special microprocessor based tester cooperating with laptop to show us in practice how to proceed with antenna tuning problems.  This was really fruitful time.

6. CONCLUSIONS

- administrations are undertaking recapitalization or planning such activities based also on co-operation with geodetic networks. The future of national DGNSS belongs rather to RAAS and large area networks rather then to local RS.

- It was proposed to organize periodically (intersessionally) working group to deal with latest DGNSS technical issues, exchanging experience in practise is  easier in a small group.

- Baltic Sea  became a European basin where neighbouring countries may overlap navigation services of the same quality. In the future they could join into Baltic RAAS.

# Annexes TO THE REPORT

### Agenda

      - Photos

Agenda

DGPS re-engineering –technical meeting in Gdynia 29th-30th of May 2008

29th of May 14.00-17.00

Venue:  Gdynia City Museum

- Welcome

- Approval of the Agenda

- Baltic countries’ DGPS status

  - DGPS recapitalization in Poland (Marek Dziewicki, PL)

  - DGPS recapitalization in Sweden( Johan Winell, S)

  - NLB report from Scotland (Archie Whyte)

- New type of transmitting antenna (Michael Hoppe, D)

- ASG EUPOS geodetic network in Poland ( Szymon Wajda, PL)

30th of May 8.00-15.00

Venue:  RS DGPS Rozewie site

Technical visit to the DGPS reference station Rozewie.

Transportation will be arranged by MOG.

- Practical application of ATU measuring unit

- General discussion and conclusions.

.