IALA e-Nav Committee – eNAV 5 (15-19 Sept 08)

WG 1 – Working Paper

Priority Analysis for User Needs

DRAFT – Working Paper

Introduction

Under the draft framework for the implementation process for the e-Navigation strategy, the IMO have requested that “the initial user needs should be further reviewed and prioritized by 2009”.

WG1 therefore discussed approaches to the prioritisation of the high level user needs identified in the draft strategic plan (section 8). The working group commenced by conducting a ‘benefits analysis assessment’ based on the stated benefits of e-Navigation as stated in the same strategy (section 6). These benefits included those for improved safety; environmental protection; security and efficiency. This benefits analysis was further broken down by main user groups comprising mariners; VTS; SAR; Pilots; Administrations and Shipowners.

The task of the assessment was to identify how important such benefits were to each stakeholder group and what impact the implementation of e-Navigation would have on improving or realising these benefits.

Participants in this assessment were representative of a wide range of maritime stakeholder including those under review. Annex 1 contains the raw data from this assessment.

Findings:

- Prioritisation of needs without a rigid specification for of attributes based on effect, cost or risk for example resulted in a subjective output. Wider consultation on the perceived benefits would reduce the effect of such subjectivity.

- There are clear benefits to all stakeholders from the effective implementation of e-Navigation, although from different perspectives.

- All sectors will gain from improved safety, some from an operational point of view and some from an investment point of view.

- It will ultimately be necessary to identify where amongst the variety of stakeholders investment in e-Navigation will come from in relation to where the benefits will be achieved.

Conclusions:

- It was noted that early notification should be given to governments and authorities as to the importance of e-Navigation and the need to address long term budgetary issues.

- Direct operators such as mariners, VTS & SAR personnel, and pilots may benefit the most from operational improvements gained from effective and coordinated displays, streamlined data exchange and communication, and decision support systems.

- The possible benefits to Administrations and Shipowners may tend to be the long term effects from fewer incidents and the resultant cost savings from incident response, environmental impact and the potential improvements in effective and efficient ship movements.

- The need for improved information flow and system integration between all stakeholder was recognised as a high priority, however system boundaries will need to be carefully managed and the implementation should be done in a phased approach. Improving the effectiveness of transmission and presentation for safety information such as Navtex navigational warnings was considered a high priority.

- Improved decision support systems were considered paramount to reducing error and improving safety. Improvement of critical safety alarms and alerts were considered a fundamental starting point for reducing burdens and improving decision making.

- A common operating platform for Mariners and Pilots such as S-Mode was considered as a high priority to gain benefits from improved decision making based on more effective operation and from effective training.

- Aspects of having a Common Operating Picture (with consideration for the needs for a User Defined Operating Picture) was considered a priority to gain benefits from coordinating an effective maritime domain awareness and incident response across a wide range of shore based authorities on a national, regional and international basis.

- System integrity and resilience was seen as absolutely critical, however scored low priority. It was concluded that this was due to the historic reliability of current systems despite the known weaknesses and potential consequences of failure.

- e-Navigation has the potential to improve teamwork within a ship, shore facility, and across remote users through improved data exchange and communication techniques. However, this improvement would only be recognised if training and procedures were developed and implemented in conjunctions with user needs.

Recommendations:

The following recommendations pertain to the prioritisation of user needs as stated and agreed in the IMO Draft Strategy for e-Navigation (section 8).

- Noting the consequences of e-Navigation on planning of Government Agencies tasked with providing maritime and shore services (hydrographic surveys and charting, MSI and integrated coastal management, etc... ), it is recommended that e-Navigation requirements are likely to impact the aforesaid long term strategy be communicated to them at an early stage through the relevant international organisations as appropriate.

- The development of a “Common Maritime Information/Data Structure” is a high priority, as it forms an essential foundation upon which e-Navigation needs to be developed. A common maritime information/data structure will help facilitate effective data exchange and communication, improved decisions based on up to date information, and foster a sharing of information to improve domain awareness, risk assessment and response effectiveness.

- The transmission to vessels of Maritime Safety Information (MSI) should be integrated and presented on the vessels navigation displays in a clear and unambiguous manner taking into account the best practice for graphical displays and the human element. Such information should be accessible via the ECDIS and INS and be integrated into an alert management system such as defined within the IMO INS standard.

- A phased approach to report transmissions between ship & shore, shore & ship and shore to shore should be established based on the goal of reducing the administrative workload to the operators from the frequency of transmitting routine messages.

- A phased approach to developing a common operating picture of the maritime domain awareness should be developed to promote improved safety, security and incident response across a national, regional and international basis.

- An S-Mode should be developed for shipboard displays to ensure that all mariners and pilots are better able to execute critical tasks to support safe navigation.

Annex I

Benefits Analysis Assessment

| Benefits |  | Mariners | VTS | SAR | Pilots | Administrations | Ship owners | The three benefit areas we thing are most important |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Safety |  |  |  |  |  |  |  |  |
|  | Decision support | 4 | 4 | 5 | 4 | 2 | 4 | 11111 |
|  | Indications/alerts | 4 | 4 | 2 | 2 | 2 | 1 | 11 |
|  | ENC coverage | 5 | 2 | 2 | 2 | 2 | 4 | 1 |
|  | S-Mode (or VTS common operating picture) | 5 | 3 | 5 | 5 | 4 | 5 | 111 |
|  | System resilience | 4 | 4 | 5 | 4 | 5 | 4 | 1 |
|  | Integration (ship/shore) | 3 | 5 | 5 | 4 | 5 | 3 | 11111 |
| Protection of the environment |  |  |  |  |  |  |  |  |
|  | Reduce incidents | 4 | 4 | 2 | 4 | 5 | 4 | 1 |
|  | Reduce emissions | 2 | 2 | 1 | 1 | 4 | 5 |  |
|  | Incident response | 2 | 4 | 5 | 2 | 4 | 4 |  |
| Security |  |  |  |  |  |  |  |  |
|  | Passive shore domain awareness | 2 | 3 | 4 | 4 | 4 | 3 | 1 |
| Efficiency |  |  |  |  |  |  |  |  |
|  | Speed of type approval | 1 | 1 | 1 | 1 | 4 | 1 |  |
|  | Standard reporting | 5 | 5 | 2 | 3 | 4 | 5 | 1 |
|  | Better watchkeeping | 5 | 4 | 4 | 3 | 3 | 4 |  |
|  | Existing system integration (including equipment compatibility and interoperability (includes data exchange) and meeting of user requirements | 4 | 4 | 5 | 4 | 3 | 4 | 1111111 |
| Teamwork |  |  |  |  |  |  |  |  |
|  | Improved Human Resource Management (teamwork) | 4 | 4 | 4 | 4 | 3 | 3 | 1 |

1 LOW priority – 5 HIGH priority (generally taking into account the current situation)

1. Low priority

2. Low to medium priority

3. Medium priority

4. Medium to high priority

5. High priority

S-Mode is the proposed functionality for shipborne navigation displays using a standard, default presentation menu system and interface.