IALA TECHNICAL CLARIFICATIONS

ON

RECOMMENDATION ITU-R M.1371-3

Edition 2.1

Table of Contents

1	Introduction	3

2	Technical Clarifications of ITU-R M.1371-3	5

2.1	A8; §3.3.2, Message 5: Ship static and voyage related data	6

2.2	A8; §3.2.1 Message 23: Group Assignment Command	7

2.3	A8; § 3.22 Message 24: Static data report	10

ANNEX A	AIS message for static information from SAR aircraft.	13

Technical Clarifications of

Recommendation ITU-R M.1371-3

Edition 2.1

# Introduction

  - The purpose of AIS is to improve the maritime safety and efficiency of navigation, safety of life at sea and the protection of the marine environment.

  - It has long been realized that an automatic reporting device fitted to a ship would have the potential to increase significantly the safety of navigation and would allow the improved control and monitoring of the maritime environment.  Taking up this challenge, the International Maritime Organization (IMO) together with the International Telecommunications Union (ITU) and the International Electrotechnical Commission (IEC) developed a new navigation system called the Automatic Identification System (AIS).

  - Recommendation ITU-R M.1371-3 is the document that describes the technical characteristics for an Automatic Identification System using Time Division Multiple Access in the VHF Maritime Mobile Band. As with any technical document some concepts are subject to interpretation. This document presents a series of clarifications for some of these concepts as agreed by the IALA e-NAV Committee.

  - Figure 1 (attached) illustrates the inter-relationship of this document to IMO SOLAS Chapter V; the IMO recommendation on Performance Standards for a Universal Shipborne Automatic Identification System (AIS), (MSC.74(69)); the IMO Guidelines for the Onboard Operational Use of the Shipborne AIS (A.917(22)); the ITU Recommendation on the Technical Characteristics for an Automatic Identification System (AIS) using Time Division Multiple Access in the Maritime Mobile Band (ITU-R M.1371-3); and the IEC Standard 61993-Part 2: Universal Shipborne Automatic Identification System (AIS) Operational Performance Requirements, Methods of testing and required test results; the IALA Guidelines on Automatic Identification Systems (AIS).

  - As shown in Figure 1, each of the referenced documents in paragraph 1.4 including this document has a specific focus in relation to an overall AIS as follows.

This document, "Recommendation on Technical Clarification of ITU-1371-3", deals specifically with the VHF Data Link (VDL) of an AIS. It acknowledges the fact that some of the specific technical paragraphs in ITU-R M.1371-3 might require interpretations as to their precise, intended meaning.  Consequently, this document presents a set of unifying recommendations that will, hopefully, result in a standardized application of these paragraphs by potential manufacturers, system integrators as well as users of the systems.  Adherence to these recommendations will ensure the viability of the VDL and the interoperability among AIS units obtained from different sources.

  - However, this document also covers some other specific, technical details of various possible AIS station or device configurations that are only mentioned in general terms in ITU-R M.1371.

  - All of the identified documents have a specific focus and address specific user needs. Nevertheless, IALA also recognized that there is a need for a high level document that consolidates all of technical and operational aspects of the sub-systems described in Figure 1.  Consequently, IALA is at this time also drafting a general IALA Guideline on AIS which will incorporate all of the elements in Figure 1 in a descriptive manner.

| IALA Documents |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NAVGUIDE [AIS Manual] |  |  |  |  |  |  |
| Guidelines: AIS as an eNAV service |  |  |  |  |  |  |
| A-123 Provision of shore based AIS | A-124 AIS Service |  |  | A-126 AIS in marine aids to navigation | A-xxx Matrix of AIS stations (out of IALA Technical Clarifications) | A-yyy Future recommendation |
|  | Annex FATDMA planning | Annex …. | Annex …. |  |  |  |
| Non IALA Documents |  |  |  |  |  |  |
| IEC 61993-2 Class A | IEC 62287-1/-2 Class B “CS”/”SO” |  |  | IEC 62320-1 Base station | IEC 62320-2 AtoN station | Future IEC standards |
| ITU-R documentation on AIS, e.g. ITU-R M.1371-3 |  |  |  |  |  | IALA Technical Clarifications on 1371-3 |
| IMO documentation on AIS, e.g. AIS Performance standards |  |  |  |  |  |  |

Figure 1  Documents Matrix

# Technical Clarifications of ITU-R M.1371-3

Important General Remark

Every clarification of this Chapter is presented in a uniform way:

- Every clarification is introduced in the appropriate context of Recommendation ITU-R M.1371-3, which is always quoted as a whole section. This is done to ensure, that all clarifications are to be understood in their appropriate context.

- The verbatim text of Recommendation ITU-R M.1371-3 is given in italics; additions by the clarification are given upright and underlined as of previous clarification editions and upright and double underlined in the present edition. Deleted text portions are identified with strike-out as of previous clarification editions and double strike out in the present edition.

- For every clarification a rationale is given to allow complete understanding as to why a clarification was needed also. Eventually, the clarifications will be incorporated into a future revision of Recommendation ITU-R M.1371-3).

- A date of when the clarification was drafted is given.

- A comment contains additional information on the clarification itself. For instance, some clarifications have already been incorporated in the first edition of the IEC standard for Class A shipborne mobile AIS stations (IEC 61993-2 latest edition). A future substantial change of the clarification result in the creation of a legacy issue. Any future change for those clarifications should therefore be done with the utmost caution.

## A8; §3.3.2, Message 5: Ship static and voyage related data

### Proposed Clarifying Text: Should only be used by Class A shipborne and SAR aircraft AIS stations when reporting static or voyage related data.

### Rationale for Clarification: This proposed change will: (1)  allow the use of this Message by SAR Aircraft, and, (2)  harmonises hazard or pollutant categories (i.e. A, B, C and D) used in the 2nd digit of Identifiers to be used by ships to report their type with new categories (i.e. X. Y. Z and OS) implemented under the 2004 amendments to MARPOL Annex II on 1 January 2007.

### Date of amendment: 17-09-2008

### Comment

Most existing Class A stations do not process Message 24 (output on the PI and display on the MKD).  Message 24 can be transmitted by other AIS stations, i.e. Base Stations, SAR Aircraft, to provide additional static data.  Message 5 is processed by AIS stations, permitting its use by SAR Aircraft ensures that this important information is received by all AIS stations, legacy or new.

.Future Class A stations, based on ITU=R M.1371-3, should be able to process message 24.

For guidance for the input of static SAR aircraft information see Annex A.

TABLE 50

| Identifiers to be used by ships to report their type |  |
| --- | --- |
| Identifier No. | Special craft |
| 50 | Pilot vessel |
| 51 | Search and rescue vessels |
| 52 | Tugs |
| 53 | Port tenders |
| 54 | Vessels with anti-pollution facilities or equipment |
| 55 | Law enforcement vessels |
| 56 | Spare – for assignments to local vessels |
| 57 | Spare – for assignments to local vessels |
| 58 | Medical transports (as defined in the 1949 Geneva Conventions and Additional Protocols) |
| 59 | Ships according to RR Resolution No. 18 (Mob-83) |

| Identifiers to be used by ships to report their type |  |  |  |
| --- | --- | --- | --- |
| Other ships |  |  |  |
| First digit(*) | Second digit(*) | First digit(*) | Second digit(*) |
| 1 – Reserved for future use | 0 – All ships of this type | – | 0 – Fishing |
| 2 – WIG | 1 – Carrying DG, HS, or MP, IMO hazard or pollutant category AX(**) | – | 1 – Towing |

TABLE 50 (end)

| Identifiers to be used by ships to report their type |  |  |  |
| --- | --- | --- | --- |
| Other ships |  |  |  |
| First digit(1) | Second digit(1) | First digit(1) | Second digit(1) |
| 3 – See right column | 2 – Carrying DG, HS, or MP, IMO hazard or pollutant category BY(**) | 3 – Vessel | 2 – Towing and length of the tow exceeds 200 m or breadth exceeds 25 m |
| 4 – HSC | 3 – Carrying DG, HS, or MP, IMO hazard or pollutant category CZ(**) | – | 3 – Engaged in dredging or underwater operations |
| 5 – See above | 4 – Carrying DG, HS, or MP, IMO hazard or pollutant category DOS(**) | – | 4 – Engaged in diving operations |
|  | 5 – Reserved for future use | – | 5 – Engaged in military operations |
| 6 – Passenger ships | 6 – Reserved for future use | – | 6 – Sailing |
| 7 – Cargo ships | 7 – Reserved for future use | – | 7 – Pleasure craft |
| 8 – Tanker(s) | 8 – Reserved for future use | – | 8 – Reserved for future use |
| 9 – Other types of ship | 9 – No additional information | – | 9 – Reserved for future use |

DG: Dangerous Goods.

HS: Harmful Substances.

MP: Marine Pollutants.

(*)  NOTE   –  The identifier should be constructed by selecting the appropriate first and second digits.

(**) NOTE  – The digits 1, 2, 3 and 4 reflecting  categories X, Y, Z and OS formerly were categories A, B, C and D.

## A8; §3.2.1 Message 23: Group Assignment Command

### Proposed Clarifying Text:

The Group Assignment Command is transmitted by a Base station when operating as a controlling entity (see § 4.3.3.3.2 Annex 7 and §3.20). This message should be applied to a mobile station within the defined region and as selected either by its position, by its “Ship and Cargo Type” or by its “Station Type”. The receiving station should consider all selector fields concurrently.  It controls the following operating parameters of a mobile station;

- transmit/ receive mode;

- reporting interval; and

- the duration of a quiet time.

Table 73

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 23; always 23 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0-3; default = 0; 3 = do not repeat any more |
| Source ID | 30 | MMSI of assigning station |
| Spare | 2 | Spare. Shall be set to zero |
| Longitude 1 | 18 | Longitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (180, East = positive, West = negative) |
| Latitude 1 | 17 | Latitude of area to which the group assignment applies; upper right corner (north-east); in 1/10 min (90, North = positive, South = negative) |
| Longitude 2 | 18 | Longitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (180, East = positive, West = negative) |
| Latitude 2 | 17 | Latitude of area to which the group assignment applies; lower left corner (south-west); in 1/10 min (90, North = positive, South = negative) |
| Station type | 4 | 0 = all types of mobiles (default); 1 = Class A mobile station only reserved for future use; 2 = all types of Class B mobile stations; 3 = SAR airborne mobile station; 4 = Class B “SO” mobile stations only AtoN station; 5 = Class B “CS” shipborne mobile station only; 6 = inland waterways; 7 to 9 = regional use and; 10 to 15 = for future use |
| Type of ship and cargo type | 8 | 0 = all types (default) 1...99 see Table 50 100...199 reserved for regional use 200...255 reserved for future use |
| Spare | 22 | Not used. Should be set to zero. Reserved for future use. |
| Tx/Rx mode | 2 | This parameter commands the respective stations to one of the following modes: 0 = TxA/TxB, RxA/RxB (default); 1 = TxA, RxA/RxB , 2 = TxB, RxA/RxB, 3 = reserved for future use |
| Reporting interval | 4 | This parameter commands the respective stations to the reporting interval given in Table 74 |
| Quiet time | 4 | 0 = default = no quiet time commanded; 1-15 = quiet time of 1 to 15 min. |
| Spare | 6 | Not used. Should be set to zero. Reserved for future use. |
| Number of bits | 160 | Occupies one-time period |

Table 74

Reporting interval settings for use with Message 23

| Reporting interval field setting | Reporting interval for Message 23 |
| --- | --- |
| 0 | As given by the autonomous mode |
| 1 | 10 minutes |
| 2 | 6 minutes |
| 3 | 3 minutes |
| 4 | 1 minute |
| 5 | 30 seconds |
| 6 | 15 seconds |
| 7 | 10 seconds |
| 8 | 5 seconds |
| 9 | 2 seconds (not applicable to the Class B “CS”) |
| 9 | Next shorter reporting interval |
| 11 10 | Next longer reporting interval |
| 11 | 2 seconds (not applicable to the Class B “CS”) |
| 12-15 | Reserved for future use |

NOTE – When the dual channel transmission is suspended by Tx/Rx mode command 1 or 2, the required reporting interval should be maintained using the remaining transmission channel.

### Rationale for Clarification:

Clarify the concurrent evaluation of parameters position, ship and cargo type and station type:

This proposed change harmonises the Message 23 reporting interval settings with the Class-B “CS” testing standard (IEC 62287-1 Ed.1).

### Date of amendment: 21 September 2007

### Comment

No comment.

## A8; § 3.22 Message 24: Static data report

### Proposed Clarifying Text:

Message 24 Part A and Part B may be used by any AIS station to associate a MMSI with the static information.

Message 24 Part A and Part B shall be used by Class B “CS” shipborne mobile equipment. The message consists of two parts. Message 24B shall be transmitted within 1 min following Message 24A.

In case of an interrogation for a Class B “CS” on a Message 24, the response shall include Part A and Part B.

Table 75

Message 24 Part A

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 24; always 24 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| Part number | 2 | Identifier for the message part number; always 0 for Part A |
| Name | 120 | Name of the MMSI-registered vessel. Maximum 20 characters 6-bit ASCII, @@@@@@@@@@@@@@@@@@@@ = not available = default |
| Number of bits | 160 | Occupies one-time period |

Table 76

Message 24 Part B

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for Message 24; always 24 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. 0 = default; 3 = do not repeat any more |
| User ID | 30 | MMSI number |
| Part number | 2 | Identifier for the message part number; always 1 for Part B |
| Type of ship and cargo type | 8 | 0 = not available or no ship = default 1-99 = as defined in § 3.3.2 100-199 = reserved, for regional use 200-255 = reserved, for future use |
| Vendor ID | 42 | Unique identification of the Unit by a number as defined by the manufacturer (option; “@@@@@@@“ = not available = default). See Table XX |
| Call sign | 42 | Call sign of the MMSI-registered vessel. 7 X 6 bit ASCII characters, “@@@@@@@“ = not available = default |
| Dimension of ship/reference for position. Or, for unregistered daughter vessels, use the MMSI of the mother ship. | 30 | Dimensions of ship in meters and reference point for reported position (see Figure 41 and §3.3.3). Or, for an unregistered daughter vessel, use the MMSI of the associated mother ship in this data field |
| Spare | 6 |  |
| Number of bits | 168 | Occupies one-time period |

Table XX – Vender ID field

| Bit | Information | Description |
| --- | --- | --- |
| (MSB) 41 …...... 24 (18bits) | Manufacturer’s ID | The Manufacturer's ID bits indicate the manufacture’s mnemonic code consisting of three 6 bit ASCII characters. 1) |
| 23 …...... 20 (4bits) | Unit Model Code | The Unit Model Code bits indicate the binary coded series number of the model. The first model of the manufacture uses “1” and the number is incremented at the release of a new model. The code reverts to “1” after reaching to “15”. The “0” is not used. |
| 19 …...... 0 (LSB) (20bits) | Unit Serial Number | The Unit Serial Number bits indicate the manufacture traceable serial number. When the serial number is composed of numeric only, the binary coding should be used. If it includes figure(s), the manufacture can define the coding method. The coding method shall be mentioned in the manual. |

1) IALA is presently considering to create an open international register of manufacturers ID’s

### Rationale for Clarification:

In the ITU-R M.1371-3, the vender ID field (42bits) is prepared in the Message 24B for the Class-B AIS station, however the detail is not defined.

The Table XX proposes the clarifications needed for the additional details.

Part B is needed to allow AIS stations to transmit static data.

### Date of amendment: 17 September 2008

### Comment

No comment.

ANNEX A	AIS message for static information from SAR aircraft.

- Background

The e-Nav Committee was requested to give guidance on the transmission of static information for SAR aircrafts.  Presently ITU-R M. 1371-3 does not specify any message for transmission of static information for SAR aircraft.  There are however two possible messages for this purpose:

- Msg 5  designed for use by Class A shipborne AIS

- Msg 24A and 24B,  designed as a general message for static information for any AIS station

2	Discussion

The two messages have very similar information content, not specifically for SAR aircraft.  However they do include relevant information.  They also include data fields which are not applicable for SAR aircraft.

Msg 5 is forwarded to the presentation interface on all existing shipborne AIS stations.  It is not clear if Msg 5 for a SAR aircraft is associated with Msg 9 (SAR aircraft position report) in a MKD. It is presented correctly at least by some presentation systems, while others may need a SW upgrade.

The transmission of Msg 5 from SAR aircraft requires an amendment of ITU-R M.1371-3 to allow the use for SAR Aircraft.

Msg 24 is “unknown” to most existing Class A shipborne AIS stations and will not be forwarded to the presentation interface.  A SW upgrade (if possible) of these AIS units is required together with a SW upgrade of their display systems.  This may strongly reduce the value of the transmissions since there is no requirement for SW upgrades of AIS units.

Msg 24 may be transmitted by SAR aircraft without any changes of existing standards.

- Proposal

It is proposed that both messages are allowed for use by SAR aircraft and they may be used in parallel to achieve the highest probability for a correct presentation, now and in the future.  The IALA Clarifications of ITU-R M.1371 should include an amendment of the text for Msg 5 to allow the use by SAR aircraft.

It is not proposed to alter the interrogation response from Msg 15 for SAR aircrafts due to its limited use.

The reporting rate may be decided by the responsible administration.

The data fields in Msg 5 are proposed to be used as follows:

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message 5 |
| Repeat indicator | 2 | Used by the repeater to indicate how many times a message has been repeated. |
| User ID | 30 | MMSI number |
| AIS version indicator | 2 | 0 = station compliant with Recommendation ITU-R M.1371-1 1 = station compliant with Recommendation ITU-R M.1371-3 2-3 = station compliant with future editions |
| IMO number | 30 | Not applicable for SAR aircraft. Set to 0 or possible new code |
| Call sign | 42 | 7 = 6 bit ASCII characters, @@@@@@@ = not available = default |
| Name | 120 | Maximum 20 characters 6 bit ASCII Should be set to “SAR AIRCRAFT NNNNNNN” where NNNNNNN equals the aircraft registration number “@@@@@@@@@@@@@@@@@@@@” = not available = default |
| Type of ship and cargo type | 8 | Should be set to “0 “= not available or no ship = default |
| Overall Dimension/ reference for position | 30 | The use of this field may be decided by the responsible administration. If used it should indicate the maximum dimensions of the craft. As a default A=B=C=D should be set to “0” Reference point for reported position. Also indicates the dimension of ship (m) |
| Type of electronic position fixing device | 4 | 0 = undefined (default) 1 = GPS 2 = GLONASS 3 = combined GPS/GLONASS 4 = Loran-C 5 = Chayka 6 = integrated navigation system 7 = surveyed 8 = Galileo, 9-15 = not used |
| ETA | 20 | The use of this field may be decided by the responsible administration Estimated time of arrival; MMDDHHMM UTC Bits 19-16: month; 1-12; 0 = not available = default Bits 15-11: day; 1-31; 0 = not available = default Bits 10-6: hour; 0-23; 24 = not available = default Bits 5-0: minute; 0-59; 60 = not available = default |
| Maximum present static draught | 8 | Not applicable for SAR aircraft. Should be set to 0 = not available  default; |
| Destination | 120 | The use of this field may be decided by the responsible administration Maximum 20 characters using 6-bit ASCII; @@@@@@@@@@@@@@@@@@@@ = not available |
| DTE | 1 | Data terminal ready (0 = available, 1 = not available = default) |
| Spare | 1 | Spare. Not used. Should be set to zero. Reserved for future use. |
| Number of bits | 424 | Occupies 2 slots |

The data fields of Msg 24A and 24B should be used in a corresponding way.

Highlighted text has been used to indicate how the messages may be used for SAR aircraft.