e-Navigation Committee						e-NAV 5/5/3

5th Session								15 Sept 2008

Rapporteur’s Report on GNSS

Introduction

This is a an updated version of the report provided for e-NAV 4 and subsequently used as the basis for an article in Bulletin 2008/2. It draws on published information in journals and on websites.

# GPS

There are 30 usable satellites in the constellation at present. The last launch was on 15 March, the next was due in September, but has been delayed by faulty components in the launch vehicle. The last two Block IIR-M satellites are now scheduled for launch in October and December. Accuracies are well within the declared (13m 95%) target and monitoring has been improved by an enhanced ground control segment. Satellites being launched now are Block IIR-M, broadcasting the second civil frequency, L2C, which is scheduled to become operational in 2014. The first IIF satellite with the additional L5 signal is due for launch in 2008, with an operational date of 2016 for L5. GPS III is still at the design stage, with first launch scheduled for 2013 and is not due to be fully operational until 2021.

The U.S. Naval Research Laboratory has awarded a contract to demonstrate High Integrity Global Positioning System (GPS) technology concepts. This will combine signals from the Iridium Low Earth Orbit telecommunications system with GPS to enhance navigation availability, integrity, accuracy and jam-resistant capabilities.

# GLONASS

There are currently 13 operating satellites, with a further three unusable at time of writing. The last launch was on 25 December 2007, but the Russian Government is committed to restoring a full system by late 2009. Recent launches have included the improved GLONASS M satellites with a second civil signal. The new GLONASS-K with a third civil signal on L3, should start launching in 2009. It will also carry differential corrections, integrity information and search and rescue functions. The future GLONASS K-M system is at the requirement definition stage.

# GALILEO

The two test-bed satellites, GIOVE A and B, continue to broadcast signals, including navigation signals for test purposes. The European Commission, supported by ESA, has launched the procurement process for the deployment phase of Galileo. Additional public funding has been allocated for this phase, with the intention of handing over to a commercial operating consortium later. The planned operational date is now 2013.

The standardisation effort continues with documentation in place to submit Galileo for recognition as a component of the World Wide Radio Navigation System, as soon as sufficient satellites are deployed to provide performance data. The IMO Performance Standard and IEC receiver specification have been completed and new RTCM messages for broadcasting differential corrections have been agreed for inter-operability tests.

The European Parliament has approved a bill introducing military use of Galileo, making it a dual-use system, in common with GPS and GLONASS.

# COMPASS

China’s Beidou/Compass regional satellite navigation system employs two GEOs, with a third as a backup, to provide 100 m horizontal accuracy. A MEO was launched in April 2007, assumed to be the start of Beidou 2, either a full 30 MEO constellation, or 4 GEOs + 12 MEOs, or some other combination of MEOs, GEOs and IGSOs, with a proposed operational date of 2012.

# QZSS

The Japanese Aerospace Exploration Agency has published an interface specification for comment, with a baseline constellation of three Highly Elliptical Orbit satellites to begin launching in 2009. It is proposed to transmit L1, L2 and L5 signals, compatible and inter-operable with GPS.

## SBAS

## The North American WAAS is operational for non-safety critical applications and to support non-precision approach. The European EGNOS is in its Initial Operations Phase with two Inmarsat 3 satellites and is scheduled to become operational during 2008. The Japanese MSAS has been commissioned for aviation use, with two MTSATs operating. The Indian GAGAN system has carried out test transmissions with one Inmarsat 4 satellite and these are expected to resume in October.

## IALA DGNSS

IALA has recommended that members operating the beacon DGNSS service, which is the international standard for maritime augmentation, should consider modernization to ensure that levels of service can be maintained and future requirements can be met (R-135). This recommendation took into account the potential alternatives, in particular Satellite Based Augmentation Services (SBAS) and the Automatic Identification System (AIS). The simplest option may be replacement of existing hardware with similar dedicated Reference Stations and Integrity Monitors (RSIM), but this would limit the potential for development and there are other possibilities, including software RSIM, Virtual Reference Station Networks and integration with SBAS.

## GNSS Strategy

At some stage (post 2012) there should be two or three GNSS available, providing a total of more than 60 satellites, with multiple frequencies, inherent integrity monitoring and warning. This number of satellites will make Receiver Autonomous Integrity Monitoring (RAIM) a reliable proposition. The requirement for separate (satellite or ground-based) augmentation may need to be justified at that stage and it may be appropriate to consider a schedule for phasing out existing augmentation systems with those developments in mind. However, the concerns about the vulnerability of GNSS will remain and there will still be a case for alternative electronic position fixing systems and for independent verification and warning systems.