e-NAV5/9/9

St Johns Output 8

IALA Recommendation A-124

Annex 14

FATDMA Planning and Operation of an AIS Service

# Revision

| # | Date | Description | Initials |
| --- | --- | --- | --- |
| V 0.1 | 2007-05-29 | Version 0.1 | JKJ |
| V 0.1.1 | 2007-06-01 | Reorganised into newer template | AIW |

# Table of Contents

1	Revision	2

2	Table of Contents	3

3	Introduction	5

4	Reference	6

5	Definitions	6

6	Abbreviations	6

7	FATDMA Planning and Operation of an AIS Service	6

8	General	7

9	Definitions	8

9.1	Fundamental considerations for terminology	8

9.2	Terms related to FATDMA reservations in the time domain in general	10

9.2.1	Terms exclusively related to the relative slot number used for actual transmissions of FATDMA reservations	11

9.2.2	Terms exclusively related to the absolute slot numbers of FATDMA reservations	12

9.2.3	Relationship between absolute and relative FATDMA terms	13

9.3	Terms related to the geographical areas for which FATDMA reservation planning is done	13

9.4	Terms related to the usage of FATDMA reserved slots (“Usage Designation”)	14

9.4.1	General considerations in regard to periodicity of transmissions using FATDMA reserved slots	14

9.4.2	General considerations in regard to timing of transmissions using FATDMA reserved slots	14

9.4.3	Terminology for categories for usage of FATDMA reserved slots	15

9.5	Terms related to activities of entities involved in the FATDMA (“VDL Usage Stake Holders”)	15

9.5.1	Terms related to roles and activities of entities related to FATDMA	15

9.5.2	Roles and activities involved in making and using FATDMA reservations	15

9.6	Mapping of FATDMA related aspects to Layers of the AIS Service	16

10	Introduction to relevant aspects of the FATDMA reservations	20

11	Fundamental philosophy statements for FATDMA Configuration	22

11.1	Guiding principles for setting up FATDMA plans	22

11.1.1	Rules in regard to the applicability of the FATDMA plan	22

11.1.2	Rules to minimise the usage of the VDL by FATDMA reservations	22

12	FATDMA Reserved Slot Use Cases	23

12.1	Rules for usage categories	23

12.2	Use Cases for FATDMA reservation usage category 1 - AIS VDL management slots	23

12.2.1	Transmission of Data Link Management Messages (message 20)	24

12.2.2	Transmission of Base Station Report (message 4) at the default reporting rate	25

12.2.3	Transmission of Channel Management Messages (message 22)	26

12.3	Use Cases for FATDMA reservation usage category 2 - High timing requirements slots	27

12.3.1	Transmission of Base Station Reports when a base station (in the given area) is acting as a semaphore	27

12.3.2	Position Reports of any kind of AIS mobile stations, when assigned a “FATDMA protected” slot (“safe slot” concept)	28

12.3.3	Transmission of Safety Related Messages (messages 12 and 14) by base station(s)	28

12.3.4	Transmission of Acknowledgements on received addressed messages (message 7 and 13) by base station(s)	29

12.3.5	Transmission of DGNSS broadcast binary messages (message 17) with high timing requirements contents, such as integrity alerts, by base station(s)	29

12.3.6	Re-transmission of Position Reports (and other high-timing requirement messages) of any kind of AIS mobile stations	31

12.4	Use Cases for FATDMA reservation usage category 3 - General purpose pre-reserved slots	31

12.4.1	Transmission of any assignment command (message 16; message 23) by base station(s)	32

12.4.2	Transmission of UTC and Date inquiry message (message 10) by base station(s)	33

12.4.3	Transmission of DGNSS broadcast binary messages (message 17) with relaxed timing requirements contents by base station(s)	33

12.4.4	Transmission of Interrogation message (message 15) by base station(s)	39

12.4.5	Transmission of Binary Messages (message 6 and 8) by base station(s)	39

12.4.6	Transmission of Aids-to-Navigation Reports (message 21)	39

12.4.6	Designing a FATDMA schedule	40

12.4.7	40

13	The definition of FATDMA schemes by space, time and frequency usage separation	40

13.1	The universal grid scheme – separation by space and frequency	40

13.2	FATDMA schemes – separation in time	41

14	Allocation of the defined FATDMA schemes to competent authorities	43

14.1	Summary of the previous definitions and their consequences	43

14.2	Stepwise allocation of the defined FATDMA schemes to competent authorities	44

14.2.1	Step 1: International allocation of FATDMA schemes	44

14.2.2	Step 2: Domestic allocation of slots out of the FATMA schemes for that country	45

14.3	FATDMA configuration data exchange between Administrations at run-time	45

Attachment A: Algorithm to calculate the FATDMA geographical grid scheme on a global basis	46

Attachment B: Absolute slot number and usage category designation for the FATDMA schemes	48

# Introduction

This Annex introduces the configuration of FATDMA of Physical AIS Shore Stations (PSS) by one or more competent authorities in a given area.

The purpose of this Annex can be summarized as followed:

- Provide to competent authorities a comprehensive „cookbook-like“ recipe to create a FATDMA plan

- Provide to adjacent and affected national competent authorities guidance to co-ordinate their joint FATDMA plan in the adjacent area

- Provide to (national / other domestic) competent authorities guidance how to operate their co-ordinated FATDMA plan correctly and optimize the benefit of both mobile AIS stations and the competent authorities in a given area.

- Provide a basis for required and / or stipulated type(s) of components of a competent authority

A range of AIS stations make use of FATDMA reserved timeslots for their transmissions.  Taking IMO resolution MSC.140(76) into consideration, which states, that “Administrations should take steps necessary to ensure the integrity of the radio channels used for AIS in their waters”, and since since FATDMA reservations in general affect the bandwidth availability of the AIS VDL, proper management of FATDMA is required to protect the VDL.

The complexity of FATDMA management increases in proportion to

- the slot usage requirements by different competent authorities in their envisaged coverage areas,

- the number of transmitting entities using the FATDMA reserved slots, and

- their relative geographical proximity.

Therefore, fundamental usage rules for FATDMA plans, FATDMA schemes and FATDMA schedules need to be established to resolve the complexity of FATDMA configuration in any given area.

This annex provides guidance and rules for the efficient use of FATDMA.

- Basic overview of the annex including purpose, justification, and any regulations or requirements addressed by the annex.

- Position of this annex within the overall structure of A124 using the following format:

| A124 |  |
| --- | --- |
|  | General |
|  | Service Model |
|  | Design Considerations |
|  | VDL Management - Annex 14 FATDMA Planning and Operation of an AIS Service - Annex 15 Assigned mode operation of an AIS Service - Annex 17 Channel Management by an AIS Service - Annex 18 General Loading Management by an AIS Service |
|  | Implementation, Installation, and Maintenance |
|  | Functional Components of the AIS Service |

# Reference

[ITU 1371] Recommendation ITU-R M.1371 (current revision and appropriate IALA Technical Clarifications).

[62320-1] IEC Standard 62320-1 – AIS base stations

[62320-2] IEC Standard 62320-2 - Aids-to-Navigation AIS Stations

[62320-3] IEC Standard 62320-3 – Simplex repeater station  (to be developed)

[62320-4] IEC Standard 62320-4 - General purpose Base station  (to be developed)

[61993-2] IEC Standard 61993-2 - Class A mobile AIS stations

[62287-1] IEC Standard 62287-1 - Class B (CSTDMA) mobile AIS stations

[62287-2] IEC Standard 62287-2 - Class B (SOTDMA) mobile AIS stations (to be developed)

[A-126] IALA Recommendation A-126 on AIS for Aids-to-Navigation

# Definitions

Additional definitions not included in A124:

# Abbreviations

Additional abbreviations not included in A124:

# FATDMA Planning and Operation of an AIS Service

Body of Annex to be determined by the content however the following should be considered:

Interfaces (logical/physical) to other functional blocks or to other E-Nav services

Functional Description

Requirements to Other Components

Special Considerations

Use of information

Risks

Clearly define any data structures

Use pictorial examples whenever possible

Define operating characteristics

# Table of Contents

# General

When setting up a FATDMA plan, there are three dominating aspects which need to be introduced:

- The required usage of FATDMA reserved slots, answering the questions: What would the FATDMA reserved slots used for by AIS stations?

- What geographical criteria apply to co-ordination of FATDMA schedules and schemes when using FATDMA reserved slots in a (larger) given area of a FATDMA plan?

- What rules are applicable for allocating FATDMA reserved slots to national competent authorities and other (domestic) competent authorities? In particular, what rule base governs the allocation of FATDMA schemes internationally?

These three aspects will be dealt with in the above order in the following sections after the introduction of important definitions.

# Definitions

This section defines relevant terminology used for FATDMA Configuration.

## Fundamental considerations for terminology

In order to make efficient usage of the available AIS VDL time slots in a given area a “proper plan” is needed. This “proper plan” may be called FATDMA Area Slot Map. It has two aspects:

- Setting up the FATMDA Area Slot Map (at planning time)

- Using the FATDMA Area Slot Map (at run-time).

These aspects and the composition of the FATMDA Area Slot Map are illustrated in the following figure. An explanation of the various aspects will be given afterwards (the figure employs the UML notation).

Figure 1: FATDMA Area slot map

The aspects which should be considered when planning FATDMA reservations on the AIS VDL in a given area, are described below:

- Geographical area aspect: Every FATDMA reservation is done within a defined area.

- Time domain aspect: The actual FATDMA reservations are done using the time domain. The time domain in the AIS is subdivided into time slots. Every FATDMA reservation consists of time slots reserved for a specific use. These FATDMA reservations can be described in terms using absolute time slot terminology and relative time slot terminology as follows:

- absolute slot numbers of FATDMA reservations: At planning time, the FATDMA reservations can only be planned using absolute slot numbers within the slot frame epoch. This requires an absolute reference point, which is the start of a specific frame epoch which in turn is related to the start of an UTC minute or hour. Since UTC is a universal concept, any absolute point in time can be determined thereby.

- Terms related to the relative slot numbers used for actual transmissions of FATDMA reservations: At run-time, the FATDMA reservation is being done be transmitting the Data Link Management Command (message 20). The parameters given by this command determine the number and location of the slots reserved by mobile stations. The parameters given determine the absolute position of the FATDMA allocated slots by reference to the absolute slot number of that slot, in which the Data Link Management Command was received. Therefore, these terms are relative by nature.

- Usage domain aspects: The usage domain is subdivided into two aspects:

- At planning time: Every FATDMA reservation is used for a purpose. Therefore, the usage aspect of FATDMA reservations must be considered. Each and every FATDMA reserved slot can and must have a designated usage allocated to it; otherwise its reservation would not be justified. This concept is called “Usage Designation”. It is further subdivided into Usage Categories.

- At run-time: The different potential users of FATDMA reservations (“VDL usage stake holders”) are identified as – by default and in accordance with the rules of the AIS VDL – all defined varieties of AIS stations, mobile or fixed. Their usage requirements as well as the overall requirement to maintain the viability of the AIS VDL need to be reflected in the Usage Designations at planning time.

- Roles of participating entities:

- At planning time the national competent authorities of individual countries sets up their FATDMA plan in accordance with the rules in this Annex. Should there be more than one national competent authority adjacent to the same geographical area, co-ordination would be done to guarantee the inter-operability of the individual FATDMA plans and create one joint FATDMA plan.

- At run-time every FATDMA reservation is done by one entity, called base station. Every time slot within this FATDMA reservation can be used by a variety of different entities participating in the AIS VDL, i. e. the AIS base stations proper, the AIS simplex repeaters and the different classes of AIS stations other than AIS base station.

In the following sections, the precise terms will be introduced.

## Terms related to FATDMA reservations in the time domain in general

The following terms are related to FATDMA reservations in the time domain in general, i.e. those terms are part of both the relative as well as the absolute FATMA terminology sets:

- FATDMA block: A FATDMA block consists of consecutive slots.

- FATDMA block size: The FATDMA block size is the number of consecutive slots of a block. The FATDMA block size can be between 1 and 5.

- FATDMA reservation: A FATDMA reservation is an announcement of a base station to create one or more FATDMA block(s) using the FATDMA reservation parameters. The FATDMA reservation is both an activity of a base station as well as the stationary knowledge of receiving mobile AIS stations, namely that the slots contained within the blocks have been reserved by one base station.

- FATDMA reservation parameters: Each FATDMA reservation is described by the following parameters (see figure). The FATDMA offset number is a relative term exclusively and will be explained in the next section.

Figure 2:
 Composition of FATDMA reservation

- FATDMA reservation time out: This value indicates, how many minutes the FATDMA reservation should be considered valid by the mobile AIS stations. The number of minutes results by adding one (1) to the values 0 to 7 in the FATDMA reservation(s) in message 20. Therefore, a FATMDA reservation can be made with a FATDMA reservation time out between 1 and 8 minutes.

- FATDMA increment: The FATDMA increment is the offset of slots between the first slot of the first FATDMA block and the first slot of the next FATDMA block of the same FATDMA reservation in the same frame. The FATDMA increment is then used repeatedly to create further FATDMA blocks in the same frame until the end of this frame is reached and no further FATDMA block could be accommodated. If the FATDMA increment is set to zero (0), then there will just one FATDMA block reserved by this FATDMA reservation.

- Data Link Management message (message 20): Actual FATDMA reservations are being made by transmitting the Data Link Management message (message 20) from a base station to mobile AIS stations.

- FATDMA reservation No N: Each Data Link Management message can include up to four (4) FATDMA reservations. Within the Data Link Management message, the FATDMA reservations are numbered from 1 to 4. The following figure illustrates, that all those four FATDMA reservations in one Data Link Management message inherit the above qualities of a FATDMA reservation, but may be different in terms of actual parameter setting.

Figure 3:
 Inheritance relationship between FATDMA reservation definition in general and individual FATDMA reservations

- Message-20-slot: The Message-20-slot is the slot, in which the Data Link Management Message (message 20) was transmitted. It has an absolute slot number. Starting with the Message-20-slot number the FATDMA offset number is counted (see below; relative terminology).

### Terms exclusively related to the relative slot number used for actual transmissions of FATDMA reservations

The following terms are exclusively related to the relative slot number used for actual transmission of FATDMA reservations.

- FATDMA offset number: Every FATDMA reservation has a parameter FATDMA offset number. The FATDMA offset number denotes the offset from the slot in which Data Link Management Message (message 20) was received to the first slot of the first FATDMA block to be reserved by the receiving station.

The relative composition of one (1) FATDMA reservation is illustrated by the following example figure.  In this example the parameters are as follows:

- Message 20 is transmitted in the slot with an absolute number of n (numbered 1 to n relative to frame start. Hence the Message-20-slot has the absolute slot number n. Note: The absolute number of the start slot of  the first FATDMA block can only be determined, if the absolute value of the Message-20-slot is known (which is n+25 in this example).

- FATDMA offset number :			25

- FATDMA block size:		 	5

- FATDMA reservation time-out:	 	3

- FATDMA increment:			375

The following figure illustrates this example.

oFATDMA Offset number

bBlock size

FATDMA Incrementincrement

n   n+1                                            n+ 25               n+29                          n+400              n+404                           n+775         n+779

free slot

reserved slot

Figure 4:Example for one FATDMA reservation within a Data Link Management message transmitted in Message-20-slot n

Additional FATDMA blocks are reserved in this example, each of which has a FATDMA block size of 5, starting at absolut slot numbers: n+1150, n+1525, n+1900 (if end of frame is not reached by then).

### Terms exclusively related to the absolute slot numbers of FATDMA reservations

Certain FATDMA related terms refer to an absolute framework of time and / or slot numbers. In addition, there are FATDMA related terms defined for areas, which may also be considered absolute (see next section).

- FATDMA start slot: The FATDMA start slot is the slot number of the first slot of the first FATDMA block within a frame (refer to Table 8 of Recommendation ITU-R M.1371-1). Therefore, the FATDMA start slot is an absolute term.

- FATDMA epoch: Due to the time out behaviour of FATDMA reservations and due to the need to devise FATDMA plans which minimise the actual number of FATDMA reservations (see next sections), a time span of more than a frame (i. e. one UTC-based minute) needs to be considered. These larger time spans are called FATDMA epochs. Each FATDMA epoch is 6 minutes long. The start of a FATDMA epoch is correlated with the beginning of the UTC hour as indicated by the FATDMA epoch number. Any FATDMA epoch starts with the first second of the first minute correlated with the UTC hour as given by the FATDMA epoch number and ends with the end of second 59 of the 6th minute after the start minute of the epoch.

- FATDMA epoch number: Each FATDMA epoch has assigned a unique number within the duration of one (1) hour. The FATDMA epoch number of each and every of the FATDMA epoch of one hour is correlated to an absolute start minute within the UTC hour as given in the table below:

| FATDMA epoch number x | Start minute of FATDMA epoch Number x | End minute of FATDMA epoch Number x |
| --- | --- | --- |
| 0 | 0 | 5 (included in epoch 0) |
| 1 | 6 | 11 (included in epoch 1) |
| 2 | 12 | 17 (included in epoch 2) |
| 3 | 18 | 23 (included in epoch 3) |
| 4 | 24 | 29 (included in epoch 4) |
| 5 | 30 | 35 (included in epoch 5) |
| 6 | 36 | 41 (included in epoch 6) |
| 7 | 42 | 47 (included in epoch 7) |
| 8 | 48 | 53 (included in epoch 8) |
| 9 | 54 | 59 (included in epoch 9) |

### Relationship between absolute and relative FATDMA terms

The following relationships exist between the above absolute and relative FATDMA related terms. Some FATDMA related terms are used identically, in an absolute as well as a relative context. In those cases, the term appears twice in the same line of the table. If there are different terms used in the same line of the table, the two terms correspond to each other, and a transformation / correlation is needed or performed at least at run-time. The following table gives the appropriate correspondence.

| Absolute terms used for FATDMA reservations | Relative terms used for FATDMA reservations |
| --- | --- |
| FATDMA plan | - no equivalent - |
| FATDMA schedule | - no equivalent - |
| FATDMA scheme | - no equivalent - |
| FATDMA epoche | - no equivalent - |
| FATDMA reservation | FATDMA reservation |
| FATDMA block | FATDMA block |
| FATDMA start slot | FATDMA offset number |
| FATDMA increment | FATDMA increment |
| FATDMA block size | FATDMA block size |
| Message20-slot | Message20-Slot |
| - no equivalent - | Message20 |

Table: Correspondence between absolute and relative FATDMA terminology.

## Terms related to the geographical areas for which FATDMA reservation planning is done

The terms related to the geographical areas for which FATDMA reservations are planned and performed at run-time are arranged in order to assist in the structured mutual FATDMA reservation planning of neighbouring administrations:

- FATDMA plan: A FATDMA plan is the highest level of FATDMA reservation planning. A FATDMA plan comprises all FATDMA schedules (see below), which are considered relevant to the FATDMA plan. Therefore, a FATDMA plan also comprises the area covered by the FATDMA schedules relevant to the FATDMA plan. It is the purpose of this Annex to describe a global FATDMA plan, which translates seamlessly to FATDMA schedules (regional or national) and FATDMA schemes (see below).

- FATDMA schedule: A FATDMA schedule comprises the relevant FATDMA schemes of one (1) or more Physical AIS Shore stations (PSS) in the area under consideration. The FATDMA schemes combined in a FATDMA schedule may be of the same or different competent authorities. Any FATDMA schedule needs to be consistent at its interfaces to adjacent FATDMA schedules.

- FATDMA scheme: A FATDMA scheme comprises the relevant FATDMA reservations of one (1) PSS. Hence, a FATDMA scheme may comprise the FATDMA reservations of one or more transmitting AIS base station(s) of that PSS.

- FATDMA Configuration: This general term denotes all aspects of FATDMA planning and FATDMA reservations made at run-time to be done by one or more competent authorities in a given area. It is an umbrella term.

The following schematic drawing illustrates the relationship between the various terms related to areas (note: each transmitting base station (BS) transmits Data Link Management Message (message 20) to set up FATDMA reservation(s).

FATDMA plan

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

FATDMA

Scheme

FATDMA

Scheme

FATDMA

Scheme

FATDMA

Scheme

FATDMA

Scheme

FATDMA Schedule

FATDMA Schedule

Figure 5:
 Hierarchy of FATDMA plan, FATDMA schedule and FATDMA scheme

## Terms related to the usage of FATDMA reserved slots (“Usage Designation”)

This section introduces the different terms for usage of FATDMA reserved slots.

### General considerations in regard to periodicity of transmissions using FATDMA reserved slots

When setting up a FATDMA plan, the required periodicity of a transmission needs to be considered. In regard to periodicity the reserved FATDMA slots can be used for:

- Short term repetitive pattern spanning a number of time slots within the same frame

- Intermediate term repetitive pattern spanning one frame

- Long term repetitive pattern spanning one (1) FATDMA epoch

There may be also one-shot transmissions, which are called spurious transmissions.

### General considerations in regard to timing of transmissions using FATDMA reserved slots

The timing of transmissions using FATDMA reserved slots is largely independent of the periodicity aspect introduced above, and needs to be considered in following ways:

- Urgency of transmission of a message(s): Regardless of their periodicity some AIS messages need to be transmitted with a very short delay. Besides the requirement to provide for prioritised queuing on the PSS Controlling Unit level, the urgency of transmission using FATDMA reserved slots requires the FATDMA plan to provide sufficient closely spaced, general-purpose FATDMA reserved time slots to allow for urgent transmissions.

- Timely availability of FATDMA reserved slots when allowing for dynamic features of FATDMA plans: When introducing dynamic features of a FATDMA plan, there is a  requirement to provide additional FATDMA reserved slots before the actual net data transmission can be made in these FATDMA reserved slots. Additional FATDMA reservations are to considered “timely”, in regard to the intended net data transmission, only if they allow for the following transient processes to be completed:

- Transmission of additional FATDMA reservation(s) by AIS base station(s) to be completed = initiation of dynamic FATDMA reservation

- Allow AIS mobile stations to clear the slots previously allocated by their own transmissions on both AIS channels completely. This process can not be fastened and – for un-garbled operation – will require 8 minutes from transmission of the additional FATDMA reservation.

This consideration substantiates the introductory general rule, that dynamic FATDMA reservations should be avoided or should be minimised.

### Terminology for categories for usage of FATDMA reserved slots

All of the above considerations are simplified when categorising the usages of FATDMA reserved slots into the following categories.

- FATDMA reservation usage category 1: AIS VDL management slots

These FATDMA reserved slots will be used for the transmission of any message, which is necessary for the management of the AIS VDL.

- FATDMA reservation usage category 2: High timing requirements slots

These FATDMA reserved slots will be used for the transmission of messages, which need to be transmitted complying to high timing requirements.

- FATDMA reservation usage category 3: General purpose pre-reserved slots

These FATDMA reserved slots will be used for the transmission of any message, which does not fall under FATDMA reservation usage category 1 or 2.

## Terms related to activities of entities involved in the FATDMA (“VDL Usage Stake Holders”)

### Terms related to roles and activities of entities related to FATDMA

The following terms are related to activities of entities involved in setting up and executing a FATDMA plan.

- FATDMA reservation: The FATDMA reservation, as viewed as an announcement, is an activity of a transmitting AIS base station.

### Roles and activities involved in making and using FATDMA reservations

A FATDMA reservation can be made by one entity participating in the AIS VDL and it will be used by one or more entities participating in the AIS VDL.

A FATDMA reservation can only be made by a base station of a competent authority.

- FATDMA reservation for transmissions of its own data and when acting as a semaphore;

- FATDMA reservation for transmissions of other base station/Limited Base Station (LBS);

- FATDMA reservation for transmissions of simplex repeater station(s)

- FATDMA reservation for transmissions for mobile AIS stations

An existing FATDMA reservation can therefore be used by one of the following entities participating in the AIS VDL:

- own AIS base station

- other AIS base station(s) (of own or other competent authority)

- other Limited AIS base station(s) (of own or other competent authority)

- other simplex repeater station(s) (of own or other competent authority)

- mobile AIS stations operating either in

- assigned mode, hard assignment (Class A, Class B, AtoN AIS stations) or

- responding to an interrogation (assigned by an AIS base station)

- fixed-slot usage AtoN mobile AIS stations

The following figure provides an overview of the different roles in regard to the FATDMA.

Figure 6: Activity of AIS base stations and AIS mobile stations for FATDMA reservation

Mapping of FATDMA related aspects to Layers of the AIS Service

The terms defined in this section can be associated to the different hierarchical layers of the AIS Service. This association indicates on what hierarchical layer of the AIS Service certain correlations between the "absolute" and "relative" realms need to be performed.

In accordance with the definition of the AIS base station in IEC 62320-1 there are two principal modes of operation for base stations:

- the independent mode: the base station in independent mode is independently, i. e. autonomously, performing tasks delegated to it from the PSS layer, such as autonomously creation and transmission of message 20 upon configuration from the PSS layer. To that end certain configuration PI sentences are used by the PSS. The configuration comprises the generally pre-programmed transmission timing, which is also given in absolute terms. If the independent mode of base stations is employed by a competent authority the PSS needs to set up and supervise the proper operation of all of its base station(s) in independent mode.

- the dependent mode: the base station in dependent mode transmits only pre-composed messages of whatever kind, which it receives from the PSS at a individually pre-determined time slot, the number of which is given in absolute terms. The base station does not compose messages itself when operating in dependent mode. Certain PI sentences are used by the PSS to forward the messages to be transmitted as well as the individually pre-determined time slot for transmission.

In both cases the task of translating from the absolute FATDMA-related terminology used throughout the AIS Service and the relative FATDMA-related terminology used on the AIS VDL is a task of  the PSS, which may be delegated completely to the AIS base station when operating in independent mode. When operating in dependent mode, the AIS base station still translates the transmission timing from the pre-determined absolute slot number it receives to the (relative) point in time, when the pre-determined transmission slot actually occurs.

Table: Association of terms related to FATDMA and Layers of the AIS and the AIS Service

| Term related to FATDMA | AIS VDL | AIS base station proper | PSS Controlling Unit ("PSS") | AIS Service Management | AIS Service as a whole | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| FATDMA Reservation (relative point of view) | X | X | - | - | - | Reservation announcement by base station on the AIS VDL as such; therefore only known to AIS VDL and AIS base station; similar to message 20 |
| FATDMA Reservation (absolute point of view) | - | X | X | X | X | Absolute equivalent of reservation |
| Message 20 | X | X (when AIS base station operates in independent mode) | X (only when AIS base station operates in dependent mode) | - | - | Consists of a set of 4 reservations; message 20 will be composed in AIS base station upon configuration settings from PSS when operating in independent mode or will be composed in PSS when AIS base station operates in dependent mode. |
| Message-20-slot | X | X | - | - | - | Message-20-slot is determined by the start slot |
| FATDMA start slot | - | X | X | X | X | Absolute term to be known for FATDMA scheme, schedule and plan; refer to Table 8 of Rec. ITU-R M.1371-1+Techn. Clarifications LME.FTST |
| FATDMA offset number | X | X (when AIS base station operates in independent mode) | X (only when AIS base station operates in dependent mode) | - | - | Relative term; refer to Table 33 of Rec. ITU-R M.1371-1+Techn. Clarifications |
| FATDMA block | X | X | X | X | X | FATDMA blocks are known both in relative and absolute terminology throughout the AIS Service and the AIS VDL |
| FATDMA increment | X | X | X | X | X | If FATDMA increments are used at all, they are known to all layers; refer to Rec. ITU-R M.1371-1 + Techn. Clarifications Table 8, LME.FTI |
| FATDMA block size | X | X | X | X | X | FATDMA block size is known both in relative and absolute terminology throughout the AIS Service and the AIS VDL (refer to Table 8 Rec. ITU-R M.1371-1; LME.FTBS) |
| FATDMA plan | - | -- | - | X | X | AIS VDL only recognizes the effects in terms of concrete reservations |
| FATDMA scheme | - | - | X | X | X | Group of all reservations of one PSS |
| FATDMA schedule | - | - | - | X | X | Group of all reservations of one or all PSS in given area |

# Introduction to relevant aspects of the FATDMA reservations

The following rules for FATDMA reservations are taken from Recommendation ITU-R M.1371 (current revision and IALA Technical Clarifications).

A FATDMA reservation is made by a base station transmitting Data Link Management Message, message 20.

A FATDMA reservation consists of

- a start slot, the slot where the FATDMA reservation starts in the frame,

- the block size, the number of consecutive reserved slots,

- an increment, the number of slots between the start of each reserved block and

- a time out value, how long the FATDMA reservation is valid.

The term FATDMA block denotes the effect of a FATDMA reservation on the AIS VDL.

Each Data Link Management Message (message 20) can reserve up to four (4) independent and different FATDMA reservations.

The blocks of FATDMA reservations should be evenly distributed across the frame. The values recommended for "increment" are as follows (refer to Technical Clarifications §2.48.1; message 20): 2, 3, 5, 6, 9, 10, 15, 18, 25, 30, 45, 50, 75, 90, 125, 225, 250, 375, 450, 750, 1125. Use of one of these values guarantees symmetric slots reservations throughout each frame.

Using the value 0 for "increment" means, that only one reservation is made in the frame.

The lower the "increment" value, the more slots will be reserved.

In normal operation the base station should use FATDMA for its own transmissions. For this, slots have to be reserved by a base station using Data Link Management Message (message 20). If there are no reserved FATDMA slots available the base station will transmit using RATDMA.

The FATDMA reservation is timed out after 3 minutes (default) in the mobile AIS stations. The time out value for the FATDMA reservation within the mobile AIS station can be configured between 1 and 8 minutes by the Data Link Management Message (message 20). The FATDMA reservation should be repeated before a time out occurs, if the FATDMA reservation is to be maintained.

The Data Link Management Message applies only to the AIS channel on which it is transmitted.

The effects of an FATDMA reservation on other AIS stations is as follows:

- Mobile stations will not include any FATDMA reserved slots in their candidate slot map at all, if the reserving base station is within 120 nautical miles. If the position of the reserving base station, as derived from message 4, is more than 120 nautical miles away from the mobile, Class A mobile stations should ignore the reservation.

- Other base stations will not include any FATDMA reserved slots in their candidate slot map when using RATDMA. However, at the discretion of the competent authority, a base station may intentionally FATDMA reserve and use the same time slots within 120 nautical miles, if no interference is likely.

# Fundamental philosophy statements for FATDMA Configuration

Before any details for FATDMA Configuration can be considered, some fundamental “philosophy” statements need to be set up.

## Guiding principles for setting up FATDMA plans

There are several possibilities to create such FATDMA plans, FATDMA schemes and FATDMA schedules. The following principles were adhered to when developing the FATDMA plan of this Annex:

### Rules in regard to the applicability of the FATDMA plan

- The rules underlying the FATDMA plan should be as simple as possible.

- The rules underlying the FATDMA plan should be globally applicable. The global applicability would provide for a fair and equal basis for setting up the FATDMA plans.

- The resulting FATDMA plan should cover the overwhelming majority of FATDMA configurations world-wide in just one planning step.

- When adjacent countries are required to co-ordinate this should require only the minimum number of parties involved, i. e. only the actually affected countries would treat the co-ordination issue independently of other co-ordination issues with other countries in a different area.

- There should not be a need to frequent or even run-time co-ordination between adjacent and affected countries.

- The most fundamental assumption is, that adjacent and affected national administrations are willing to protect the joint operation of the AIS VDL and to achieve its smooth operation in their area of (joint) responsibility. It is assumed that all affected national administrations will exhibit due diligence to arrive at a solution where co-ordination is needed.

- It is also assumed, that not all aspects of co-ordination can be satisfactorily achieved by FATMDA planning alone. Other mitigation methods consistent with the AIS system layout as a whole would be needed, and those mitigation methods are hinted at in this Annex but are fully described in other chapters of this document.

### Rules to minimise the usage of the VDL by FATDMA reservations

- The number of FATDMA reserved slots should be minimised in any given area. This would translate into the following consecutive rules:

- The FATDMA Configuration should be such, that FATDMA reservations are only done when there is a current (run-time) need for FATDMA reserved slots.

- The FATDMA block size should not exceed 5 consecutive slots (refer to Technical Clarifications §2.18.1 "Candidate Slots").

- However, this rule should not be applied rigidly, since the sophistication and the complexity of the components of the AIS Service(s) involved should be decreased rather than increased by a properly completed FATDMA plan. This translates into the following rule: The FATDMA plan should impose only a minimum of run-time dynamic FATDMA reservations. A maximum of stationary FATDMA reservations should be achieved.

- The FATDMA plan should provide for all capabilities of the AIS.

- The FATDMA plan should provide a maximum flexibility of AIS VDL operations.

- FATDMA reservations should be done, also using the frequency separation of the two working channels of the AIS (A and B) for the same slots. By default, mobile stations include the slots in the opposite working channel in the same slot only as a last resort into their candidate slot set, hence acting as if a de-facto FATDMA reservation was made in these other slots as well. Not to use these slots for FATDMA reservations would be a waste of available slots.

# FATDMA Reserved Slot Use Cases

This section describes the functional cases that require FATDMA reservations, the parameters that need to be considered, how to go about deciding the slot reservation profile and proposes usage recommendations for each case. The selection of the actual AIS base station reserving the slots will be discussed in another section.

Note: While the main concepts, such as the usage categories, are in place and are mature in this section, the detailed usage recommendations on individual messages should be reviewed and may be amended at the next revision of A-124.

## Rules for usage categories

The following rules, developed from the fundamental rules described in Section 4 apply to the  usage category definitions:

- At least one stationary FATDMA slot should be reserved for each of the defined usage-categories. This is necessary to minimise the dynamics of FATDMA allocation: All FATDMA reserved slots of a FATDMA plan, reserved for a given usage-category, should be reserved in such a way, that the appropriate FATDMA reservations comply exactly with the anticipated periodicity of the messages to be transmitted. To that end the FATDMA plan should take into account

- the required periodicity within one frame, which translates into the appropriate FATDMA increment and

- the usage of the individual minutes of an FATDMA epoch.

- In a single FATDMA reservation with more than one FATDMA reserved slot, the available slots can be used for different categories of usage independently of each other. This means, that slots reserved for different usage-categories can be bundled together in one FATDMA reservation, while their respective individual usage may be completely different.

## Use Cases for FATDMA reservation usage category 1 - AIS VDL management slots

The FATDMA reserved slots of usage category 1 will be used for the transmission of any message, which is necessary for the management of the AIS VDL.

The following use cases have been identified for FATDMA reservation usage category 1:

- Transmission of Data Link Management message  (message 20) by base station(s)

- Transmission of Base Station Report (message 4) with default reporting rate by base station(s)

- Transmission of Channel Management message (message 22) by base station(s)

Each of the above use cases will be dealt with in more detail in consecutive sections.

### Transmission of Data Link Management Messages (message 20)

When defining the usage of Data Link Management messages (message 20),  the following aspects should be taken into account:

- At least one FATDMA reservation must be made before FATDMA reserved slots become available. The transmission of a Data Link Management message 20 is needed to make any FATDMA reservations in a given area.

- When starting a new FATDMA reservation, i. e. when there has been none previously,

- the FATDMA reserved slots will be available in the next frame after the frame during which the transmission of message 20 occurred.

- the FATDMA reserved slots can only safely be used after 8 frames have elapsed because mobile AIS stations need to clear the FATDMA reserved slots.

- Depending on the total number of FATDMA reservations needed for a given geographical area, there may be more than one transmission of Data Link Management message 20 needed to accommodate all required FATDMA reservations in that area.

- Every FATDMA reservation times out after 3 minutes (default), if no different time out value is given within the FATDMA reservation. A different value may be between 1 and 8 minutes.

- Should a particular FATDMA reservation be maintained continuously longer than the above pre-set time out value, the FATDMA reservation needs to be refreshed periodically before the FATDMA reservation expires.

- Due to the time out value up to 8 minutes, one refreshment transmission of message20 per FATDMA epoch would be sufficient to maintain that FATDMA reservation.

- The overlap of 2 minutes between maximum time out pre-set and FATDMA epoch length, would allow the distribution of the required transmissions of message 20 over the first two frames of any FATDMA epoch, hence allowing for an even distribution of message 20 transmissions in the first two frames of the first FATDMA epoch of any new continuous FATDMA reservation.

- When updating the refreshment of a continuous FATDMA reservation at the minimum rate, i. e. once per epoch, AIS mobile stations entering the area under consideration would learn about this FATDMA reservation with a delay of up to nearly 6 minutes.

- Note: The periodicity of transmissions of message 20 should not be confused with the FATDMA increment of one FATDMA reservation. One FATDMA reservation may reserve many FATDMA blocks throughout the same frame, but may time-out after 8 minutes maximum because it was not refreshed.

Based on the above aspects two different use cases of transmission of message20 can be identified:

5.2.1.1 Transmission of Data Link Management message 20 to establish a short-lived FATDMA reservation (no repetitive pattern):

Short-lived FATDMA reservations should be used to accommodate dynamic demand for FATDMA reserved slots. 
Usage recommendation:

- When using a short-lived FATDMA reservation, its time-out should be pre-set to the duration of a FATDMA epoch, i. e. 6 minutes (or time-out value 5), maximum.

- Due to the purpose of short-lived FATDMA reservations, it should be carefully considered whether more than one FATDMA block should be reserved by this short lived FATDMA reservation, i. e. the FATDMA increment parameter should be used with care.

5.2.1.2  Transmission of Data Link Management message 20 to establish a continuous FATDMA reservation (long term repetitive pattern):

Continuous FATDMA reservations should be used to accommodate the stationary demand for FATDMA reserved slots.
Usage recommendation:

- Data Link Management message (message 20) should be transmitted at least once per epoch to maintain a continuous FATDMA reservation.

- When transmitting message 20, the time-out value should be set to 6 minutes, thus spanning one entire FATDMA epoch.

- The transmissions of message 20 for different continuous FATDMA reservations should be evenly distributed over the FATDMA epoch.

- Depending on the general risk assessment of message loss due to VDL traffic loading and / or interference in the given area, there should be up to 6 transmissions of the Data Link Management message (message 20) in one FATDMA epoch to maintain the same continuous FATDMA reservation, thus creating redundancy and providing for a more rapid reaction of new mobile AIS stations entering the given area to the FATMDA reservations. Those transmissions of message 20 for the same continuous FATDMA reservation should also be evenly distributed over the FATDMA epoch, i. e. one message 20 per 3 frames (2 transmissions per epoch), one message 20 per 2 frames (3 transmissions per epoch), one message 20 per one frame (6 transmissions per epoch).

### Transmission of Base Station Report (message 4) at the default reporting rate

Assignment of the transmission of Base Station Report (message 4) at the default reporting rate is justified, because a message 4 with a valid position allows all mobile AIS stations to correlate this position with the transmission(s) of Data Link Management message 20 using the same MMSI. The correlation is used by a mobile AIS station to determine whether the 120 nm intentional slot reuse rule should be used. This is an essential AIS VDL management rule because it  effectively counteracts the potential blocking of a large number of slots by the reception of non-relevant  message 20 from distant base stations during times of  good RF propagation.

When defining the usage of Base Station Report (message 4) at the default reporting rate, the following aspects should be taken into account:

- Recommendation ITU-R M.1371 states, that the default reporting rate by a base station should be once per 10 seconds.

- Therefore, the transmission of Base Station Reports exhibits a short-term repetitive pattern.

- This translates into six (6) slots reserved for that purpose per frame for a reporting interval of 375 slots, this translates into a FATDMA increment of 750 (message 20) in one FATDMA reservation on each channel.

Usage recommendation:

When transmitting Base Station Reports (message 4) with default reporting rate by one base station,

- there should be one (1) slot reserved with an increment of 750 on each channel, and

- the required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation (see above).

### Transmission of Channel Management Messages (message 22)

When defining the usage of Channel Management messages (message 22), the following aspects should be considered:

- The Channel Management message (message 22) provides the capability to change the  fundamental operation settings of the AIS frequency channels A and B.

- A mobile AIS station will store up to eight (8) different Channel Management settings, which were either received by Channel Management message, by a DSC telecommand or by manual setting aboard.

- If a new Channel Management setting is received, which partly or in total overlaps any of the previous Channel Management settings, the older Channel Management setting will be erased and replaced by the new Channel Management setting.

- Amongst other parameters, every Channel Management setting includes a geographical area (rectangle) for which this parameter set would be activated.

- The AIS mobile station uses the Channel Management setting corresponding to the geographical area in which it is currently located.

- The geographical area given for any Channel Management setting can be very large.

- The transient characteristics of a change in Channel Management settings involve at least the time span of two FATDMA epochs (compare IALA AIS Guidelines, Vol. I, chapter on Channel Management). Therefore, the transmission of Channel Management messages should be considered to exhibit long-term repetitive pattern.

- The timing requirements for the transmission of Channel Management messages as such are relaxed, i. e. the mobile AIS stations act upon a Channel Management setting when they enter the area designated by the Channel Management setting regardless of when they received the Channel Management setting.

- To ensure, that all mobile AIS stations operate on the default AIS frequency settings with the default Channel Management settings, a competent authority should transmit Channel Management messages on a regular basis with the default Channel management settings for its area of responsibility. These transmissions would instruct the mobile AIS stations to “return to default”. Due to the long-term repetitive pattern of the Channel Management message the VDL load created by these “return to default” transmissions would not be high.

Usage Recommendation:

- The geographical area parameter of any Channel Management message (message 22) should be as small as possible. It may be equal to the size of the geographical area of the FATDMA plan, FATDMA schedule or even FATDMA scheme.

- Channel Management messages should be transmitted at least once per FATDMA epoch.

- Depending on the general risk assessment of message loss due to VDL traffic loading and / or interference in the given area, there should be up to 6 transmissions of the Channel Management message (message 22) in one FATDMA epoch, thus creating redundancy and providing for a more rapid reaction of new mobile AIS stations entering the given area. Those transmissions of message 22 should also be evenly distributed over the FATDMA epoch, i. e. one message 22 per 3 frames (2 transmissions per epoch), one message 22 per 2 frames (3 transmissions per epoch), one message 22 per one frame (6 transmissions per epoch).

- The required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation (see above).

- Even when there is no channel management scheme in place, which deviates from the default Channel Management settings in any regard, the Channel Management message should be transmitted by administrations with default Channel Management settings (return to AIS1 and AIS2 in the geographical area) to erase „wrong“ and „old“ frequency settings in mobile AIS station’s memories.

## Use Cases for FATDMA reservation usage category 2 - High timing requirements slots

These FATDMA reserved slots will be used for the transmission of messages, which need to be transmitted within a defined time period.

The following use cases have been identified for FATDMA reservation usage category 2:

- Transmission of Base Station Report (message 4) when a base station in the given area is acting as a semaphore

- Transmission of Position Reports of any kind of AIS mobile stations, when assigned a “FATDMA protected” slot (“safe slot” concept)

- Transmission of Safety Related message (message 12 and 14)

- Transmission of Acknowledgements on received addressed messages (message 7 and 13) by base station(s)

- Transmission of DGNSS broadcast binary messages (message 17) with high timing requirements contents, such as integrity alerts, by base station(s)

- Re-transmission of Position Reports (and other messages) of any kind of AIS mobile stations

### Transmission of Base Station Reports when a base station (in the given area) is acting as a semaphore

The assignment of the transmission of Base Station Report (message 4) with increased reporting rate due to a base station switching into semaphore mode is justified, because the reporting interval is decreased to a value below 4 seconds. This constitutes a high timing requirement.

When defining the usage of Base Station Report (message 4) with increased reporting rate due to a base station acting as a semaphore, the following aspects should be taken into account:

- Recommendation ITU-R M.1371 state, that the increased reporting rate by a base station should be once per 3 1/3 seconds.

- Therefore, the transmission of Base Station Reports exhibits a short-term repetitive pattern.

- This translates into eighteen (18) slots reserved for that purpose per frame for a reporting interval of 125 slots or into a FATDMA increment of 250 (message 20) in one FATDMA reservation on each channel.

- Since the requirement for a base station to act as a semaphore is infrequent and transient in nature, one or several short-lived FATDMA reservations may suffice for the duration of the base station acting as a semaphore.

When transmitting Base Station Reports (message 4) with an increased reporting rate due,

- there should be one (1) slot reserved with an increment of 250 on each channel, and

- the required FATDMA reserved slots should be allocated by one Short-Lived FATDMA reservation with time-out value pre-set to maximum, i. e. 8 minutes, and consecutive Short-Lived FATDMA reservations with the same parameters as long as the base station is acting as a semaphore (see above).

- Instead, the required FATDMA reserved slots may be taken from a continuously and in advance reserved pool of usage category 2 slots within the same FATDMA scheme, FATDMA schedule or FATDMA plan. Although the latter option would smooth the operation of the VDL by avoiding transient FATDMA reservations, more VDL capacity will be used when it will not actually be needed.

### Position Reports of any kind of AIS mobile stations, when assigned a “FATDMA protected” slot (“safe slot” concept)

To assign the transmission of Position Reports of any kind of mobile AIS stations, which have been assigned to specific “FATDMA protected” slots given by a previous assignment command, to the FATDMA usage category 2 („High timing requirements slots”) is justified, because the position reports will invariably occur in the slots pre-determined by the previous assignment command, but the resulting reporting rate of the mobile AIS station may still be high (Class A mobile stations will allow only to increase the reporting rate up to once per second). This constitutes a high timing requirement.

When defining the usage the following aspects need to be taken into consideration:

- assigned transmissions of mobile stations either hard assignment (message 2, 18)

Usage recommendation:

When reserving FATDMA protected slots for assigned mode mobile stations

- The number of slots and the increment required depend on the defined reporting rate

- The required amount of slots should be allocated by one short lived FATDMA reservation preset to x minutes (depending on the timeout of the assignment)

### Transmission of Safety Related Messages (messages 12 and 14) by base station(s)

The assignment of the transmission of Safety Related Messages (messages 12 and 14) is justified, because it is a general operational requirement that safety related messages are being transmitted to the VDL with minimum latency after being released. This constitutes a high timing requirement.

When defining the usage the following aspects need to be taken into consideration:

Safety related message transmissions are of a non-periodic nature. Repeated broadcast of safety related messages within a timeframe of epoch magnitude may well be considered a nuisance rather than relevant information.

The number of slots required for safety related messages can dynamically vary between 1 and 5, however reserving blocks of timeslots larger than 3 on a continuous basis is generally discouraged.

Usage recommendation:

When reserving FATDMA protected slots for Safety Related Messages

- Safety related messages should be transmitted using available high timing or general purpose FATDMA reserved timeslots if possible.

- Addressed messages may be transmitted (up to 3 times) using RATDMA, since the acknowledge message will indicate whether or not the message was received. When the acknowledge is received, no repeated transmissions are necessary.

- If general purpose timeslots are not available, the required FATDMA reserved slots should be allocated by a Short-Lived FATDMA reservation, due to the non-periodic nature of the transmission.

### Transmission of Acknowledgements on received addressed messages (message 7 and 13) by base station(s)

The assignment of the transmission of Acknowledgements on received addressed messages (message 7 and 13) is justified, because the acknowledgement on a correctly received addressed message is required and must be received within 4 seconds by the originating station of the addressed message in order to prevent re-transmission of the same addressed message by the originating station, which would result in unnecessary VDL load. This constitutes a high timing requirement.

When defining the usage the following aspects need to be taken into consideration:

Acknowledgements are non-periodic of nature, and most likely occur infrequently.

Usage recommendation:

When reserving FATDMA slots for Acknowledgements of received addressed messages

- Acknowledgement messages should be transmitted using available high timing or general purpose FATDMA reserved timeslots if possible.

- RATDMA may be used if no FATDMA slots are available – to stop the addressed message from being repeated.

- If FATDMA reserved slots are required, they should be allocated by a Continuous FATDMA reservation.

### Transmission of DGNSS broadcast binary messages (message 17) with high timing requirements contents, such as integrity alerts, by base station(s)

The assignment of the transmission of DGNSS broadcast binary messages (message 17) with high timing requirements contents, such as integrity alerts, is justified, because of the relevant operational requirement in the field of GNSS.

Note: While the transmission of e. g. integrity alerts of DGNSS message to mobile AIS stations may exhibit a high timing requirement, this may not be true to any other DGNSS message content. Therefore, all DGNSS messages contents should be examined in regard to their timing requirements. DGNSS Message contents identified not to exhibit high timing requirements for transmission should be treated under the usage category 3 (“General purpose pre-reserved slots”).

When defining the usage the following aspects need to be taken into consideration:

- The most stringent demand with respect to the DGNSS corrections update rate is the integrity functionality. This holds true in particular to GPS since S/A was removed from GPS.

- The time for broadcasting a DGNSS integrity warning to the user, from the time the integrity problem is identified within the DGNSS correction source(s), i. e. a DGNSS reference station, to the reception of that DGNSS integrity alert at the user receiver equipment (here: the AIS mobile equipment) should not exceed 10 seconds. This results in a small latency budget for the AIS Service, which in turn translates into a high timing requirement for transmissions of DGNSS integrity alerts.

- There can be more than one integrity alert simultaneously, since more than one satellite may exhibit flawed behaviour. Up to four (4) DGNSS integrity alerts may be included in one DGNSS broadcast message using one (1) FATDMA reserved slot.

- This DGNSS integrity alert message may substitute regular DGNSS correction messages, which have a less stringent timing behaviour (therefore, those are included in FATDMA reservation usage category 3).

- Since the AIS Service does not create the DGNSS corrections and integrity alerts itself but rather receives them from an external DGNSS correction source (for instance the fully monitored IALA beacon reference station system, as stipulated by IALA Recommendation R – 121 Edition 1.1 Dec 2004 ), the replacement of regular DGNSS corrections with DGNSS integrity alerts – as need may be – will be done at the external DGNSS correction source. Therefore, the AIS Service will only be required to provide a transparent means to transmit the DGNSS correction broadcasts received from the external source within proper timing bounds.

- Only part of the above latency budget of 10 seconds can be used by the AIS Service. The other part would be needed by the DGNSS correction source to establish and distribute the integrity warning.

Usage recommendation:

- Provisions should be made to transmit a DGNSS broadcast message containing at least one DGNSS integrity alert, one slot every 6 seconds with an increment of 225, and

- the required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation.

- This would require to use FATDMA reserved slots of FATDMA reservation usage category 2.

- This defines the minimum update rate for DGNSS broadcast messages and the minimum number of FATMDA reserved slots for this service.

- While fulfilling this requirement there are still some alternatives to setup a regular DGNSS correction broadcast scheme. They will be discussed under the appropriate heading in FATDMA reservation usage category 3 below.

### Re-transmission of Position Reports (and other high-timing requirement messages) of any kind of AIS mobile stations

The VDL access schemes for Simplex Repeater Stations are either FATDMA or RATDMA. There are 3 types of Simplex Repeater Station

- FATDMA only

- RATDMA only

- FATDMA  and RATDMA

Simplex Repeater Stations can be configured to repeat either all received messages or  selected sub-sets of the received messages.

When defining the usage the following aspects need to be taken into consideration:

- A Simplex Repeater Station should repeat the message within 4 seconds (150 slots) and if it cannot find free slots within 4 seconds the message should not be repeated.

- The greatest impact on the VDL is when a Simplex Repeater Station is configured to repeat all messages – this will double the load on the VDL.

- Above 50% VDL loading starts to have an impact on Mobile AIS Stations ability to find free slots. Hence it is considered that the Simplex Repeater Station should not use more than 25% of the VDL bandwidth for its transmissions.

- RATDMA enabled devices should monitor that the total VDL load does not exceed 50%, and reduce it’s transmissions if this occurs.

Usage recommendation:

- The required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation.

- A Competent Authority should exercise great care when implementing a Simplex Repeater Station as inappropriate configuration could have a detrimental effect on the VDL.

- FATDMA reservations should not exceed 1125 timeslots (25 % of the VDL bandwidth)

- The Competent Authority should monitor the use of FATDMA Simplex Repeater Stations to ensure that the VDL loading does not exceed 50% in the vicinity of the Station for any significant period as this would adversely affect the ability of mobile stations to find free slots for their own transmissions.

## Use Cases for FATDMA reservation usage category 3 - General purpose pre-reserved slots

These FATDMA reserved slots will be used for the transmission of any message, which does not fall under FATDMA reservation usage category 1 or 2.

The following use cases have been identified for FATDMA reservation usage category 3:

- Transmission of any assignment command (message 16; [message 23]) by base station(s)

- Transmission of UTC and Date Inquiry message (message 10) by base station(s)

- Transmission of DGNSS broadcast binary messages (message 17) with relaxed timing requirements contents by base station(s)

- Transmission of Binary Messages (message 6 and 8) by base station(s)

- Transmission of Interrogation message (message 15) by base station(s)

- Transmission of Aids-to-Navigation reports (message 21; [new AtoN message])

- Re-transmission of messages other than position reports

### Transmission of any assignment command (message 16; message 23) by base station(s)

When defining the usage of any assignment command (message 16 or message 23), the following aspects should be considered:

- There are two possible options of the assignment command which impact the behaviour of mobile AIS stations: assignment of a new reporting rate behaviour while leaving the actual slot selection to the mobile AIS station (so called “soft” assignment) and assignment of the usage of specific pre-determined slots by the assigned mobile AIS station (so called “hard” assignment).

- Assignment Command (message 16) and Group Assignment Command (message 23) differ in that the Assignment Command applies to individual mobile AIS stations and provides for both “soft” and “hard” assignment, while the Group Assignment Command provides the capabilities to address group of mobile AIS stations determined by their respective device class.

- Every assignment times out after 3 minutes (default), if no different time out value is given within the assignment command. A different value may be between 1 and 8 minutes. The effect of the [Go silent] parameter for Class B mobile AIS stations within the Group Assignment Command (message 23) will last for up to 15 minutes, however.

- All transmissions of assignment commands do not – by default – exhibit a repetitive pattern. Therefore the transmission of assignment commands would require slot reserved in the usage category 3 (“General purpose pre-reserved slots”)

- Should a particular assignment be maintained over a longer period than 8 minutes, the assignment needs to be refreshed periodically before the present assignment expires. In this case, the following should be considered:

- Due to the time out value up to 8 minutes, one refreshment transmission of the assignment command per FATDMA epoch would be sufficient to maintain that assignment.

- When updating the refreshment of an assignment at the minimum rate, i. e. once per epoch, AIS mobile stations entering the area under consideration would learn about this assignment with a delay of up to nearly 6 minutes.

- While a mobile AIS station commanded to change into slot assignment mode (“hard” assignment) exhibits a high timing requirement for the pre-reserved slots, the assignment command as such has no high timing requirement for transmission. [The latency between the trigger event (assignment request) and the reaction shown by the assigned mobile AIS station(s) will be determined not by the timing of the transmission of the assignment command (message 16 or message 23) but by the present frame: Any assignment will be effective with the next frame earliest. Hence the latency can be up to one minute, which is the duration of one frame.]

NOTE: this will be checked with Class A, Class B etc. and clarification will be provided, in a future release of this document.

Usage recommendation:

- Assignment commands should be transmitted in one (1) of the next available slots reserved in the usage category 3 (“General purpose pre-reserved slots”).

- Should a particular assignment be maintained over a longer period than 8 minutes, the assignment needs to be refreshed periodically before the present assignment expires. In this case, the following recommendation applies:

- The assignment command (message 16 or message 23) should be transmitted at least once per epoch for a un-interrupted assignment as long as the desired duration of the assignment.

- When transmitting the assignment command, the time-out value should be set to 6 minutes, thus spanning one entire FATDMA epoch.

- Depending on the general risk assessment of message loss due to VDL traffic loading and / or interference in the given area, there should be up to 6 transmissions of the assignment command to maintain the un-interrupted assignment, thus creating redundancy and providing for a more rapid reaction of new mobile AIS stations entering the given area to the assignments. Those transmissions of assignment command for the same desired assignment should also be evenly distributed over the FATDMA epoch, i. e. one assignment command per 3 frames (2 transmissions per epoch), one assignment command per 2 frames (3 transmissions per epoch), one assignment command per one frame (6 transmissions per epoch).

### Transmission of UTC and Date inquiry message (message 10) by base station(s)

UTC and Date inquiry message may be used to query a mobile stations time syncronisation.

When defining the usage the following aspects need to be taken into consideration:

- The transmission has relaxed timing requirements

Usage recommendation:

- Reserving FATDMA timeslots for this purpose is not recommended.

- If a pool of general purpose FATDMA timeslots are available such a timeslot should be used, otherwise RATDMA can be used.

### Transmission of DGNSS broadcast binary messages (message 17) with relaxed timing requirements contents by base station(s)

The DGNSS broadcast binary message (message 17) is designated for distribution of DGNSS information. Broadcasting DGNSS corrections from AIS base station(s) on the AIS VDL enables the receiving AIS stations to provide position reports with integrity and improved accuracy.

#### Timing requirements depending on DGNSS correction message content

While the transmission of e. g. integrity alerts of DGNSS correction message to mobile AIS stations exhibits a high timing requirement, this may not be true to any other DGNSS correction message content. Therefore, all DGNSS correction messages contents should be examined in regard to their timing requirements. DGNSS Message contents identified not to exhibit high timing requirements for transmission should be treated here under the usage category 3 (“General purpose pre-reserved slots”).

The requirements of DGNSS correction messages containing high timing requirement integrity information were introduced under the usage category 2 (“High timing requirement slots”).

#### Responsibilities of the external DGNSS Correction Source compared to that of the AIS Service

The actual source for the DGNSS correction messages to be transmitted via the AIS is external to the AIS Service. That source, which should be set up and operated in accordance with the appropriate IALA recommendations, originates a constant flow of DGNSS correction messages, which could contain either DGNSS integrity alerts, thus leading to the “high timing requirements” as discussed above, and / or regular DGNSS corrections.

Since the AIS Service does not generate the DGNSS corrections and should not be involved in the sophistication of the generation of DGNSS corrections but rather act like an ideal transmission scheme for those DGNSS correction messages, it is the responsibility of the external DGNSS Correction Source to schedule the correct flow of DGNSS correction messages to the AIS Service, thus inserting DGNSS integrity alerts when required instead of regular DGNSS correction messages. On the other hand, it is the responsibility of the AIS Service to provide FATDMA reserved slots both for the DGNSS integrity alerts (see “high timing requirement section”) and for the regular DGNSS corrections (discussed here).

#### Degradation of DGPS accuracy with age of corrections

An important aspect when transmitting DGNSS corrections via AIS is the additional channel load in consequence of the necessary update rate of the DGNSS corrections, i. e. the DGNSS corrections update rate. Therefore, some consideration must be given to DGNSS correction update rate scenarios.

The following figure shows how the errors in a differentially corrected position fix increase with increasing age of correction in the absence of S/A [reference take from M. Hoppe’s paper]. The age-of-correction and hence the DGNSS broadcast message transmission interval for the same satellite could be extended from a value of less than 10 seconds (which is used in the IALA DGNSS MF beacon system) to about 3 minutes without the errors in position accuracy increasing by more than 0.1 m. Indeed, errors in position accuracy remain below 1m for all DGNSS broadcast message transmission intervals up to 8.5 minutes.

The following table shows the increase of position accuracy error for various intervals.

| Time interval between DGNSS broadcast message transmissions [minutes] | Position accuracy error at end of time interval [95%; in meters; absence of S/A] |
| --- | --- |
| 0.5 | Less than 0.1 |
| 1.0 | Less than 0.1 |
| 2.0 | Less than 0.1 |
| 3.0 | 0.2 |
| 4.0 | 0.4 |
| 5.0 | 0.6 |

#### Resulting scenarios for regular DGNSS correction message transmission using FATDMA reservations usage category 3

From the above considerations follows, that there are some different scenarios possible with regard to the DGNSS correction message broadcast.  There are identified the following five (5) different scenarios (see table below).

| Scenario | Consecutive Number of FATDMA reserved time slots needed per DGNSS correction message | Number of corrected satellites/bits needed (incl. 80 bit for header) | DGNSS correction message update rate | Resulting data age [s] for complete set of corrections | Slots needed for scenario and frame | Resulting AIS VDL Channel load [%] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | One Channel usage | Two Channel usage |
| Scenario 1 | 3 | 12 / 560 bits | 5 | 5 | 36 | 1.6 | 0.8 |
| Scenario 2 | 1 | 2 / 160 bits | 5 | 30 | 12 | 0.53 | 0.26 |
| Scenario 3 | 3 | 12 / 560 bits | 30 | 30 | 6 | 0.26 | 0.13 |
| Scenario 4 | 2 plus 1 | 8 / 400 bits 2 / 160 bits | 30 5 | 5 – 30 | 6 12 | 0.8 | 0.4 |
| Scenario 5 | 2 | 8 / 400 bits | 5 | 5 for 8 GNSS satellites; 10 for 12 GNSS satellites | 24 | 1 | 0.5 |

Scenario 4 implies the reservation of two FATMDA blocks. “FATDMA block 1” uses two (2) time slots of FATDMA reservation usage category 3 to transmit DGNSS corrections for up to 8 GNSS satellites with an update rate of 30 seconds. “FATDMA block 2” uses one (1) time slot of FATDMA reservation usage category 2 with an update rate of 5 seconds; this “FATDMA block 2” would be used for integrity alerts, whenever they appear, or would be used on a regular basis for regular DGNSS corrections.

Scenario 5 implies that the DGNSS correction source provides only DGNSS correction messages, regular and / or integrity warnings, to the 8 most relevant GNSS at the simultaneously.

#### Assessment of the scenarios presented

The pros and cons of the different scenarios are as follows.

##### Scenario 1

Advantages:

- Lowest possible age-of-correction for complete GNSS satellite set

- Integrity alerts already included in DGNSS correction messages:

- No need for additional FATDMA reservation usage category 2 slots (“high timing requirement slots”) due to DGNSS broadcasts

- No need for AIS Service to provide additional algorithm for discerning the different timing requirements, i. e. straight forward connection of AIS Service to DGNSS correction source possible

- No dynamic FATDMA reassignment needed for regular DGNSS correction messages

Disadvantages:

- Largest AIS VDL load of all scenarios considered due to DGNSS correction broadcast

##### Scenario 2

Advantages:

- Low AIS VDL channel load

- No dynamic FATDMA reassignment needed for regular DGNSS correction messages

Disadvantages:

- Relatively high data age for complete set of GNSS satellites

- requirement for additional FATDMA reservation usage category 2 slots(“high timing requirement slots”) when more than 4 GNSS satellites would need integrity alerts simultaneously, the probability of which should be considered low.

- An additional algorithm is needed in AIS Service to make sure that the most critical DGNSS correction data is transmitted with the highest priority for those max. 4 GNSS satellite’s integrity alerts.

##### Scenario 3

Advantages:

- Lowest AIS VDL channel load of all alternatives

- No dynamic FATDMA reassignment needed for regular DGNSS correction messages

Disadvantages:

- Relatively high data age for complete set of GNSS satellites

- Requirement for additional FATDMA reservation usage category 2 slots(“high timing requirement slots”) when any one GNSS satellite would need integrity alert.

##### Scenario 4

Advantages:

- Relatively low AIS VDL channel load

- No dynamic FATDMA reassignment needed for regular DGNSS correction messages

- Low age-of-correction for those DGNSS corrections transmitted using the one FATDMA reservation usage category 2 slot

Disadvantages:

- Requirement for one additional FATDMA reservation usage category 2 slot (“high timing requirement slots”) in any case, for use for regular DGNSS correction broadcast or for GNSS integrity alerts when any one GNSS satellite would need integrity alerts simultaneously.

- An additional algorithm is needed in AIS Service to make sure that the most critical DGNSS correction data is transmitted with the highest priority for those max. 4 GNSS satellite’s integrity alerts.

##### Scenario 5

Advantages:

- Data age is less than 10 seconds

- Integrity alerts already included in DGNSS correction messages:

- No need for additional FATDMA reservation usage category 2 slots (“high timing requirement slots”) due to DGNSS broadcasts

- No need for AIS Service to provide additional algorithm for discerning the different timing requirements, i. e. straight forward connection of AIS Service to DGNSS correction source possible

- No dynamic FATDMA reassignment needed for regular DGNSS correction messages

Disadvantages:

- Second highest AIS channel load of all scenarios considered due to DGNSS correction broadcast

- Maximum of 8 relevant GNSS satellites are corrected.

#### Provision for future additional need for DGNSS broadcast

The selection of one of the above scenarios may also be determined by the different GNSS to which DGNSS corrections should be provided. There may be a need to transmit DGNSS correction messages for GALILEO in the future. For the following reasons any such future requirement can be accommodated as long as the high timing requirements for the appropriate integrity alerts are met by FATDMA reservation usage category 2 time slots.

- The responsibility of the composition of the proper flow of DGNSS correction messages lies with the external DGNSS correction source, regardless to what GNSS the messages apply to.

- The degradation of position accuracy due to age-of-correction is graceful as shown above.

#### Recommended use

All of the above DGNSS correction broadcast scenarios have their trade-offs. All have in common that they do not require dynamic FATDMA reassignment provided that proper FATDMA reservation planning is done beforehand. All above scenarios would fulfil both requirements for DGNSS integrity alerts and regular DGNSS broadcast corrections.

Therefore, a competent authority may chose one of the above scenarios depending on the local situation. A competent authority may also change the DGNSS correction broadcast scenario over time.

### Transmission of Interrogation message (message 15) by base station(s)

A detailed description of the use case will be developed for a future release of this document.

Refer to ITU 3.3.8.2.11

### Transmission of Binary Messages (message 6 and 8) by base station(s)

A detailed description of the use case will be developed for a future release of this document.

The number of slots required for safety related messages can dynamically vary between 1 and 5 – message sizes above 3 timeslots are discouraged.

### Transmission of Aids-to-Navigation Reports (message 21)

A detailed description of the use of AIS on AtoN’s is available in IALA recommendation A-126.

When defining the usage the following aspects need to be taken into consideration:

- Two slots required for each AtoN message

- Update rate may vary, refer to A-126

- AtoN AIS stations may use slots, which have been reserved by a base station by FATDMA; therefore the AtoN AIS station could be configured to broadcast in fixed slots assigned by competent authority of FATDMA scheme

Usage recommendation:

- FATDMA reservations for AtoN’s should use a Continuous FATDMA reservation.

- To reduce bandwidth loading of the VDL, AtoN transmissions should be coordinated such that several AtoN’s may use the same timeslots within different frames of the epoch.

### Designing a FATDMA schedule

The use cases concerning VDL management transmissions and periodic transmissions with high timing requirements require dedicated patterns of timeslots available in the FATDMA schedule for a given Physical Shore station.  However, the use cases regarding  high timing requirement transmissions with non-periodic nature, and general purpose use with relaxed timing requirements, indicate that these transmissions could actually make more efficient use of a pool of Pre-Reserved Slots and a priority transmission queue.

When designing the layout of such a pool, the following considerations should be taken:

- The pool must have sufficient size to accommodate the required usage capacity within one frame to minimize the latency of the actual transmission and provide for relaxed queuing.

- It must have sufficient repetitive occurrence to accommodate the high timing requirements

- The transmission queue must be capable of giving high timing transmissions priority over relaxed timing transmissions.

# The definition of FATDMA schemes by space, time and frequency usage separation

After the previous section has introduced the purpose of FATDMA reservations on a detailed level and has given the conditions which apply for certain use case categories of FATDMA reservations in some detail, this section deals with the question: What should be the geographical criteria to determine co-ordinated operation?

## The universal grid scheme – separation by space and frequency

The geographical grid defines a global method for selecting a default FATDMA scheme for any area under consideration: It should be noted that the area is the most important aspect, not the actual location of the transmitting entities using FATDMA reserved slots. It is the total of transmitted, i. e. injected, messages of all relevant AIS stations using FATDMA reserved slots, that count in a given area, i. e. cell, not the station and its location as such in the first place.

It is the goal of the geographical grid to ensure, that identical FATDMA schemes by default are geographically separated more than 120 nautical miles, by arranging a set of FATDMA schemes in a geographical grid in such a manner, that no FATDMA scheme is reused within 120 nautical miles.

The geographical grid is meant as a geographical starting point from which further planning and co-ordination must take place, if needed at all.

The geographical grid is created by dividing the world into cells as illustrated below, by an algorithm, based on the WGS84 datum. The precise algorithm for calculating the geographical grid is given in Attachment A to this Annex. The following picture just gives an impression in principle.

The illustration contains a grid with a repetitive pattern of 36 different FATDMA schemes in an 6 x 6 matrix (called a ‘master cell’), where each cell is approximately 30 x 30 nautical miles in size. Employing the two working frequencies A and B of the AIS would yield 72 different FATDMA schemes in each master cell, while each individual cell would have the two FATDMA schemes created by using the very same time slot reservation pattern but only in the mirrored working frequency.

Identical FATDMA schemes are repeated in a distance of the size 120 nm + 2*cell size, i.e. in a distance of 180 nautical miles.

The above considerations yield one number of a FATDMA scheme. To denote the frequency separation, the two FATDMA schemes are identified by adding the Roman numerals “I” and “II” to the number, e. g. 59-I and 59-II. It should be noted that neither FATDMA scheme works exclusively on AIS working channel A or B exclusively, but alternate between these working channels constantly, but in a frequency mirrored manner. Hence, the Roman numerals were chosen to avoid confusion with the working channel designation A and B.

## FATDMA schemes – separation in time

Above the definition was introduced how to arrive at a cell number for the geographical cell, any given location on earth is in. Further the frequency separation was included. Both FATDMA schemes thus derived are identical in regard to their time slot reservation and usage characteristics, but employ these on mutually mirrored frequencies. It is now necessary to define to what absolute FATDMA reservations and what precise usage categories the above FATDMA scheme numbers translate to.

This is done in a table in Attachment B, in accordance with the following rationale:

As described in the section on usage categories, it is desirable that the FATDMA schemes are confined to a certain part of the time domain of the AIS VDL in order to provide sufficient free time slots for mobiles to allocate under all dynamical situations. The sum of all FATDMA schemes defined here constitute a repetitive pattern of 5 + 5 + 4 timeslots for every 25 timeslots:

| x | x | x | x | x |  |  |  |  |  | x | x | x | x | x |  |  |  | x | x | x | x |  |  |  | x | x | x | x | x |  |  |  |  |  | x | x | x | x | x |  |  |  | x | x | x | x |  |  |  | x | x | x | x | x |  |  | .... |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  | 5 |  |  |  |  | 10 |  |  |  |  | 15 |  |  |  |  | 20 |  |  |  |  | 25 |  |  |  |  | 30 |  |  |  |  | 35 |  |  |  |  | 40 |  |  |  |  | 45 |  |  |  |  | 50 |  |  |  |  | 55 |  |  |

This repetitive pattern of every 25 timeslots ensures, that mobiles’ transmissions fits neatly into the remaining timeslots, since the associated reporting rates of 2 seconds, 3 1/3 seconds, 6 seconds, 10 seconds... like those reporting rates defined for Class A mobiles translate into one transmission every 75, 125, 225, 375.. timeslots.

These FATDMA slot reservations are the theoretical maximum at planning time and will not be used all simultaneously in the overwhelming majority of cases globally in the same cell. For the few remaining cases, where special considerations are needed, see below.

The above FATDMA schemes are divided into slots assigned to specific usage categories as given in the Attachment B. The assignment to usage categories rest upon the following rationale. Each FATDMA scheme consists of:

- Usage category 1 time slots – VDL management

- Three (3) timeslots per frame and working channel for Base Station report in non-semaphore mode

- One (1) timeslots per frame and working channel for Data Link Management and / or Channel Management messages

- Usage category 2 time slots – High Timing requirements: Six (6) timeslots per frame and working channel for Base Station reports in semaphore mode or other high timing requirement transmissions; these time slots can be used for Usage category 3 (General purpose) in the absence of Usage category 2 messages.

- Usage category 3 time slots – General purpose: Seven (7) timeslots per frame and working channel for general purpose use or possibly for Usage Categories 1 or 2 (if urgent need should appear)

To re-iterate what was said before: Only time slots needed at run-time should be actually reserved.

Consequential action in particular in those (few) cases when the above FATDMA reservations would not suffice: The requirement for implementing queuing techniques (including priority schemes) as a mitigation measure on the part of the sources for transmission (not necessarily the AIS base stations of the national competent authority alone) is obvious.

After this definition part, the following sections will introduce the rules for allocating the above FATDMA schemes, referenced by their number, to competent authorities, both national and domestic competent authorities.

# Allocation of the defined FATDMA schemes to competent authorities

## Summary of the previous definitions and their consequences

The sections so far have introduced the definitions and concepts needed for a FATDMA plan. The means developed for setting up the FATDMA plan can be summarised by the following two parts:

- The geographical grid scheme introduced in the previous section provides for separation of FATDMA reservations in space, taking into account the 120 nautical mile rule.

- The FATDMA schemes, identified by their FATDMA scheme code, provide the combination of separation of FATDMA reservations in time, i. e. different time slots, and frequency, i. e. different working channels A and B, while preserving the alternating use of –those two working channels. The FATDMA scheme layouts have been informed by usage considerations as well, i. e. they assign certain usage categories to certain slots in the FATDMA schemes.

As a consequence, the FATDMA schemes, identified by their FATDMA scheme code, together with the geographical grid, provide a means to allocate FATDMA schemes for each geographical area, isolated from other geographical areas and their allocated FATDMA schemes – at least in a first, and in most cases sufficient step. Also, this is globally applicable.

Also the means exhibit a certain simplicity as it is built only on the concept of an area, as opposed to location: transmitting stations, using FATDMA reservations, inject their messages into that given area, i. e. one geographical grid cell, while the concrete location from where this transmission occurs within that geographical grid cell is less relevant due to the separations in space introduced by the geographical grid scheme.

From a practical point of view, this means that there will be minimum effort required to co-ordinate between adjacent countries and their national competent authorities because of the isolation of the FATDMA schemes in a given area: By introducing the geographical grid, only bi-lateral or tri-lateral co-ordination would be required in areas, where on the geographical grid cell falls into more than one country.

There are consequences that need to be discussed at a later planning stage, if they are relevant at all:

- The upper limit of the VDL loading due to FATDMA reservations can only be assessed in a given area after the concrete FATDMA plan for that area has been established.

- There may be restrictions to the usage of FATDMA reserved slots over time in areas, where a larger number of transmitting stations are planned to inject messages: Not every message to be transmitted may be transmitted at once when a FATDMA reserved slot allocated to that transmitting station arrives; there may be the need for queuing. These restrictions are due to the very nature of the AIS; a mitigation measure in the long run would be to increase the number of working frequencies of the AIS (which would require more than two receiving processes in mobile AIS stations).

- Proper, and even sophisticated coverage planning would be needed in some cases, employing directional antennas, lower antenna heights of the transmitting station(s) and even transmission power attenuation of the transmitting station(s), making use of or at least taking into account the local topography: The goal would be keep the injected message confined to that geographical grid cell if at all possible.

However, it is anticipated that these consequences will only be relevant to a few places globally, thus justifying extra planning efforts at those places (instead of not having a simple global plan for the large majority of cases). In addition, it is good to know that there are mitigation measures in place for areas where the most simple measures would no longer work.

## Stepwise allocation of the defined FATDMA schemes to competent authorities

Based on the above definitions and having considered the consequences, the following simple two-step approach for planning can be taken.

### Step 1: International allocation of FATDMA schemes

#### Step 1A: Check need for international co-ordination

All FATDMA schemes that fall completely – judging by the area of their geographical grid cell - within the EEZ (Exclusive Economical Zone) of a country belong to that country. They are immediately subject to domestic allocation rules (see step 2 below).

By means of the FATDMA scheme definition of the previous section this should cover the overwhelming majority of cases globally already in just one step.

#### Step 1B: Need to international allocation co-ordination arises

There are FATDMA schemes, which fall - by the geographical area of their geographical grid cell - into the adjacent EEZ of different countries. To these only apply the following rules.

- Thus affected countries and only the thus affected countries need to negotiate the allocation on a peer-to-peer basis and create a memorandum of understanding (or similar document) on the usage of the FATDMA scheme in question. They may use one or more of the following options, combining them as deemed necessary.

- Option A: One or more than one country can contribute an unused and immediately adjacent FATDMA scheme from within its own area (EEZ), where it can be guaranteed by that country that this cell will remain unused (e. g. due to topology).

- Option B:  The cell management is assigned solely to one of the two countries, while both may use the FATDMA scheme on a mutually agreed basis.

- Option C: Both countries set up a detailed usage allocation based on the usage categories of the slots of the FATDMA scheme.

- Option D: There may be an alternating use (e. g. based on epoch) of the same slots of a FATDMA scheme. In this case the countries need to agree on the time schedule for the alternating in advance.

- The result of the international allocation co-ordination is communicated internationally and appropriately.

- The affected countries are encouraged to do a maximum of co-ordination at planning time; there should not be a large need for run-time exchange of FATDMA scheduling data between countries.

### Step 2: Domestic allocation of slots out of the FATMA schemes for that country

- The national competent authority uses the internationally allocated FATDMA scheme on a primary basis; other competent authorities of that country use individual slots as a subset out of that FATDMA scheme due to allocation by the national competent authority on a secondary basis.

- The national authority represents the complete set of usage requirements (also of other competent authorities) to the international co-ordination (see step 1).

- The VDL management capabilities of the AIS (by AIS base stations) as well as the usage of AIS repeater stations are completely reserved to the national competent authority. They are exclusively considered part of the primary allocation domain.

- The other competent authorities should exclusively use other classes of AIS stations, such as the AIS AtoN station and / or Limited AIS Base Station, the definition of which are currently under development at IEC.

- There should be memorandum of understanding (or some similar document) between the national competent authority and other domestic competent authority stating the distribution of individual slots and their intended usage within the FATDMA schemes assigned to that country by international allocation.

## FATDMA configuration data exchange between Administrations at run-time

The previous chapters have shown that careful considerations between adjacent competent authorities are needed at planning time. Furthermore, a FATDMA set-up may not be changed frequently under „real life“ conditions: coverage planning and statutory rights of the competent authorities have been taken into account to arrive at the optimum FATDMA plan or FATDMA schedule for a given area.

While a theoretical possibility to exchange run-time data on actual FATDMA schedules (or maybe even FATDMA schemes) exists on the level of the ASM , this option should not be employed widely and is discouraged by this Recommendation to be used at all.

# Attachment A: Algorithm to calculate the FATDMA geographical grid scheme on a global basis

In order to achieve a well defined global geographical distribution an algorithm is used for calculating the position of each individual cell in the geographical grid. Upon input of any geographical position the algorithm will render the number of the FATDMA scheme valid for that geographical position, i. e. the number of the FATDMA scheme used in that cell.

The reference point of the geographical cell would be the [top left] corner for cells on the northern hemisphere, and the [bottom left] corner for cells on the southern hemisphere.

LAT =  (WGS 84 latitude in decimal degrees)

LON =  (WGS 84 latitude in decimal degrees)

// Note: The grid consists of a repeated pattern of 6 x 6 cells, each cell 30 x 30 nautical miles in size.

SingleCellSizeInNauticalMiles = 30;

NoOfSingleCellsAlongOneSideOfMasterCell = 6;

// Note: The repeatable pattern of 6 x 6 cells is denoted a ‘master cell’

MasterCellSizeInNauticalMiles =

SingleCellSizeInNauticalMiles * NoOfSingleCellsAlongOneSideOfMasterCell;
// Note: At Equator, one degree longitude approximately resembles 60 nm i.e. one minute longitude resembles one nautical mile

NoOfMasterCellsAroundEquator = 360 * 60  / MasterCellSizeInNauticalMiles;

//Note: At any latitude, one minute latitude resembles one nautical mile

MasterCellSizeInDegreesLatitude = MasterCellSizeInNauticalMiles / 60.0d;

//Note: abs(x) denotes the absolute value of x; int(x) denotes truncated integer value of x

MasterCellRowNo = int( abs(LAT) / MasterCellSizeInDegreesLatitude );

MasterCellMeanLatitude = (MasterCellRowNo+0.5) * MasterCellSizeInDegreesLatitude

//Note: cos(x) denotes cosine of x in radians

NoOfMasterCellsAroundMasterCellRow = 
int( NoOfMasterCellsAroundEquator * cos( 2 * π * MasterCellMeanLatitude / 360 ) );

SingleCellWidthInDegrees = 
360 / (NoOfSingleCellsAlongOneSideOfMasterCell * NoOfMasterCellsAroundMasterCellRow);

RowNumberInsideMasterCell = 
int ( abs(LAT) * 60 / SingleCellSizeInNauticalMiles ) - NoOfSingleCellsAlongOneSideOfMasterCell * MasterCellRowNo;

// Note: first Southern Hemisphere is assumed, then compensate if Northern Hemisphere

if (LAT>0)

{ RowNumberInsideMasterCell =

NoOfSingleCellsAlongOneSideOfMasterCell - RowNumberInsideMasterCell - 1;

};

ColumnNumberInsideMasterCell = int (abs(LON) / SingleCellWidthInDegrees);

while (ColumnNumberInsideMasterCell > (NoOfSingleCellsAlongOneSideOfMasterCell – 1 ) )

{ColumnNumberInsideMasterCell = 
ColumnNumberInsideMasterCell – NoOfSingleCellsAlongOneSideOfMasterCell

};

// Note: first positive longitude is assumed, compensate if negative longitude (west of Greenwich)

if (LON<0)

{ ColumnNumberInsideMasterCell =
(NoOfSingleCellsAlongOneSideOfMasterCell - 1) - ColumnNumberInsideMasterCell

};

ResultingCellNumber  = 
RowNumberInsideMasterCell * NoOfSingleCellsAlongOneSideOfMasterCell + ColumnNumberInsideMasterCell + 1

# Attachment B: Absolute slot number and usage category designation for the FATDMA schemes

The following table shows the absolute slot numbers, working channel usage, and usage category designations for the FATDMA schemes.

|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell Reference |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |
| 1-I | 0 | 1 | 2501 or 7502 | 1251 or 3752 | 1 | 2501 or 7502 | Base Station report* | 0 |
|  | 312 | 1 | 0 or 1125 | 877 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 602 | 0, 1, 2 or 3 | 0 or 1125 | 12 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 1-II | 125 | 1 | 2501 or 7502 | 01 or 5002 | 1 | 2501 or 7502 | Base Station report* | 125 |
|  | 877 | 1 | 0 or 1125 | 312 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 12 | 0, 1, 2 or 3 | 0 or 1125 | 602 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 2-I | 1 | 1 | 2501 or 7502 | 1261 or 3762 | 1 | 2501 or 7502 | Base Station report* | 1 |
|  | 313 | 1 | 0 or 1125 | 878 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 612 | 0, 1, 2 or 3 | 0 or 1125 | 27 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 2-II | 126 | 1 | 2501 or 7502 | 11 or 5012 | 1 | 2501 or 7502 | Base Station report* | 126 |
|  | 878 | 1 | 0 or 1125 | 313 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 27 | 0, 1, 2 or 3 | 0 or 1125 | 612 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 3-I | 10 | 1 | 2501 or 7502 | 1351 or 3852 | 1 | 2501 or 7502 | Base Station report* | 10 |
|  | 314 | 1 | 0 or 1125 | 879 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 619 | 0, 1, 2 or 3 | 0 or 1125 | 37 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 3-II | 135 | 1 | 2501 or 7502 | 101 or 5102 | 1 | 2501 or 7502 | Base Station report* | 135 |
|  | 879 | 1 | 0 or 1125 | 314 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 37 | 0, 1, 2 or 3 | 0 or 1125 | 619 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 4-I | 11 | 1 | 2501 or 7502 | 1361 or 3862 | 1 | 2501 or 7502 | Base Station report* | 11 |
|  | 627 | 1 | 0 or 1125 | 52 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 327 | 0, 1, 2 or 3 | 0 or 1125 | 902 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 4-II | 136 | 1 | 2501 or 7502 | 111 or 5112 | 1 | 2501 or 7502 | Base Station report* | 136 |
|  | 52 | 1 | 0 or 1125 | 627 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 902 | 0, 1, 2 or 3 | 0 or 1125 | 327 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 5-I | 19 | 1 | 2501 or 7502 | 1441 or 3942 | 1 | 2501 or 7502 | Base Station report* | 19 |
|  | 628 | 1 | 0 or 1125 | 53 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 337 | 0, 1, 2 or 3 | 0 or 1125 | 912 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 5-II | 144 | 1 | 2501 or 7502 | 191 or 5192 | 1 | 2501 or 7502 | Base Station report* | 144 |
|  | 53 | 1 | 0 or 1125 | 628 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 912 | 0, 1, 2 or 3 | 0 or 1125 | 337 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 6-I | 20 | 1 | 2501 or 7502 | 1451 or 3952 | 1 | 2501 or 7502 | Base Station report* | 20 |
|  | 629 | 1 | 0 or 1125 | 54 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 352 | 0, 1, 2 or 3 | 0 or 1125 | 887 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 6-II | 145 | 1 | 2501 or 7502 | 201 or 5202 | 1 | 2501 or 7502 | Base Station report* | 145 |
|  | 54 | 1 | 0 or 1125 | 629 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 887 | 0, 1, 2 or 3 | 0 or 1125 | 352 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 7-I | 21 | 1 | 2501 or 7502 | 1461 or 3962 | 1 | 2501 or 7502 | Base Station report* | 21 |
|  | 362 | 1 | 0 or 1125 | 927 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 637 | 0, 1, 2 or 3 | 0 or 1125 | 62 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 7-II | 146 | 1 | 2501 or 7502 | 211 or 5212 | 1 | 2501 or 7502 | Base Station report* | 146 |
|  | 927 | 1 | 0 or 1125 | 362 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 62 | 0, 1, 2 or 3 | 0 or 1125 | 637 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 8-I | 25 | 1 | 2501 or 7502 | 1501 or 4002 | 1 | 2501 or 7502 | Base Station report* | 25 |
|  | 363 | 1 | 0 or 1125 | 928 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 652 | 0, 1, 2 or 3 | 0 or 1125 | 77 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 8-II | 150 | 1 | 2501 or 7502 | 251 or 5252 | 1 | 2501 or 7502 | Base Station report* | 150 |
|  | 928 | 1 | 0 or 1125 | 363 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 77 | 0, 1, 2 or 3 | 0 or 1125 | 652 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |

| Default FATDMA schemes / 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 9-I | 26 | 1 | 2501 or 7502 | 1511 or 4012 | 1 | 2501 or 7502 | Base Station report* | 26 |  |
|  | 364 | 1 | 0 or 1125 | 929 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 662 | 0, 1, 2 or 3 | 0 or 1125 | 87 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 9-II | 151 | 1 | 2501 or 7502 | 261 or 5262 | 1 | 2501 or 7502 | Base Station report* | 151 |  |
|  | 929 | 1 | 0 or 1125 | 364 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 87 | 0, 1, 2 or 3 | 0 or 1125 | 662 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 10-I | 35 | 1 | 2501 or 7502 | 1601 or 4102 | 1 | 2501 or 7502 | Base Station report* | 35 |  |
|  | 677 | 1 | 0 or 1125 | 102 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 369 | 0, 1, 2 or 3 | 0 or 1125 | 937 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 10-II | 160 | 1 | 2501 or 7502 | 351 or 5352 | 1 | 2501 or 7502 | Base Station report* | 160 |  |
|  | 102 | 1 | 0 or 1125 | 677 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 937 | 0, 1, 2 or 3 | 0 or 1125 | 369 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 11-I | 36 | 1 | 2501 or 7502 | 1611 or 4112 | 1 | 2501 or 7502 | Base Station report* | 36 |  |
|  | 678 | 1 | 0 or 1125 | 103 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 377 | 0, 1, 2 or 3 | 0 or 1125 | 952 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 11-II | 161 | 1 | 2501 or 7502 | 361 or 5362 | 1 | 2501 or 7502 | Base Station report* | 161 |  |
|  | 103 | 1 | 0 or 1125 | 678 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 952 | 0, 1, 2 or 3 | 0 or 1125 | 377 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 12-I | 44 | 1 | 2501 or 7502 | 1691 or 4192 | 1 | 2501 or 7502 | Base Station report* | 44 |  |
|  | 679 | 1 | 0 or 1125 | 104 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 387 | 0, 1, 2 or 3 | 0 or 1125 | 962 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 12-II | 169 | 1 | 2501 or 7502 | 441 or 5442 | 1 | 2501 or 7502 | Base Station report* | 169 |  |
|  | 104 | 1 | 0 or 1125 | 679 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 962 | 0, 1, 2 or 3 | 0 or 1125 | 387 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 13-I | 45 | 1 | 2501 or 7502 | 1701 or 4202 | 1 | 2501 or 7502 | Base Station report* | 45 |  |
|  | 402 | 1 | 0 or 1125 | 977 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 687 | 0, 1, 2 or 3 | 0 or 1125 | 112 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 13-II | 170 | 1 | 2501 or 7502 | 451 or 5452 | 1 | 2501 or 7502 | Base Station report* | 170 |  |
|  | 977 | 1 | 0 or 1125 | 402 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 112 | 0, 1, 2 or 3 | 0 or 1125 | 687 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 14-I | 46 | 1 | 2501 or 7502 | 1711 or 4212 | 1 | 2501 or 7502 | Base Station report* | 46 |  |
|  | 403 | 1 | 0 or 1125 | 978 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 702 | 0, 1, 2 or 3 | 0 or 1125 | 119 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 14-II | 171 | 1 | 2501 or 7502 | 461 or 5462 | 1 | 2501 or 7502 | Base Station report* | 171 |  |
|  | 978 | 1 | 0 or 1125 | 403 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 119 | 0, 1, 2 or 3 | 0 or 1125 | 702 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 15-I | 50 | 1 | 2501 or 7502 | 1751 or 4252 | 1 | 2501 or 7502 | Base Station report* | 50 |  |
|  | 404 | 1 | 0 or 1125 | 979 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 712 | 0, 1, 2 or 3 | 0 or 1125 | 127 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 15-II | 175 | 1 | 2501 or 7502 | 501 or 5502 | 1 | 2501 or 7502 | Base Station report* | 175 |  |
|  | 979 | 1 | 0 or 1125 | 404 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 127 | 0, 1, 2 or 3 | 0 or 1125 | 712 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 16-I | 51 | 1 | 2501 or 7502 | 1761 or 4262 | 1 | 2501 or 7502 | Base Station report* | 51 |  |
|  | 727 | 1 | 0 or 1125 | 137 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 412 | 0, 1, 2 or 3 | 0 or 1125 | 987 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 16-II | 176 | 1 | 2501 or 7502 | 511 or 5512 | 1 | 2501 or 7502 | Base Station report* | 176 |  |
|  | 137 | 1 | 0 or 1125 | 727 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 987 | 0, 1, 2 or 3 | 0 or 1125 | 412 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

| Default FATDMA schemes / 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 17-I | 60 | 1 | 2501 or 7502 | 1851 or 4352 | 1 | 2501 or 7502 | Base Station report* | 60 |  |
|  | 728 | 1 | 0 or 1125 | 138 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 427 | 0, 1, 2 or 3 | 0 or 1125 | 994 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 17-II | 185 | 1 | 2501 or 7502 | 601 or 5602 | 1 | 2501 or 7502 | Base Station report* | 185 |  |
|  | 138 | 1 | 0 or 1125 | 728 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 994 | 0, 1, 2 or 3 | 0 or 1125 | 427 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 18-I | 61 | 1 | 2501 or 7502 | 1861 or 4362 | 1 | 2501 or 7502 | Base Station report* | 61 |  |
|  | 729 | 1 | 0 or 1125 | 139 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 452 | 0, 1, 2 or 3 | 0 or 1125 | 1002 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 18-II | 186 | 1 | 2501 or 7502 | 611 or 5612 | 1 | 2501 or 7502 | Base Station report* | 186 |  |
|  | 139 | 1 | 0 or 1125 | 729 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1002 | 0, 1, 2 or 3 | 0 or 1125 | 452 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 19-I | 69 | 1 | 2501 or 7502 | 1941 or 4442 | 1 | 2501 or 7502 | Base Station report* | 69 |  |
|  | 462 | 1 | 0 or 1125 | 1012 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 737 | 0, 1, 2 or 3 | 0 or 1125 | 152 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 19-II | 194 | 1 | 2501 or 7502 | 691 or 5692 | 1 | 2501 or 7502 | Base Station report* | 194 |  |
|  | 1012 | 1 | 0 or 1125 | 462 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 152 | 0, 1, 2 or 3 | 0 or 1125 | 737 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 20-I | 70 | 1 | 2501 or 7502 | 1951 or 4452 | 1 | 2501 or 7502 | Base Station report* | 70 |  |
|  | 463 | 1 | 0 or 1125 | 1013 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 744 | 0, 1, 2 or 3 | 0 or 1125 | 162 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 20-II | 195 | 1 | 2501 or 7502 | 701 or 5702 | 1 | 2501 or 7502 | Base Station report* | 195 |  |
|  | 1013 | 1 | 0 or 1125 | 463 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 162 | 0, 1, 2 or 3 | 0 or 1125 | 744 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 21-I | 71 | 1 | 2501 or 7502 | 1961 or 4462 | 1 | 2501 or 7502 | Base Station report* | 71 |  |
|  | 464 | 1 | 0 or 1125 | 1014 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 752 | 0, 1, 2 or 3 | 0 or 1125 | 177 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 21-II | 196 | 1 | 2501 or 7502 | 711 or 5712 | 1 | 2501 or 7502 | Base Station report* | 196 |  |
|  | 1014 | 1 | 0 or 1125 | 464 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 177 | 0, 1, 2 or 3 | 0 or 1125 | 752 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 22-I | 75 | 1 | 2501 or 7502 | 2001 or 4502 | 1 | 2501 or 7502 | Base Station report* | 75 |  |
|  | 762 | 1 | 0 or 1125 | 187 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 437 | 0, 1, 2 or 3 | 0 or 1125 | 1027 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 22-II | 200 | 1 | 2501 or 7502 | 751 or 5752 | 1 | 2501 or 7502 | Base Station report* | 200 |  |
|  | 187 | 1 | 0 or 1125 | 762 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1027 | 0, 1, 2 or 3 | 0 or 1125 | 437 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 23-I | 76 | 1 | 2501 or 7502 | 2011 or 4512 | 1 | 2501 or 7502 | Base Station report* | 76 |  |
|  | 763 | 1 | 0 or 1125 | 188 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 477 | 0, 1, 2 or 3 | 0 or 1125 | 1037 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 23-II | 201 | 1 | 2501 or 7502 | 761 or 5762 | 1 | 2501 or 7502 | Base Station report* | 201 |  |
|  | 188 | 1 | 0 or 1125 | 763 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1037 | 0, 1, 2 or 3 | 0 or 1125 | 477 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 24-I | 85 | 1 | 2501 or 7502 | 2101 or 4602 | 1 | 2501 or 7502 | Base Station report* | 85 |  |
|  | 764 | 1 | 0 or 1125 | 189 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 487 | 0, 1, 2 or 3 | 0 or 1125 | 1052 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 24-II | 210 | 1 | 2501 or 7502 | 851 or 5852 | 1 | 2501 or 7502 | Base Station report* | 210 |  |
|  | 189 | 1 | 0 or 1125 | 764 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1052 | 0, 1, 2 or 3 | 0 or 1125 | 487 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

| Default FATDMA schemes / 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 25-I | 86 | 1 | 2501 or 7502 | 2111 or 4612 | 1 | 2501 or 7502 | Base Station report* | 86 |  |
|  | 494 | 1 | 0 or 1125 | 1062 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 777 | 0, 1, 2 or 3 | 0 or 1125 | 202 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 25-II | 211 | 1 | 2501 or 7502 | 861 or 5862 | 1 | 2501 or 7502 | Base Station report* | 211 |  |
|  | 1062 | 1 | 0 or 1125 | 494 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 202 | 0, 1, 2 or 3 | 0 or 1125 | 777 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 26-I | 94 | 1 | 2501 or 7502 | 2191 or 4692 | 1 | 2501 or 7502 | Base Station report* | 94 |  |
|  | 495 | 1 | 0 or 1125 | 1063 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 787 | 0, 1, 2 or 3 | 0 or 1125 | 212 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 26-II | 219 | 1 | 2501 or 7502 | 941 or 5942 | 1 | 2501 or 7502 | Base Station report* | 219 |  |
|  | 1063 | 1 | 0 or 1125 | 495 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 212 | 0, 1, 2 or 3 | 0 or 1125 | 787 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 27-I | 95 | 1 | 2501 or 7502 | 2201 or 4702 | 1 | 2501 or 7502 | Base Station report* | 95 |  |
|  | 496 | 1 | 0 or 1125 | 1064 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 802 | 0, 1, 2 or 3 | 0 or 1125 | 227 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 27-II | 220 | 1 | 2501 or 7502 | 951 or 5952 | 1 | 2501 or 7502 | Base Station report* | 220 |  |
|  | 1064 | 1 | 0 or 1125 | 496 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 227 | 0, 1, 2 or 3 | 0 or 1125 | 802 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 28-I | 96 | 1 | 2501 or 7502 | 2211 or 4712 | 1 | 2501 or 7502 | Base Station report* | 96 |  |
|  | 812 | 1 | 0 or 1125 | 237 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 502 | 0, 1, 2 or 3 | 0 or 1125 | 1102 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 28-II | 221 | 1 | 2501 or 7502 | 961 or 5962 | 1 | 2501 or 7502 | Base Station report* | 221 |  |
|  | 237 | 1 | 0 or 1125 | 812 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1102 | 0, 1, 2 or 3 | 0 or 1125 | 502 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 29-I | 100 | 1 | 2501 or 7502 | 2251 or 4752 | 1 | 2501 or 7502 | Base Station report* | 100 |  |
|  | 813 | 1 | 0 or 1125 | 238 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 512 | 0, 1, 2 or 3 | 0 or 1125 | 1087 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 29-II | 225 | 1 | 2501 or 7502 | 1001 or 6002 | 1 | 2501 or 7502 | Base Station report* | 225 |  |
|  | 238 | 1 | 0 or 1125 | 813 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1087 | 0, 1, 2 or 3 | 0 or 1125 | 512 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 30-I | 101 | 1 | 2501 or 7502 | 2261 or 4762 | 1 | 2501 or 7502 | Base Station report* | 101 |  |
|  | 814 | 1 | 0 or 1125 | 239 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 527 | 0, 1, 2 or 3 | 0 or 1125 | 1077 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 30-II | 226 | 1 | 2501 or 7502 | 1011 or 6012 | 1 | 2501 or 7502 | Base Station report* | 226 |  |
|  | 239 | 1 | 0 or 1125 | 814 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1077 | 0, 1, 2 or 3 | 0 or 1125 | 527 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 31-I | 110 | 1 | 2501 or 7502 | 2351 or 4852 | 1 | 2501 or 7502 | Base Station report* | 110 |  |
|  | 537 | 1 | 0 or 1125 | 1113 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 827 | 0, 1, 2 or 3 | 0 or 1125 | 244 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 31-II | 235 | 1 | 2501 or 7502 | 1101 or 6102 | 1 | 2501 or 7502 | Base Station report* | 235 |  |
|  | 1113 | 1 | 0 or 1125 | 537 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 244 | 0, 1, 2 or 3 | 0 or 1125 | 827 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 32-I | 111 | 1 | 2501 or 7502 | 2361 or 4862 | 1 | 2501 or 7502 | Base Station report* | 111 |  |
|  | 538 | 1 | 0 or 1125 | 1114 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 837 | 0, 1, 2 or 3 | 0 or 1125 | 252 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 32-II | 236 | 1 | 2501 or 7502 | 1111 or 6112 | 1 | 2501 or 7502 | Base Station report* | 236 |  |
|  | 1114 | 1 | 0 or 1125 | 538 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 252 | 0, 1, 2 or 3 | 0 or 1125 | 837 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

| Default FATDMA schemes / 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 33-I | 18 | 1 | 2501 or 7502 | 1431 or 3932 | 1 | 2501 or 7502 | Base Station report* | 18 |  |
|  | 552 | 1 | 0 or 1125 | 1112 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 2 | 0, 1, 2 or 3 | 0 or 1125 | 852 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 33-II | 143 | 1 | 2501 or 7502 | 181 or 5182 | 1 | 2501 or 7502 | Base Station report* | 143 |  |
|  | 1112 | 1 or 2 | 0 or 1125 | 552 | 1 or 2 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 852 | 0, 1, 2 or 3 | 0 or 1125 | 2 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 34-I | 43 | 1 | 2501 or 7502 | 1681 or 4182 | 1 | 2501 or 7502 | Base Station report* | 43 |  |
|  | 562 | 1 | 0 or 1125 | 553 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 262 | 0, 1, 2 or 3 | 0 or 1125 | 862 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 34-II | 168 | 1 | 2501 or 7502 | 431 or 5432 | 1 | 2501 or 7502 | Base Station report* | 168 |  |
|  | 553 | 1 | 0 or 1125 | 562 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 862 | 0, 1, 2 or 3 | 0 or 1125 | 262 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 35-I | 68 | 1 | 2501 or 7502 | 1931 or 4432 | 1 | 2501 or 7502 | Base Station report* | 68 |  |
|  | 577 | 1 | 0 or 1125 | 1118 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 277 | 0, 1, 2 or 3 | 0 or 1125 | 869 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 35-II | 193 | 1 | 2501 or 7502 | 681 or 5682 | 1 | 2501 or 7502 | Base Station report* | 193 |  |
|  | 1118 | 1 | 0 or 1125 | 577 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 869 | 0, 1, 2 or 3 | 0 or 1125 | 277 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 36-I | 93 | 1 | 2501 or 7502 | 2181 or 4682 | 1 | 2501 or 7502 | Base Station report* | 93 |  |
|  | 587 | 1 | 0 or 1125 | 993 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 302 | 0, 1, 2 or 3 | 0 or 1125 | 1119 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 36-II | 218 | 1 | 2501 or 7502 | 93 or 593 | 1 | 2501 or 7502 | Base Station report* | 218 |  |
|  | 993 | 1 | 0 or 1125 | 587 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1119 | 0, 1, 2 or 3 | 0 or 1125 | 302 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

Name change currently considered in revision process of Recommendation ITU-R M.1371-1.

The value 2250 should not be used, as it implies a once per frame transmission; the value zero results in the same.

Means to avoid interference could include low, directional antennas and taking advantage of local land topography.

When acting as a semaphore, the reporting rate of a base station increases to a higher update rate. Therefore more FATDMA reserved slots would be needed and the timing requirements for transmissions of Base Station Reports in semaphore mode would be much more stringent. Therefore, this case is being dealt with at FATDMA usage category 2 (“High timing requirements slots” in the section below.

The requirements concerning DGNSS integrity alert (To-To-Alert TTA max. 10 sec) can be met by a maximum of 4 GNSS satellites at the same time.

The requirements concerning DGNSS integrity alert can only be met by provision of additional FATDMA reservation usage category 1 time slots.

Only the 8 most relevant GNSS satellites are corrected.

Name change currently considered in revision process of Recommendation ITU-R M.1371-1.