e-NAV Committee	e-NAV5/9/14

5th Session	September 15-19, 2008

Agenda items 9 & 7

U.S. Coast Guard

Using AIS to Notify Competent Authorities of Local GNSS Anomalies

Introduction

- In April 2001 the captain of a research vessel based in Monterey California USA complained that signal reception of GPS was unavailable for the whole of Monerey’s Moss Landing Harbor. When the problem persisted for a month local engineers eventually isolated the problem to unintentional radio interference coming from three commercially available VHF/UHF television antenna located on different vessels in the harbor.  The emission from just one antenna was sufficient to jam GPS reception throughout all of Moss Landing Harbor and an area at least 1 kilometer out to sea.  That same year the US Volpe National Transportation Systems Center concluded that “Because of the increasing reliance of transportation upon GPS, the consequences of loss of the GPS signal can be severe…both in terms of safety and environmental and economic damage to the nation, unless the threats are mitigated.”

- Even today, competent authorities responsible for the safety of maritime navigation generally have no ready means of knowing when a local GNSS anomaly or interference situation occurs, unless someone notices the problem and makes them aware of it, or unless the authority has equipment in the right place at the right time to detect the problem itself.  A possible solution may be to take advantage of the AIS transmissions being made by some 50,000 AIS-equipped vessels and received by national shore-based AIS stations.  Through attention to certain message contents, AIS can be a means of notifying competent authorities of the status of, or  of the presence of local interference to, the GNSS service for the local area surrounding AIS-equipped vessels.

Discussion

- The U.S. Coast Guard submitted two technical papers to the 23-27 June 2008 Intercessional Meeting of IALA eNAV AIS-TWG held at St. Johns, Newfoundland, proposing a possible solution to this problem and asking how shipborne AIS be best used to notify competent authorities operating a shore-based AIS network of a local GNSS anomaly or interference, be it GPS, GLONASS, Galileo, LORAN C/Chayka or some other system.  Although AIS is capable of performing this function, it is not its primary purpose and certain limitations, differences in interpretation of standards and differences in design of both AIS equipment and the GNSS equipment which feeds it appear to exist.  Since those differences in interpretation of standards and differences in design of AIS and GNSS equipment can affect safety of navigation, a review is perhaps warranted.

Recommendation

- The U.S. Coast Guard proposes the following be considered:

  - That guidance be developed for competent authorities on using AIS responses to monitor local GNSS anomalies in their areas of responsibility.

  - That specific recommendations to standards committees responsible for both GNSS equipment and AIS equipment be developed to clarify certain requirements regarding data interfaces to GNSS devices be considered.  For example, new tests of data interface sentences output from external navigation devices and input to AIS devices may need to be precisely specified in standards for both types of equipment to ensure clarity exists in the meaning of certain sentences, and sentences do not cause contradictory interpretations when more than one output sentence is used.

  - Expanding GNSS RAIM data transmitted in AIS Messages 1,2,3 from one to two bits in order to provide more useful information to mariners and shore authorities.

  - That the use and availability of an estimated (dead reckoning) position mode on SOLAS-certified GNSS receivers be reviewed.  Do equipment manufacturers avoid use of that mode in order to simplify the SOLAS certification process?  Should its availability be encouraged?  Should the GNSS device integral to AIS have an estimated position mode capability, or is an estimated position transmitted by AIS potentially more hazardous than a no-position-available signal (latitude/longitude = 91/181)?

  - The IEC certification standard for Galileo shipborne equipment is currently being developed, but it’s been years since the GPS and GLONASS shipborne equipment standard have been updated.  Should those standards be reviewed, especially in light of the increased dependence upon reliable and accurate information contained in data interface sentences?

  - That a binary message application be considered, designed to inform competent shore authorities or mariners of a local GNSS outage, anomaly or interference.  For example, that message could include more detailed RAIM information than is currently available, without need to modify the structure of Messages 1,2,3.

  - That the role of Class B CSTDMA in detecting GNSS anomalies, if any, be considered.  Class B CSTDMA devices stop transmitting whenever GNSS information becomes lost.

The Hunt for RFI, 1 Jan 2003, GPS World, http://www.gpsworld.com/gpsworld/article/articleDetail.jsp?id=43404&pageID=1&sk=&date=

Vulnerability Assessment of the Transportation Infrastructure Relying on the Global Positioning System, August 29, 2001, Volpe National Transportation Systems Center, www.navcen.uscg.gov/archive/2001/Oct/FinalReport-v4.6.pdf.

Using AIS transmissions to monitor local GPS/GNSS Anomalies and Interference, and MMSI and Sync State Statistics Collected from AIS transmission, dated 4 June and 13 June 2008 respectively.

e.g. No RAIM process available, Expected RAIM error is <10m, Expected RAIM error is >10m.  See Rec. ITU-R M.1371-3 table 47.