## 3.5	Long Range Identification and Tracking

Competent authorities with responsibility for aids to navigation, port security and other shoreside activities are often faced with the requirement to maintain surveillance of maritime approaches to their ports and port facilities for safety, security, and environmental protection.

These authorities are pursuing vessel tracking technologies to assist in the detection, classification, identification, and tracking of vessels. Among these technologies, Long Range Identification and Tracking (LRIT) is being implemented internationally for tracking ships globally.

Long range identification and tracking (LRIT) is a cooperative surveillance capability. In the simplified LRIT concept (Figure 1, courtesy of Inmarsat), a ship carries radio communications equipment that reports identification, position and time to authorities tracking that ship.  However, the final implementation of LRIT is more complicated as explained below.

Figure 1. Early LRIT Concept

The International Maritime Organization (IMO) Maritime Safety Committee (MSC), at its 81st session in May 2006 (MSC 81), adopted amendments to the Safety of Life at Sea Convention (SOLAS) for the long-range identification and tracking (LRIT) of ships (IMO MSC.202(81), SOLAS V/19-1). MSC 81 also approved performance standards and functional requirements for LRIT (MSC.210(81), later amended MSC.263(84)).

The SOLAS regulation on LRIT

Regulation 19-1 of SOLAS Chapter V (Safety of Navigation) entered into force on 1 January 2008, with ships required to transmit LRIT information in a phased-in period beginning 31 December 2008.  The regulation requires cargo ships of 300 gross tons and above, passenger ships, and mobile offshore drilling units on international voyages to be fitted with a system to automatically transmit LRIT information. Ships that operate exclusively within sea area A1 (essentially within VHF range of shore) and fitted with an automatic identification system (AIS) are not required to comply with the LRIT regulation.

Contracting governments, subject to certain restrictions, can receive LRIT information transmitted by ships as follows:

- Flag states: All flag ships worldwide.

- Port states: All ships indicating an intention to enter a port facility, at a distance or time set by the port state, but not in internal waters of another contracting government.

- Coastal states: All ships, regardless of flag, within a distance of 1000 nautical miles of the coast, but not in internal waters of another contracting government, nor in the territorial sea of the contracting government whose flag the ship is entitled to fly.

Administrations (the government of the state whose flag the ship is entitled to fly) may deny coastal states access to LRIT information at any time. Despite the regulation’s broad range to 1000 nautical miles for coastal states, it is important to note the first provision of the regulation:

Nothing in this regulation or the provisions performance standards and functional requirements adopted by the Organization in relation to the long-range identification and tracking of ships shall prejudice the rights, jurisdiction or obligations of States under international law, in particular, the legal regimes of the high seas, the exclusive economic zone, the contiguous zone, the territorial seas or the straits used for international navigation and archipelagic sea lanes.

Contracting governments must bear all communications costs for long-range identification and tracking information that they request and receive. A master of a ship may, for the protection of navigational information or when s/he considers LRIT operation may compromise the safety or security of the ship, switch off the LRIT shipboard equipment. Search and rescue (SAR) services of a contracting government may receive long-range identification and tracking information free of charge for SAR purposes.

LRIT Performance Standards and Functional Requirements

The approved performance standards and functional requirements for long-range identification and tracking lay out the LRIT system architecture (Figure 2) and describe how the long-range identification and tracking system works.

Figure 2. LRIT System Architecture

# In this architecture, the Administration determines whether its ships will report to a national, regional/cooperative, or the international LRIT data center. Each of these types of centers may use multiple communications service providers. The architecture is also designed to accommodate multiple application service providers.  Building on the basic concept noted above, a ship carries radio communications equipment that reports identification, position and time to the national, regional/cooperative, or international LRIT data centre tracking that ship. The Administration of the ship can access the LRIT information directly from the data centre.  Other Contracting Governments that are entitled to that information (i.e., port and coastal states) can request the information through their data centre and thence through the international LRIT Data Exchange.  The LRIT information is routed to the requesting data centre through the data exchange.  The rules for routing of information and the geographic polygons for all contracting governments are held in the Data Distribution Plan.

# In lieu of a dedicated LRIT data centre, an existing ship reporting or vessel monitoring system that may be able to function as national LRIT data centre within the LRIT system architecture. Examples of these include Victoria (Russian Federation); GICOMS (Republic of Korea); and AUSREP (Australia).