AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation A-124

On

Automatic Identification System (AIS) Shore Station and Networking Aspect relating to the AIS Service

Edition 1.3

December 2008

Edition 1.0 issued December 2002

Edition 1.1 issued December 2004

Edition 1.2 issued December 2005

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  http://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| September 2005 | Addition of Annex F – FATDMA Configuration of Physical AIS Shore Stations | Guidance on configuration of FATDMA is now available. Full revision of A-124 to occur after publication of IEC AIS non-ship-based testing standard 62320-1. |
| Jan 2007 | Section F | Editorial changes in Section F (number of reserved slots) |
| 2008-09 | Document | Removal of portions now covered by IEC documents. Addition of Annexes for DGPS correction broadcasts and for FATDMA planning. |

Recommendation on Automatic Identification System (AlS)

Shore Stations and networking aspects relating to the AIS Service

Draft Edition 1.3

(Recommendation A-124)

THE COUNCIL:

RECALLING the function of IALA with respect to Safety of Navigation, the efficiency of maritime transport and the protection of the environment;

RECOGNISING that IMO has concluded that AIS will improve the safety of navigation and the protection of the environment;

RECOGNISING ALSO that there is a mandatory carriage requirement for AIS equipment on SOLAS Convention vessels that entered into force on 1 July 2002 and will be complete by [2008];

RECOGNISING FURTHER that the Association has recommended that National Members providing aids to navigation services, including VTS, consider the introduction of AIS facilities in their shore infrastructure ( IALA Recommendation A-123)

NOTING that National Members intending to introduce AIS facilities in their shore infrastructure need information on the operational and technical aspects of such facilities;

NOTING ALSO that IALA has contributed substantially to the development of AIS through collaboration with IMO, ITU and IEC;

CONSIDERING that there is an urgent need for shore based AIS infrastructure to facilitate the interchange of data with shipping;

CONSIDERING ALSO the proposal of the [11th] session of the AIS Committee for the requirement for approval of the draft recommendation and subsequent publishing to the IALA Web site;

APPROVES the “Standard for AIS Shore Stations and networking aspects related to the AIS Service” set out in the annex to this recommendation for consideration by National Members; and,

RECOMMENDS that National Members introducing AIS facilities into their shore infrastructure take into account the “Standard for AIS Shore Stations and networking aspects related to the AIS Service” set out in the annex to this recommendation.

Table of Contents

Part I – General	6

1	IMO Provisions For AIS Shore Infrastructure	6

2	Additional Operational Applications of AIS Shore Infrastructure As Noted By ITU	6

3	AIS As A Maritime, Safety-Related, Information Service	7

4	The AIS Service within The Shore-Based Technical Environment	7

5	The Functional Interface Between The Shore-Based Applications And The AIS Service	9

6	The Layered Structure of the AIS Service	11

Part II - Installation Considerations for AIS Physical Shore Stations	18

7	Number and Location of AIS Physical Shore Stations	18

8	AIS Base Station Types	19

9	Limited Base Station(LBS)	20

10	Simplex Repeating functionality to Extend a Coverage Area	21

11	DGNSS Correction Broadcasts From AIS Shore Stations	23

12	Guidelines For FATDMA Configuration Set-Up Of AIS Shore Stations	23

13	Guidance For Channel Management By AIS Shore Stations	23

14	Network Considerations	23

Annex A: Services Operating on the VDL Simultaneously 	25

Annex B: Interfacing the DGNSS Correction Service to The AIS Base Station	26

1	Annex B Introduction	26

2	Annex B Abbreviation	26

3	Annex B Performance requirements for a DGNSS Service	26

4	Annex B Functional descriptions	28

Annex C – FATDMA Planning and Operation of an AIS Service	33

1	Annex C Introduction	33

2	Annex C General	33

3	Annex C Definitions	34

4	Annex C Introduction to relevant aspects of the FATDMA reservations	46

5	Annex C Fundamental philosophy statements for FATDMA Configuration	47

6	Annex C FATDMA Reserved Slot Use Cases	48

7	Annex C The definition of FATDMA schemes by space, time and frequency usage separation	60

8	Annex C Allocation of the defined FATDMA schemes to competent authorities	63

Annex D – Example of the Logical AIS Shore Station (LSS) Layer	73

1	Annex D Use Case of the Logical AIS Shore Station (LSS)	73

Annex E – Example of functionality of the AIS Service Management	80

1	Annex E Use Cases of the AIS Service Management (ASM)	80

# Part I – General

# IMO Provisions For AIS Shore Infrastructure

IMO's SOLAS Convention, as revised, Regulation 19, §2.4.5, states with regard to the purpose of the AIS:

"AIS shall

1.	provide automatically to appropriate equipped shore stations, other ships and aircraft information, including ship's identity, type, position, course, speed, navigational status and other safety-related information;

2.	receive automatically such information from similarly fitted ships;

3.	monitor and track ships; and

4.	exchange data with shore-based facilities."

In addition, the IMO Performance Standards for the AIS state:

"1.2	The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment, and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

.1	in a ship-to-ship mode for collision avoidance;

.2	as a means for littoral States to obtain information about a ship and its cargo; and

.3	as a VTS tool, i.e. ship-to-shore (traffic management).

1.3	The AIS should be capable of providing to ships and to competent authorities, information from the ship, automatically and with the required accuracy and frequency, to facilitate accurate tracking. Transmission of the data should be with the minimum involvement of ship's personnel and with a high level of availability.

1.4	The installation, in addition to meeting the requirements of the Radio Regulations, applicable ITU-R Recommendations and the general requirements as set out in resolution A.694(17), should comply with the following performance standards."

From this the provision of IMO for AIS shore infrastructure can be inferred. It should be noted however, that there is no obligation arising from IMO conventions for any competent authority to implement a VTS nor to implement AIS into existing VTS. However, since IMO has noted, that AIS improves the safety of navigation and the operation of VTS, competent authorities should consider implementing AIS within VTS.

IMO's provision for AIS shore infrastructure was taken up by ITU-R when creating Recommendation ITU-R M.1371, which included an AIS base station. When drafting a test standard for Class A and Class B shipborne AIS stations, IEC also took the existence and role of AIS base and repeater stations into consideration.

# Additional Operational Applications of AIS Shore Infrastructure As Noted By ITU

ITU also recognised the potential of the AIS for areas of shore-based application, other than ship reporting and VTS, namely maritime, safety-related information services, Aids-to-Navigation, and Search and Rescue. Refer to  Recommendation ITU-R M.1371-3.

"The ITU Radiocommunication Assembly considering (...)

d)	that such a system should be used primarily for surveillance and safety of navigation purposes in ship to ship use, ship reporting and vessel traffic services (VTS) applications. It could also be used for other maritime safety related communications, provided that the primary functions were not impaired;

e)	that such a system would be autonomous, automatic, continuous and operate primarily in a broadcast, but also in an assigned and in an interrogation mode using time division multiple access (TDMA) techniques;

f)	that such a system would be capable of expansion to accommodate future expansion in the number of users and diversification of applications, including vessels which are not subject to IMO AIS carriage requirement, aids to navigation and search and rescue;

Hence, the VTS would not be the only shore-based application to which the AIS would be of relevance. Therefore, the design of the technical AIS shore infrastructure, should be such that it would not limit the use of AIS information to VTS's needs. This statement does not reduce the prominent role of the VTS as the primary shore-based user of the AIS information.

# AIS As A Maritime, Safety-Related, Information Service

AIS provides a competent authority with an information service for shore-based VTS, traffic management schemes, ship reporting systems and other shore-based safety-related services.

This AIS information service consists of information delivery between ships and shore and vice versa. Thus the service of information exchange between ships and maritime, safety-related shore services, such as VTS, is one important part of the AIS (refer to SOLAS Regulation 19, §2.4.5, Nr. 4, as cited above). This information comprises, amongst others, the maritime, safety-related data items listed in IMO SOLAS Regulation 19 (see above).

# The AIS Service within The Shore-Based Technical Environment

Figure 4.1 shows an overview of the functional technical layers needed to process data derived from various shore-based sensor services to present the user with a complete traffic picture. The layers above the different services will be dependent on user requirements and complexity of the infrastructure and will not be described herein.

Figure 4.1: Functional technical layers

Radio

Direction

Finding

Service

User, e.g.  VTS-Operator or MRCC-Operator.

Provide Human-Machine-Interface

Provide Operational Application(s) for this User

(based on all available ship data)

Ship Data Processing and Storing Services

Long

Range

Monitoring

Service

Ship's Position / Dynamic Data Correlation Service

.....

.....

(additional services)

.....

(additional services)

.....

(addi-tional

services)

AIS Service

Radar Service

.....

(additional

position

related

services)

In a VTS environment the AIS-derived information of the AIS Service would be integrated e.g. with the Radar Service, the Direction Finding Service, Ship Data Processing Services, etc.

According to need, the competent authority may define more than one AIS Service, e.g. East Coast AIS Service and West Coast AIS Service, which may have different properties. When defining more than one AIS Service within one competent authority, it is beneficial to have complete operational, geographical, physical, and data separation between these services to avoid interference. Should a complete separation of different AIS Services, also of different neighbouring competent authorities, not be possible, the competent authority or authorities should take measures to minimise the potentially harmful interference of their AIS Service, e.g. FATDMA planning (Annex D).

For simplicity's sake, this document, in its present edition, confines itself to the treatment of one AIS Service in one competent authority, only.

# The Functional Interface Between The Shore-Based Applications And The AIS Service

The functional interface between the AIS Service and shore-based applications is known as the  Basic AIS Services (BAS).   The BAS should facilitate the integration of the AIS Service shore-based applications, such as radar-based VTS environments.

The BAS encapsulates the technical details of both the AIS technology and of the layout and local configuration of shore-based AIS.

## Basic AIS Services

The purpose and functions of the AIS can be expressed in terms of services provided to the recipient. The most fundamental services of the AIS are called Basic AIS Services (BAS). They make use of the diverse features of both  the AIS VHF Data Link (VDL) and of the different AIS stations. The BAS are described in a common format.

This description of the BAS does not make redundant the referenced documents, i.e. the appropriate international standards, nor does it introduce new system features. However, each description of a BAS integrates informative items from various sources into functional terms that are essential to understand what this BAS is delivering.

The BAS are subdivided into "external" and "internal". The "external" BAS are used to deliver information about ships and shipboard applications. The "internal" BAS are used to make use of the special capabilities of the AIS base station with regard to the AIS VDL.

The BAS are the interface between the AIS Service and "higher", application-oriented levels. Therefore, their description also takes into account the processing at the fixed AIS stations, the Physical AIS Shore Station (PSS), and the Logical AIS Shore Station (LSS).

## Basic AIS Service Overview

The following provides an overview of all the Basic AIS Services

### External BAS

- A_STAT		Static ship data from Class A shipborne mobile AIS stations

- A_DYN		Dynamic ship data from Class A shipborne mobile AIS stations

- A_VOY		Voyage related ship data from Class A shipborne mobile AIS stations

- SAFE_AD	Safety related addressed message

- SAFE_BR	Safety related broadcast message

- INT_TDMA	interrogation via AIS VDL

- INT_DSC		interrogation via DSC Ch 70

- B_DAT		Ship data from Class B shipborne mobile AIS stations

- SAR_DAT	Data from SAR airborne AIS stations

- ATON_DAT	Data from AtoN AIS stations

- TRANS_IAI	Transparent transmission within the International Application Identifier branch

- TRANS_RAI	Transparent transmission within the Regional Application Identifier branch

### Internal BAS

- BASE_DAT	Data on base station (base station's own data)1

- ASGN_RATE	Assignment of reporting rate to mobile station(s)

- ASGN_SLOT	Assignment of transmission slots to mobile station(s)

- DGNS_COR	DGNSS corrections to mobiles

- CH_TDMA	Channel management by TDMA

- CH_DSC		Channel management by DSC

- PWR_LEV	Power level setting to mobiles

- FATDMA		(announce) configuration of FATDMA reservations of base station(s)

# The Layered Structure of the AIS Service

Figure 6.1: Layered structure of AIS Service

Technical Operation /

Technical Maintenance

MMI

Logical AIS Shore Station (LSS)

Status reports to higher levels

MMI

Physical AIS Shore Station (PSS)

MMI

Fixed AIS Stations Layer (Base stations, repeater stations)

MMI

HF components for the VHF maritime mobile Service

MMI

AIS VDL

(TDMA; optionally DSC)

The AIS Service

Co-loca-tion with other VHF services

MMI

Net Data Output From / Input to External Basic AIS Services

AIS Service
Management

Technical Operation / Technical Maintenance (in principle for each layer)

* Note that this symbolic representation does not imply anything concerning the number of required personnel for the technical operation or technical maintenance of the AIS service.

## The Functional Layers For The AIS Service And The AIS Shore Infrastructure

The functional Layers of the AIS Service and AIS shore infrastructure are as follows:

- The ASM (AIS Service Manager) layer is responsible for managing the AIS Service for the higher levels.

- The LSS layer processes the data derived from the different physical AIS shore stations and provides the Basic AIS Services to the applications during run-time;

- The PSS layer consists of one or more fixed AIS stations;

- The fixed AIS station layer consists of the actual AIS basestation or repeater hardware;

- The RF components layer providing  VHF/RF domain equipment for fixed AIS stations, i.e. antenna(s) and other pieces of on-site infrastructure;

- The AIS VHF Data Link (VDL) layer which can only be accessed through the Fixed AIS station with the correct VHF-/RF-domain equipment.

Each of these layers may consist of different entities, which in total must deliver the complete functionality of the layer. On the fixed AIS  station layer, for example, the entities may be AIS base stations or AIS simplex repeater stations, or both.

In addition the following fundamental technical prerequisites must be considered during the planning process:

- The physical coverage of all physical AIS shore stations of one competent authority should always exceed the required coverage area of all logical AIS shore stations defined by that competent authority, where the same service level in terms of availability, reliability etc. applies.

- Each of the layers of the AIS Service requires a certain amount of processing capacity. The competent authority should provide the needed processing capacity to ensure proper operation of the AIS Service.

- The layered stack does not expressively state transportation of data over distances. Between each layer a transportation process is required; however, the distances and capacity requirements of which are totally dependent on local conditions. It is assumed, that the transportation processes do not constitute a bottleneck which determine the overall functionality. This can be achieved by standard modern technology even with high capacity needs.

- Between each layer there are functional interfaces.

## The AIS Service Management (ASM)

Since the AIS Service of a competent authority will, in most cases, comprise more than one LSS, PSS, and fixed AIS station, there is a need for a top layer, which acts as a controlling entity for the whole of the AIS Service.  The ASM:

"owns" all the logical and physical shore stations, i.e. it invokes, initialises, configures and terminates the logical and physical shore station software processes at run-time;

determines the network communication relationships between physical shore stations and their associated logical shore stations for them to use during run-time;

determines the communication relationships between the logical shore stations and the applications associated with them, i.e. this top level acts as a "switch-board" for the data exchange relationships between the different processes.

Figure 6.2

Example of an AIS Service with clients and providers of AIS Data. This example describes an AIS Service, which includes Physical Shore Stations (PSS), Logical Shore Stations (LSS) and an AIS Service Management (ASM) entity. Several PSS are assigned to at least one LSS, which combines the data from these PSS in order to cover a certain area. In this example the AIS Service includes several LSS. Clients will receive data from the LSS distributing the data of interest to that certain client. The management of the AIS Service is done by the ASM. This entity controls the data flow in the system and configures the different entities in order to fulfil the given requirements.

## Logical AIS Shore Station (LSS)

A LSS is a software process that interfaces the PSS of the AIS service to the applications.  The LSS must consider the following:

- operational aspects of the applications using the AIS service, and

- technical aspects which arise when operating a network of PSS.

The software process of a logical AIS shore station can run on any appropriate computer at any location. It is required, however, that there are reliable data transportation means between all associated physical AIS shore stations and the AIS Service.

## Physical AIS Shore Station (PSS)

A PSS consists of at least the following components:

- one AIS base station and/or one AIS repeater station;

- power supply;

- VHF-/RF-domain equipment, at minimum simply a cable and a VHF antenna;

- if the PSS houses an AIS base station a means to transport data to and from the AIS base station is required (an AIS repeater may operate without this data transport means);

- a means to protect the above component against environmental influence and damage, e.g. a shelter building or a housing case.

In addition, a PSS will have a UTC source of its own. This UTC source may be internal to the AIS base station such as a GNSS receiver, or just internal to the PSS, such as an atomic clock or a Loran timing receiver.

In addition, there may be optional AIS-related functions added to the PSS' set-up, e.g. DGNSS correction source, AtoN station's functionality source, remote control equipment, or logging devices. Also, the PSS will, in most cases, comprise a control device, which, for instance, monitors the integrity of operation of some or all devices of that PSS, or which performs filtering functions for those AIS base and repeater stations housed by this PSS.

## Fixed AIS Stations Layer

The following fundamental design philosophy statement(s) should apply throughout the descriptions for Fixed AIS Stations:

- For maintenance purposes all fixed AIS stations should have modular construction.

- The functionality of fixed AIS stations should be able to be upgraded at minimal expense and minimal downtime for the fixed AIS station;

- All fixed AIS devices should have status monitoring, and the status information should be output via an appropriate interface.

The functionality of different fixed AIS stations, as identified by Recommendation ITU-R M.1371-3, is described separately, although some functionalities may run simultaneously at run-time in one device as indicated by Table 6.1.

Table 6.1: Possible / Impossible combinations of functionality of fixed AIS stations at run-time

|  | AIS base station functionality | AtoN-AIS-station functionality | AIS simplex repeating |
| --- | --- | --- | --- |
| AIS base station functionality | N/A | Possible | Possible |
| AtoN-AIS station functionality | Possible | N/A | Possible |
| AIS simplex repeater | Possible | Possible | N/A |

It is possible, in principle, that manufacturers develop fixed AIS stations, which may also be configured as AIS simplex repeater stations.

### AIS Base Station

The AIS base station is the most basic AIS-related entity of any AIS shore infrastructure. Conceptually, it is a "bare bones" black-box like device .  The AIS Base Station allows for the full control of the AIS VHF data link.  The AIS Service in a littoral or port state is operated by the national competent authority for the whole of the national coverage area. This national competent authority has the need to control the AIS Service and the AIS VHF data link, by employing means like channel management, protected slot allocations, and assigned mode and the AIS base station is the only means through which these requirements can be accomplished.

However, there are other competent authorities responsible for local operations of e. g. individual harbours, locks, marinas, off-shore structures. These regularly have an interest in monitoring the vessel traffic in their locally confined area of competence and exchange limited operational information with ships in regard to their area of competence, e. g. docking scheduling, tugs operation.  For those purposes the concept of a Limited Base Station (LBS) was developed.

### AIS Repeaters

The AIS essentially is a simplex system, and the AIS easily provides for a simplex repeater or simplex repeating process.

The main application of simplex repeating is to extend the coverage, or to overcome local obstacles to radio propagation.

Within the AIS Service, simplex AIS repeaters are located on the same layer as AIS base stations, i.e. they have direct access to the VHF/RF domain equipment for reception and transmission of VDL messages.

This document describes the simplex repeater station. More detailed planning considerations and application notes are given in the IALA AIS Guidelines (Edition 1.1, Volume I, Part II).

## VHF-/RF Domain Equipment

The VHF-/RF domain equipment routes and processes the base station’s VHF output signal  for transmission on the VDL between the different AIS stations.  Antennas, cables and filters are components of the VHF-/RF equipment.

## The AIS VHF Data Link

The AIS VHF data link (VDL) is understood as the medium for exchange of data between different AIS stations; by default, using ITU-assigned channels AIS1 and AIS2 in the VHF maritime mobile service band. Channel mangement may be used, on the AIS channels, or by using DSC Channel 70 – see section 13 below.

The channels AIS1 and AIS2 are divided in time slots, 1 minute consists of 2250  slots per channel, giving in total 4500 slots.

The VDL is also described in Recommendation ITU-R M.1371-3. Refer also to the IALA Recommendation on Technical Clarifications of Recommendation ITU-R M.1371-3.

## References

The following documents are referenced throughout the document by the using the abbreviation given in brackets.

### IMO References

- [SOLAS] “IMO Resolution MSC.90(73) Annex 7, Adoption of amendments to the international convention for the safety of life at sea, 1974, as amended”

- [A3] "IMO Resolution MSC.74(69) Annex 3 Recommendation on performance standards for AIS"

- [IMOCircFreqMan] "IMO Sub-Committee on Safety of Navigation (NAV); Liaison statement to ITU-R Working Party 8B" (Report NAV47/13, Annex 15; 26 July 2001) on use of Frequency management in the AIS”

- [RNAV] “Draft Revision of "IMO Resolution A.815(19):1995 World wide radio navigation system"; needed for requirements of any DGNSS service provided by AIS stations”

- [VDLProt]”Resolution MSC.140 (76) Recommenadtion for the protection of the AIS VHF Dala Link. (05-12-2002)”

- [IMOCircBinMsg] “SN/Circ. 236 Guidance on the application of AIS binary messages”

### ITU References

- [M1371] "Recommendation ITU-R M.1371, Technical characteristics for a automatic identification system using time division multiple access in the VHF maritime mobile band."

- [M825] "Recommendation ITU-R M.825, Characteristics of a Transponder System using Digital Selective Calling Techniques for use with Vessel Traffic Services and Ship-to-Ship Identification"

- [RR] ITU-R Radio Regulations, Appendix S18

- [M1084] "Recommendation ITU-R M.1084, Interim Solutions for Improved Efficiency in the Use of the Band 156-174 MHz by Stations in the Maritime Mobile Service"

- [M823] "Recommendation ITU-R M.823; Technical characteristics of differential transmissions for Global Navigation Satellite Systems from maritime radio beacons in the frequency band 283.5-315 kHz in Region 1 and 285-325 kHz in Regions 2 and 3"

- [ReplyFreqMan] “Liaison Statement of ITU-R Working Party 8B to IMO with regard to Frequency Management (reply to IMOCircFreqMan).”

- [585] “Recommendation ITU-R M.585, Assignment and use of maritime mobile service identities”.

- [210]  “Recommendation ITU-T E.210, Ship station identification for VHF/UHF and maritime mobile-satellite services.”

### IALA References

- [Clarifications] “IALA Recommendation on Technical Clarifications on Recommendation ITU-R M.1371-3”

- [1028/1029] “IALA Guidelines on Universal ship-borne Automatic Identification System (AIS)” (2 volumes).

- [R-121] “Recommendation on Maritime Radio Beacons”

- [A-123] “Recommendation on the provision of shore based AIS”

- [A-126] “Recommendation A-126 The Use of the Automatic Identification System (AIS) in Marine Aids to Navigation Services”

- [1026] “Guidelines on AIS as a VTS tool”

- [1050] “Guideline on the Management and Monitoring of AIS information”

- [1059] “Guideline on the comparison of AIS Stations”

### IEC References

- [62320-1] “Maritime navigation and radiocommunication equipment and systems -Automatic identification systems (AIS) - Part 1: AIS Base Stations - Minimum operational and performance requirements, methods of testing and required test results” (2007)

- [62320-2] “Maritime navigation and radiocommunication equipment and systems -Automatic identification systems (AIS) - Part 2: AIS AtoN Stations - Minimum operational and performance requirements, methods of testing and required test results” (2007)

- [61993-2] "Maritime Navigation and Radiocommunication Equipment and Systems – Automatic Identification systems (AIS); Part 2: Class A shipborne equipment of the Universal Automatic Identification system (AIS) – Operational and performance requirements, methods of test and required test results" (2001)

- [62287-1] "Maritime Navigation and Radiocommunication Equipment and Systems - Class-B ship-borne equipment of the universal automatic identification system (AIS) Part 1: Carrier sense time division multiple access (CSTDMA) techniques

- [61162-series] "Maritime Navigation and Radiocommunication Equipment and Systems"

- [61162-1] "Part 1: Single talker and single listeners", as revised 2001.

- [61162-2] "Part 2: Single talker and multiple listeners, high-speed transmissions", as revised 2001.

- [61162-3] "Part 3: Multiple talker and multiple listeners, high-speed network bus" (future publication).

- [60950] IEC 60950 “Information Technology Equipment – Safety - Part 1:General Requirements”

- [61097] IEC 61097-14 “Global Maritime Distress and Safety System – Part 14: AIS search and rescue transmitter (AIS-SART) – Operational and performance requirements, methods of testing and required test results.“

# Part II - Installation Considerations for AIS Physical Shore Stations

# Number and Location of AIS Physical Shore Stations

A careful study needs to be undertaken to establish practically the number and location of base and repeater stations required to achieve full and reliable coverage of the expected traffic load. Although VHF reception is greatly influenced by antenna location and height, operation in a heavy electronic environment may necessitate the installation of additional base stations in order to reduce susceptibility to interference.

## RF Coverage Area

AIS VHF operational coverage should encompass the following considerations.

In coastal areas, extending seaward at least 20 nautical miles (nm) from the territorial sea baseline as defined by the competent authority, and including navigable waters within an area extending inshore to the extent of AIS coverage defined by the competent authority.

For regions along inland rivers and lakes, including the navigable portion of the rivers and lakes and adjacent land areas, the extent of AIS coverage is defined by the competent authority.

The basic principles defined by IMO Resolution A.801(19) Provision of Radio Services For The Global Maritime Distress and Safety Service (GMDSS) for establishing Sea Area A1 for GMDSS VHF (156-174 MHz) can be used to make an approximate rough estimate of AIS radio coverage.

"Sea Area A1 is that area which is within a circle of radius A nautical miles over which the radio propagation path lies substantially over water. The radius A is equal to the transmission distance between a ship’s VHF antenna at a height (h) of 4 metres above sea level and the height (H) of the antenna of the VHF coast station which lies at the centre of the circle.

The following formula should be used to calculate the range A in nautical miles

A=2.5(Square root of H (in metres) + Square root of h (in metres))

H is the height of the coast station VHF receiving antenna and h is the height of the ship’s transmitting antenna, which is assumed to be 4 metres.

The following table gives the range in nautical miles (n.m.) for typical values of H:

| h (m) | H (m) | Range (nm) |
| --- | --- | --- |
| 4 | 50 | 23 |
| 4 | 100 | 30 |

Table 16.1 RF coverage related to antenna height”

The table gives just a rough estimation of the RF coverage. The given range can be different during the time as function of the weather and atmospheric conditions. Also the geographic situation (mountains, etc.) can influence the coverage of the RF coverage.

# AIS Base Station Types

The Base Station may be designed for dependent only operation or independent operation. Both are under some control of the Physical Shore Station (PSS)

–	A dependent Base Station accesses the VHF data link (VDL) using only the combinaion of linked TSA+VDM sentences (see Table 1), as provided by the PSS.

–	An independent Base Station accesses the VDL using either the combination of linked TSA+VDM sentences as provided by the PSS or by using internal control. When operated as an independent Base Station the unit may be delegated certain autonomous functionality under the supervisory control of the PSS.

The PSS, or external controlling entity, is responsible for Base Station configuration, transmission scheduling, and processing of received information. Presentation Interface (PI) text sentences are used to configure the Base Station, schedule message transmissions, and output information.

When TSA and VDM sentences are used, the PSS is responsible for ensuring the integrity of the VDL.

## 8.1 Functional Definition of the Presentation Interface of the AIS Base Station

The Presentation Interface (PI) of an AIS base station consists of at least one input/output port.  The purpose of the PI data port is to:

- Exchange VDL data with the shore station

- configure the base station

- enable real time control of the base station

- provide an output for BIIT alarms and status

Optionally there can be additional ports as part of the PI such as DGNSS input, UTC timing input, etc. These will be discussed in the appropriate sections of this document.

The messages of the PI are subdivided in following functional blocks (refer to Figure 8.1).

Figure 8.1: Functional blocks of PI messages

AIS Base

station

AIS and application related PI messages

AIS system management messages

Remote control messages

BIIT messages

Presentation

interface

# Limited Base Station(LBS)

## Requirement for limited Base Station

The national competent authority has the need to control the AIS Service and the AIS VHF data link, by employing means like channel management, protected slot allocations, and assigned mode. The AIS Base Station allows for the full control of the AIS VHF data link.

There are however other competent authorities responsible for local operations of e. g. individ-ual harbours, locks, marinas, off-shore structures. These regularly have an interest in monitoring the vessel traffic in their locally confined area of competence and exchange limited operational information. In order allow these users to independently access the AIS VDL, without danger of disruption to the AIS service, the concept of a limited base station was devised.

The LBS is capable of receiving all AIS messages within its radio range, but in the case of addressed AIS messages the LBS only receives messages addressed to itself.

The LBS does not exhibit any means of controlling the AIS Service and the AIS VHF data link and cannot broadcast any navigational signals, except for Aids to Navigation messages when authorised .

The LBS may transmit binary and safety related messages, and may also transmit acknowledgements to addressed messages.

Finally, any LBS are subject to the channel management messages (TDMA and DSC Channel 70) of the national AIS Service, if performed by the national AIS Service.  Therefore, the LBS reacts like a mobile station in that it reacts to received Msg 22 and/or DSC-based AIS Channel Management command.

## Comparison of the LBS to the Base Station

Table 10.1 presents the maximum capabilities of a LBS in comparison to the AIS Base Station.  LBSs that are less capable than that shown in Table 10.1 can be achieved by hardware or software configuration means.

Table 10.1 –Comparison of  Limited Base Station and Base Station

| Msg ID | Description | LBS | Base Station |
| --- | --- | --- | --- |
| 1 | SOTDMA Position Report | Nil | nil |
| 2 | Assigned Position Report | Nil | nil |
| 3 | ITDMA Position Report | Nil | nil |
| 4 | Base Station static report | Nil | Yes |
| 5 | Static Voyage Data | Nil | nil |
| 6 | Binary Addressed Message | Yes1 | Yes |
| 7 | Binary Addressed Ack | Yes | Yes |
| 8 | Binary Broadcast Message | Yes1 | Yes |
| 9 | SAR Position Report | Nil | nil |
| 10 | UTC/Date request | Nil | Yes |
| 11 | UTC/Date | Nil | nil |
| 12 | Safety Addressed Message | Yes1 | Yes |
| 13 | Safety Addressed Ack | Yes | Yes |
| 14 | Safety Broadcast Message | Yes1 | Yes |
| 15 | Interrogation | Yes1 | Yes |
| 16 | Assigned mode command | Nil | Yes |
| 17 | DGNSS broadcast | Nil | Yes |
| 18 | Class B position report | Nil | nil |
| 19 | Extended Class B position report | Nil | nil |
| 20 | Data Link Management | Nil | Yes |
| 21 | Aids-to-navigation report | Yes1 | Yes |
| 22 | Channel Management | Nil | Yes |
| 23 | Group Assignment command | Nil | Yes |
| 24 | Static data report | Yes1 | Yes |
| 25 | Single slot binary message | [Nil] | Yes |
| 26 | Multiple slot binary message | Yes1 | Yes |

Footnote 1: Total Link Load should be less than 20 slots per frame due to RATDMA access scheme.  This applies to Binary Msgs (6, 8), Safety Messages (12, 14), Interrogation (15) and ATON Report (21).

# Simplex Repeating functionality to Extend a Coverage Area

## Definition of AIS Simplex Repeater

The AIS Simplex Repeater is a store and forward process in which a different time slot is used for retransmission.  The retransmission could be on the same channel as that in which it was received (per ITU-R M.1084).

The AIS Simplex Repeater may apply filters to assist in reducing the amount of data to be re-peated over the VDL.  These filters could be those that acted upon such items as area, mobile station reporting rate and AIS message type.

The following considerations have to be taken into account:

- The turn around time from receiving the message on the antenna port to retransmitting the mes-sage should be as short as possible and should not exceed four seconds.

- The simplex repeater should not change the content of the message, but should increment the repeat indicator by one (to a maximum of three) or set the repeat indicator to a pre-configured value.  If the repeat indicator of the received message is set to three, the received message should not be repeated.

- The simplex repeater should not change the content of the message, but should change the SOTDMA communication state or ITDMA if relevant.

- The AIS Simplex Repeater should use FATDMA reserved slots for all repeated messages.  If this is not possible, the RATDMA access scheme may be used.

## Using the AIS Base Station for Simplex Repeating functionality

The base station receives AIS messages from the VDL and PI.  Received VDL messages are sent to an external application for processing via the PI as VDM sentences.  VDM sentences received via the PI are retransmitted.  These messages are not processed by the base station.  If received VDL messages are retransmitted (simplex repeating), some messages need modifications before retransmission.  The external application is responsible for providing the modified VDM sentences.

The process of assembling the binary data to broadcast on the VDL is similar to the process of assembling an encapsulated ABM or BBM. The difference is that the entire content is transmitted. See IEC: 61993-2 for details.

General Guidance for the use of position reports; a retransmitted report should be rejected if the time stamp of the position report is identical or older than the current report being used for that unit (identified by MMSI).

A position report shall not be retransmitted:

- if the time stamp indicates that the age of the position report is greater than 30 seconds,

- if the time stamp of the position report is identical or older than the current report being used for that unit (identified by MMSI), OR

- if M.1371-3 section 4.6.1 applies (“Repeat Indicator” equals 3).

Table 10.1: Condition Table governing Retransmission - contains each type of AIS VDL Message

| Message | Name | Comm. state | Instructions General: increment repeat indicator (1371-3) | Subject to message delay rules (time critical) |
| --- | --- | --- | --- | --- |
| 1 | Class A position report | SOTDMA | Table 25 | Yes |
| 2 | Class A position report | SOTDMA | Table 25 | Yes |
| 3 | Class A position report | ITDMA | Table 26 | Yes |
| 4 | Base Station Report | SOTDMA | (none, not repeated) |  |
| 5 | Static and Voyage related data | - | Nil | No |
| 6 | Binary Addressed message | - | Nil | Yes |
| 7 | Binary acknowledgement | - | Nil | Yes |
| 8 | Binary broadcast message | - | Nil | No |
| 9 | SAR position | ITDMA/SOTDMA | (none, not repeated) |  |
| 10 | UTC/date inquiry | - | (none, not repeated) |  |
| 11 | UTC/date response | SOTDMA | (none, not repeated) |  |
| 12 | Addressed safety related message | - | Nil | Yes |
| 13 | Safety related acknowledgement | - | Nil | Yes |
| 14 | Safety related broadcast message | - | Nil | No |
| 15 | Interrogation | - | Only repeat interrogations with slot offset zero (0) do not repeat interrogation containing slot offset | Yes |
| 16 | Assigned mode command | - | Only repeat if assigning reporting rate do not repeat if assigning slot reporting increment | No |
| 17 | DGNSS broadcast message | - | (None, not repeated) |  |
| 18 | Standard Class B equipment position report | ITDMA/SOTDMA | Table 25 for SOTDMA Table 26 for ITDMA | Yes |
| 19 | Extended Class B equipment position report | - | Nil | Yes |
| 20 | Data link management message | - | (None, not repeated) |  |
| 21 | Aids-to-navigation report | - | Nil | No |
| 22 | Channel management | - | --- | No |
| 23 | Group Assignment command | - | nil | Yes |
| 24 | Static data | - | Nil | No |
| 25 | Single slot binary message | - | Only repeat if addressed Do not repeat if broadcast | yes but only if addressed |
| 26 | Multiple slot binary message | ITDMA/SOTDMA | Only repeat if addressed Do not repeat if broadcast | yes but only if addressed |

# DGNSS Correction Broadcasts From AIS Shore Stations

Refer to Annex B for a discussion of this matter.

# Guidelines For FATDMA Configuration Set-Up Of AIS Shore Stations

Refer to Annex C for a description of the considerations and the process of setting up FATDMA configurations.

# Guidance For Channel Management By AIS Shore Stations

The ITU has provided for administrations to designate “regional frequency channels for AIS” where channels 2087 and 2088 are unavailable and, if necessary, to derive new channels.

Channel management may be used to mitigate throughput breakdown caused by (local) RF interference, or blocking of one or both of the default operating channels by locally switching over to alternate operating channels.

Channel management is performed, when mobile AIS stations are switched from their default operating settings to any different operating setting, which may only differ in one parameter. The whole set of channel management settings and region specification is called “regional operating settings”.

These settings include:

- frequency of the operating channel A

- frequency of the operating channel B

- Transmitter power level setting

- Transmit/Receive mode

- Transition zone size

- Addressed mode

- Geographical region

# Network Considerations

## Guidelines for Network Protocols

The AIS data transfer network is a fully transparent data transportation means used to deliver AIS-derived data. Since all AIS-related functionality has been encapsulated in either the PSS, or the LSS or at the AIS Service level, the AIS data transport network does not comprise any AIS-specific technology. Hence, the AIS data transfer network is a virtual network transporting AIS-related data within a possibly much larger network of the competent authority

The AIS Service should ideally run on either an IPv4 or IPv6 based Ethernet network employing:

- TCP/IP sockets

- UDP

- Multicast

The IEC tag block as described in [62320-1] shall be optionally used to timestamp data and to identify the source of data.

In cases where large amount of AIS data has to be delivered to external BAS the following can be employed to make more effective use of network bandwidth:

- Buffering of data to increase TCP packet size

- Use of UDP broadcasts

- Use of multicast groups

All TCP socket communication should ideally be secured so that unauthorised connections to the AIS service are rejected.

If required data communication over TCP can be secured by using SSL.

Annex A: Services Operating on the VDL Simultaneously

Prioriti-

ring in accor-

dance with M.1371-1

Prioriti-

ring in accor-

dance with M.1371-1

void

All mobile AIS stations in radio coverage area

All fixed AIS stations

in radio coverage area

-	Regular base station reports

-	Synchronisation support (additional) by base stations

-	Assigned mode control

-	Frequency mode control

-	Safety related messages

-	Interrogations to mobiles

-	VDL-level acknowledgements

-	FATDMA reservations

-	Application specific messages

-	DGNSS corrections

-	Simplex repeated messages

-	Responses to interrogations from mobiles

-	Re-transmit of addressed messages

-	Scheduled ship position reports (all Classes)

-	Static and voyage Related Data Report (scheduled)

-	Safety Related Messages

-	A-to-N Reports (scheduled)

-	SAR Aircraft reports (scheduled)

-	Extra reports (all kind of mobiles) due to assignment

-	Responses to interrogation

-	VDL-level acknowledgements

-	Application specific messages

-	Synchronisation support (additional) by mobiles

-	Interrogations from mobiles

-	Re-transmit of addressed messages

capacity

loss due

to garbling and

interference

4.500 slots/

minute

(theoretical

maximum)

void

capacity

loss due

to garbling and

interference

# Annex B: Interfacing the DGNSS Correction Service to The AIS Base Station

# Annex B Introduction

The purpose of Differential GNSS (DGNSS) is to increase accuracy and integrity of vessel EPFS. A base station may transmit DGNSS corrections via AIS VDL if it is connected to a DGNSS reference station. This service can be provided automatically and may be considered a back-up service for the radiobeacon DGNSS service.

This annex describes how a base station may transmit DGNSS corrections, and ensure that a consistent approach is taken whilst implementing a DGNSS service over AIS. In particular, this annex covers the use of Message 17 for the broadcast of differential corrections. Annex C provides guidance on how to reserve slots for Message 17. This annex does not define how the differential corrections are generated.

There are currently no mandated requirements to provide DGNSS corrections via AIS. Distributing DGNSS corrections via AIS is an added value service, which may improve the accuracy of AIS position reports, but which is unlikely to affect the position accuracy of the external GNSS devices.

# Annex B Abbreviation

RSIM – Reference Station and Integrity Monitor

# Annex B Performance requirements for a DGNSS Service

## Accuracy, Availability and Continuity

An AIS-based DGNSS service should comply with the current IMO requirements for the accuracy, availability and continuity for a World-wide Radionavigation System. These are currently described in IMO Resolution A.953(23), and summary of the requirements are given in Table 1. Also in the table is the number of stations needed to meet the IMO requirements. This is based on IALA Recommendation R-121.

| Area | Absolute horizontal accuracy (95%) | Availability | Continuity | Augmentation |
| --- | --- | --- | --- | --- |
| Ocean | ≤ 100 m | >99.8% over 30 days | N/A | None |
| Coastal/harbour with low level of risk | ≤ 10 m | >99.5% over 2 years | >99.85% over 3 hours | Single station |
| Coastal/harbour with high level of risk | ≤ 10 m | >99.8% over 2 years | >99.97% over 3 hours | 2 or more stations |

Table 1 - Accuracy, availability and continuity requirements for GNSS by IMO

At the time of writing, the definition of Availability and Continuity for an AIS service has not been defined. The definition of continuity is currently being discussed at IMO. The competent authority should define these criteria locally and ensure that their AIS DGNSS service complies with the IMO requirements using these definitions.

## Integrity

The competent authority should refer to IALA Recommendation R-121 for recommendations of implementing integrity checks of the DGNSS system. This should include a separate integrity check of the contents of the transmitted Message 17.

It should be noted that, in addition to the requirements highlighted above, the time for broadcasting an integrity warning to the user, from the time the integrity problem is

identified, should not be greater than 10 seconds. When it has been determined that the error condition no longer exists, there may be a delay, similar to that used for initially identifying the error, before the error warning is withdrawn.

## AIS Requirements

DGNSS Corrections should be transmitted over the AIS VDL using Message 17 only. In accordance with ITU-R M.1371, the surveyed position of the DGNSS reference station must be given in Message 17. In accordance with IALA Recommendation R-121, the reference station antenna should be surveyed to a 3-D accuracy of 0.2m. Corrections may be given for GPS, GLONASS, and any other future operational GNSS services (e.g. Galileo, Compass). Recommended RTCM messages to be transmitted in AIS Message 17 are:

- RTCM Messages 1 or 9 for GPS corrections

- RTCM Message 31 or 34 for GLONASS corrections

At the time of writing, it is recommended that no other RTCM Message Types be used as these will use valuable bandwidth. Future RTCM Messages Type may be relevant for transmission, although consideration must be given to legacy mobile users.

Message 17 should preferably be transmitted using FATDMA (Usage category 2 – Annex 14) with a transmission scheme that allows the system to meet the 10-second TTA requirement. It is recommended that RATDMA is not used for Message 17 transmissions due to the possibility of the message not being broadcast in time to meet the TTA requirement. The probability of a slot collision is also increased with the use of RATDMA, particularly in busy areas.

### VDL Loading Requirements

With regular, high volume data being broadcast on the AIS VDL, the potential issues regarding the loading of the VDL must be considered. The slots allocated to the service should comply with the FATDMA Plan (see Annex 14). Through trials, it has been found that most Class A stations will revert to an uncorrected position state after the age of the corrections exceed 30 seconds, and therefore, an new set of corrections should be made available every 30 seconds or less. This should not affect the 10-second TTA requirement.

# Annex B Functional descriptions

## Overall System Description

Figure 1 – DGNSS Corrections within the AIS shore-based infrastructure

The AIS shore-based infrastructure allows for DGNSS corrections to be broadcast to AIS users in the vicinity of the physical shore stations. Figure 1 shows how DGNSS corrections enter the AIS shore-based infrastructure, and that the information flows down to the VDL. The majority of the AIS shore-based infrastructure remains identical to the general structure of the AIS Service, with the ASM controlling the configuration of the LSS and PSS, and reporting back the status of the AIS Service back to the External Services.

### DGNSS Corrections

The DGNSS corrections are always provided by an External Service, and are not generated by the AIS service. Section 9.2 consider the options in terms of suitable sources of DGNSS corrections.

### DGNSS Corrections within the Logical Shore Station

The DGNSS corrections being provided to the AIS Service are in a data format defined by RTCM SC-104. The information can also be provided by the DGNSS_COR object defined in this document. The DGNSS correction data from the External Service is encapsulated in an IEC 61162 VDM sentence by the Logical Shore Station (LSS) for processing by the Physical Shore Station (PSS). The LSS ensures that the latest full set of corrections is used, and that they are transmitted at the correct time. The LSS also prioritises the DGNSS corrections so that any new integrity alarms that have been identified by the DGNSS corrections source are transmitted in the next Message 17 slot(s).

### Service Planning

The competent authority should consider the impact of providing an AIS DGNSS service on the VDL. It is not recommended that an AIS DGNSS service be deployed over a wide area (it is better to deploy a radiobeacon DGNSS service for this purpose). However, an AIS DGNSS service could be considered where it is necessary to obtain high accuracy and high integrity position fixes from vessels.

The competent authority should minimise the number of base stations providing the AIS DGNSS service. This may require careful planning using any available terrain information.

### Service Limitations

At the time of writing, a DGNSS service over AIS has several limitations in the service it is capable of providing.

On most vessels, the external GNSS is not capable of switching between an external radiobeacon DGNSS receiver and an output provided by AIS. The implication of this is that an AIS DGNSS service cannot be considered as the backup to an existing radiobeacon service and visa versa. It should be noted that a Class A and B stations has the ability to switch between an external DGNSS receiver and a Message 17 source (priority given to Message 17), although this can only be used to differentially correct the internal GNSS receiver.

There is also concern on the difference between the position being broadcast by the vessel over AIS and the position being used by the vessel’s commander. The IEC standard for Class A and B stations asks that the position being broadcast is prioritised based on the status of the internal and external position fixing systems (EPFS). If the external EPFS is differentially corrected, then this is the position broadcast and used by the vessel’s commander. However, for most vessels, the external EPFS is not differentially corrected (and may not be capable of do so in its current configuration), and so the AIS will broadcast its internal differentially corrected (by Message 17) GNSS position. The vessel’s commander will continue to use the external GNSS, and a situation will occur where other vessels are more confident of a ship’s position than the commander of the vessel.

A competent authority should be satisfied with the benefits of providing a DGNSS corrections service over AIS given the potential limitations above.

## Interfacing a DGNSS Reference Station and Integrity Monitor

### Overall Description

In order to provide the correction data to the AIS base station, a DGNSS Reference Station and Integrity Monitor (RSIM) is required. The purpose of the RSIM is to generate pseudorange corrections for the Satellite Vehicles (SV) (note: SV is usually an abbreviation for Space Vehicle) in view, and ensure that the corrected position is within limits. When the position is out-of-tolerance, the Integrity Monitor raises an alarm for the appropriate SV, and notifies users not to use it in their navigation solutions.

The AIS shore stations should be capable of using such a data stream to provide a DGNSS service over AIS. There are many ways this is possible, some of which are highlighted below. It should be noted that where the RSIM and the AIS transmitting station are physically separated, the integrity and reliability of the intermediate communications structure should be given careful consideration since any failure will cause the DGNSS service to be disrupted.

### Correction Sources

#### Independent RSIM

It is recommended that an AIS DGNSS Service relies on a dedicated RSIM that provides corrections for transmission by AIS only. This provides an independent DGNSS service which can be used as a back-up to other DGNSS services (e.g. radiobeacon DGNSS).

It should be noted that the cost to implement and operate an independent RSIM for an AIS DGNSS Service will be greater than to utilise an existing RSIM as described in the MF radiobeacon option below. It may be possible to use one central RSIM for mutliple AIS base stations, although care should be taken to ensure that spatial decorrolation is considered.

#### MF Radiobeacon

If the AIS base station is in an area covered by an MF radiobeacon service, it may be possible to use this to provide the RTCM data stream required to generate Message 17. A suitable MF receiver will be required. The output from this receiver would be passed through the RTCM interface of the LSS for further processing.

Care should be taken when utilising such a system to provide an DGNSS service over AIS because it is reliant on an external system to provide the differential data. Any failures of the radiobeacon service will be reflected almost immediately in the DGNSS service over AIS. If two or more radiobeacon services operate in the area, it may be possible to utilise them to increase availability of the AIS service. The competent authority should also ensure that the MF signal can be received reliably 24 hours a day (it is affected by skywave propagation during night hours that can cause fading). If the MF radiobeacon receiver is installed at a distance from the transmitting AIS base station, then care should be taken to ensure that spatial decorrolation of the corrections is considered. A much better solution would be a direct datalink to the RSIM at the beacon station location.

#### Data interface Guidance RTCM Input/Output in Binary Format

- The LSS interface for the input of DGNSS corrections should comply with the Data Interface Specification defined in ITU-R M.823. The information enters through a dedicated input port. The competent authority should also refer to RTCM Recommended Standards for Differential GNSS Service Version 2.3.

### Processing Multiple Sources of DGNSS Corrections

In some locations, the system may have access to more than one reference station. In this case, a decision on how to deal with them must be made in order to provide a single set of corrections for each navigation system. The base station may be configured to transmit the data for the nearest reference station. However, a more sophisticated solution would be to receive data from all sources, and interpolate according to distance from “Virtual RSIM” position. This will provide a more accurate set of corrections for vessels near the transmitting AIS base station.

### Processing Correction Data

ITU-R M.1371 provides the standard Message 17 format to be transmitted by the AIS base station. The data for the fields are generated by the LSS and directly from the data stream. Table 2 shows the data source for each field in Message 17. It should be noted that the preamble and parity bits are discarded, and will be replaced by the receiving AIS mobile stations. The header contains the Station Health message. This should NOT be discarded.

| AIS Message 17 |  | Data source |
| --- | --- | --- |
| Static data |  |  |
| Parameter | Number of Bits |  |
| Message ID | 6 | Set by AIS shore-based Service |
| Repeat Indicator | 2 | Set by AIS shore station |
| Source ID | 30 | Set by AIS shore station |
| Spare | 2 | - |
| Longitude | 18 | Set by AIS shore station (refer to remark below) |
| Latitude | 17 | Set by AIS shore station (refer to remark below) |
| Spare | 5 | - |
| Differential Correction data (header) |  |  |
| Parameter | Number of Bits |  |
| Message Type | 6 | RTCM Header, Word 1, d9-d14 |
| Station ID | 10 | RTCM Header, Word 1, d15-d24 |
| Z count | 13 | RTCM Header, Word 2, d1-d13 |
| Sequence number | 3 | RTCM Header, Word 2, d14-d16 |
| N | 5 | RTCM Header, Word 2, (d17-d21) - 2 1) |
| Health | 3 | RTCM Header, Word 2, d22-d24 |
| Differential Correction data |  |  |
| Correction Data | Up to 13 SV | RTCM Data, Word 3 onwards 2) |

Table 2 - Data source for fields in Message 17. Differential data to be transmitted are Words 3 onwards without the parity bits.

1)The frame length in RTCM header is two more than the number of words (N) following the header

2)16 fill bits are used for RTCM word (N+2) if number of satellites Ni = 2,5,8 or 11

The reference station position fields should be filled with the position of the differential correction source, and not the position of the transmitting AIS base station. This information is either manually configured into the transmitting Base Station, or is determined by the evaluation of the RTCM Message Type 3. If a virtual RSIM has been defined, then its position should be transmitted in Message 17.

## Transmission Schemes

When selecting a transmission scheme to provide a DGNSS service over AIS, it is important to consider the potential loading on the VDL. The FATDMA planning allows the possibility to transmit a full set of corrections (containing up to 13 SV data in a 3-slot message) approximately every 15 seconds (see Annex 14). This easily meets the 30-second correction age requirement, but does not meet the 10-second TTA requirement. This requirement is met by reserving a single slot every 3 1/3 seconds (increment of 250 slots per channel) that will enable a message to be sent for a single SV. This message will simply disable the use of the SV in the navigational solution. The transmission scheme is summarised in Figure 1.

Figure 1 - Proposed FATDMA transmission scheme for transmitting DGNSS corrections over AIS for one GNSS system. The diagram shows the sequence for one frame. Not to scale.

The advantage of this scheme is that a number of GNSS systems could be serviced with the minimum additional loading on the VDL. The regular 1-slot reservations could be shared for all the GNSS systems.

It is unlikely that multiple GNSS failures will occur at the same time, and therefore, the regular 1-slot reservations could be used for all the GNSS systems. This practice will substantially reduce the VDL loading when providing a DGNSS service for more than one GNSS system. Table 3 provides a brief summary of the VDL loading for a number of GNSS systems.

| Number of GNSS Systems | Slots per frame required for full constellation data | Slots per frame required for the TTA slots | Total number of slots | VDL Loading (over two channels) |
| --- | --- | --- | --- | --- |
| 1 | 12 | 18 | 30 | 0.67% |
| 2 | 24 | 18 | 42 | 0.93% |
| 3 | 36 | 18 | 54 | 1.20% |

Table 3 - Loading on the VDL when one base station provides DGNSS service for multiple GNSS systems.

### Multiple Base Stations

[It is envisaged that in some areas, the coverage of base stations providing DGNSS over AIS would overlap. From a time-slot reservation point of view, it is better for each base station to provide their own TTA slot reservations, using different time slots. The implication of this is that the VDL loading will increase linearly with the amount of base stations in the local area. It is up to the competent authority to ensure that the loading of the VDL is not adversely affected.

If Base Stations are transmitting different sets of corrections for the same GNSS system, then the competent authority should be aware that vessel positions may shift slightly due to spatial decorrelation as different sets of corrections are applied to the GNSS unit.]

# Annex C – FATDMA Planning and Operation of an AIS Service

# Annex C Introduction

This Annex introduces the configuration of FATDMA of Physical AIS Shore Stations (PSS) by one or more competent authorities in a given area.

The purpose of this Annex can be summarized as followed:

- Provide to competent authorities a comprehensive „cookbook-like“ recipe to create a FATDMA plan

- Provide to adjacent and affected national competent authorities guidance to co-ordinate their joint FATDMA plan in the adjacent area

- Provide to (national / other domestic) competent authorities guidance how to operate their co-ordinated FATDMA plan correctly and optimize the benefit of both mobile AIS stations and the competent authorities in a given area.

- Provide a basis for required and / or stipulated type(s) of components of a competent authority

A range of AIS stations make use of FATDMA reserved timeslots for their transmissions.  Taking IMO resolution MSC.140(76) into consideration, which states, that “Administrations should take steps necessary to ensure the integrity of the radio channels used for AIS in their waters”, and since since FATDMA reservations in general affect the bandwidth availability of the AIS VDL, proper management of FATDMA is required to protect the VDL.

The complexity of FATDMA management increases in proportion to

- the slot usage requirements by different competent authorities in their envisaged coverage areas,

- the number of transmitting entities using the FATDMA reserved slots, and

- their relative geographical proximity.

Therefore, fundamental usage rules for FATDMA plans, FATDMA schemes and FATDMA schedules need to be established to resolve the complexity of FATDMA configuration in any given area.

This annex provides guidance and rules for the efficient use of FATDMA.

# Annex C General

When setting up a FATDMA plan, there are three dominating aspects which need to be introduced:

- The required usage of FATDMA reserved slots, answering the questions: What would the FATDMA reserved slots used for by AIS stations?

- What geographical criteria apply to co-ordination of FATDMA schedules and schemes when using FATDMA reserved slots in a (larger) given area of a FATDMA plan?

- What rules are applicable for allocating FATDMA reserved slots to national competent authorities and other (domestic) competent authorities? In particular, what rule base governs the allocation of FATDMA schemes internationally?

These three aspects will be dealt with in the above order in the following sections after the introduction of important definitions.

# Annex C Definitions

This section defines relevant terminology used for FATDMA Configuration.

## Terminology

In order to make efficient usage of the available AIS VDL time slots in a given area a “proper plan” is needed. This “proper plan” may be called FATDMA Area Slot Map. It has two aspects:

- Setting up the FATMDA Area Slot Map (at planning time)

- Using the FATDMA Area Slot Map (at run-time).

These aspects and the composition of the FATMDA Area Slot Map are illustrated in the following figure. An explanation of the various aspects will be given afterwards (the figure employs the UML notation).

Figure 1: FATDMA Area slot map

The aspects which should be considered when planning FATDMA reservations on the AIS VDL in a given area, are described below:

- Geographical area aspect: Every FATDMA reservation is done within a defined area.

- Time domain aspect: The actual FATDMA reservations are done using the time domain. The time domain in the AIS is subdivided into time slots. Every FATDMA reservation consists of time slots reserved for a specific use. These FATDMA reservations can be described in terms using absolute time slot terminology and relative time slot terminology as follows:

- absolute slot numbers of FATDMA reservations: At planning time, the FATDMA reservations can only be planned using absolute slot numbers within the slot frame epoch. This requires an absolute reference point, which is the start of a specific frame epoch which in turn is related to the start of an UTC minute or hour. Since UTC is a universal concept, any absolute point in time can be determined thereby.

- Terms related to the relative slot numbers used for actual transmissions of FATDMA reservations: At run-time, the FATDMA reservation is being done be transmitting the Data Link Management Command (message 20). The parameters given by this command determine the number and location of the slots reserved by mobile stations. The parameters given determine the absolute position of the FATDMA allocated slots by reference to the absolute slot number of that slot, in which the Data Link Management Command was received. Therefore, these terms are relative by nature.

- Usage domain aspects: The usage domain is subdivided into two aspects:

- At planning time: Every FATDMA reservation is used for a purpose. Therefore, the usage aspect of FATDMA reservations must be considered. Each and every FATDMA reserved slot can and must have a designated usage allocated to it; otherwise its reservation would not be justified. This concept is called “Usage Designation”. It is further subdivided into Usage Categories.

- At run-time: The different potential users of FATDMA reservations (“VDL usage stake holders”) are identified as – by default and in accordance with the rules of the AIS VDL – all defined varieties of AIS stations, mobile or fixed. Their usage requirements as well as the overall requirement to maintain the viability of the AIS VDL need to be reflected in the Usage Designations at planning time.

- Roles of participating entities:

- At planning time the national competent authorities of individual countries sets up their FATDMA plan in accordance with the rules in this Annex. Should there be more than one national competent authority adjacent to the same geographical area, co-ordination would be done to guarantee the inter-operability of the individual FATDMA plans and create one joint FATDMA plan.

- At run-time every FATDMA reservation is done by one entity, called base station. Every time slot within this FATDMA reservation can be used by a variety of different entities participating in the AIS VDL, i. e. the AIS base stations proper, the AIS simplex repeaters and the different classes of AIS stations other than AIS base station.

In the following sections, the precise terms will be introduced.

## Terms related to FATDMA reservations in the time domain in general

The following terms are related to FATDMA reservations in the time domain in general, i.e. those terms are part of both the relative as well as the absolute FATMA terminology sets:

- FATDMA block: A FATDMA block consists of consecutive slots.

- FATDMA block size: The FATDMA block size is the number of consecutive slots of a block. The FATDMA block size can be between 1 and 5.

- FATDMA reservation: A FATDMA reservation is an announcement of a base station to create one or more FATDMA block(s) using the FATDMA reservation parameters. The FATDMA reservation is both an activity of a base station as well as the stationary knowledge of receiving mobile AIS stations, namely that the slots contained within the blocks have been reserved by one base station.

- FATDMA reservation parameters: Each FATDMA reservation is described by the following parameters (see figure). The FATDMA offset number is a relative term exclusively and will be explained in the next section.

Figure 2: Composition of FATDMA reservation

- FATDMA reservation time out: This value indicates, how many minutes the FATDMA reservation should be considered valid by the mobile AIS stations. The number of minutes results by adding one (1) to the values 0 to 7 in the FATDMA reservation(s) in message 20. Therefore, a FATMDA reservation can be made with a FATDMA reservation time out between 1 and 8 minutes.

- FATDMA increment: The FATDMA increment is the offset of slots between the first slot of the first FATDMA block and the first slot of the next FATDMA block of the same FATDMA reservation in the same frame. The FATDMA increment is then used repeatedly to create further FATDMA blocks in the same frame until the end of this frame is reached and no further FATDMA block could be accommodated. If the FATDMA increment is set to zero (0), then there will just one FATDMA block reserved by this FATDMA reservation.

- Data Link Management message (message 20): Actual FATDMA reservations are being made by transmitting the Data Link Management message (message 20) from a base station to mobile AIS stations.

- FATDMA reservation No N: Each Data Link Management message can include up to four (4) FATDMA reservations. Within the Data Link Management message, the FATDMA reservations are numbered from 1 to 4. The following figure illustrates, that all those four FATDMA reservations in one Data Link Management message inherit the above qualities of a FATDMA reservation, but may be different in terms of actual parameter setting.

Figure 3: Inheritance relationship between FATDMA reservation definition in general and individual FATDMA reservations

- Message-20-slot: The Message-20-slot is the slot, in which the Data Link Management Message (message 20) was transmitted. It has an absolute slot number. Starting with the Message-20-slot number the FATDMA offset number is counted (see below; relative terminology).

### Terms exclusively related to the relative slot number used for actual transmissions of FATDMA reservations

The following terms are exclusively related to the relative slot number used for actual transmission of FATDMA reservations.

- FATDMA offset number: Every FATDMA reservation has a parameter FATDMA offset number. The FATDMA offset number denotes the offset from the slot in which Data Link Management Message (message 20) was received to the first slot of the first FATDMA block to be reserved by the receiving station.

The relative composition of one (1) FATDMA reservation is illustrated by the following example figure.  In this example the parameters are as follows:

- Message 20 is transmitted in the slot with an absolute number of n (numbered 1 to n relative to frame start. Hence the Message-20-slot has the absolute slot number n. Note: The absolute number of the start slot of  the first FATDMA block can only be determined, if the absolute value of the Message-20-slot is known (which is n+25 in this example).

- FATDMA offset number :			25

- FATDMA block size:		 	5

- FATDMA reservation time-out:	 	3

- FATDMA increment:			375

The following figure illustrates this example.

offset number

block size

increment

n   n+1                                            n+ 25               n+29                          n+400              n+404                           n+775         n+779

free slot

reserved slot

Figure 4:Example for one FATDMA reservation within a Data Link Management message transmitted in Message-20-slot n

Additional FATDMA blocks are reserved in this example, each of which has a FATDMA block size of 5, starting at absolut slot numbers: n+1150, n+1525, n+1900 (if end of frame is not reached by then).

### Terms exclusively related to the absolute slot numbers of FATDMA reservations

Certain FATDMA related terms refer to an absolute framework of time and / or slot numbers. In addition, there are FATDMA related terms defined for areas, which may also be considered absolute (see next section).

- FATDMA start slot: The FATDMA start slot is the slot number of the first slot of the first FATDMA block within a frame (refer to Table 8 of Recommendation ITU-R M.1371-1). Therefore, the FATDMA start slot is an absolute term.

- FATDMA epoch: Due to the time out behaviour of FATDMA reservations and due to the need to devise FATDMA plans which minimise the actual number of FATDMA reservations (see next sections), a time span of more than a frame (i. e. one UTC-based minute) needs to be considered. These larger time spans are called FATDMA epochs. Each FATDMA epoch is 6 minutes long. The start of a FATDMA epoch is correlated with the beginning of the UTC hour as indicated by the FATDMA epoch number. Any FATDMA epoch starts with the first second of the first minute correlated with the UTC hour as given by the FATDMA epoch number and ends with the end of second 59 of the 6th minute after the start minute of the epoch.

- FATDMA epoch number: Each FATDMA epoch has assigned a unique number within the duration of one (1) hour. The FATDMA epoch number of each and every of the FATDMA epoch of one hour is correlated to an absolute start minute within the UTC hour as given in the table below:

| FATDMA epoch number x | Start minute of FATDMA epoch Number x | End minute of FATDMA epoch Number x |
| --- | --- | --- |
| 0 | 0 | 5 (included in epoch 0) |
| 1 | 6 | 11 (included in epoch 1) |
| 2 | 12 | 17 (included in epoch 2) |
| 3 | 18 | 23 (included in epoch 3) |
| 4 | 24 | 29 (included in epoch 4) |
| 5 | 30 | 35 (included in epoch 5) |
| 6 | 36 | 41 (included in epoch 6) |
| 7 | 42 | 47 (included in epoch 7) |
| 8 | 48 | 53 (included in epoch 8) |
| 9 | 54 | 59 (included in epoch 9) |

### Relationship between absolute and relative FATDMA terms

The following relationships exist between the above absolute and relative FATDMA related terms. Some FATDMA related terms are used identically, in an absolute as well as a relative context. In those cases, the term appears twice in the same line of the table. If there are different terms used in the same line of the table, the two terms correspond to each other, and a transformation / correlation is needed or performed at least at run-time. The following table gives the appropriate correspondence.

| Absolute terms used for FATDMA reservations | Relative terms used for FATDMA reservations |
| --- | --- |
| FATDMA plan | - no equivalent - |
| FATDMA schedule | - no equivalent - |
| FATDMA scheme | - no equivalent - |
| FATDMA epoche | - no equivalent - |
| FATDMA reservation | FATDMA reservation |
| FATDMA block | FATDMA block |
| FATDMA start slot | FATDMA offset number |
| FATDMA increment | FATDMA increment |
| FATDMA block size | FATDMA block size |
| Message20-slot | Message20-Slot |
| - no equivalent - | Message20 |

Table: Correspondence between absolute and relative FATDMA terminology.

## Terms related to the geographical areas for which FATDMA reservation planning is done

The terms related to the geographical areas for which FATDMA reservations are planned and performed at run-time are arranged in order to assist in the structured mutual FATDMA reservation planning of neighbouring administrations:

- FATDMA plan: A FATDMA plan is the highest level of FATDMA reservation planning. A FATDMA plan comprises all FATDMA schedules (see below), which are considered relevant to the FATDMA plan. Therefore, a FATDMA plan also comprises the area covered by the FATDMA schedules relevant to the FATDMA plan. It is the purpose of this Annex to describe a global FATDMA plan, which translates seamlessly to FATDMA schedules (regional or national) and FATDMA schemes (see below).

- FATDMA schedule: A FATDMA schedule comprises the relevant FATDMA schemes of one (1) or more Physical AIS Shore stations (PSS) in the area under consideration. The FATDMA schemes combined in a FATDMA schedule may be of the same or different competent authorities. Any FATDMA schedule needs to be consistent at its interfaces to adjacent FATDMA schedules.

- FATDMA scheme: A FATDMA scheme comprises the relevant FATDMA reservations of one (1) PSS. Hence, a FATDMA scheme may comprise the FATDMA reservations of one or more transmitting AIS base station(s) of that PSS.

- FATDMA Configuration: This general term denotes all aspects of FATDMA planning and FATDMA reservations made at run-time to be done by one or more competent authorities in a given area. It is an umbrella term.

The following schematic drawing illustrates the relationship between the various terms related to areas (note: each transmitting base station (BS) transmits Data Link Management Message (message 20) to set up FATDMA reservation(s).

FATDMA plan

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

BS

FATDMA

Scheme

FATDMA

Scheme

FATDMA

Scheme

FATDMA

Scheme

FATDMA

Scheme

FATDMA Schedule

FATDMA Schedule

Figure 5: Hierarchy of FATDMA plan, FATDMA schedule and FATDMA scheme

## Terms related to the usage of FATDMA reserved slots (“Usage Designation”)

This section introduces the different terms for usage of FATDMA reserved slots.

### General considerations in regard to periodicity of transmissions using FATDMA reserved slots

When setting up a FATDMA plan, the required periodicity of a transmission needs to be considered. In regard to periodicity the reserved FATDMA slots can be used for:

- Short term repetitive pattern spanning a number of time slots within the same frame

- Intermediate term repetitive pattern spanning one frame

- Long term repetitive pattern spanning one (1) FATDMA epoch

There may be also one-shot transmissions, which are called spurious transmissions.

### General considerations in regard to timing of transmissions using FATDMA reserved slots

The timing of transmissions using FATDMA reserved slots is largely independent of the periodicity aspect introduced above, and needs to be considered in following ways:

- Urgency of transmission of a message(s): Regardless of their periodicity some AIS messages need to be transmitted with a very short delay. Besides the requirement to provide for prioritised queuing on the PSS Controlling Unit level, the urgency of transmission using FATDMA reserved slots requires the FATDMA plan to provide sufficient closely spaced, general-purpose FATDMA reserved time slots to allow for urgent transmissions.

- Timely availability of FATDMA reserved slots when allowing for dynamic features of FATDMA plans: When introducing dynamic features of a FATDMA plan, there is a  requirement to provide additional FATDMA reserved slots before the actual net data transmission can be made in these FATDMA reserved slots. Additional FATDMA reservations are to considered “timely”, in regard to the intended net data transmission, only if they allow for the following transient processes to be completed:

- Transmission of additional FATDMA reservation(s) by AIS base station(s) to be completed = initiation of dynamic FATDMA reservation

- Allow AIS mobile stations to clear the slots previously allocated by their own transmissions on both AIS channels completely. This process can not be fastened and – for un-garbled operation – will require 8 minutes from transmission of the additional FATDMA reservation.

This consideration substantiates the introductory general rule, that dynamic FATDMA reservations should be avoided or should be minimised.

### Terminology for categories for usage of FATDMA reserved slots

All of the above considerations are simplified when categorising the usages of FATDMA reserved slots into the following categories.

- FATDMA reservation usage category 1: AIS VDL management slots

These FATDMA reserved slots will be used for the transmission of any message, which is necessary for the management of the AIS VDL.

- FATDMA reservation usage category 2: High timing requirements slots

These FATDMA reserved slots will be used for the transmission of messages, which need to be transmitted complying to high timing requirements.

- FATDMA reservation usage category 3: General purpose pre-reserved slots

These FATDMA reserved slots will be used for the transmission of any message, which does not fall under FATDMA reservation usage category 1 or 2.

## Terms related to activities of entities involved in the FATDMA (“VDL Usage Stake Holders”)

### Terms related to roles and activities of entities related to FATDMA

The following terms are related to activities of entities involved in setting up and executing a FATDMA plan.

- FATDMA reservation: The FATDMA reservation, as viewed as an announcement, is an activity of a transmitting AIS base station.

### Roles and activities involved in making and using FATDMA reservations

A FATDMA reservation can be made by one entity participating in the AIS VDL and it will be used by one or more entities participating in the AIS VDL.

A FATDMA reservation can only be made by a base station of a competent authority.

- FATDMA reservation for transmissions of its own data and when acting as a semaphore;

- FATDMA reservation for transmissions of other base station/Limited Base Station (LBS);

- FATDMA reservation for transmissions of simplex repeater station(s)

- FATDMA reservation for transmissions for mobile AIS stations

An existing FATDMA reservation can therefore be used by one of the following entities participating in the AIS VDL:

- own AIS base station

- other AIS base station(s) (of own or other competent authority)

- other Limited AIS base station(s) (of own or other competent authority)

- other simplex repeater station(s) (of own or other competent authority)

- mobile AIS stations operating either in

- assigned mode, hard assignment (Class A, Class B, AtoN AIS stations) or

- responding to an interrogation (assigned by an AIS base station)

- fixed-slot usage AtoN mobile AIS stations

The following figure provides an overview of the different roles in regard to the FATDMA.

Figure 6: Activity of AIS base stations and AIS mobile stations for FATDMA reservation

## Mapping of FATDMA related aspects to Layers of the AIS Service

The terms defined in this section can be associated to the different hierarchical layers of the AIS Service. This association indicates on what hierarchical layer of the AIS Service certain correlations between the "absolute" and "relative" realms need to be performed.

In accordance with the definition of the AIS base station in IEC 62320-1 there are two principal modes of operation for base stations:

- the independent mode: the base station in independent mode is independently, i. e. autonomously, performing tasks delegated to it from the PSS layer, such as autonomously creation and transmission of message 20 upon configuration from the PSS layer. To that end certain configuration PI sentences are used by the PSS. The configuration comprises the generally pre-programmed transmission timing, which is also given in absolute terms. If the independent mode of base stations is employed by a competent authority the PSS needs to set up and supervise the proper operation of all of its base station(s) in independent mode.

- the dependent mode: the base station in dependent mode transmits only pre-composed messages of whatever kind, which it receives from the PSS at a individually pre-determined time slot, the number of which is given in absolute terms. The base station does not compose messages itself when operating in dependent mode. Certain PI sentences are used by the PSS to forward the messages to be transmitted as well as the individually pre-determined time slot for transmission.

In both cases the task of translating from the absolute FATDMA-related terminology used throughout the AIS Service and the relative FATDMA-related terminology used on the AIS VDL is a task of  the PSS, which may be delegated completely to the AIS base station when operating in independent mode. When operating in dependent mode, the AIS base station still translates the transmission timing from the pre-determined absolute slot number it receives to the (relative) point in time, when the pre-determined transmission slot actually occurs.

Table: Association of terms related to FATDMA and Layers of the AIS and the AIS Service

| Term related to FATDMA | AIS VDL | AIS base station proper | PSS Controlling Unit ("PSS") | AIS Service Management | AIS Service as a whole | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| FATDMA Reservation (relative point of view) | X | X | - | - | - | Reservation announcement by base station on the AIS VDL as such; therefore only known to AIS VDL and AIS base station; similar to message 20 |
| FATDMA Reservation (absolute point of view) | - | X | X | X | X | Absolute equivalent of reservation |
| Message 20 | X | X (when AIS base station operates in independent mode) | X (only when AIS base station operates in dependent mode) | - | - | Consists of a set of 4 reservations; message 20 will be composed in AIS base station upon configuration settings from PSS when operating in independent mode or will be composed in PSS when AIS base station operates in dependent mode. |
| Message-20-slot | X | X | - | - | - | Message-20-slot is determined by the start slot |
| FATDMA start slot | - | X | X | X | X | Absolute term to be known for FATDMA scheme, schedule and plan; refer to Table 8 of Rec. ITU-R M.1371-1+Techn. Clarifications LME.FTST |
| FATDMA offset number | X | X (when AIS base station operates in independent mode) | X (only when AIS base station operates in dependent mode) | - | - | Relative term; refer to Table 33 of Rec. ITU-R M.1371-1+Techn. Clarifications |
| FATDMA block | X | X | X | X | X | FATDMA blocks are known both in relative and absolute terminology throughout the AIS Service and the AIS VDL |
| FATDMA increment | X | X | X | X | X | If FATDMA increments are used at all, they are known to all layers; refer to Rec. ITU-R M.1371-1 + Techn. Clarifications Table 8, LME.FTI |
| FATDMA block size | X | X | X | X | X | FATDMA block size is known both in relative and absolute terminology throughout the AIS Service and the AIS VDL (refer to Table 8 Rec. ITU-R M.1371-1; LME.FTBS) |
| FATDMA plan | - | -- | - | X | X | AIS VDL only recognizes the effects in terms of concrete reservations |
| FATDMA scheme | - | - | X | X | X | Group of all reservations of one PSS |
| FATDMA schedule | - | - | - | X | X | Group of all reservations of one or all PSS in given area |

# Annex C Introduction to relevant aspects of the FATDMA reservations

The following rules for FATDMA reservations are taken from Recommendation ITU-R M.1371 (current revision and IALA Technical Clarifications).

A FATDMA reservation is made by a base station transmitting Data Link Management Message, message 20.

A FATDMA reservation consists of

- a start slot, the slot where the FATDMA reservation starts in the frame,

- the block size, the number of consecutive reserved slots,

- an increment, the number of slots between the start of each reserved block and

- a time out value, how long the FATDMA reservation is valid.

The term FATDMA block denotes the effect of a FATDMA reservation on the AIS VDL.

Each Data Link Management Message (message 20) can reserve up to four (4) independent and different FATDMA reservations.

The blocks of FATDMA reservations should be evenly distributed across the frame. The values recommended for "increment" are as follows (refer to Technical Clarifications §2.48.1; message 20): 2, 3, 5, 6, 9, 10, 15, 18, 25, 30, 45, 50, 75, 90, 125, 225, 250, 375, 450, 750, 1125. Use of one of these values guarantees symmetric slots reservations throughout each frame.

Using the value 0 for "increment" means, that only one reservation is made in the frame.

The lower the "increment" value, the more slots will be reserved.

In normal operation the base station should use FATDMA for its own transmissions. For this, slots have to be reserved by a base station using Data Link Management Message (message 20). If there are no reserved FATDMA slots available the base station will transmit using RATDMA.

The FATDMA reservation is timed out after 3 minutes (default) in the mobile AIS stations. The time out value for the FATDMA reservation within the mobile AIS station can be configured between 1 and 8 minutes by the Data Link Management Message (message 20). The FATDMA reservation should be repeated before a time out occurs, if the FATDMA reservation is to be maintained.

The Data Link Management Message applies only to the AIS channel on which it is transmitted.

The effects of an FATDMA reservation on other AIS stations is as follows:

- Mobile stations will not include any FATDMA reserved slots in their candidate slot map at all, if the reserving base station is within 120 nautical miles. If the position of the reserving base station, as derived from message 4, is more than 120 nautical miles away from the mobile, Class A mobile stations should ignore the reservation.

- Other base stations will not include any FATDMA reserved slots in their candidate slot map when using RATDMA. However, at the discretion of the competent authority, a base station may intentionally FATDMA reserve and use the same time slots within 120 nautical miles, if no interference is likely.

# Annex C Fundamental philosophy statements for FATDMA Configuration

Before any details for FATDMA Configuration can be considered, some fundamental “philosophy” statements need to be set up.

## Guiding principles for setting up FATDMA plans

There are several possibilities to create such FATDMA plans, FATDMA schemes and FATDMA schedules. The following principles were adhered to when developing the FATDMA plan of this Annex:

### Rules in regard to the applicability of the FATDMA plan

- The rules underlying the FATDMA plan should be as simple as possible.

- The rules underlying the FATDMA plan should be globally applicable. The global applicability would provide for a fair and equal basis for setting up the FATDMA plans.

- The resulting FATDMA plan should cover the overwhelming majority of FATDMA configurations world-wide in just one planning step.

- When adjacent countries are required to co-ordinate this should require only the minimum number of parties involved, i. e. only the actually affected countries would treat the co-ordination issue independently of other co-ordination issues with other countries in a different area.

- There should not be a need to frequent or even run-time co-ordination between adjacent and affected countries.

- The most fundamental assumption is, that adjacent and affected national administrations are willing to protect the joint operation of the AIS VDL and to achieve its smooth operation in their area of (joint) responsibility. It is assumed that all affected national administrations will exhibit due diligence to arrive at a solution where co-ordination is needed.

- It is also assumed, that not all aspects of co-ordination can be satisfactorily achieved by FATMDA planning alone. Other mitigation methods consistent with the AIS system layout as a whole would be needed, and those mitigation methods are hinted at in this Annex but are fully described in other chapters of this document.

### Rules to minimise the usage of the VDL by FATDMA reservations

- The number of FATDMA reserved slots should be minimised in any given area. This would translate into the following consecutive rules:

- The FATDMA Configuration should be such, that FATDMA reservations are only done when there is a current (run-time) need for FATDMA reserved slots.

- The FATDMA block size should not exceed 5 consecutive slots (refer to Technical Clarifications §2.18.1 "Candidate Slots").

- However, this rule should not be applied rigidly, since the sophistication and the complexity of the components of the AIS Service(s) involved should be decreased rather than increased by a properly completed FATDMA plan. This translates into the following rule: The FATDMA plan should impose only a minimum of run-time dynamic FATDMA reservations. A maximum of stationary FATDMA reservations should be achieved.

- The FATDMA plan should provide for all capabilities of the AIS.

- The FATDMA plan should provide a maximum flexibility of AIS VDL operations.

- FATDMA reservations should be done, also using the frequency separation of the two working channels of the AIS (A and B) for the same slots. By default, mobile stations include the slots in the opposite working channel in the same slot only as a last resort into their candidate slot set, hence acting as if a de-facto FATDMA reservation was made in these other slots as well. Not to use these slots for FATDMA reservations would be a waste of available slots.

# Annex C FATDMA Reserved Slot Use Cases

This section describes the functional cases that require FATDMA reservations, the parameters that need to be considered, how to go about deciding the slot reservation profile and proposes usage recommendations for each case. The selection of the actual AIS base station reserving the slots will be discussed in another section.

Note: While the main concepts, such as the usage categories, are in place and are mature in this section, the detailed usage recommendations on individual messages should be reviewed and may be amended at the next revision of A-124.

## Rules for usage categories

The following rules, developed from the fundamental rules described in Section 4 apply to the  usage category definitions:

- At least one stationary FATDMA slot should be reserved for each of the defined usage-categories. This is necessary to minimise the dynamics of FATDMA allocation: All FATDMA reserved slots of a FATDMA plan, reserved for a given usage-category, should be reserved in such a way, that the appropriate FATDMA reservations comply exactly with the anticipated periodicity of the messages to be transmitted. To that end the FATDMA plan should take into account

- the required periodicity within one frame, which translates into the appropriate FATDMA increment and

- the usage of the individual minutes of an FATDMA epoch.

- In a single FATDMA reservation with more than one FATDMA reserved slot, the available slots can be used for different categories of usage independently of each other. This means, that slots reserved for different usage-categories can be bundled together in one FATDMA reservation, while their respective individual usage may be completely different.

## Use Cases for FATDMA reservation usage category 1 - AIS VDL management slots

The FATDMA reserved slots of usage category 1 will be used for the transmission of any message, which is necessary for the management of the AIS VDL.

The following use cases have been identified for FATDMA reservation usage category 1:

- Transmission of Data Link Management message  (message 20) by base station(s)

- Transmission of Base Station Report (message 4) with default reporting rate by base station(s)

- Transmission of Channel Management message (message 22) by base station(s)

Each of the above use cases will be dealt with in more detail in consecutive sections.

### Transmission of Data Link Management Messages (message 20)

When defining the usage of Data Link Management messages (message 20),  the following aspects should be taken into account:

- At least one FATDMA reservation must be made before FATDMA reserved slots become available. The transmission of a Data Link Management message 20 is needed to make any FATDMA reservations in a given area.

- When starting a new FATDMA reservation, i. e. when there has been none previously,

- the FATDMA reserved slots will be available in the next frame after the frame during which the transmission of message 20 occurred.

- the FATDMA reserved slots can only safely be used after 8 frames have elapsed because mobile AIS stations need to clear the FATDMA reserved slots.

- Depending on the total number of FATDMA reservations needed for a given geographical area, there may be more than one transmission of Data Link Management message 20 needed to accommodate all required FATDMA reservations in that area.

- Every FATDMA reservation times out after 3 minutes (default), if no different time out value is given within the FATDMA reservation. A different value may be between 1 and 8 minutes.

- Should a particular FATDMA reservation be maintained continuously longer than the above pre-set time out value, the FATDMA reservation needs to be refreshed periodically before the FATDMA reservation expires.

- Due to the time out value up to 8 minutes, one refreshment transmission of message20 per FATDMA epoch would be sufficient to maintain that FATDMA reservation.

- The overlap of 2 minutes between maximum time out pre-set and FATDMA epoch length, would allow the distribution of the required transmissions of message 20 over the first two frames of any FATDMA epoch, hence allowing for an even distribution of message 20 transmissions in the first two frames of the first FATDMA epoch of any new continuous FATDMA reservation.

- When updating the refreshment of a continuous FATDMA reservation at the minimum rate, i. e. once per epoch, AIS mobile stations entering the area under consideration would learn about this FATDMA reservation with a delay of up to nearly 6 minutes.

- Note: The periodicity of transmissions of message 20 should not be confused with the FATDMA increment of one FATDMA reservation. One FATDMA reservation may reserve many FATDMA blocks throughout the same frame, but may time-out after 8 minutes maximum because it was not refreshed.

Based on the above aspects two different use cases of transmission of message20 can be identified:

5.2.1.1 Transmission of Data Link Management message 20 to establish a short-lived FATDMA reservation (no repetitive pattern):

Short-lived FATDMA reservations should be used to accommodate dynamic demand for FATDMA reserved slots. 
Usage recommendation:

- When using a short-lived FATDMA reservation, its time-out should be pre-set to the duration of a FATDMA epoch, i. e. 6 minutes (or time-out value 5), maximum.

- Due to the purpose of short-lived FATDMA reservations, it should be carefully considered whether more than one FATDMA block should be reserved by this short lived FATDMA reservation, i. e. the FATDMA increment parameter should be used with care.

5.2.1.2  Transmission of Data Link Management message 20 to establish a continuous FATDMA reservation (long term repetitive pattern):

Continuous FATDMA reservations should be used to accommodate the stationary demand for FATDMA reserved slots.
Usage recommendation:

- Data Link Management message (message 20) should be transmitted at least once per epoch to maintain a continuous FATDMA reservation.

- When transmitting message 20, the time-out value should be set to 6 minutes, thus spanning one entire FATDMA epoch.

- The transmissions of message 20 for different continuous FATDMA reservations should be evenly distributed over the FATDMA epoch.

- Depending on the general risk assessment of message loss due to VDL traffic loading and / or interference in the given area, there should be up to 6 transmissions of the Data Link Management message (message 20) in one FATDMA epoch to maintain the same continuous FATDMA reservation, thus creating redundancy and providing for a more rapid reaction of new mobile AIS stations entering the given area to the FATMDA reservations. Those transmissions of message 20 for the same continuous FATDMA reservation should also be evenly distributed over the FATDMA epoch, i. e. one message 20 per 3 frames (2 transmissions per epoch), one message 20 per 2 frames (3 transmissions per epoch), one message 20 per one frame (6 transmissions per epoch).

### Transmission of Base Station Report (message 4) at the default reporting rate

Assignment of the transmission of Base Station Report (message 4) at the default reporting rate is justified, because a message 4 with a valid position allows all mobile AIS stations to correlate this position with the transmission(s) of Data Link Management message 20 using the same MMSI. The correlation is used by a mobile AIS station to determine whether the 120 nm intentional slot reuse rule should be used. This is an essential AIS VDL management rule because it  effectively counteracts the potential blocking of a large number of slots by the reception of non-relevant  message 20 from distant base stations during times of  good RF propagation.

When defining the usage of Base Station Report (message 4) at the default reporting rate, the following aspects should be taken into account:

- Recommendation ITU-R M.1371 states, that the default reporting rate by a base station should be once per 10 seconds.

- Therefore, the transmission of Base Station Reports exhibits a short-term repetitive pattern.

- This translates into six (6) slots reserved for that purpose per frame for a reporting interval of 375 slots, this translates into a FATDMA increment of 750 (message 20) in one FATDMA reservation on each channel.

Usage recommendation:

When transmitting Base Station Reports (message 4) with default reporting rate by one base station,

- there should be one (1) slot reserved with an increment of 750 on each channel, and

- the required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation (see above).

### Transmission of Channel Management Messages (message 22)

When defining the usage of Channel Management messages (message 22), the following aspects should be considered:

- The Channel Management message (message 22) provides the capability to change the  fundamental operation settings of the AIS frequency channels A and B.

- A mobile AIS station will store up to eight (8) different Channel Management settings, which were either received by Channel Management message, by a DSC telecommand or by manual setting aboard.

- If a new Channel Management setting is received, which partly or in total overlaps any of the previous Channel Management settings, the older Channel Management setting will be erased and replaced by the new Channel Management setting.

- Amongst other parameters, every Channel Management setting includes a geographical area (rectangle) for which this parameter set would be activated.

- The AIS mobile station uses the Channel Management setting corresponding to the geographical area in which it is currently located.

- The geographical area given for any Channel Management setting can be very large.

- The transient characteristics of a change in Channel Management settings involve at least the time span of two FATDMA epochs. Therefore, the transmission of Channel Management messages should be considered to exhibit long-term repetitive pattern.

- The timing requirements for the transmission of Channel Management messages as such are relaxed, i. e. the mobile AIS stations act upon a Channel Management setting when they enter the area designated by the Channel Management setting regardless of when they received the Channel Management setting.

- To ensure, that all mobile AIS stations operate on the default AIS frequency settings with the default Channel Management settings, a competent authority should transmit Channel Management messages on a regular basis with the default Channel management settings for its area of responsibility. These transmissions would instruct the mobile AIS stations to “return to default”. Due to the long-term repetitive pattern of the Channel Management message the VDL load created by these “return to default” transmissions would not be high.

Usage Recommendation:

- The geographical area parameter of any Channel Management message (message 22) should be as small as possible. It may be equal to the size of the geographical area of the FATDMA plan, FATDMA schedule or even FATDMA scheme.

- Channel Management messages should be transmitted at least once per FATDMA epoch.

- Depending on the general risk assessment of message loss due to VDL traffic loading and / or interference in the given area, there should be up to 6 transmissions of the Channel Management message (message 22) in one FATDMA epoch, thus creating redundancy and providing for a more rapid reaction of new mobile AIS stations entering the given area. Those transmissions of message 22 should also be evenly distributed over the FATDMA epoch, i. e. one message 22 per 3 frames (2 transmissions per epoch), one message 22 per 2 frames (3 transmissions per epoch), one message 22 per one frame (6 transmissions per epoch).

- The required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation (see above).

- Even when there is no channel management scheme in place, which deviates from the default Channel Management settings in any regard, the Channel Management message should be transmitted by administrations with default Channel Management settings (return to AIS1 and AIS2 in the geographical area) to erase „wrong“ and „old“ frequency settings in mobile AIS station’s memories.

## Use Cases for FATDMA reservation usage category 2 - High timing requirements slots

These FATDMA reserved slots will be used for the transmission of messages, which need to be transmitted within a defined time period.

The following use cases have been identified for FATDMA reservation usage category 2:

- Transmission of Base Station Report (message 4) when a base station in the given area is acting as a semaphore

- Transmission of Position Reports of any kind of AIS mobile stations, when assigned a “FATDMA protected” slot (“safe slot” concept)

- Transmission of Safety Related message (message 12 and 14)

- Transmission of Acknowledgements on received addressed messages (message 7 and 13) by base station(s)

- Transmission of DGNSS broadcast binary messages (message 17) with high timing requirements contents, such as integrity alerts, by base station(s)

- Re-transmission of Position Reports (and other messages) of any kind of AIS mobile stations

### Transmission of Base Station Reports when a base station (in the given area) is acting as a semaphore

The assignment of the transmission of Base Station Report (message 4) with increased reporting rate due to a base station switching into semaphore mode is justified, because the reporting interval is decreased to a value below 4 seconds. This constitutes a high timing requirement.

When defining the usage of Base Station Report (message 4) with increased reporting rate due to a base station acting as a semaphore, the following aspects should be taken into account:

- Recommendation ITU-R M.1371 state, that the increased reporting rate by a base station should be once per 3 1/3 seconds.

- Therefore, the transmission of Base Station Reports exhibits a short-term repetitive pattern.

- This translates into eighteen (18) slots reserved for that purpose per frame for a reporting interval of 125 slots or into a FATDMA increment of 250 (message 20) in one FATDMA reservation on each channel.

- Since the requirement for a base station to act as a semaphore is infrequent and transient in nature, one or several short-lived FATDMA reservations may suffice for the duration of the base station acting as a semaphore.

When transmitting Base Station Reports (message 4) with an increased reporting rate due,

- there should be one (1) slot reserved with an increment of 250 on each channel, and

- the required FATDMA reserved slots should be allocated by one Short-Lived FATDMA reservation with time-out value pre-set to maximum, i. e. 8 minutes, and consecutive Short-Lived FATDMA reservations with the same parameters as long as the base station is acting as a semaphore (see above).

- Instead, the required FATDMA reserved slots may be taken from a continuously and in advance reserved pool of usage category 2 slots within the same FATDMA scheme, FATDMA schedule or FATDMA plan. Although the latter option would smooth the operation of the VDL by avoiding transient FATDMA reservations, more VDL capacity will be used when it will not actually be needed.

### Position Reports of any kind of AIS mobile stations, when assigned a “FATDMA protected” slot (“safe slot” concept)

To assign the transmission of Position Reports of any kind of mobile AIS stations, which have been assigned to specific “FATDMA protected” slots given by a previous assignment command, to the FATDMA usage category 2 („High timing requirements slots”) is justified, because the position reports will invariably occur in the slots pre-determined by the previous assignment command, but the resulting reporting rate of the mobile AIS station may still be high (Class A mobile stations will allow only to increase the reporting rate up to once per second). This constitutes a high timing requirement.

When defining the usage the following aspects need to be taken into consideration:

- assigned transmissions of mobile stations either hard assignment (message 2, 18)

Usage recommendation:

When reserving FATDMA protected slots for assigned mode mobile stations

- The number of slots and the increment required depend on the defined reporting rate

- The required amount of slots should be allocated by one short lived FATDMA reservation preset to x minutes (depending on the timeout of the assignment)

### Transmission of Safety Related Messages (messages 12 and 14) by base station(s)

The assignment of the transmission of Safety Related Messages (messages 12 and 14) is justified, because it is a general operational requirement that safety related messages are being transmitted to the VDL with minimum latency after being released. This constitutes a high timing requirement.

When defining the usage the following aspects need to be taken into consideration:

Safety related message transmissions are of a non-periodic nature. Repeated broadcast of safety related messages within a timeframe of epoch magnitude may well be considered a nuisance rather than relevant information.

The number of slots required for safety related messages can dynamically vary between 1 and 5, however reserving blocks of timeslots larger than 3 on a continuous basis is generally discouraged.

Usage recommendation:

- Reserving FATDMA protected slots for Safety Related Messages specifically, is not recommended, due to the non periodic nature.

- If a pool of general purpose timeslots are reserved and are available within the selection interval, they can be used.

- Addressed messages may be transmitted (up to 3 times) using RATDMA, since the acknowledge message will indicate whether or not the message was received. When the acknowledgement is received, no repeated transmissions are necessary.

### Transmission of Acknowledgements on received addressed messages (message 7 and 13) by base station(s)

The assignment of the transmission of Acknowledgements on received addressed messages (message 7 and 13) is justified, because the acknowledgement on a correctly received addressed message is required and must be received within 4 seconds by the originating station of the addressed message in order to prevent re-transmission of the same addressed message by the originating station, which would result in unnecessary VDL load. This constitutes a high timing requirement.

When defining the usage the following aspects need to be taken into consideration:

Acknowledgements are non-periodic of nature, and most likely occur infrequently.

Usage recommendation:

When reserving FATDMA slots for Acknowledgements of received addressed messages

- Acknowledgement messages should be transmitted using available high timing or general purpose FATDMA reserved timeslots if possible.

- RATDMA may be used if no FATDMA slots are available – to stop the addressed message from being repeated.

- If FATDMA reserved slots are required, they should be allocated by a Continuous FATDMA reservation.

### Transmission of DGNSS broadcast binary messages (message 17) with high timing requirements contents, such as integrity alerts, by base station(s)

This service is described in detail in Annex B.

The assignment of the transmission of DGNSS broadcast binary messages (message 17) with high timing requirements contents, such as integrity alerts, is justified, because of the relevant operational requirement in the field of GNSS.

Note: While the transmission of e. g. integrity alerts of DGNSS message to mobile AIS stations may exhibit a high timing requirement, this may not be true to any other DGNSS message content. Therefore, all DGNSS messages contents should be examined in regard to their timing requirements. DGNSS Message contents identified not to exhibit high timing requirements for transmission should be treated as usage category 3 (“General purpose pre-reserved slots”).

Usage recommendation:

- Provisions should be made to have high timing requirements timeslots available for transmitting a DGNSS broadcast message containing a DGNSS integrity alert, one slot every 3 1/3 seconds, requiring an increment of 250 per channel

- Provisions should be made to have enough time slots available in usage categori 3, general purpose usage timeslots, to transmit a full set of DGNSS corrections at least twice every 30 seconds – this should be possible using 3 timeslots per GNSS system on each channel every 30 seconds.

- the required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation.

### Re-transmission of Position Reports (and other high-timing requirement messages) of any kind of AIS mobile stations

The VDL access schemes for Simplex Repeater Stations are either FATDMA or RATDMA. There are 3 types of Simplex Repeater Station

- FATDMA only

- RATDMA only

- FATDMA  and RATDMA

Simplex Repeater Stations can be configured to repeat either all received messages or  selected sub-sets of the received messages.

When defining the usage the following aspects need to be taken into consideration:

- A Simplex Repeater Station should repeat the message within 4 seconds (150 slots) and if it cannot find free slots within 4 seconds the message should not be repeated.

- The greatest impact on the VDL is when a Simplex Repeater Station is configured to repeat all messages – this will double the load on the VDL.

- Above 50% VDL loading starts to have an impact on Mobile AIS Stations ability to find free slots. Hence it is considered that the Simplex Repeater Station should not use more than 25% of the VDL bandwidth for its transmissions.

- RATDMA enabled devices should monitor that the total VDL load does not exceed 50%, and reduce it’s transmissions if this occurs.

Usage recommendation:

- The required FATDMA reserved slots should be allocated by a Continuous FATDMA reservation.

- A Competent Authority should exercise great care when implementing a Simplex Repeater Station as inappropriate configuration could have a detrimental effect on the VDL.

- FATDMA reservations should not exceed 1125 timeslots (25 % of the VDL bandwidth)

- The Competent Authority should monitor the use of FATDMA Simplex Repeater Stations to ensure that the VDL loading does not exceed 50% in the vicinity of the Station for any significant period as this would adversely affect the ability of mobile stations to find free slots for their own transmissions.

## Use Cases for FATDMA reservation usage category 3 - General purpose pre-reserved slots

These FATDMA reserved slots will be used for the transmission of any message, which does not fall under FATDMA reservation usage category 1 or 2.

The following use cases have been identified for FATDMA reservation usage category 3:

- Transmission of any assignment command (message 16; [message 23]) by base station(s)

- Transmission of UTC and Date Inquiry message (message 10) by base station(s)

- Transmission of DGNSS broadcast binary messages (message 17) with relaxed timing requirements contents by base station(s)

- Transmission of Binary Messages (message 6 and 8) by base station(s)

- Transmission of Interrogation message (message 15) by base station(s)

- Transmission of Aids-to-Navigation reports (message 21; [new AtoN message])

- Re-transmission of messages other than position reports

### Transmission of DGNSS broadcast binary messages (message 17) with relaxed timing requirement contents.

Please refer to section 6.3.5 of this annex, and Annex B.

### Transmission of any assignment command (message 16; message 23) by base station(s)

When defining the usage of any assignment command (message 16 or message 23), the following aspects should be considered:

- There are two possible options of the assignment command which impact the behaviour of mobile AIS stations: assignment of a new reporting rate behaviour while leaving the actual slot selection to the mobile AIS station (so called “soft” assignment) and assignment of the usage of specific pre-determined slots by the assigned mobile AIS station (so called “hard” assignment).

- Assignment Command (message 16) and Group Assignment Command (message 23) differ in that the Assignment Command applies to individual mobile AIS stations and provides for both “soft” and “hard” assignment, while the Group Assignment Command provides the capabilities to address group of mobile AIS stations determined by their respective device class.

- Every assignment times out after 3 minutes (default), if no different time out value is given within the assignment command. A different value may be between 1 and 8 minutes. The effect of the [Go silent] parameter for Class B mobile AIS stations within the Group Assignment Command (message 23) will last for up to 15 minutes, however.

- All transmissions of assignment commands do not – by default – exhibit a repetitive pattern. Therefore the transmission of assignment commands would require slot reserved in the usage category 3 (“General purpose pre-reserved slots”)

- Should a particular assignment be maintained over a longer period than 8 minutes, the assignment needs to be refreshed periodically before the present assignment expires. In this case, the following should be considered:

- Due to the time out value up to 8 minutes, one refreshment transmission of the assignment command per FATDMA epoch would be sufficient to maintain that assignment.

- When updating the refreshment of an assignment at the minimum rate, i. e. once per epoch, AIS mobile stations entering the area under consideration would learn about this assignment with a delay of up to nearly 6 minutes.

- While a mobile AIS station commanded to change into slot assignment mode (“hard” assignment) exhibits a high timing requirement for the pre-reserved slots, the assignment command as such has no high timing requirement for transmission. [The latency between the trigger event (assignment request) and the reaction shown by the assigned mobile AIS station(s) will be determined not by the timing of the transmission of the assignment command (message 16 or message 23) but by the present frame: Any assignment will be effective with the next frame earliest. Hence the latency can be up to one minute, which is the duration of one frame.]

NOTE: this will be checked with Class A, Class B etc. and clarification will be provided, in a future release of this document.

Usage recommendation:

- Assignment commands should be transmitted in one (1) of the next available slots reserved in the usage category 3 (“General purpose pre-reserved slots”).

- Should a particular assignment be maintained over a longer period than 8 minutes, the assignment needs to be refreshed periodically before the present assignment expires. In this case, the following recommendation applies:

- The assignment command (message 16 or message 23) should be transmitted at least once per epoch for a un-interrupted assignment as long as the desired duration of the assignment.

- When transmitting the assignment command, the time-out value should be set to 6 minutes, thus spanning one entire FATDMA epoch.

- Depending on the general risk assessment of message loss due to VDL traffic loading and / or interference in the given area, there should be up to 6 transmissions of the assignment command to maintain the un-interrupted assignment, thus creating redundancy and providing for a more rapid reaction of new mobile AIS stations entering the given area to the assignments. Those transmissions of assignment command for the same desired assignment should also be evenly distributed over the FATDMA epoch, i. e. one assignment command per 3 frames (2 transmissions per epoch), one assignment command per 2 frames (3 transmissions per epoch), one assignment command per one frame (6 transmissions per epoch).

### Transmission of UTC and Date inquiry message (message 10) by base station(s)

UTC and Date inquiry message may be used to query a mobile stations time syncronisation.

When defining the usage the following aspects need to be taken into consideration:

- The transmission has relaxed timing requirements

Usage recommendation:

- Reserving FATDMA timeslots for this purpose is not recommended.

- If a pool of general purpose FATDMA timeslots are available such a timeslot should be used, otherwise RATDMA can be used.

### Transmission of Binary Messages (message 6 and 8) by base station(s)

When defining the usage the following aspects need to be taken into consideration:

- The transmission may have either high or relaxed timing requirements

- The number of slots required for binary messages can dynamically vary between 1 and 5 – however message sizes above 3 timeslots are discouraged.

Usage recommendation:

- Reserving FATDMA protected slots for Binary Messages specifically, of non periodic nature is not recommended.

- If a pool of general purpose timeslots are reserved and are available within the selection interval, they can be used.

- Addressed messages may be transmitted (up to 3 times) using RATDMA, since the acknowledge message will indicate whether or not the message was received. When the acknowledgement is received, no repeated transmissions are necessary.

### Transmission of Aids-to-Navigation Reports (message 21)

A detailed description of the use of AIS on AtoNs is available in IALA recommendation A-126.

When defining the usage the following aspects need to be taken into consideration:

- Two slots required for each AtoN message

- Update rate may vary, refer to A-126

- AtoN AIS stations may use slots, which have been reserved by a base station by FATDMA; therefore the AtoN AIS station could be configured to broadcast in fixed slots assigned by competent authority of FATDMA scheme

Usage recommendation:

- FATDMA reservations for AtoN’s should use a Continuous FATDMA reservation.

- To reduce bandwidth loading of the VDL, AtoN transmissions should be coordinated such that several AtoN’s may use the same timeslots within different frames of the epoch.

### Designing a FATDMA schedule

The use cases concerning VDL management transmissions and periodic transmissions with high timing requirements require dedicated patterns of timeslots available in the FATDMA schedule for a given Physical Shore station.  However, the use cases regarding  high timing requirement transmissions with non-periodic nature, and general purpose use with relaxed timing requirements, indicate that these transmissions could actually make more efficient use of a pool of Pre-Reserved Slots and a priority transmission queue.

When designing the layout of such a pool, the following considerations should be taken:

- The pool must have sufficient size to accommodate the required usage capacity within one frame to minimize the latency of the actual transmission and provide for relaxed queuing.

- It must have sufficient repetitive occurrence to accommodate the high timing requirements

- The transmission queue must be capable of giving high timing transmissions priority over relaxed timing transmissions.

# Annex C The definition of FATDMA schemes by space, time and frequency usage separation

After the previous section has introduced the purpose of FATDMA reservations on a detailed level and has given the conditions which apply for certain use case categories of FATDMA reservations in some detail, this section deals with the question: What should be the geographical criteria to determine co-ordinated operation?

## The universal grid scheme – separation by space and frequency

The geographical grid defines a global method for selecting a default FATDMA scheme for any area under consideration: It should be noted that the area is the most important aspect, not the actual location of the transmitting entities using FATDMA reserved slots. It is the total of transmitted, i. e. injected, messages of all relevant AIS stations using FATDMA reserved slots, that count in a given area, i. e. cell, not the station and its location as such in the first place.

It is the goal of the geographical grid to ensure, that identical FATDMA schemes by default are geographically separated more than 120 nautical miles, by arranging a set of FATDMA schemes in a geographical grid in such a manner, that no FATDMA scheme is reused within 120 nautical miles.

The geographical grid is meant as a geographical starting point from which further planning and co-ordination must take place, if needed at all.

The geographical grid is created by dividing the world into cells as illustrated below, by an algorithm, based on the WGS84 datum. The precise algorithm for calculating the geographical grid is given in Attachment A to this Annex. The following picture just gives an impression in principle.

The illustration contains a grid with a repetitive pattern of 36 different FATDMA schemes in an 6 x 6 matrix (called a ‘master cell’), where each cell is approximately 30 x 30 nautical miles in size. Employing the two working frequencies A and B of the AIS would yield 72 different FATDMA schemes in each master cell, while each individual cell would have the two FATDMA schemes created by using the very same time slot reservation pattern but only in the mirrored working frequency.

Identical FATDMA schemes are repeated in a distance of the size 120 nm + 2*cell size, i.e. in a distance of 180 nautical miles.

The above considerations yield one number of a FATDMA scheme. To denote the frequency separation, the two FATDMA schemes are identified by adding the Roman numerals “I” and “II” to the number, e. g. 59-I and 59-II. It should be noted that neither FATDMA scheme works exclusively on AIS working channel A or B exclusively, but alternate between these working channels constantly, but in a frequency mirrored manner. Hence, the Roman numerals were chosen to avoid confusion with the working channel designation A and B.

## FATDMA schemes – separation in time

Above the definition was introduced how to arrive at a cell number for the geographical cell, any given location on earth is in. Further the frequency separation was included. Both FATDMA schemes thus derived are identical in regard to their time slot reservation and usage characteristics, but employ these on mutually mirrored frequencies. It is now necessary to define to what absolute FATDMA reservations and what precise usage categories the above FATDMA scheme numbers translate to.

This is done in a table in Attachment B, in accordance with the following rationale:

As described in the section on usage categories, it is desirable that the FATDMA schemes are confined to a certain part of the time domain of the AIS VDL in order to provide sufficient free time slots for mobiles to allocate under all dynamical situations. The sum of all FATDMA schemes defined here constitute a repetitive pattern of 5 + 5 + 4 timeslots for every 25 timeslots:

| x | x | x | x | x |  |  |  |  |  | x | x | x | x | x |  |  |  | x | x | x | x |  |  |  | x | x | x | x | x |  |  |  |  |  | x | x | x | x | x |  |  |  | x | x | x | x |  |  |  | x | x | x | x | x |  |  | .... |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  | 5 |  |  |  |  | 10 |  |  |  |  | 15 |  |  |  |  | 20 |  |  |  |  | 25 |  |  |  |  | 30 |  |  |  |  | 35 |  |  |  |  | 40 |  |  |  |  | 45 |  |  |  |  | 50 |  |  |  |  | 55 |  |  |

This repetitive pattern of every 25 timeslots ensures, that mobiles’ transmissions fits neatly into the remaining timeslots, since the associated reporting rates of 2 seconds, 3 1/3 seconds, 6 seconds, 10 seconds... like those reporting rates defined for Class A mobiles translate into one transmission every 75, 125, 225, 375.. timeslots.

These FATDMA slot reservations are the theoretical maximum at planning time and will not be used all simultaneously in the overwhelming majority of cases globally in the same cell. For the few remaining cases, where special considerations are needed, see below.

The above FATDMA schemes are divided into slots assigned to specific usage categories as given in the Attachment B. The assignment to usage categories rest upon the following rationale. Each FATDMA scheme consists of:

- Usage category 1 time slots – VDL management

- Three (3) timeslots per frame and working channel for Base Station report in non-semaphore mode

- One (1) timeslots per frame and working channel for Data Link Management and / or Channel Management messages

- Usage category 2 time slots – High Timing requirements: Six (6) timeslots per frame and working channel for Base Station reports in semaphore mode or other high timing requirement transmissions; these time slots can be used for Usage category 3 (General purpose) in the absence of Usage category 2 messages.

- Usage category 3 time slots – General purpose: Seven (7) timeslots per frame and working channel for general purpose use or possibly for Usage Categories 1 or 2 (if urgent need should appear)

To re-iterate what was said before: Only time slots needed at run-time should be actually reserved.

Consequential action in particular in those (few) cases when the above FATDMA reservations would not suffice: The requirement for implementing queuing techniques (including priority schemes) as a mitigation measure on the part of the sources for transmission (not necessarily the AIS base stations of the national competent authority alone) is obvious.

After this definition part, the following sections will introduce the rules for allocating the above FATDMA schemes, referenced by their number, to competent authorities, both national and domestic competent authorities.

# Annex C Allocation of the defined FATDMA schemes to competent authorities

## Summary of the previous definitions and their consequences

The sections so far have introduced the definitions and concepts needed for a FATDMA plan. The means developed for setting up the FATDMA plan can be summarised by the following two parts:

- The geographical grid scheme introduced in the previous section provides for separation of FATDMA reservations in space, taking into account the 120 nautical mile rule.

- The FATDMA schemes, identified by their FATDMA scheme code, provide the combination of separation of FATDMA reservations in time, i. e. different time slots, and frequency, i. e. different working channels A and B, while preserving the alternating use of –those two working channels. The FATDMA scheme layouts have been informed by usage considerations as well, i. e. they assign certain usage categories to certain slots in the FATDMA schemes.

As a consequence, the FATDMA schemes, identified by their FATDMA scheme code, together with the geographical grid, provide a means to allocate FATDMA schemes for each geographical area, isolated from other geographical areas and their allocated FATDMA schemes – at least in a first, and in most cases sufficient step. Also, this is globally applicable.

Also the means exhibit a certain simplicity as it is built only on the concept of an area, as opposed to location: transmitting stations, using FATDMA reservations, inject their messages into that given area, i. e. one geographical grid cell, while the concrete location from where this transmission occurs within that geographical grid cell is less relevant due to the separations in space introduced by the geographical grid scheme.

From a practical point of view, this means that there will be minimum effort required to co-ordinate between adjacent countries and their national competent authorities because of the isolation of the FATDMA schemes in a given area: By introducing the geographical grid, only bi-lateral or tri-lateral co-ordination would be required in areas, where on the geographical grid cell falls into more than one country.

There are consequences that need to be discussed at a later planning stage, if they are relevant at all:

- The upper limit of the VDL loading due to FATDMA reservations can only be assessed in a given area after the concrete FATDMA plan for that area has been established.

- There may be restrictions to the usage of FATDMA reserved slots over time in areas, where a larger number of transmitting stations are planned to inject messages: Not every message to be transmitted may be transmitted at once when a FATDMA reserved slot allocated to that transmitting station arrives; there may be the need for queuing. These restrictions are due to the very nature of the AIS; a mitigation measure in the long run would be to increase the number of working frequencies of the AIS (which would require more than two receiving processes in mobile AIS stations).

- Proper, and even sophisticated coverage planning would be needed in some cases, employing directional antennas, lower antenna heights of the transmitting station(s) and even transmission power attenuation of the transmitting station(s), making use of or at least taking into account the local topography: The goal would be keep the injected message confined to that geographical grid cell if at all possible.

However, it is anticipated that these consequences will only be relevant to a few places globally, thus justifying extra planning efforts at those places (instead of not having a simple global plan for the large majority of cases). In addition, it is good to know that there are mitigation measures in place for areas where the most simple measures would no longer work.

## Stepwise allocation of the defined FATDMA schemes to competent authorities

Based on the above definitions and having considered the consequences, the following simple two-step approach for planning can be taken.

### Step 1: International allocation of FATDMA schemes

#### Step 1A: Check need for international co-ordination

All FATDMA schemes that fall completely – judging by the area of their geographical grid cell - within the EEZ (Exclusive Economical Zone) of a country belong to that country. They are immediately subject to domestic allocation rules (see step 2 below).

By means of the FATDMA scheme definition of the previous section this should cover the overwhelming majority of cases globally already in just one step.

#### Step 1B: Need to international allocation co-ordination arises

There are FATDMA schemes, which fall - by the geographical area of their geographical grid cell - into the adjacent EEZ of different countries. To these only apply the following rules.

- Thus affected countries and only the thus affected countries need to negotiate the allocation on a peer-to-peer basis and create a memorandum of understanding (or similar document) on the usage of the FATDMA scheme in question. They may use one or more of the following options, combining them as deemed necessary.

- Option A: One or more than one country can contribute an unused and immediately adjacent FATDMA scheme from within its own area (EEZ), where it can be guaranteed by that country that this cell will remain unused (e. g. due to topology).

- Option B:  The cell management is assigned solely to one of the two countries, while both may use the FATDMA scheme on a mutually agreed basis.

- Option C: Both countries set up a detailed usage allocation based on the usage categories of the slots of the FATDMA scheme.

- Option D: There may be an alternating use (e. g. based on epoch) of the same slots of a FATDMA scheme. In this case the countries need to agree on the time schedule for the alternating in advance.

- The result of the international allocation co-ordination is communicated internationally and appropriately.

- The affected countries are encouraged to do a maximum of co-ordination at planning time; there should not be a large need for run-time exchange of FATDMA scheduling data between countries.

### Step 2: Domestic allocation of slots out of the FATMA schemes for that country

- The national competent authority uses the internationally allocated FATDMA scheme on a primary basis; other competent authorities of that country use individual slots as a subset out of that FATDMA scheme due to allocation by the national competent authority on a secondary basis.

- The national authority represents the complete set of usage requirements (also of other competent authorities) to the international co-ordination (see step 1).

- The VDL management capabilities of the AIS (by AIS base stations) as well as the usage of AIS repeater stations are completely reserved to the national competent authority. They are exclusively considered part of the primary allocation domain.

- The other competent authorities should exclusively use other classes of AIS stations, such as the AIS AtoN station and / or Limited AIS Base Station, the definition of which are currently under development at IEC.

- There should be memorandum of understanding (or some similar document) between the national competent authority and other domestic competent authority stating the distribution of individual slots and their intended usage within the FATDMA schemes assigned to that country by international allocation.

## FATDMA configuration data exchange between Administrations at run-time

The previous chapters have shown that careful considerations between adjacent competent authorities are needed at planning time. Furthermore, a FATDMA set-up may not be changed frequently under „real life“ conditions: coverage planning and statutory rights of the competent authorities have been taken into account to arrive at the optimum FATDMA plan or FATDMA schedule for a given area.

While a theoretical possibility to exchange run-time data on actual FATDMA schedules (or maybe even FATDMA schemes) exists on the level of the ASM , this option should not be employed widely and is discouraged by this Recommendation to be used at all.

Attachment A to Annex C: Algorithm to calculate the FATDMA geographical grid scheme on a global basis

In order to achieve a well defined global geographical distribution an algorithm is used for calculating the position of each individual cell in the geographical grid. Upon input of any geographical position the algorithm will render the number of the FATDMA scheme valid for that geographical position, i. e. the number of the FATDMA scheme used in that cell.

The reference point of the geographical cell would be the [top left] corner for cells on the northern hemisphere, and the [bottom left] corner for cells on the southern hemisphere.

LAT =  (WGS 84 latitude in decimal degrees)

LON =  (WGS 84 latitude in decimal degrees)

// Note: The grid consists of a repeated pattern of 6 x 6 cells, each cell 30 x 30 nautical miles in size.

SingleCellSizeInNauticalMiles = 30;

NoOfSingleCellsAlongOneSideOfMasterCell = 6;

// Note: The repeatable pattern of 6 x 6 cells is denoted a ‘master cell’

MasterCellSizeInNauticalMiles =

SingleCellSizeInNauticalMiles * NoOfSingleCellsAlongOneSideOfMasterCell;
// Note: At Equator, one degree longitude approximately resembles 60 nm i.e. one minute longitude resembles one nautical mile

NoOfMasterCellsAroundEquator = 360 * 60  / MasterCellSizeInNauticalMiles;

//Note: At any latitude, one minute latitude resembles one nautical mile

MasterCellSizeInDegreesLatitude = MasterCellSizeInNauticalMiles / 60.0d;

//Note: abs(x) denotes the absolute value of x; int(x) denotes truncated integer value of x

MasterCellRowNo = int( abs(LAT) / MasterCellSizeInDegreesLatitude );

MasterCellMeanLatitude = (MasterCellRowNo+0.5) * MasterCellSizeInDegreesLatitude

//Note: cos(x) denotes cosine of x in radians

NoOfMasterCellsAroundMasterCellRow = 
int( NoOfMasterCellsAroundEquator * cos( 2 * π * MasterCellMeanLatitude / 360 ) );

SingleCellWidthInDegrees = 
360 / (NoOfSingleCellsAlongOneSideOfMasterCell * NoOfMasterCellsAroundMasterCellRow);

RowNumberInsideMasterCell = 
int ( abs(LAT) * 60 / SingleCellSizeInNauticalMiles ) - NoOfSingleCellsAlongOneSideOfMasterCell * MasterCellRowNo;

// Note: first Southern Hemisphere is assumed, then compensate if Northern Hemisphere

if (LAT>0)

{ RowNumberInsideMasterCell =

NoOfSingleCellsAlongOneSideOfMasterCell - RowNumberInsideMasterCell - 1;

};

ColumnNumberInsideMasterCell = int (abs(LON) / SingleCellWidthInDegrees);

while (ColumnNumberInsideMasterCell > (NoOfSingleCellsAlongOneSideOfMasterCell – 1 ) )

{ColumnNumberInsideMasterCell = 
ColumnNumberInsideMasterCell – NoOfSingleCellsAlongOneSideOfMasterCell

};

// Note: first positive longitude is assumed, compensate if negative longitude (west of Greenwich)

if (LON<0)

{ ColumnNumberInsideMasterCell =
(NoOfSingleCellsAlongOneSideOfMasterCell - 1) - ColumnNumberInsideMasterCell

};

ResultingCellNumber  = 
RowNumberInsideMasterCell * NoOfSingleCellsAlongOneSideOfMasterCell + ColumnNumberInsideMasterCell + 1

Attachment B to Annex C: Absolute slot number and usage category designation for the FATDMA schemes

The following table shows the absolute slot numbers, working channel usage, and usage category designations for the FATDMA schemes.

|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cell Reference |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |
| 1-I | 0 | 1 | 2501 or 7502 | 1251 or 3752 | 1 | 2501 or 7502 | Base Station report* | 0 |
|  | 312 | 1 | 0 or 1125 | 877 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 602 | 0, 1, 2 or 3 | 0 or 1125 | 12 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 1-II | 125 | 1 | 2501 or 7502 | 01 or 5002 | 1 | 2501 or 7502 | Base Station report* | 125 |
|  | 877 | 1 | 0 or 1125 | 312 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 12 | 0, 1, 2 or 3 | 0 or 1125 | 602 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 2-I | 1 | 1 | 2501 or 7502 | 1261 or 3762 | 1 | 2501 or 7502 | Base Station report* | 1 |
|  | 313 | 1 | 0 or 1125 | 878 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 612 | 0, 1, 2 or 3 | 0 or 1125 | 27 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 2-II | 126 | 1 | 2501 or 7502 | 11 or 5012 | 1 | 2501 or 7502 | Base Station report* | 126 |
|  | 878 | 1 | 0 or 1125 | 313 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 27 | 0, 1, 2 or 3 | 0 or 1125 | 612 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 3-I | 10 | 1 | 2501 or 7502 | 1351 or 3852 | 1 | 2501 or 7502 | Base Station report* | 10 |
|  | 314 | 1 | 0 or 1125 | 879 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 619 | 0, 1, 2 or 3 | 0 or 1125 | 37 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 3-II | 135 | 1 | 2501 or 7502 | 101 or 5102 | 1 | 2501 or 7502 | Base Station report* | 135 |
|  | 879 | 1 | 0 or 1125 | 314 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 37 | 0, 1, 2 or 3 | 0 or 1125 | 619 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 4-I | 11 | 1 | 2501 or 7502 | 1361 or 3862 | 1 | 2501 or 7502 | Base Station report* | 11 |
|  | 627 | 1 | 0 or 1125 | 52 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 327 | 0, 1, 2 or 3 | 0 or 1125 | 902 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 4-II | 136 | 1 | 2501 or 7502 | 111 or 5112 | 1 | 2501 or 7502 | Base Station report* | 136 |
|  | 52 | 1 | 0 or 1125 | 627 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 902 | 0, 1, 2 or 3 | 0 or 1125 | 327 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 5-I | 19 | 1 | 2501 or 7502 | 1441 or 3942 | 1 | 2501 or 7502 | Base Station report* | 19 |
|  | 628 | 1 | 0 or 1125 | 53 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 337 | 0, 1, 2 or 3 | 0 or 1125 | 912 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 5-II | 144 | 1 | 2501 or 7502 | 191 or 5192 | 1 | 2501 or 7502 | Base Station report* | 144 |
|  | 53 | 1 | 0 or 1125 | 628 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 912 | 0, 1, 2 or 3 | 0 or 1125 | 337 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 6-I | 20 | 1 | 2501 or 7502 | 1451 or 3952 | 1 | 2501 or 7502 | Base Station report* | 20 |
|  | 629 | 1 | 0 or 1125 | 54 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 352 | 0, 1, 2 or 3 | 0 or 1125 | 887 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 6-II | 145 | 1 | 2501 or 7502 | 201 or 5202 | 1 | 2501 or 7502 | Base Station report* | 145 |
|  | 54 | 1 | 0 or 1125 | 629 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 887 | 0, 1, 2 or 3 | 0 or 1125 | 352 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 7-I | 21 | 1 | 2501 or 7502 | 1461 or 3962 | 1 | 2501 or 7502 | Base Station report* | 21 |
|  | 362 | 1 | 0 or 1125 | 927 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 637 | 0, 1, 2 or 3 | 0 or 1125 | 62 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 7-II | 146 | 1 | 2501 or 7502 | 211 or 5212 | 1 | 2501 or 7502 | Base Station report* | 146 |
|  | 927 | 1 | 0 or 1125 | 362 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 62 | 0, 1, 2 or 3 | 0 or 1125 | 637 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 8-I | 25 | 1 | 2501 or 7502 | 1501 or 4002 | 1 | 2501 or 7502 | Base Station report* | 25 |
|  | 363 | 1 | 0 or 1125 | 928 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 652 | 0, 1, 2 or 3 | 0 or 1125 | 77 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |
| 8-II | 150 | 1 | 2501 or 7502 | 251 or 5252 | 1 | 2501 or 7502 | Base Station report* | 150 |
|  | 928 | 1 | 0 or 1125 | 363 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |
|  | 77 | 0, 1, 2 or 3 | 0 or 1125 | 652 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |

| Default FATDMA schemes / 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 9-I | 26 | 1 | 2501 or 7502 | 1511 or 4012 | 1 | 2501 or 7502 | Base Station report* | 26 |  |
|  | 364 | 1 | 0 or 1125 | 929 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 662 | 0, 1, 2 or 3 | 0 or 1125 | 87 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 9-II | 151 | 1 | 2501 or 7502 | 261 or 5262 | 1 | 2501 or 7502 | Base Station report* | 151 |  |
|  | 929 | 1 | 0 or 1125 | 364 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 87 | 0, 1, 2 or 3 | 0 or 1125 | 662 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 10-I | 35 | 1 | 2501 or 7502 | 1601 or 4102 | 1 | 2501 or 7502 | Base Station report* | 35 |  |
|  | 677 | 1 | 0 or 1125 | 102 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 369 | 0, 1, 2 or 3 | 0 or 1125 | 937 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 10-II | 160 | 1 | 2501 or 7502 | 351 or 5352 | 1 | 2501 or 7502 | Base Station report* | 160 |  |
|  | 102 | 1 | 0 or 1125 | 677 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 937 | 0, 1, 2 or 3 | 0 or 1125 | 369 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 11-I | 36 | 1 | 2501 or 7502 | 1611 or 4112 | 1 | 2501 or 7502 | Base Station report* | 36 |  |
|  | 678 | 1 | 0 or 1125 | 103 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 377 | 0, 1, 2 or 3 | 0 or 1125 | 952 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 11-II | 161 | 1 | 2501 or 7502 | 361 or 5362 | 1 | 2501 or 7502 | Base Station report* | 161 |  |
|  | 103 | 1 | 0 or 1125 | 678 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 952 | 0, 1, 2 or 3 | 0 or 1125 | 377 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 12-I | 44 | 1 | 2501 or 7502 | 1691 or 4192 | 1 | 2501 or 7502 | Base Station report* | 44 |  |
|  | 679 | 1 | 0 or 1125 | 104 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 387 | 0, 1, 2 or 3 | 0 or 1125 | 962 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 12-II | 169 | 1 | 2501 or 7502 | 441 or 5442 | 1 | 2501 or 7502 | Base Station report* | 169 |  |
|  | 104 | 1 | 0 or 1125 | 679 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 962 | 0, 1, 2 or 3 | 0 or 1125 | 387 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 13-I | 45 | 1 | 2501 or 7502 | 1701 or 4202 | 1 | 2501 or 7502 | Base Station report* | 45 |  |
|  | 402 | 1 | 0 or 1125 | 977 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 687 | 0, 1, 2 or 3 | 0 or 1125 | 112 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 13-II | 170 | 1 | 2501 or 7502 | 451 or 5452 | 1 | 2501 or 7502 | Base Station report* | 170 |  |
|  | 977 | 1 | 0 or 1125 | 402 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 112 | 0, 1, 2 or 3 | 0 or 1125 | 687 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 14-I | 46 | 1 | 2501 or 7502 | 1711 or 4212 | 1 | 2501 or 7502 | Base Station report* | 46 |  |
|  | 403 | 1 | 0 or 1125 | 978 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 702 | 0, 1, 2 or 3 | 0 or 1125 | 119 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 14-II | 171 | 1 | 2501 or 7502 | 461 or 5462 | 1 | 2501 or 7502 | Base Station report* | 171 |  |
|  | 978 | 1 | 0 or 1125 | 403 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 119 | 0, 1, 2 or 3 | 0 or 1125 | 702 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 15-I | 50 | 1 | 2501 or 7502 | 1751 or 4252 | 1 | 2501 or 7502 | Base Station report* | 50 |  |
|  | 404 | 1 | 0 or 1125 | 979 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 712 | 0, 1, 2 or 3 | 0 or 1125 | 127 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 15-II | 175 | 1 | 2501 or 7502 | 501 or 5502 | 1 | 2501 or 7502 | Base Station report* | 175 |  |
|  | 979 | 1 | 0 or 1125 | 404 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 127 | 0, 1, 2 or 3 | 0 or 1125 | 712 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 16-I | 51 | 1 | 2501 or 7502 | 1761 or 4262 | 1 | 2501 or 7502 | Base Station report* | 51 |  |
|  | 727 | 1 | 0 or 1125 | 137 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 412 | 0, 1, 2 or 3 | 0 or 1125 | 987 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 16-II | 176 | 1 | 2501 or 7502 | 511 or 5512 | 1 | 2501 or 7502 | Base Station report* | 176 |  |
|  | 137 | 1 | 0 or 1125 | 727 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 987 | 0, 1, 2 or 3 | 0 or 1125 | 412 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

| Default FATDMA schemes / 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 17-I | 60 | 1 | 2501 or 7502 | 1851 or 4352 | 1 | 2501 or 7502 | Base Station report* | 60 |  |
|  | 728 | 1 | 0 or 1125 | 138 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 427 | 0, 1, 2 or 3 | 0 or 1125 | 994 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 17-II | 185 | 1 | 2501 or 7502 | 601 or 5602 | 1 | 2501 or 7502 | Base Station report* | 185 |  |
|  | 138 | 1 | 0 or 1125 | 728 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 994 | 0, 1, 2 or 3 | 0 or 1125 | 427 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 18-I | 61 | 1 | 2501 or 7502 | 1861 or 4362 | 1 | 2501 or 7502 | Base Station report* | 61 |  |
|  | 729 | 1 | 0 or 1125 | 139 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 452 | 0, 1, 2 or 3 | 0 or 1125 | 1002 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 18-II | 186 | 1 | 2501 or 7502 | 611 or 5612 | 1 | 2501 or 7502 | Base Station report* | 186 |  |
|  | 139 | 1 | 0 or 1125 | 729 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1002 | 0, 1, 2 or 3 | 0 or 1125 | 452 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 19-I | 69 | 1 | 2501 or 7502 | 1941 or 4442 | 1 | 2501 or 7502 | Base Station report* | 69 |  |
|  | 462 | 1 | 0 or 1125 | 1012 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 737 | 0, 1, 2 or 3 | 0 or 1125 | 152 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 19-II | 194 | 1 | 2501 or 7502 | 691 or 5692 | 1 | 2501 or 7502 | Base Station report* | 194 |  |
|  | 1012 | 1 | 0 or 1125 | 462 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 152 | 0, 1, 2 or 3 | 0 or 1125 | 737 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 20-I | 70 | 1 | 2501 or 7502 | 1951 or 4452 | 1 | 2501 or 7502 | Base Station report* | 70 |  |
|  | 463 | 1 | 0 or 1125 | 1013 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 744 | 0, 1, 2 or 3 | 0 or 1125 | 162 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 20-II | 195 | 1 | 2501 or 7502 | 701 or 5702 | 1 | 2501 or 7502 | Base Station report* | 195 |  |
|  | 1013 | 1 | 0 or 1125 | 463 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 162 | 0, 1, 2 or 3 | 0 or 1125 | 744 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 21-I | 71 | 1 | 2501 or 7502 | 1961 or 4462 | 1 | 2501 or 7502 | Base Station report* | 71 |  |
|  | 464 | 1 | 0 or 1125 | 1014 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 752 | 0, 1, 2 or 3 | 0 or 1125 | 177 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 21-II | 196 | 1 | 2501 or 7502 | 711 or 5712 | 1 | 2501 or 7502 | Base Station report* | 196 |  |
|  | 1014 | 1 | 0 or 1125 | 464 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 177 | 0, 1, 2 or 3 | 0 or 1125 | 752 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 22-I | 75 | 1 | 2501 or 7502 | 2001 or 4502 | 1 | 2501 or 7502 | Base Station report* | 75 |  |
|  | 762 | 1 | 0 or 1125 | 187 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 437 | 0, 1, 2 or 3 | 0 or 1125 | 1027 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 22-II | 200 | 1 | 2501 or 7502 | 751 or 5752 | 1 | 2501 or 7502 | Base Station report* | 200 |  |
|  | 187 | 1 | 0 or 1125 | 762 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1027 | 0, 1, 2 or 3 | 0 or 1125 | 437 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 23-I | 76 | 1 | 2501 or 7502 | 2011 or 4512 | 1 | 2501 or 7502 | Base Station report* | 76 |  |
|  | 763 | 1 | 0 or 1125 | 188 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 477 | 0, 1, 2 or 3 | 0 or 1125 | 1037 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 23-II | 201 | 1 | 2501 or 7502 | 761 or 5762 | 1 | 2501 or 7502 | Base Station report* | 201 |  |
|  | 188 | 1 | 0 or 1125 | 763 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1037 | 0, 1, 2 or 3 | 0 or 1125 | 477 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 24-I | 85 | 1 | 2501 or 7502 | 2101 or 4602 | 1 | 2501 or 7502 | Base Station report* | 85 |  |
|  | 764 | 1 | 0 or 1125 | 189 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 487 | 0, 1, 2 or 3 | 0 or 1125 | 1052 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 24-II | 210 | 1 | 2501 or 7502 | 851 or 5852 | 1 | 2501 or 7502 | Base Station report* | 210 |  |
|  | 189 | 1 | 0 or 1125 | 764 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1052 | 0, 1, 2 or 3 | 0 or 1125 | 487 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

| Default FATDMA schemes / 4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 25-I | 86 | 1 | 2501 or 7502 | 2111 or 4612 | 1 | 2501 or 7502 | Base Station report* | 86 |  |
|  | 494 | 1 | 0 or 1125 | 1062 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 777 | 0, 1, 2 or 3 | 0 or 1125 | 202 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 25-II | 211 | 1 | 2501 or 7502 | 861 or 5862 | 1 | 2501 or 7502 | Base Station report* | 211 |  |
|  | 1062 | 1 | 0 or 1125 | 494 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 202 | 0, 1, 2 or 3 | 0 or 1125 | 777 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 26-I | 94 | 1 | 2501 or 7502 | 2191 or 4692 | 1 | 2501 or 7502 | Base Station report* | 94 |  |
|  | 495 | 1 | 0 or 1125 | 1063 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 787 | 0, 1, 2 or 3 | 0 or 1125 | 212 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 26-II | 219 | 1 | 2501 or 7502 | 941 or 5942 | 1 | 2501 or 7502 | Base Station report* | 219 |  |
|  | 1063 | 1 | 0 or 1125 | 495 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 212 | 0, 1, 2 or 3 | 0 or 1125 | 787 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 27-I | 95 | 1 | 2501 or 7502 | 2201 or 4702 | 1 | 2501 or 7502 | Base Station report* | 95 |  |
|  | 496 | 1 | 0 or 1125 | 1064 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 802 | 0, 1, 2 or 3 | 0 or 1125 | 227 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 27-II | 220 | 1 | 2501 or 7502 | 951 or 5952 | 1 | 2501 or 7502 | Base Station report* | 220 |  |
|  | 1064 | 1 | 0 or 1125 | 496 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 227 | 0, 1, 2 or 3 | 0 or 1125 | 802 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 28-I | 96 | 1 | 2501 or 7502 | 2211 or 4712 | 1 | 2501 or 7502 | Base Station report* | 96 |  |
|  | 812 | 1 | 0 or 1125 | 237 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 502 | 0, 1, 2 or 3 | 0 or 1125 | 1102 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 28-II | 221 | 1 | 2501 or 7502 | 961 or 5962 | 1 | 2501 or 7502 | Base Station report* | 221 |  |
|  | 237 | 1 | 0 or 1125 | 812 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1102 | 0, 1, 2 or 3 | 0 or 1125 | 502 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 29-I | 100 | 1 | 2501 or 7502 | 2251 or 4752 | 1 | 2501 or 7502 | Base Station report* | 100 |  |
|  | 813 | 1 | 0 or 1125 | 238 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 512 | 0, 1, 2 or 3 | 0 or 1125 | 1087 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 29-II | 225 | 1 | 2501 or 7502 | 1001 or 6002 | 1 | 2501 or 7502 | Base Station report* | 225 |  |
|  | 238 | 1 | 0 or 1125 | 813 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1087 | 0, 1, 2 or 3 | 0 or 1125 | 512 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 30-I | 101 | 1 | 2501 or 7502 | 2261 or 4762 | 1 | 2501 or 7502 | Base Station report* | 101 |  |
|  | 814 | 1 | 0 or 1125 | 239 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 527 | 0, 1, 2 or 3 | 0 or 1125 | 1077 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 30-II | 226 | 1 | 2501 or 7502 | 1011 or 6012 | 1 | 2501 or 7502 | Base Station report* | 226 |  |
|  | 239 | 1 | 0 or 1125 | 814 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1077 | 0, 1, 2 or 3 | 0 or 1125 | 527 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 31-I | 110 | 1 | 2501 or 7502 | 2351 or 4852 | 1 | 2501 or 7502 | Base Station report* | 110 |  |
|  | 537 | 1 | 0 or 1125 | 1113 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 827 | 0, 1, 2 or 3 | 0 or 1125 | 244 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 31-II | 235 | 1 | 2501 or 7502 | 1101 or 6102 | 1 | 2501 or 7502 | Base Station report* | 235 |  |
|  | 1113 | 1 | 0 or 1125 | 537 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 244 | 0, 1, 2 or 3 | 0 or 1125 | 827 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 32-I | 111 | 1 | 2501 or 7502 | 2361 or 4862 | 1 | 2501 or 7502 | Base Station report* | 111 |  |
|  | 538 | 1 | 0 or 1125 | 1114 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 837 | 0, 1, 2 or 3 | 0 or 1125 | 252 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 32-II | 236 | 1 | 2501 or 7502 | 1111 or 6112 | 1 | 2501 or 7502 | Base Station report* | 236 |  |
|  | 1114 | 1 | 0 or 1125 | 538 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 252 | 0, 1, 2 or 3 | 0 or 1125 | 837 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

| Default FATDMA schemes / 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Channel A |  |  | Channel B |  |  | Usage Configuration | Base station report starting slot (CBM sentence) |  |
| Cell Reference |  |  |  |  |  |  |  |  |  |
|  | starting slot | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode | starting slot 1: semaphore mode 2: non – semaphore mode | reservation block size | Increment 1: semaphore mode 2: non – semaphore mode |  |  |  |
| 33-I | 18 | 1 | 2501 or 7502 | 1431 or 3932 | 1 | 2501 or 7502 | Base Station report* | 18 |  |
|  | 552 | 1 | 0 or 1125 | 1112 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 2 | 0, 1, 2 or 3 | 0 or 1125 | 852 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 33-II | 143 | 1 | 2501 or 7502 | 181 or 5182 | 1 | 2501 or 7502 | Base Station report* | 143 |  |
|  | 1112 | 1 or 2 | 0 or 1125 | 552 | 1 or 2 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 852 | 0, 1, 2 or 3 | 0 or 1125 | 2 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 34-I | 43 | 1 | 2501 or 7502 | 1681 or 4182 | 1 | 2501 or 7502 | Base Station report* | 43 |  |
|  | 562 | 1 | 0 or 1125 | 553 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 262 | 0, 1, 2 or 3 | 0 or 1125 | 862 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 34-II | 168 | 1 | 2501 or 7502 | 431 or 5432 | 1 | 2501 or 7502 | Base Station report* | 168 |  |
|  | 553 | 1 | 0 or 1125 | 562 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 862 | 0, 1, 2 or 3 | 0 or 1125 | 262 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 35-I | 68 | 1 | 2501 or 7502 | 1931 or 4432 | 1 | 2501 or 7502 | Base Station report* | 68 |  |
|  | 577 | 1 | 0 or 1125 | 1118 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 277 | 0, 1, 2 or 3 | 0 or 1125 | 869 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 35-II | 193 | 1 | 2501 or 7502 | 681 or 5682 | 1 | 2501 or 7502 | Base Station report* | 193 |  |
|  | 1118 | 1 | 0 or 1125 | 577 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 869 | 0, 1, 2 or 3 | 0 or 1125 | 277 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 36-I | 93 | 1 | 2501 or 7502 | 2181 or 4682 | 1 | 2501 or 7502 | Base Station report* | 93 |  |
|  | 587 | 1 | 0 or 1125 | 993 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 302 | 0, 1, 2 or 3 | 0 or 1125 | 1119 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |
| 36-II | 218 | 1 | 2501 or 7502 | 93 or 593 | 1 | 2501 or 7502 | Base Station report* | 218 |  |
|  | 993 | 1 | 0 or 1125 | 587 | 1 | 0 or 1125 | Data Link Management + general purpose * |  |  |
|  | 1119 | 0, 1, 2 or 3 | 0 or 1125 | 302 | 0, 1, 2 or 3 | 0 or 1125 | General purpose * |  |  |

# Annex D – Example of the Logical AIS Shore Station (LSS) Layer

# Annex D Use Case of the Logical AIS Shore Station (LSS)

This chapter describes the use cases of the Layer of the Logical AIS Shore Stations recognised so far. The use case list constitutes the basis for appropriate description of the functionalities of the use cases.

The chapter is subdivided in accordance with the three different relationships a LSS can exhibit:

- use cases with regard to the Basic AIS Services

- use cases with regard to the mobile AIS stations which are taken care of by a LSS

- use cases with regard to the configuration of the Physical AIS Shore Stations assigned to a LSS

## Use Cases of the Logical AIS Shore Station with regard to Basic AIS Services

These use cases can be further subdivided into five (5) different categories in accordance with their principle working mechanism.

### BAS-related Use Cases Category I

These use cases share the following common characteristic: The data flow direction is from mobile to fixed. The use cases are implicitly triggered by AIS data input received from the Physical Shore Station layer, which then is processed and forwarded to higher levels. These use cases perform their function upon a trigger event and return to standby until the next trigger event, only.

Status

Configuration

Figure 52.1

Data to client(s)

LSS

data input =

implicit trigger event

Status

Configuration

Use Cases of this category:

The generation of the following BAS constitutes the use cases of this category:

A_STAT  (without any application-specific message processing)

A_DYN

A_VOY 	(without any application-specific message processing)

B_DAT

SAR_DAT

ATON_DAT

SAFE_AD   	(information flow direction mobile-to-fixed)

SAFE_BR   	(information flow direction mobile-to-fixed)

TRANS_IAI 	(information flow direction mobile-to-fixed)

TRANS_RAI 	(information flow direction mobile-to-fixed)

These use cases may also have a diagnostic functionality for individual mobile AIS stations.

### BAS-related Use Cases Category II

These use cases share the following common characteristic: The data flow direction is from fixed to mobile. The use cases are implicitly triggered by AIS data input received from higher levels, which then is processed and forwarded to lower levels, i.e. the Physical Shore Station layer. These use perform their function upon a trigger event and return to standby until the next trigger event, only.

Status

Configuration

Figure 52.2

Data input from client(s) =

implicit trigger event

LSS

Status

Configuration

processed data

Use Cases of this category:

The generation of the following BAS constitutes the use cases of this category:

SAFE_AD   		(information flow direction fixed-to-mobile)

SAFE_BR   		(information flow direction fixed-to-mobile)

TRANS_IAI  		(information flow direction fixed-to-mobile)

TRANS_RAI  	(information flow direction fixed-to-mobile)

DGNS_COR

### BAS-related Use Cases Category III

These use cases share the following common characteristic: These use cases are triggered expressively by a trigger information input received from higher levels (trigger event). This triggers a use case, which assembles a request for certain data, which then is processed and forwarded to lower levels, i.e. the Physical Shore Station layer. Upon receipt of the answer to the data request, the data is processed to be forwarded to higher levels (to the originator of the trigger event). These use cases perform their function upon a trigger event and return to standby until the next trigger event, only. The total information flow direction is fixed-mobile-fixed.

Figure 52.3

Status

Configuration

LSS

Requested Data to Client(s)

Trigger

Information

Status

Configuration

Use Cases of this category:

The generation of the following BAS constitutes the use cases of this category:

INT_TDMA   	(information flow direction fixed-mobile-fixed)

INT_DSC  	(information flow direction fixed-mobile-fixed)

### BAS-related Use Cases Category IV

These use cases share the following common characteristic: The use cases are triggered expressively by a trigger information input received from the Physical AIS Shore Station layer. This constitutes the trigger event. It triggers a use case, which assembles the reply for certain data, which then is processed and forwarded to lower levels, i.e. the Physical Shore Station layer, to be forwarded to the originator of the trigger event. An indication on the request as such may be given to higher levels. These use cases perform their function upon a trigger event and return to standby until the next trigger event, only. The total information flow direction is mobile-fixed-mobile.

Figure 52.4

Status

Configuration

## LSS

Requested

data

Indication

Status

Configuration

Use Cases of this category:

The generation of the following BAS constitutes the use case of this category:

INT_TDMA   (information flow direction mobile-fixed-mobile)

Note: The BAS INT_DSC is allowed only for the information flow direction fixed-mobile-fixed.

### BAS-related Use Cases Category V

These use cases share the following common characteristic: These use cases are triggered expressively by a trigger information input received from higher levels (trigger event). This triggers a use case, which, based on the configuration input from higher levels, generates and forwards certain AIS VDL management commands to the PSS layer. Upon receipt of the (implicit) answers to these management commands, they will be evaluated by the use case, and indications may be forwarded to higher levels (to the originator of the trigger information).

These use cases by default exhibit a continuously and autonomously behaviour, but one-shot behaviour may also be achieved by appropriate trigger information input. The total information flow direction is fixed-mobile-fixed.

Figure 52.5

Status

Configuration

## LSS

Indication to

Trigger Source

Trigger Information

Status

Configuration

Use Cases of this category:

The generation of the following BAS constitute the use cases of this category:

BASE_DAT

ASGN_RATE

ASGN_SLOT

CH_TDMA

CH_DSC

PWR_LEV

FATDMA

## Other Use Cases with regard to BAS

In addition, there are the following use cases of LSS layer with regard to BAS:

- Format transformation of VDL Message format to BAS format (and vice versa)

- Fault detection and management of with regard to the BAS

- Provide MMI for technical operation / maintenance of LSS layer

## Use Cases with regard to the mobile AIS stations which are taken care of by a LSS

While the above use cases are defined in regard to an BAS, the following use cases are defined in regard to regularly more than one BAS for one or more individual mobile AIS stations. The following Use Cases with regard to the mobile AIS have been identified:

- Management of the type-of-mobile-filter functionality of the LSS layer: This use case defines the mix of relevant type-of-mobiles for a given LSS. Types of mobiles could be Class A Shipborne Mobile AIS stations, Class B Shipborne AIS stations, SAR Airborne AIS Stations, and AtoN AIS Stations. 
Higher levels may configure "their" LSS such, that this LSS forwards only data of relevant types-of-mobiles to them. 
Example: An application (higher level) may be interested only in data from Class A and Class B shipborne mobile AIS stations. This application would then configure "its" LSS such, that only Class A and Class B data would be forwarded, but not data from AtoN or SAR Airborne AIS stations. The latter would have been received however, and would possibly be used by different LSS and their associated applications.

- Management of the type-of-ship-filter functionality of the LSS layer: This use case defines the mix of relevant type-of-ships for a given LSS. Types of ship could be determined by evaluating the appropriate data field in Class A and Class B Shipborne Mobile AIS stations reports. 
Higher levels may configure "their" LSS such, that this LSS forwards only data of relevant types-of-ships to them. 
Example: An application (higher level) may be interested only in data from ships identifying themselves as "passenger ships". This application would then configure "its" LSS such, that only data from "passenger ships" would be forwarded, but not data from any other type of ship. The latter would have been received however, and would possibly be used by different LSS and their associated applications.

- Management of the type-of-AtoN-filter functionality of the LSS layer: This use case defines the mix of relevant type-of-AtoN for a given LSS. Types of AtoN could be determined by evaluating the appropriate data field in AtoN AIS station reports. 
Higher levels may configure "their" LSS such, that this LSS forwards only data of relevant types-of-AtoN to them.

- Management of the assignments of responsibilities of a PSS and/or fixed AIS station for a given mobile AIS station:  In particular, but not exclusively, for any information flow direction fixed-to-mobile, it is necessary to determine, which PSS and/or even fixed AIS station would be "responsible" for transmitting data from any BAS to that mobile AIS station. Since mobiles regularly move through the coverage areas of many fixed AIS stations and / or PSS, the assignment of responsibilities must be dynamic. The assignment would be based on the mobile's VDL-related identity.

- Geographical Filtering for information flow direction fixed-to-mobile: Which PSS would be responsible for outgoing data?

- Geographical Filtering for information flow direction mobile-to-fixed: Which LSS would be responsible for data received by a fixed AIS station / PSS?

- Remove duplicate data for information flow direction mobile-to-fixed

- Reduce data rate forwarded by this LSS to higher levels for information flow direction mobile-to-fixed: Upon configuration from higher levels, this use case would filter the data rate forwarded from a given LSS to its associated application. This use case would be employed when an application would have no need of data from mobiles at the rate the data is transmitted by that mobile on the AIS VDL. As a benefit, the reliability of the data flow regularly increases by employing this use case. Note: There is a similar use case defined at the PSS layer, for a situation, when a fixed AIS station / PSS would be sitting at a location with insufficient AIS data transfer network capacity to the associated LSS.

- Fault detection and management of with regard to above use cases

- Provide MMI for technical operation / maintenance of above use cases

## Use Cases with regard to the configuration of the PSS assigned to a LSS

The following represent a class of use cases, completely internal to the shore-based AIS Service. They are relevant for the management of the assignment of LSS and PSS.

- Management of the assignments of regularly more than one PSS to a given LSS

- Configuration management of individual PSS assigned to a given LSS within the margin of configuration rights given to that LSS by the ASM

- Fault detection and management of with regard to above use cases

- Provide MMI for technical operation / maintenance of above use cases

# Annex E – Example of functionality of the AIS Service Management

# Annex E Use Cases of the AIS Service Management (ASM)

## Use Cases of the ASM with regard to the management of the BAS assignment to (individual) LSS

The common characteristics of these use cases are such, that they manage the responsibilities of any and all LSS in particular with regard to the internal BAS.

In some cases, the internal BAS control critical features of the VDL. In this case, the ASM must ensure, that responsibilities assigned to different LSS do not interfere with each other. This would mean in some cases, that only one LSS should be given the right to employ certain internal BAS.

The following use cases in this category have been identified:

- Management of the assignment of internal BAS to LSS

- LSS selection based on geographical and possibly other, configurable criteria for incoming data from higher levels for information flow direction fixed-to-mobile

- Frequency management for the whole of the AIS Service: This use cases controls the use of the internal BAS CH_TDMA, CH_DSC, PWR_LEV by LSS layer.

- Management of the AIS Service-wide slot reservation scheme using internal BAS: FATDMA

- Management of the AIS Service-wide assignment functionalities of mobiles using internal BAS: ASGN_RATE, ASGN_SLOT

- Management of the AIS Service-wide distribution of DGNSS corrections: DGNSS corrections are provided by some radio navigation service, e.g. IALA beacon reference stations, outside the AIS Service proper. This use case manages the distribution of the correction data input to the appropriate LSS. It is using the internal BAS DGNS_COR

- Management of identity of the LSS, PSS and / or fixed AIS station: Instead of their own pre-configured MMSI fixed AIS stations may use different MMSI, which may be provided by applications on higher layers and / or by a LSS and / or by a PSS. This use case manages the use of different MMSI’s within the AIS Service

- Diagnostics toolbox for individual AIS mobile stations

- Diagnostics and failure management of entities within the AIS Service

- Fault detection and management of with regard to above use cases

- Provide MMI for technical operation / maintenance of above use cases

## Use Cases with regard to the configuration of any or all individual LSS except BAS

The common characteristics of these use cases are, that they manage the existence and configuration of any or all individual LSS.  Following use cases have been identified:

- Creation of an individual LSS

- Termination of an individual LSS

- Configuration management of an individual LSS with regard to its configurable parameters except BAS-related parameters

- Provide MMI for technical operation / maintenance of all LSS

- Fault detection and fault management for LSS functionality except for BAS-related functionality

- Tools for planned maintenance at LSS level, such as "Garbage Collection"

## Management of the assignments within the AIS Service

The common characteristics of these use cases are, that they all manage the assignments between individual functional entities within the AIS Service:

- Overall management of the assignments of LSS to the / more than one AIS Service

- Overall management of the assignments of PSS to LSS

- Overall management of the assignments of  fixed AIS stations to PSS

- (Supplementary) management functionality of the virtual AIS data transport network within the overall network of the competent authority

- Perform basic configuration of individual PSS upon their initialisation

- Perform basic configuration of individual fixed AIS stations upon their initialisation

- Fault detection and fault management for assignments within the AIS Service

- Provide MMI for technical operation / maintenance of above use cases

## Initialisation and Termination of the AIS Service

The following use cases have been identified:

- Initialisation of the AIS Service as a whole, including set-up of the minimum number of LSS for internal functions of the AIS Service

- Termination of the AIS Service as a whole.

1 The Base Station without control functions has limited internal BAS functions.

The value 2250 should not be used, as it implies a once per frame transmission; the value zero results in the same.

Means to avoid interference could include low, directional antennas and taking advantage of local land topography.

When acting as a semaphore, the reporting rate of a base station increases to a higher update rate. Therefore more FATDMA reserved slots would be needed and the timing requirements for transmissions of Base Station Reports in semaphore mode would be much more stringent. Therefore, this case is being dealt with at FATDMA usage category 2 (“High timing requirements slots” in the section below.

Name change currently considered in revision process of Recommendation ITU-R M.1371-1.