Paper on IALA Response to ITU

IALA (along with IMO and IEC) was invited by ITU (in WRC-07 Resolution 357) to contribute to studies for future use of the spectrum, considering the future requirements envisioned by ITU (Florida2009-Output02b) (compare also eNAV 5/7/3). IALA may consider responding to ITU in terms of IALA’s e-Navigation vision and strategy which should address the issues identified by ITU. To help facilitate this response to ITU and to help identify and address future needs and requirements, a draft “e-Navigation Master Plan” is included for consideration at e-NAV6.

This is not intended to overtake the orderly decision making process concerning satellite detection of AIS. The response to ITU should be for purposes stated by ITU as a matter of urgency because:

- IALA has strong position to ITU.

- The WRC-11 preparation is now underway and needs IAIA input.

- WRC-11 Agenda Item 1.10 is relevant to e-Navigation.

Its purpose is to examine the frequency allocation requirements with regard to operation of safety systems for ships and ports and the related regulatory provisions, in accordance with Resolution 357(WRC-07);

- Resolution 357 resolves:

1. that WRC-11 consider amendments to provisions of the Radio

Regulations necessary to provide for the operation of ship and

port security and maritime safety systems;

2. that WRC-11 consider additional allocations to the maritime

mobile service below 1 GHz to support the requirements

identified in resolves 1.

3. that WRC-11 consider additional allocations to the maritime

mobile-satellite service in frequency bands allocated to the

maritime mobile service between 156 and 162.025MHz to support

the requirements identified in resolves 1,

- This agenda item includes RFID, MSI broadcasting, AIS satellite

detection and VHF data communication.

- Therefore, WRC-11 can revise Appendix 18 channel arrangement/usage

and may add additional spectrums for VHF data communications and

also for AIS satellite detection.

- Thus, IALA should prepare and submit IALA position to WRC-11 on this

agenda item for better progress on both issues.

Subject Matter Addressed in the Attachments:

  - Contribution to draft e-Navigation Master Plan (Florida2009-Output 02), including the following attachments:

    - Explanation of WRC-11 Agenda Item 1.10 (Florida2009-Output02a)

    - WRC-07 Resolution 357 (Florida2009-Output02b) (compare also eNAV 5/7/3)

  - Satellite Detection of AIS

    - draft new ITU report (Improved Satellite Detection of AIS) (Annex 5 to ITU-R WP5B Chairman’s report of the October 2008 meeting) (Florida2009-Output06)

    - Consequentially required revision to Recommendation ITU-R M.1371-3 Annex 4 (Florida2009-Output07)

  - VHF Data Service

    - Draft Revision (to add Annexes 3 and 4) to Recommendation ITU-R M.1842 (ITU Document 5/117, 7 November 2008) (Florida2009-Output08)

    - MARINTEK Report (IMO-ITU EG 4-5-7 Annex) (Florida2009-Output09)

    - Report ITU-R M.2122 (2007) (Florida2009-Output10)