| INTERNATIONAL MARITIME ORGANIZATION | #### IMO | E |
| --- | --- | --- |

Ref.:  T2-OSS/1.4	30 January 2009

LIAISON STATEMENT TO ITU-R WP 5b, IALA, IEC TC 80 and CIRM

AUTOMATIC IDENTIFICATION SYSTEM (AIS)

SEARCH AND RESCUE TRANSMITTER (AIS-SART)

1	IMO would like to thank ITU-R Working Party 5B for the liaison statement (Document 5B/TEMP/50(Rev.1)) concerning the proposed text message within message 14 to identify the modes of the AIS SART active and test, as well as the navigational status to be used by this device when activated.

2	The Sub-Committee on Radiocommunications and Search and Rescue (COMSAR), at its thirteenth session (19 to 23 January 2009), noted, in relation to the proposed text message within message 14, that ITU-R WP 5B finds both suggestions acceptable.  The Sub-Committee has decided to stay with its initial position that the messages should be as simple as possible; therefore the following wording should be used for the Broadcast Safety Related Message 14:

.1	For the active SART, use the text “SART ACTIVE”.

.2	For the test mode, use the text “SART TEST”.

3	The COMSAR Sub-Committee also considered the ITU-R WP 5B suggestion relating to the navigational status in messages 1, 2 and 3.  The Sub Committee believes that the reserved number 14 should be activated for the use by AIS-SART.

The message used should define the navigational status as “AIS-SART” when the device is activated.  The Sub-Committee further agreed that when the device is under test the navigational status should be set to 15 “NOT DEFINED”.

4	Action required:

.1	ITU-R is invited to consider the decisions taken by the Sub-Committee when the next version ITU-R M.1371 is prepared;

.2	IALA is invited to consider the decisions taken by the Sub-Committee by upgrading the IALA technical clarifications to ITU-R M.1371;

.3	IEC TC 80 is invited to consider the decisions taken by the Sub-Committee for the relevant test standards using such technology;  and

.4	CIRM is invited to inform their members about the decisions taken by the Sub-Committee.

____________