| From: e-NAV Committee |  |
| --- | --- |
| To: IMO MSC | March 12, 2009 |

Liaison Statement to IMO

IALA Technical Clarifications on AIS Navigation Status considering the Advent of Navigation Light Controllers and Foreseen Equipment Interfacing

- Introduction

At the 53rd Session of the IMO Navigation Sub-committee performance standards were developed for navigation lights, navigation light controllers (NLC) and associated equipment; subsequently adopted at the 83rd Session of the IMO Marine Safety Committee.  (Resolution MSC.253(83) (adopted on 8 October 2007) Adoption of the performance standards for navigation lights, navigation light controllers and associated equipment.)

- Discussion

In their report to MSC, the Sub-committee recognized the need for standardized serial interface for navigation lights controllers to enable them to communicate with other marine navigation and communication systems (i.e. AIS and VDR) and invited the IEC to develop a suitable interface; the NLC should have a bi-directional interface to transfer alarms to external systems and receive acknowledgements of alarms from external systems; and, that these interface should comply with the relevant international standards, such as IEC 61162.

ITU-R M1371 AIS Message 1, Navigation Status field should faithfully reflect the ships navigation status at all times; and, mirror what is reflected in the navigation shapes and lights being displayed per the COLREGS.  Unfortunately, mariners tend to neglect the proper upkeep of this date field; something, which IALA has brought to the attention of Council and IMO.  Automated upkeep via an NLC would mitigate this problem.

- Recommendation

In order to ensure this automation is possible and done correctly, it is imperative that all understand what a given AIS Navigation Status, particularly in the context of the COLREGS and thus the display of lights and shapes.  To that end, the IALA proposes amending the Navigation Status parameter of AIS Message 1, 2, and 3 as follows:

Navigation Status

0 = under way using engine (Rule 23(a)) or Rule 25(e)), 1 = at anchor (Rule 30(a-c)), 2 = not under command (Rule 27(a)), 3 = restricted manoeuvrability (Rule 27(b-h)), 4 = constrained by her draught (Rule 28), 5 = moored, 6 = aground (Rule 30(d)), 7 = engaged in fishing-trawling (Rule 26(b)), 8 = under way sailing (Rule 25), 9 = engaged in fishing other than trawling (Rule 26(c)), 10 = air-cushion vessel in non-displacement mode or WIG craft taking off, landing or in flight (Rule23(b-c)),  11 = power-driven vessel towing astern (Rule 24(a)), 12 = power-driven vessel pushing ahead or towing alongside (Rule 24(b)), 13= in distress or requiring assistance (Rule 37),  14 = AIS-SART, seeking to attract attention (Rule 36), 15 = not defined = default

Citing the specific COLREG Rules with an associated navigation status will ensure mariners understanding of the navigation status parameter and facilitate the development of NLC’s and their applicable standards.

The following is brought to IMO’s attention because of the addition of ‘seeking to attract attention’ to status 14 = AIS-SART; be done so to meet the intent of IMO to equate status 14 with AIS-SART, yet, we provide some amplifying detail that comes from COLREG Rule 36.  Also, note, we have added status ‘13 = in distress or requiring assistance’, fully recognizing that AIS is not a distress alerting device nor should be relied upon as the primary means of informing SAR authorities of a distress situation; however, there might be a circumstance(s) when mariners will need to avail themselves of all means available to signal distress or inform others of their need for assistance; this provides an effective means of them doing so.  Finally, please note, that IEC 61993-2 Class A Test Standard is currently in maintenance status, which affords an opportunity to amend the standard with these changes and thus ensure that 2nd generation AIS Class A units broadcast a navigation status that accurately reflects that status of a ship and to the lights or shapes shall be displayed.

IMO NAV53/INF.10 - AIS  Incorrect Transmisisons