e-Navigation Committee

6th Session										9 March 2009

Developments in GNSS

1.	Introduction

This Rapporteur’s briefing is provided for e-NAV, based on information published in journals and on websites.

2.	GPS

There are 31 operational satellites in the constellation Around 12 of these are operating without redundancy in one or more of their essential systems. Deployment of replacement satellites is still held up by problems with the launch vehicle and the next launch is now expected in March 2009. An experimental L5 signal is planned to be available from this satellite (IIR-M7). This is significant as there is an ITU deadline of August 2009 for securing the frequency, which had been expected to be met by the launch of IIF satellites. The six IIR-M satellites in orbit have been broadcasting the L2C signal, without data or modulation, but newly uploaded software will allow them to provide the new signal with a message structure, which has several advantages over L1.

The next generation GPS IIIA has undergone a successful Integrated Baseline Review, allowing the development and production of the first two satellites with a planned launch in 2014.

3.	GLONASS

19 satellites are in operation and one in maintenance, the three launched on 25 Dec 2008 having now been commissioned. A further 6 are expected to be deployed in two launches this year. The Russian Space Agency (Roscosmos), has declared that GLONASS accuracy will approach that of GPS from 2010 and that the constellation will increase to 30 satellites by 2011.  It is claimed that the present constellation gives 83% global coverage. The new GLONASS K satellites will start launching in 2010 and CDMA signals on L1 & L5 are promised.

4.	GALILEO

Positive results are being claimed from the Galileo test satellites GIOVE-A and B, with a signal availability of 99.8% from GIOVE-A, over nearly two years. GIOVE-B underwent successful in-orbit testing last July, but was taken out of service for two weeks in September, because of high levels of space radiation. Prime contractors have been short-listed for manufacturing the space and ground segments for the operational system. The first operational satellites are due for launch in 2010, with Full Operational Capability scheduled for 2013.

5.	COMPASS

The PR of China system Beidou/Compass is currently in a demonstration phase with 3 GEO satellites. Ten satellites are expected to be launched in the next two years to provide PNT capability in the Asia-Pacific region.

The constellation is eventually planned to comprise 5 geo-synchronous and 30 MEO satellites, providing 10 services, 5 of them open, with a standalone accuracy of 10 m and a real-time wide area differential service giving 1 m. Completion date has recently been announced as 2015.

6.	QZSS

This system is under development by the Japan Aerospace Exploration Agency and will be interoperable and compatible with GPS, providing signals at L1, L1C, L2C and L5, including a sub-meter augmentation with integrity function. Coordination meetings have been held with the EU Galileo signal task force to ensure RF compatibility and efforts are also being made to secure RF compatibility with Compass.

7.	SBAS

The WAAS Performance Standard has been published, defining the signal in space characteristics, navigation message and performance requirements. The service, provided for aircraft approach, is designed to give 16 m horizontal and 20 m vertical accuracy, with 6.2 sec time to alert.

The Safety of Life service of EGNOS is undergoing qualification testing, with certification and declaration of “entry into service” planned for the second half of 2009. Once this service achieves certification under the EU’s Single European Sky regulations it will be available for use by the aviation sector and for maritime and other transport sectors. An operational Signal in Space has been available for the past year and the Open Service should be declared usable shortly. Receivers currently available on the consumer market for leisure craft already use the open signal.

The Japanese MSAS has been commissioned for aviation use, with two MTSATs operating. The Indian GAGAN system has carried out test transmissions with one Inmarsat 4 satellite.