| System | Band | Frequency Channel | Bandwidth Data rate | Status | Life Span | Ownership | Mode | Service | Purposes | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Distress and safety communication within GMDSS |  |  |  |  |  |  |  |  |  |  |
| MF/HF Voice | MF/HF | 2182 kHz 4125 kHz 6215 kHz 8291 kHz 12290 kHz 16420 kHz | 3 kHz | Current | Long | Regional and International | Analogue voice. | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Distress communication | Long distance, > 250 nm |
| MF/HF DSC | MF/HF . | 2187.5 kHz 4207.5 kHz 6312.0 kHz 8414.5 kHz 12577 kHz 16804.5 kHz | 0.5 kHz 100 bps | Current | Long | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Distress alerting | Long distance > 250 nm |
| 121.5 DF | VHF. | 121.5 MHz |  | Current | Short | International | Carrier (Analogue) | Mobile to mobile. | Location. | Line of sight |
| VHF DSC | VHF. | 156.525 MHz (Ch 70) | 25 kHz 1200 bps | Current | Long | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Distress alerting | Line of sight |
| VHF voice | VHF . | 156.300 MHz (Ch 06) 156.650 MHz (Ch 13) 156.800 MHz (Ch 16) | 25 kHz | Current | Long | International | Voice | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Distress communication | Line of sight |
| Two way VHF | VHF . | 156.025 to 161.950 MHz. | 25 kHz | Current | Long | International | Voice | Mobile to mobile. | On scene communication | Line of sight |
| AIS SART | VHF | 161.975 MHz, 162.025 MHz | 25 kHz 9600 bps/ TDMA | Near future | Long | International | digital | Mobile to mobile | Homing | Line of sight |
| EPIRB; | UHF | 406 MHz |  | Current | Long | International | Digital | Mobile to satellite | Distress alerting | COSPAS-SARSAT Satellite; Global converge |
| Satellite INMARSAT C, B, F | UHF | Tx 1626.5 to 1646.5 MHz. Rx 1525.0 to 1545.0 MHz. |  | Current | Long | 3rd party | Digital. Voice and data. | Satellite to earth. Earth to satellite. | Distress alerting, distress communication. | Global converge |
| RADAR SART, x-Band | SHF | 9.2 – 9.5 GHz |  | Current | medium | International | analogue | Mobile to mobile | Homing | Line of sight |
| Maritime safety information promulgation |  |  |  |  |  |  |  |  |  |  |
| NAVTEX | MF/HF | 518 kHz 490 kHz and 4209.5 kHz (for local language) | 0.5 kHz | Current | Medium | International | Text | Fixed to mobile. | Reception of maritime safety information. | Long distance, > 250 nm Broadcast to mobile only. |
| Narrow band direct print. (NBDP) | HF. | . 4210 kHz 6314 kHz 8416.5 kHz 12509 kHz 16806.5 kHz 19680.5 kHz 22376.0 kHz 26100.5 kHz | 0.5 kHz | Current | Medium | International. | Text | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Reception of maritime safety information | Long distance, > 250 nm |
| EGC | UHF | Tx 1626.5 to 1646.5 MHz. Rx 1525.0 to 1545.0 MHz. |  | Current | Long | international | Digital data. | Satellite to mobile | Reception of maritime safety information (Safety Net) | Global coverage |
| Safety of navigation |  |  |  |  |  |  |  |  |  |  |
| Loran and e-Loran | LF | 100 kHz |  | Current | Very Long | Regional | Carrier (Analogue) | Fixed to mobile. | Positioning | Long distance |
| DGNSS | MF | 285 to 325 kHz | 50/100 bps | Current | Medium. | Regional. | Digital | Fixed to mobile. | Augmentation of positioning | Medium distance |
| AIS | VHF | 161.975MHz (AIS 1) 162.025MHz (AIS 2) | 25 kHz 9600 bps/ TDMA | Current | Long | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Collision avoidance, Environmental protection, Security, VTS, Augmentation of positioning, communication for navigational and safety related purposes. | Line of sight |
| AIS ATON | VHF | 161.975MHz (AIS 1) 162.025MHz (AIS 2) | 25 kHz 9600 bps/ TDMA | Current | Long | International | Data (Digital) | ATON (fixed/floating) to mobile/shore | Navigational aid | Line of sight |
| VHF voice | VHF Mobile. | 156.025 to 161.950 MHz. | 25 kHz | Current | Long | International | Voice | Mobile to mobile. Fixed to mobile. Mobile to fixed. | communication for navigational and safety related purposes | Line of sight |
| GNSS | UHF | GPS: 1227.6 MHz 1575.42 MHz GLONASS: Compass Galileo |  | Current Current Future Future | Long Long Long Long | International International Regional International | Digital | Satellite to earth | Positioning | Global coverage |
| LRIT | Satellite | Tx 1626.5 to 1646.5 MHz. Rx 1525.0 to 1545.0 Mhz. |  | Future | Short | 3rd party | Data (Digital) | Inmarsat | Security. Vessel tracking. Environmental protection. | Global coverage |
| Radar S Band. |  | 2.9 to 3.1 GHz |  | Current | Long | International |  | Mobile and fixed | Collision avoidance, Navigational aid. | Line of sight |
| Radar X Band |  | 9.2 to 9.5 GHz |  | Current | Long | International |  | Mobile and fixed. | Collision avoidance,. Navigational aid | Line of sight |
| RACON |  | 2.9 to 3.1 GHz and 9.2 to 9.5 GHz |  | Current | Long | International |  | Fixed. | Navigational aid | Line of sight |
| General communication |  |  |  |  |  |  |  |  |  |  |
| HF Voice | MF/HF | 1.6 to 26.5 MHz | 3 kHz | Current | Long | Regional and International | Analogue voice. (ALE) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General voice communication. | Long distance, > 250 nm |
| TOR/SITOR | MF & HF. | 1.6 to 26.5 MHz. | 0.5 kHz | Near Dead | Short. | Regional. | Text | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General text communication. | Long distance, > 250 nm |
| Narrow band direct print. (NBDP) | MF & HF. | 1.6 to 26.5 MHz. | 0.5 kHz | Current | Long for GMDSS. Short for general communication. | International. | Text | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General text communication. . | Long distance, > 250 nm |
| HF Voice | MF/HF | 1.6 to 26.5 MHz | 3 kHz | Current | Long | Regional and International | Analogue voice. (ALE) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General voice communication. | Long distance, > 250 nm |
| HF Data | MF/HF | 1.6 to 26.5 MHz | 0.5 – 3 kHz | Current | Long | Regional and International | Digital data. | Fixed to mobile. Mobile to fixed. | General data communication. | Long distance, > 250 nm |
| VHF voice | VHF MMBe | 156.025 to 161.950 MHz. | 25 kHz | Current | Long | international | Voice | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General voice communication | Line of sight |
| VHF data | VHF MMBe | 156.025 to 161.950 MHz. | 25 kHz | Current | Long | regional | data | Mobile to mobile. Fixed to mobile. Mobile to fixed. | General data communication. | Line of sight |
| Satellite | UHF | Tx 1626.5 to 1646.5 MHz. Rx 1525.0 to 1545.0 MHz |  | Current | Long | global 3rd party | Digital. Voice and data. | Satellite to earth. Earth to satellite. | General voice and data communication. | Global coverage |
| GSM | UHF |  |  | Current | Long | regional 3rd party | Digital. Voice and data. | Cellular. Mobile-fixed-mobile. | General voice and data communication | Medium range in locality |
| GPRS | UHF |  |  | Current | Long | regional 3rd party | Digital. Voice and data. | Cellular. Mobile-fixed-mobile. | General voice and data communication | Short range in locality |
| UMTS/3G | UHF |  |  | Current | Long | regional 3rd party | Digital. Voice and data. | Cellular. Mobile-fixed-mobile. | General voice and data communication | Short range in locality |
| IEEE 802.11 (WiFi) | UHF/ SHF | 2.4 – 2.5 GHz 5.17 – 5.33 GHz 4.9 GHz | 11 Mbps 54 - 300 Mbps 54 Mbps | Current | Long | local 3rd party | Digital data | LAN Peer-to peer | Internet access | Sort in locality < 100m <10 km |
| IEEE 802.16 (WiMax) | UHF/ SHF | 2.5 GHz 3.3 GHz | 75 Mbps | Current | Long | local 3rd party | Digital data | LAN Peer-to peer | Internet access | Short range in locality < 50 km |
| IEEE 802.15.4 (ZigBee) | UHF | 868-870 MHz 902 – 928 MHz 2.4 GHz | 20 kbps 40 kbps 250 kbps | Current | Long | local 3rd party | Digital data |  |  | Short range in locality < 30 |
| Future development |  |  |  |  |  |  |  |  |  |  |
| HF Digital data. | HF | 4 to 26 MHz. | 10 to 20 kHz. | Near Future | Long term | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Binary data communication for navigational and safety related purposes. | KENTA (France) Has range advantage over VHF 40 to 250 NM (>250 sky wave). |
| AIS 3rd Channel | VHF | TBD | 25kHz 9600bps TDMA | Near Future | Long term | International | Data (Digital) | Mobile to satellite. | Security. Environmental protection. Vessel tracking. GMDSS (AIS EPIRB) | Closely related to current AIS. Same transponder on ship |
| AIS 4th Channel | VHF | TBD | 25kHz 9600bps TDMA | Near Future | Long term | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Binary data communication for navigational and safety related purposes. Security and tracking. | Closely related to current AIS. Same transponder on ship |
| AIS MMB channel management | VHF | AIS 1 and AIS 2 | 25kHz 9600bps TDMA | Near Future | Long term | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | To manage data transfer over channels in the MMB. | Eg. “go to channel XX to get chart update for Tokyo Bay. |
| VHF Digital data. | VHF Mobile. | 156.025 to 161.950 MHz. | 25 to 100 kHz. See note 3. | Near Future | Long term | International | Data (Digital) | Mobile to mobile. Fixed to mobile. Mobile to fixed. | Binary data communication for navigational and safety related purposes. | ITU 1842 SC 123 Managed or autonomous systems. |

Notes

- MMB = Maritime Mobile Band.

- Move to change MMB bandwidth to 6 ¼ or 12 ½ kHz.

- Table of performance for VHF data.

| Bandwidth kHz | Data rate kbps | Modulation technique |
| --- | --- | --- |
| 25 | 43 | Π/4 DQPSK |
| 50 | 156 | 16 QAM |
| 100 | 397 | 16 QAM |