AIS Message 5 & use of internal GNSS (USA)

AIS Message 5 – Types of Electronic Positioning Device

ITU-R M.1371 ‘Message 5 - Ship static & voyage related data’ includes a parameter for the ‘Type of electronic position fixing device’ being used for positioning reporting by the AIS station. This 4 bit field is described as follows:

0 = undefined (default)

1 = GPS

2 = GLONASS

3 = combined GPS

4 = Loran-C

5 = Chayka

6 = integrated navigation system

7 = surveyed

8 = Galileo,

9-15 = not used

IEC 61993-2, ‘Sec. 6.2.1 - Source of AIS positioning reporting’ allows for the use of the internal GNSS as a source of AIS position reporting.

Current version of 1371 does not account for the use of the internal GNSS receiver as a source for AIS position reporting, i.e. ‘type of electronic positioning device’.

This matter should be clarified as to whether Message 5 should broadcast a: 1 = GPS, 0 = undefined or a value of 9-15 to denote that neither of the device(s) 0 - 8 was used as the positioning source.  A better solution would be to create a value for ‘internal GNSS’, i.e. 9 =  Internal GNSS; thus properly reflect the position source used by the AIS station.

We invite to Committee to take this into consideration in IALA Technical Clarifications on ITU-R M.1371.