e-Navigation Committee						e-NAV6/9/1

6th Session								9 March 2009

eLoran Standardisation

This review of the standardisation process required for eLoran has been prepared by the Vice Chair to assist the Committee in planning its work on this topic.

Standardisation Plan for e-Loran (v9)

Organisations & Status of Documentation

# ILA – International Loran Association

The ILA published an eLoran Definition Document in 2007 and this is the starting point for preparing performance standards and an interface control document. The GAUSS group, which was set up to develop standards for Loran, is now a working group of the ILA and is preparing a draft receiver standard for eLoran.

# RTCM – Technical Standards

RTCM has set up a Special Committee (SC-127) for eLoran and it is expected that a new RTCM performance standard will be developed, including a final signal specification and differential correction characteristics. The draft receiver standard being developed by the GAUSS group can be expected to end up in SC-127 and could then go to IEC.

# ITU – Technical Characteristics & Spectrum Requirements

Recommendation M.589-3 (08/01), already provides technical characteristics for data transmission and interference protection. It may need revision and the addition of an annex giving more detail of the modulation system to be adopted.

# IMO – Performance Standards & WWRNS Recognition

IMO Resolution A.818(19) contains performance standards based on long out-of-date technology. A new resolution will be needed before a World Wide Radio Navigation System (WWRNS) submission can be made. The first step will be to draft an input to the Maritime Safety Committee (MSC) proposing that the subject be put on the agenda for the sub-committee on Safety of Navigation (NAV). Alternatively it may be possible to deal with it under the e-Navigation task, if it is accepted that eLoran is a necessary component of e-Navigation. In either case the earliest it is likely to be considered is NAV 56 in mid-2010, assuming an input is made to MSC 87 (late 2009) with approval possible by MSC 89 at the end of 2010.

# IEC – Equipment Test Standards

IEC document 61075 Ed. 1.0 (1991-07) “Loran-C receivers for ships – Minimum performance standards, methods of testing and required test results” is based on an RTCM standard from the 1980s and is not relevant to modern Loran. It was proposed for reconfirmation in late 2008, with its next revision date in 2011. A new work item proposal will need to be submitted to IEC TC80 (next meeting 8/9 Sept. 2009) to develop an eLoran standard and support from several members will be needed to make it a task for a Working Group. The WG would need the new IMO Performance Standard to be in place, but could draw on the work already carried out by RTCM SC-127.

# IALA – Operational Standards for Service Providers, Database of stations & GRIs

Operational standards for eLoran could be developed by a task group within the Technical WG of the IALA e-NAV Committee, drawing on NELS, FERNS and USCG documentation. This would require the support of several members of the Committee, but could start during e-NAV 6 in March 2009 and be completed at e-NAV 7 in September 2009. The result could be an IALA Recommendation similar to the Performance and Monitoring standard for DGNSS (R-121). The database of transmitting stations and GRIs will also need to be updated.

Suggested Timeline for eLoran Standardisation Process