The IALA e-Navigation Committee

Contribution to draft e-Navigation strategic master plan

1	Introduction 2

2	Drivers for the implementation of e-Navigation strategy 2

The upcoming WRC-11, the necessity to defend frequency allocations and the necessity to preserve frequencies for e-Navigation purposes 2

The agreed introduction of the ECDIS as a mandatory carriage requirement 2

3	1st and 2nd generation (1G, 2G) maritime digital technologies to be replaced by e-Navigation compliant 3rd generation (3G) maritime digital technologies eventually 3

1G maritime digital technology exist 3

2G maritime digital concepts/technologies exist 3

New demands leading to 3G maritime digital concepts/technologies 3

4	“Shopping list” for 3G maritime VHF voice/data communication service 4

5	“Shopping list” for AIS (V.2) – e-Navigation-compliant AIS 4

6	General legacy strategy 5

7	The final situation on the VHF maritime mobile service band 5

8	Conclusion for work programme of international organizations involved 7

9	Resulting “big” international working projects 7

10	Most pressing next steps 7

11	Attachment 7

# Introduction

IMO has developed a general e-Navigation strategy, which was approved by IMO/MSC85 (Nov/Dec 2008). The future e-Navigation concept should be essentially user-requirement driven, but there are certain key areas of functionality, such as voice and data communications, vessel identification and tracking, where the core set of user-requirements are known from the existing systems. These user-requirements may receive a capability and capacity expansion in the process but the fundamental requirements will remain the same as presently known.

Hence, for the sake of the development of a more refined strategic master plan, i.e. to provide a global co-ordinating framework for the variety of specialized working areas, the following considerations may be appropriate.

# Drivers for the implementation of e-Navigation strategy

## The upcoming WRC-11, the necessity to defend frequency allocations and the necessity to preserve frequencies for e-Navigation purposes

ITU has developed a framework for e-Navigation related topics to be discussed at WRC 2011, by virtue of Agenda item 1.10 and WRC-07 Resolution 357. Refer to attachment.

It is essential to have in place, in proper time before WRC-11, strong requirement language from IMO justifying the need for improved frequency allocations for communication services. Such services in the maritime mobile frequency band supporting voice and data transmissions in general, and for a new AIS (V.2) in particular, must be established to cater for current requirements, and anticipated needs due to the implementation of IMO’s e-Navigation strategy.

Attention is drawn to the fact that there are parties external to the maritime community on the move to “share” essential portions of the maritime VHF mobile service band. Such “sharing” may seriously limit the maritime availability of the frequencies for an indefinite time period into the future. These proposals can only be countered if there are sufficiently substantiated proposals by IMO for the future use of the frequency band, both for the near and the more distant future.

## The agreed introduction of the ECDIS as a mandatory carriage requirement

ECDIS (S-57-based as default, ideally S-100-based) is being introduced as mandatory carriage requirement in accordance with IMO agreed phase-in schedule (2012 – 2018). ECDIS would provide the long-awaited fully graphical shipboard user presentation interface to display not only charts but a larger variety of data objects, including data from AIS and other sensors.

Obviously, the phase-in schedule of mandatory ECDIS equipment may set the pace for the (mandatory) introduction of other new and/or revised maritime digital technologies within the context of e-Navigation and the e-Navigation at large. It is important that the opportunity is not missed to establish target dates for the introduction of other technologies, so that they are ready in time for the ECDIS phase-in schedule.

# 1st and 2nd generation (1G, 2G) maritime digital technologies to be replaced by e-Navigation compliant 3rd generation (3G) maritime digital technologies eventually

## 1G maritime digital technology exist

- such as VHF-DSC component of the GMDSS

- they have been built on the technological concepts of the 1960/1970s, involving sophisticated operation of equipment both shipboard and ashore

## 2G maritime digital concepts/technologies exist

- such as present AIS based on M.1371-3, henceforth called AIS (V.1) for the purpose of this document,  and LRIT.

- The AIS (V.1) was designed to be essentially a hands-off system, involving only the minimum of user interaction for its routine operation.

- AIS (V.1), i.e. present AIS, has proved to be a very powerful tool for various applications in the field of navigation, however the functionalities designed into the present AIS (V.1) have not yet fully been explored, namely the various transmission capabilities.

- AIS (V.1) has already taken on some functionalities for GMDSS/SAR:

  - general purpose includes SAR (cf. M.1371 preamble),

  - dedicated SAR aircraft AIS station,

  - number of persons on board data transmission required by some authorities,

  - certain manufacturers include “distress button” for Class B AIS devices, and

  - AIS SART station, application specific messaging for SAR purposes,

  - Appendix 15 RR (WRC-07) defines the frequencies used for GMDSS, and it includes the frequencies AIS1/2 used by AIS SART stations.

- Is there any operational functionality in present VHF-DSC spec (M.493), which could not be fulfilled by amended AIS? (Hence, leading to AIS (V.2), see below, and replacing the present VHF DSC eventually)

## New demands leading to 3G maritime digital concepts/technologies

- Need for global tracking of ships led to the creation of both LRIT and AIS-by-satellite detection; AIS (V.1) was not built to cater for satellite tracking support. Certain improvements in the present AIS (V.1), a 2G digital technology, while leaving the core of the present technology core of the AIS untouched, would lead to AIS (V.2), a 3G digital technology.

- Presently there is only partial compliance with OSI Open System technology design principles. For example the present shipboard AIS equipment comprises basically all OSI levels, including the application level (notably existence and limitations of the MKD as an integral part of the AIS device). The confusion regarding the definition of application specific messages in the international maritime community adds to that.

- 3G maritime digital VHF voice/data system proposals are being discussed at ITU but the set of operational requirements taking into account the concepts of e-Navigation is missing

- To fulfill user-requirements, system design should be information and functionality led rather than technology led. It is the information transmitted (even if contained in a transparent container such as voice data), which is of relevance to the users.

- The Universal Maritime Data Model (UMDM)/Universal Data Object Identifier (UDOI) have not been currently adopted by any existing concept/technology (analogue/1G/2G). However the universal maritime data modelling is an essential key element of IMO’s e-Navigation strategy. Hence, 3G technologies must be required to comply with the UMDM / UDOI.

# “Shopping list” for 3G maritime VHF voice/data communication service

- Complete functional replacement of the present analogue VHF voice communication service, i.e. ship-ship/ship-shore/shore-ship

- Reliable ship-ship communications without the need to route voice/data via shore or space based system

- Virtual voice/data channels (visible to users) operating on physical channels; decoupling of virtual voice/data channel (management) from physical frequency channel (management)

- Higher data transfer capacity

- “Common root” for management of the 3G maritime services by using VHF Ch 70, using AIS messaging format (not DSC messaging format) (considering the co-existence in accordance with Recommendation ITU-R M.822)

- UMDM compliant

# “Shopping list” for AIS (V.2) – e-Navigation-compliant AIS

- much of the present AIS (V.1) can be retained, i.e. essentially, the core functionalities of the M.1371; some features of the present AIS are too confined to cater for an e-Navigation compliant AIS (e.g. some message compositions)

- OSI-level compliant equipment design, i.e. completely “modem-only” like functionality and display left to separate shipboard device(s) (e.g. new ECDIS).

- Improved set of messages:

- Replacement for some old messages, when they have proved to be not supporting proper data modelling (e.g. static ship data)

- Reduction of tailored messages to essentially needed messages (“statements of the AIS device of its own”), while increasing the proportion of the transparent data container messaging (“passing information of other components/entities”): properly designed application specific message realm (e.g. also “safety related texts” would be some application specific messages)

- New flexibility for applications

- Functionality to frequency mapping:

- Two internationally dedicated frequencies (with improved location in the band and improved protection) reserved for exchange of navigational data proper

- One or two internationally dedicated frequencies for exchange of application specific messages, only (ship-ship, ship-shore, shore-ship)

- application specific message transmission shore-ship using regionally assigned frequencies from a candidate list, subject to run-time channel management

- These measures will significantly reduce the complexity of FATDMA co-ordination

- One or two internationally dedicated frequencies to support AIS satellite detection

- Complete management of all frequencies used by AIS (V.2) by using Ch 70 (with AIS messages, not DSC messages) (considering the co-existence in accordance with Recommendation ITU-R M.822)

- Absorbs all of the operational functionality (after an evaluation by today’s  user-requirements) of the VHF-DSC (Ch 70, M.493)

- Implementation of AIS satellite detection will absorb the operational functionality of the LRIT

# General legacy strategy

- Of course, the analogue VHF data communication will be replaced by the 3G maritime digital VHF voice/data service, eventually.

- Old technologies systems (1G / 2G) will remain in place for a period of time after the complete introduction of the new technologies; a plan for phasing out these old technologies will be developed together with the plan for phase-in of the new technologies; assets used by 1G / 2G technologies, such as dedicated frequencies, will be assigned to the new technologies eventually.

- New technologies (3G) will not be technologically backwards compatible in order not to stifle their capabilities from the outset (design). Rather separate assets, in particular frequencies, will be used from the outset.

- Where there is one common root in the VHF maritime mobile band needed to manage and control the new digital technologies, VHF Ch 70 will be used by sharing in a legacy friendly way (i.e. friendly to VHF-DSC Ch 70).

# The final situation on the VHF maritime mobile service band

After the completion of the phase-out, the VHF maritime mobile service band will look as follows:

- global system perspective: On the global scale only two systems remain in the VHF maritime mobile band

- AIS (V.2)

- New maritime digital VHF voice/data service

- i.e. no VHF-DSC and no AIS (V.1) and no analogue voice communications

- frequency management perspective:

- larger international dedicated frequency portion assigned to AIS (V.2) for international use as described above

- some frequencies assigned for AIS (V.2) regional use

- larger international dedicated frequency portions assigned to new maritime digital VHF voice/data service

- common root for management of both AIS (V.2) and new maritime digital VHF voice/data service at Ch 70, using AIS TDMA message format (not DSC encoding)

- some frequencies assigned to local use (for specific technologies)

- strong requirements for (largely exclusive) ownership of the maritime mobile band by the maritime community confirmed

# Conclusion for work programme of international organizations involved

Using the phase-in plan of IMO for the ECDIS as a common time master/pace maker will have beneficial effects for all working areas involved: It will focus their work on the common goal and will require proper timing of individual supporting projects.

# Resulting “big” international working projects

- Detailed international (normative) description of user-requirements based on evaluation of the user-requirements which can be derived / “re-engineered” from existing systems:

- Which were the original user requirements?

- Are they still valid / satisfactorily?

- Definition of the UMDM/UDOI

- Definition of the e-Navigation architecture

- Definition of the AIS (V.2): Revision (supported by drafting at IALA) of the IMO Performance Standard for AIS

- Frequency planning at WRC-level: URGENT need to prepare positions and input to WRC-11

- Incorporation of the AIS (V.2) and the VHF voice/data system in GMDSS as replacement for DSC technology in VHF maritime mobile band

- Satellite detection of AIS fully developed

- Definition of the shore-based e-Navigation architecture to cater for new services and integrate heritage services

- …

# Most pressing next steps

- Revision of Appendix 18 RR at WRC-11 that support the above vision:

- Refining the frequency allocations for AIS (V.2) including AIS satellite detection capabilities;

- designation of four Appendix 18 duplex channels for the new digital maritime VHF data exchange service (candidate technology described in  Rec. ITU-R M.1842, Annex 4);

- designation of at least two Appendix 18 duplex channels for the new digital maritime VHF voice service (candidate technology described in Rec. ITU-R M.1842, Annex 3);

- Initial quantitative assessment of required throughput capacity for the new digital maritime data exchange / voice service.

- Develop a draft refined integrated working plan for consideration by the relevant international bodies involved and thereby identify the critical path(s)

# Attachment

The highly relevant agenda item 1.10 of the WRC-11 and the WRC-07 Resolution 357 is brought to the attention of the e-Navigation Committee as attached.