19 January 2009

Kurt Schwehr’s  Comments as posted on his BLOG site: 
http://schwehr.org/blog/archives/2009-01.html#e2009-01-19T23_10_49.txt

### “Reasons why not to have the same message in multiple DAC/FI locations”

Last week at the IALA AIS working group, one of the vendors pointed out that messages are appearing with multiple DAC/FI combinations. For example, if a competent authority tells a developer to use 366/54, and then the message is used in a different location, the developer will use for example, 303/54 to transmit the same message content. This causes two problems that I can think of: 

First, if a Electronic Chart System (ECS) is setup to work with the first 366/54 and the vendor is not aware of 303/54 being the same message content, then the vessel will not recognize the message when it transits to Alaska. This is adding extra complexity to every ECS/ECDIS system that might end up in multiple locations on the Earth. 

Second, this reuse drastically collapses the space of available messages from over 17 thousand to something like 120 to 250. That is implying that for a particular area, there is only a DAC of 1 for international IMO messages, a local DAC, and possibly a 3rd or 4th regional DAC that can be used in a particular area. 

There is no reason not to reuse the same DAC/FI else where in the world. If you are a competent authority, I strongly urge you to reuse other binary application messages from other DACs and keep the original DAC/FI. If the message, was originally 366/54, use 366/54 no matter where you are in the world. You can authorize the use of transmitting that DAC/FI pair in your part of the world. 

If you see that someone else has a great message, by all means use it, but please don't change the DAC/FI. To quote the ITU R1371-3 specification for AIS (P. 58, Annex 5, 1.2):

It is recommended that the administrator of application specific

messages base the DAC selection on the maritime identification digit

(MID) of the administrator's country or region. It is the intention

that any application specific message can be utilized worldwide. The

choice of the DAC does not limit the area where the message can be

used.

[emaphasis added] 

So if you are creating a new type of message, put it in your DAC. If you are using an existing message, do not put it in another DAC. 

Putting a DAC in a message should not specify where in the world the transmitter is located. Location can be determined based on other AIS messages and/or the MMSI of a known station.

Posted by Kurt | Permalink

## 01.19.2009 23:10