Draft IALA Recommendation 
e-Nav 201

on

Shore-based e-Navigation system architecture

Working towards Edition 1.0

2009-03-05

IALA Recommendation Preamble

THE COUNCIL

RECALLING that one of the aims of the Association is to foster safe, economic and efficient movement of vessels and the protection of the environment through the improvement and harmonisation of aids to navigation, vessel traffic services and other means world-wide.

NOTING

- IALA Recommendation e-Nav 101 on the e-Navigation architecture – the shore-based perspective [1]

- IMO’s strategic plan regarding e-Navigation; [2]

- IMO has expressed an interest in the contribution of IALA to the work on e-Navigation; [3]

RECOGNISING that the e-Navigation architecture will assist in the development and maintenance of peer-to-peer applications between ship and shore, shore and ship and between shore and shore, particularly with respect to the international harmonisation and international standardisation, in the following fields

- technical shore-based e-Navigation services;

- technical means for communication;

- data modelling and referential data;

- Human Machine Interface presentations;

RECOGNISING ALSO the responsibilities of IALA National Members regarding international and/or regional e-Navigation applications:

- that the shore-based e-Navigation systems of National Members are embedded into local, national, regional and global topologies of systems;

- that there are data/information flow chains within the local, national, regional and global topologies of systems which support local, national, regional, and global e-Navigation applications;

- that each shore-based e-Navigation system contributes in a specific way to those data/information flow chains;

- that the disruption of this contribution would have an effect on the e-Navigation application supported;

- that the shore-based e-Navigation system in itself consists of components, which support the data/information flow chain through that system;

- that therefore certain requirements regarding quality parameters, such as accuracy, reliability, continuity and others, will need to be taken into account when designing the common shore-based e-Navigation system architecture;

RECOGNISING FURTHER that the common shore-based e-Navigation system architecture laid out in this Recommendation will assist National Members in the efficient deployment of their services to the mariner and to the maritime community by

- allowing the shore-based system of a National Member to participate in the harmonised data exchange ship-shore and shore-ship;

- easing the integration of standardised technical services and components;

- creating a global market for those standardised technical services and components;

- ensuring maintenance- and development-friendly architecture of the technical services and components;

- improving the efficiency in organisational terms by exploiting commonalities;

- improving the life cycle management;

- harmonising data exchange to other shore-based stakeholders’ systems (local, national, regional, global);

- complying with the anticipated performance requirements from IMO, i.e. rendering the shore-based systems “e-Navigation compliant”;

- obtaining cost benefits from the above bullets;

RECOMMENDS

that National Members and other appropriate authorities providing marine aids to navigation services adopt the e-Navigation design and implementation principles set out in the Annex to this recommendation.

ANNEX

# Revision history

| # | Date | Description | Initials |
| --- | --- | --- | --- |
| V 0.1 | 20.Dec 2007 | Version 1.00 | JHO |
| V 0.2 | 20.Feb 2008 | Version 1.01 (as agreed in Dec. 2007 meeting) | JHO |
| V 0.3 | 17. Apr 2008 | Version 1.03 (Norrköping Meeting; April 2008) | Arch TWG |
| V 0.4 | 22. Oct 2008 | Version 1.06 (St-Germain-en-Laye Meeting; Oct 08) | Arch TWG |
| V 0.5 | 03. Mar 2009 | Version 1.10 (Kiel meeting; March 09) | Arch TWG |

# Table of Contents

Revision history	4

Table of Contents	5

1	Introduction	6

2	The relationship between different shore-based systems within the context of e-Navigation	8

2.1	Local, national, regional, and global topologies for shore-based systems	8

2.2	Necessary Inter-System data exchange as a resulting consequence	10

2.3	The common shore-based system architecture for IALA National Members’ systems	10

2.4	The place of the common shore-based e-Navigation system within the overarching e-Navigation system architecture	11

3	Goals for the common shore-based e-Navigation system architecture	13

4	Fundamental system architecture design principles	14

5	Common shore-based system architecture general layout	15

6	Shore-based e-Navigation system architecture detail layout	19

7	Consequences and Options	25

7.1	Consequential requirements for the common shore-based e-Navigation system and its components	25

7.2	The usefulness of the common shore-based eNAV system architecture as a reference framework	25

7.3	Options for different administration’s systems setup	25

8	Introduction to case studies as examples how to use the common shore-based e-Navigaton system layout	26

9	References	26

10	Glossary	26

11	Abbreviations	27

Appendix 1: Example for Application Specific AIS messaging as applied to common shore-based e-Navigation system architecture	28

Appendix 2: the e-ANSI concept applied to common shore-based e-Navigation system architecture	28

Appendix 3: the GNSS anomaly detection via AIS and handling applied to common shore-based e-Navigation system architecture	28

- Introduction

This Recommendation builds on “IALA Recommendation e-NAV 101 on the e-Navigation Architecture - the Shore-based Perspective” [4].

[The relative place for this Recommendation can be found in the “IALA Recommendation e-Nav 0000 on IALA Documentation Scheme”.]

The Recommendation e-Nav 101 presented the general shore-based perspective on the e-Navigation concept, IALA’s mandate, and on the shore systems in general. This Recommendation focuses the attention on the shore side of the overall e-Navigation architecture in accordance with the mandate of IALA. This is illustrated by the shaded area in Figure 1 on the following page. The next chapter will discuss the relationship between different shore-based systems within the context of e-Navigation.

In following chapters this Recommendation then will turn towards the design and implementation principles of the common shore-based e-Navigation system deployed a by an IALA National Member in the following topics:

- Goals

- Fundamental design considerations

- General layout

- More detailed layout

- Consequences and options

This common shore-based e-Navigation system layout will be employed for sample applications mapped to the system layout in the final chapter and Annexes.

IBS

World Wide Radionavigation System (WWRS) of IMO, including GNSS, GNSS augmentation and terrestrial backup

Appli-cation 1

Application 2

Application 3

Application 4

Link technology (encapsulated)

Transceiver station

Ship’s

sensors

Shipboard

Applications

INS

shore based

components

Application-to-

application

(peer-to-peer)

functional

connection

Physical

Link (e.g. radio link)

other

ships

other

ships

mariner

Shore-based

users

Shore-based e-Navigation systems

Figure 1: 
The e-Navigation 
architecture and 
the common shore-based 
system

- The relationship between different shore-based systems within the context of e-Navigation

  - Local, national, regional, and global topologies for shore-based systems

Along the maritime and inland waterways there are different shore-based systems operating giving services to the mariners and the shore-based users. Those different shore-based systems should interact with the systems on board of ships and other shore-based systems. This can be on a local, national, regional or global scale.

- Local systems: Local systems provide services to users in geographically confined areas of waterways. Their main goal is to serve the mariner in that confined area.

- National systems: National systems provide services to users of appropriate waterways in their respective countries.

- Regional systems: Regional systems provide services to users in a specific region of the world. Such a system could for instance serve the St-Lawrence-Seaway, the whole Baltic area, the Malacca Strait, as a region.

- Global systems: Global systems provide services on a world-wide scale.

Every system comprises a set of components which provide the specific services for the different users in their appropriate area of responsibility.

In principle, every system may interact with any other system on whatever scale, i.e. the connectivity of any given system is not limited by architectural considerations.

From the individual user’s perspective, both onboard and ashore, the UN/IMO concept of a “single window” (refer to e.g. the work of IMO’s Facilitation Committee) will be beneficial because a “single window” would avoid administrative overhead. In order to support this concept, the systems involved need to interact accordingly and exchange data to the other stakeholders involved. This can be achieved by distributed systems on the various scales involved (local, national, regional, and global), which are operated by independent authorities: “Single window” does not imply “single system”.

The next figure illustrates the above systems communicate with each other and with the mariner. The lines show a substantial sample of the possible relations between systems and mariners. In order for these relationships to work efficiently there needs to be commonality of the data objects and their format.

An IALA National Member generally operates and maintains one of the national systems (compare highlighted national system in Figure 2). Hence, an IALA National Member needs to take into account the required connectivity to the other shore-base systems operated and maintained by other stakeholders (compare e.g. Annex 2 of [2] for a list).

Figure 2: Topology of different kind of shore-based systems

Figure 2 also provides an example of a vessel interacting with shore-based authorities using a “single window” provided by the national system No.3. National system No. 3 would responsible to disseminate the data required by the other shore-based systems indicated by the dotted rectangle, i.e. national system No. 4 and regional system No. 2.

  - Necessary Inter-System data exchange as a resulting consequence

For the shore-based systems to work in an efficient way, i.e. increasingly automated, standardized data exchange between those systems is necessary. Standardized data exchange between the shore-based systems will result in a more consistent and reliable system interaction and data.

Standardized data exchange will also minimize the burden on the mariner for giving information to the shore-based authorities and will ensure more reliable and more complete information about shipping.

Standardized data exchange requires both a standardized data model and standardized data exchange formats.

A standardized data model describes the data exchanged by using data type definitions. The standardized data model ideally is one universal data model including all kind of data types in the maritime world, which is called Universal Maritime Data Model (U.MDM). Also, each and every data item of that universal model should be identifiable by a unique data object identifier, which is called Universal Data Object Identifier (U.DOI).

There are several data encoding options to exchange the same data item of the U.MDM by using different data exchange formats, each of which is tailored to a specific need.

On top of that there are several data transmission technology options available to transmit the encoded data, each of which is also tailored to a specific need.

The U.MDM/U.DOI concept, the data encoding options and the data transmission technologies are not within the scope of this Recommendation. They will be addressed in different IALA Recommendations.

  - The common shore-based system architecture for IALA National Members’ systems

The shore-based systems of IALA National Members are built to serve similar purposes and to perform similar tasks in particular in the realms of Aids-to-Navigation and VTS/VTM. Therefore, already today they have similar architectures. By standardizing the architecture of IALA National Members’ shore-based systems benefits could be gained.

An additional impact is created by the globalization of the system architecture by the e-Navigation concept. E-Navigation will require as a key strategic element internationally harmonized system architecture.

Therefore, it follows that IALA National Members’ shore-based systems should have a common architecture, which would comply with the e-Navigation concept.

Hence, the focal point of this document is the description of that architecture, which is called “common shore-based e-Navigation system architecture”.

  - The place of the common shore-based e-Navigation system within the overarching e-Navigation system architecture

As a consequence of the above discussion, the Figure 3 on the next page shows the resulting place of the common shore-based e-Navigation system of IALA National Members within the overarching e-Navigation system architecture. Also, Figure 3 shows the

- interaction of shore-based operators (shore-based users) with mariners (shipboard users) in peer-to-peer applications (i.e. operational services), which are facilitated by the interaction of the shipboard technical equipment with the shore-based technical system;

- potentially user-independent, automated interaction of the shore-based technical system with the shipboard technical equipment;

- relationship between the shore-based system and its shore-based users, indicating that user requirements need to be fulfilled by appropriate technical services at its appropriate interfaces.

- interaction with other shore-based systems, named “third party” systems from the point of view of the National Member’s system, both for giving and receiving data;

- existence of so-called technical e-Navigation services as one building block of the common shore-based e-Navigation system as a forward reference to following chapters.

Figure 3: Common shore-based e-Navigation system architecture as embedded in the overarching e-Navigation system architecture and in relation to user-requirements

- Goals for the common shore-based e-Navigation system architecture

By implementing and adopting the common shore-based e-Navigation architecture the following possible benefits can be achieved. Many of them have been specifically identified also in the IMO e-Navigation strategy [2].

- Quality benefits:

- user-required functionality to the various users (including accuracy, integrity, reliability, continuity, and latency)

- demonstration of “e-Navigation compliancy” to stakeholders by means of appropriate certification for service level achievements

- application of objective criteria for each technical e-Navigation service provided

- same level of service for different users with similar requirements

- extensive usage of applicable international technical standards

- minimization of response time for fulfilment of new user requirements

- Cost related benefits:

- Full life-cycle cost evaluation facilitated by the identification of cost items, both for investment and running costs, which are known in advance

- Cost reductions for planning and development by maximum usage of applicable international standards and extensive re-use of engineering concepts

- Cost reductions for updating existing services and functionalities due to (rapid) change of technology

- Cost reductions for implementing further or changed services and/or functionalities

- Investment cost reduction: a) reduction by cost per unit due to standardization; b) reduction in numbers of units needed due to better utilization of the same kind of unit (down side: potentially increased single point of failure, hence effective mitigation strategies needed)

- Running cost reduction over full life cycle, taking also into account cut in staffing level: a) remote access/control as opposed to only-localized access/control resulting in consequential savings of working time and travel; b) automation; c) easier replacement of individual components

- Organisation and staff related benefits:

- Maintenance and widening of the expertise of technical personnel

- Achievement of common level of understanding between different, but interacting authorities, who both employ the common shore-based e-Navigation system architecture

- Optimization of the business processes and organizational procedures involved based on the use of the common shore-based e-Navigation system architecture

- potentially optimization of organizational structure

- Improvement of training efficiency for technical personnel due to the transfer of knowledge from the general (i.e. the understanding of the generic model) to the specific technology under consideration (i.e. individual technical e-Navigation service);

- Public relations and societal benefits:

- Contribution to an improved public relation image of the administration by use of an internationally accepted, advanced system architecture

- Ease of public access to information by e.g. publicly accessible information portals provided by the administration, while the administration simultaneously has the benefit of a standardized method of presenting that information to the public

- Success in achieving the IMO stated goals for improving the safety and efficiency of navigation, protection of the environment and security [2].

All the above benefits are critical to the success in achieving the IMO stated goals for the e-Navigation concept.

- Fundamental system architecture design principles

To achieve the above goals the following fundamental principles for the common shore-based e-Navigation system architecture are introduced:

- User-requirement driven system design: Only clearly and consistently stated user requirements result in technical service provided.

- Shift from technology-orientation to information-orientation while designing the system layout: All technical solutions based on data modelling as a rule.

- Introduction of the Object-Oriented Paradigm (OOP) for all technical aspects.

- Employ the principles of modularity and encapsulation, while preserving a holistic view of the system’s intended functionality.

- Application of a uniform engineering model for all technical e-Navigation services provided by the system, regardless of technology, thus exploiting commonality.

- Implementation of functional specifications, based on international standards to the largest extent possible: Procurement of technical solutions based on functional specifications as a rule.

- Open system architecture and focus on open and standardized interfaces between components and services adhered to: No proprietary interfacing as a rule.

- Employment of remote access techniques where feasible at all in orders to allow for minimal number of technical operation and maintenance centres: Components without remote access capabilities as a rule not permissible (the access capabilities must allow for open interfacing, too).

- Implementation of life-cycle management in order to prevent “quick-fix-solutions” with huge long-term costs associated: Full life cycle coverage of technical proposals required for their approval.

- Documentation of each and every functional aspect in a uniform, overall documentation system: This is a pre-requisite for any quality management system, which in turn would support concepts such as certification in general, IT security certification in accordance with ISO 27000 series, and also the VIMSAS (Voluntary IMO Member State Audit Scheme).

- Separation between technical operation and maintenance tasks and personnel on one hand (no authorisation to change functionality and/or structure of component) and system development and optimization tasks and personnel on the other hand (authorisation to change functionality and/or structure).

- Common shore-based system architecture general layout

The main building blocks of the common shore-based e-Navigation system architecture are the so-called technical e-Navigation services. The common shore-based e-Navigation system architecture describes the technical set-up of the shore-based system of an IALA National Member. Hence, any individual technical e-Navigation service contributing to the shore-based e-Navigation system is also a technical entity.

Its qualities are as follows: A technical e-Navigation service encapsulates all primary functionalities dealing with a specific technology or with a specific user, depending on the kind of the technical e-Navigation service. All technical e-Navigation services of the common shore-based e-Navigation system architecture adhere to the same object-oriented engineering model. The object-oriented model is described in a different IALA Recommendation. All technical e-Navigation services are self-contained and provide for themselves all capabilities needed for their tasks, including their own service management.

Figure 4 provides a structural overview representation of the common shore-based e-Navigation system architecture.

It should be noted, that the common shore-based e-Navigation system architecture is modelled in a client-server-fashion; the individual technical e-Navigation services can regularly assume either roles, i.e. be “clients” or “servers”, depending on their present role in a given interaction chain to support a given application-to-application data exchange within the overarching e-Navigation architecture.

Figure 4: common shore-based e-Navigation system architecture (overview)

In Figure 4 there are given some individual technical e-Navigation services (i.e. the User Interaction Service and the Gateway Service) and some groups of individual technical e-Navigation services (i.e. Data Collection and Data Transfer Services, Value Added Data Processing Services).

Basically all technical e-Navigation services shown in Figure 4 do what is implied by their name:

- The “Data Collection and Data Transfer Services” are a group of technical e-Navigation services interfacing the shore-based e-Navigation system to the physical links towards the traffic objects, the natural environment and the waterways as such. For example, the AIS Service, the Radar Service, the Visual Aids-to-Navigation Services belong to this group. Some operate bi-directionally (e.g. the AIS Service), while others operate uni-laterally on their appropriate physical links.

- The “Value Added Data Processing Services” also are a group of individual technical e-Navigation services. Their main task is to add value to (raw) data by processing, combination, comparison etc., store data and information and provide it upon request to other technical e-Navigation services.

- The “User Interaction Service” – a single individual technical e-Navigation service – is specialised to provide the Human-Machine-Interface to the primary users of the shore-based e-Navigation system, i.e. such users who are supported directly by the e-Navigation system via dedicated displays, keyboards and other human interaction devices.

- The “Gateway Service” – another individual technical e-Navigation service – is specialised on data exchange shore-to-shore. Mainly it interfaces to external systems of “third parties”, which provide data needed within the own system or which request relevant data to be forwarded to them. The “Gateway Service” can also interface different shore-based systems locally, regionally, and globally.

From Figure 4 also one further important conclusion can be derived: It is the combination of the technical e-Navigation services in their system-wide interaction, which provides the required functionality of the whole of the system to its users.

Figure 5 on the following page still provides a structural overview, but adding a bit more detail, as will be explained below.

-Radio Transmission -

Satellit

Radio Navigation / GNSS

SB

Atmosphere

Waterway

Acoustical

Transmission

General Shipping

Commercial Shipping

Yachts

Inland Waterway Barges

Ships operated by an administration

Buouy Tender

Law Enforcement Vessels

Navy Ships

Search And Rescue Vessels

Pilot Boats

Etc.

Passenger

Crew

Applications

Equipment

Ships

Visual

Transmission

Data Collec-tion

and

Data

Transfer

Services

HMI

OP/

DP

Value-added Data Proces-sing Services

HMI

OP/

DP

User

Inter-

action

Service

HMI

Service Couplers

Service Couplers

OP/

DP

OP/

DP

Physical

Measurements

EGEND:

HMI	Human-Machine-Interface

M2M	Machine-Machine-Interface

OP/DP		Technical Operating Personnel

(operates and maintains technical equipment;

no licence to change structure and/or functionality) / 
Technical Development Personnel

(plans, deploys and further develops technical equipment;

licence to change structure and/or functionality)

NOTE: 	By default, external users can request a HMI (Configuration Type to be provided by the User Interaction Service) and/or a dedicated Gateway (Machine-to-Machine-Interface; configuration type to be provided by Gateway Service), depending on individual operational requirements.

01-Nov 2007

Service Couplers within Nodes

Shore-based external systems (of third parties)

HMI

Gateway Service

M2M

M2M

M2M

M2M

M2M

M2M

HMI

It should be noted, that data exchange between different technical e-Navigation services is exclusively done by so-called Service Couplers and only in so-called Node sites, e.g. using local area network technologies at the Nodes.

- Shore-based e-Navigation system architecture detail layout

Figure 6 on the following page shows the shore-based e-Navigation system architecture in full detail. It particularly indicates the variety and plurality of different individual technical e-Navigation services and interactions with the shipping and the environment (left hand side), with shore-based primary users (right hand side), and to other shore authorities, shore-based users and shore-based stakeholders (lower side of Figure 6).

The names of the technical e-Navigation services given in Figure 6 are descriptions of what they provide to the shore-based e-Navigation system. Each of the individual technical e-Navigation services has been assigned a three-letter mnemonic, which is uniquely assigned within the common shore-based e-Navigation system architecture, and which will be used for various purposes when detailing the architecture and when encoding data items in shore-based data exchange.

Additional technical e-Navigation services may be defined in the future as prompted by new technologies and/or new functionality requirements.

The following list gives a short explanation of each of the technical e-Navigation services:

- Data Collection and Data Transfer Services:

- The AIS Service (AIS) allows the shore-based e-Navigation system to access the AIS VHF Data Link, supporting the various receiving and transmitting functions of the AIS intended for a shore-based competent authority.

- The Radar Service (RAD) delivers to the shore-based e-Navigation system all radar-derived data, i.e. radar images, radar plots, and radar tracks, each of which with radar-specific relevant quality parameters.

- The Direction Finding Service (DFS) determines the current position of a mobile VHF voice/data transmission device operating on one of the several frequency channels monitored. Thus a DF track together with relevant quality parameters (e.g. accuracy estimation) is provided to the shore-based e-Navigation system.

- The DGNSS Correction Service (DGN) delivers to the shore-based e-Navigation system all relevant measurements regarding the current “on-air” status of selected components of World Wide Radio Navigation System (WWRNS), such as particular GNSS, terrestrial augmentation and terrestrial backup-systems of other authorities. Such measurements could be forwarded to e. g. to the User Interaction Service for appropriate display to users e.g. in VTS Centres, thus fulfilling user requirements accordingly, and/or to the surveying community and/or the general public via the Gateway Service for their added value. Also, based on these run-time measurements, the DGNSS Correction Service provides own DGNSS correction data to the own shore-based e-Navigation system for transmission to vessels e.g. via the AIS Service or via the Medium Wave Broadcast Service. Hence, the DGNSS Correction Service does not transmit DGNSS corrections to shipping on its own but relies on some other technical e-Navigation service to fulfil that specific task.

- The Medium Wave Broadcast Service (MWB) ... is sometimes referred to as the IALA Radiobeacon System, however in this context it is only the transmission of the

- The VHF Communication Service (VHF) allows for shore-based bi-directional voice and/or data communication with any mobile radio station operating in the VHF maritime mobile service band. Today, this communication is done mainly using analogue voice communication technology, but there are developments under way to eventually replace the present analogue technology by high-capacity digital voice/data communication means in the VHF maritime mobile service band.

- Similarly, the Short Wave Communication Service (SWC) provides the shore-based bi-directional voice and/or data communication with any mobile station operating on the frequencies of the short wave bands assigned to maritime use.

- The GMDSS VHF DSC Service (DSC) allows the shore-based e-Navigation system to participate in the digital GMDSS DSC-Channel 70 in the VHF maritime mobile service band.

- Similarly, the Short Wave GMDSS Service (SWG) allows for the bi-directional communication in the short wave channels dedicated to the GMDSS.

- The Aviation Communication Service (AVC) allows for shore-based bi-directional voice and/or possibly data communication with any aircraft or helicopter, thus supporting operational applications such as Search and Rescue and pollution combat.

- The Fixed Visual Aids Service (FXA) provides fixed visual or classical Aids-to-Navigation to the mariner. It should be noted, that “visual” may be a misnomer to the extent that a fixed Aid-to-Navigation could also include on site racons and / or on site AtoN AIS stations, which conceptionally would be a part of the FXA.

- The Floating Visual Aids Service (FLA) provides floating visual or classical Aids-to-Navigation to the mariner. The same cautionary note regarding non-visual technologies as with the FXA applies.

- The CCTV Video Service (VID) provides the shore-based e-Navigation system with the capability to visually observe the areas of waterways covered by video camera observation.

- The Local Public Address Service (LPA) allows for acoustically addressing areas of a waterway, for acoustic voice communications and for the dissemination of other acoustic signals to mariners. Hence, the acoustic fog signals would belong to that service.

- The Environmental Sensor Service (ENS) provides the shore-based e-Navigation system with measurements of relevant parameters of the maritime environment. Such relevant sensor data could be hydrological, meteorological or of other physical nature.

- Value Added Data Processing Services:

- The Position Determination Service (POS) processes all real-time data regarding traffic objects (vessels, aircrafts, floating objects etc.) from all available and/or relevant sensory sources (e.g. AIS, radar, direction finding). Also, fusion algorithms dealing with the different sensory information may be processed here, thus providing the shore-based e-Navigation system with a multi-sensor-validated track of every traffic object under surveillance by the shore-based e-Navigation system, together with relevant quality parameters such as accuracy estimations, tracking status and reliability etc. For every traffic object surveyed not only the “last known” position is stored, but also their past tracks derived from the validated position reports.

- The Ship Data Consistency Analysis Service (SDA) collects, evaluates, stores and provides on demand non-real-time data regarding traffic objects, in particular their static and voyage related data. To that end this technical e-Navigation service draws on a variety of available sources of information for that kind of data, such as AIS reports, historical data from past voyages of the same traffic object, manually acquired data (e.g. from User Interaction Service), and third party data bases (accessed via Gateway Service). This service also provides pseudo tracking identifiers for the Position Determination Service (POS), see above, for correlation of real-time tracking data with static and voyage related data of the same traffic objects.

- The Environmental Data Evaluation Service (ENE) collects, evaluates, stores and provides on demand all data of the physical environment of the waterways relevant to safety and efficiency of shipping and to the protection of the environment. To that end this technical e-Navigatoin service draws on a variety of available sources of information for that kind of data, such as the system’s own Environmental Sensor Service (ESS), own historical data, manually acquired data, and in particular from third party data bases (accessed via Gateway Service). The Environmental Data Evaluaton Service also creates added-value information on the physical environment by evaluating the available raw data from the various sources by appropriate algorithms. For instance, tidal windows may be calculated here.

- The Vector Chart Service (VEC) collects, stores and provides on demand all data pertaining to vectorized ENCs, both drawing on official sources for ENCs but also drawing on the system’s own sources, i.e. on manuel input via the User Interaction Service. The Vector Chart Service maintains the ENCs from external sources as they are, but provides overlay layers which contain the additional information from the system’s own sources.

- The Ship Industry Database Service (SID) …

- The Data Mining Service (DMS) allows for complex queries on present and historical data available from all other technical e-Navigation services of the system. It creates, stores and provides reports to the requesting service, e.g. the User Interaction Service or the Gateway Service.

The Gateway Service (GWY) and the User Interaction Service (UIA) have been described in the previous chapter already.

Each of the above technical e-Navigation services except for the User Interaction Service provides a functional interface to any other of the technical e-Navigation services of the own shore-based e-Navigation system, i.e. on their “domestic” side. This functional interface provides the respective functionalities of a technical e-Navigation service as a set of Basic Services specific to that technical e-Navigation service.

For example, the AIS Service delivers the so called Basic AIS Services, which is a set of well defined, AIS specific functionality to access the complex features of the AIS VHF data link in an abstract representation, which encapsulates to the shore-based system the subtleties of the AIS and its complexity and thereby avoids proliferation of AIS VHF data link specifics throughout the shore-based e-Navigation system.

The User Interaction Service provides its functional interface as a Human Machine Interface to its human users. This functional interface, which may be called Operational Presentation Surface generally also is subdivided into individual functional windows with specified user interaction functionality each.

It should also be noted, that the Value Added Data Processing Services are only represented at Node sites.

It should be noted, that data exchange between different technical e-Navigation services is exclusively done by so-called Service Couplers and only in so-called Node sites, e.g. using local area network technologies at the Nodes. This has at least two important benefits: The wide-area networking topologies of the different technical e-Navigation services, which may be different from each other, are encapsulated within the service. Also, local area networks allows for maximum and highly cost-efficient performance of data exchange at the Node sites.

The Shore-Based Wide Area Network Service (SBN) is a utility service, which support any other requesting service of the shore-based e-Navigation system. It provides the feeder link and the backbone connectivity for the individual technical e-Navigation services of the shore-based e-Navigation system. Because of its supporting position in regard to its requesting services, it is encapsulated by its requesting service.

--- introduce On-Site Infrastructure (INF)

For further details on the above and additional features of the technical e-Navigation services refer to another IALA Recommendation on the generic service model of a technical e-Navigation service.

* introduce the most important features of the interaction of the technical e-Navigation services, in particular the “bus” structure at the Node sites => point to the introduction of the concept of Nodes (distribution model) within the next structural Recommendation on generic service model

* other building blocks for the system besides technical services: infrastructure (=> dependency on infrastructure already mentioned in 101, here specifically addressed for shore-side); personnel for technical maintenance and system development

Generic Functional Setup of the 
Common Shore based e-NAVigation system

Position Determination Service

(POS)

HMI

Ship Data Consistency Analysis

Service (SDA)

HMI

Environmental Data Evaluation

Service (ENE)

HMI

Vector Chart Service

(VEC)

HMI

Data Mining Service

(DMS)

HMI

Shipping Industry Database Service

(SID)

HMI

OP/

DP

SB

OP/

DP

SB

OP/

DP

SB

OP/

DP

OP/

DP

OP/

DP

SB

Notices To Mariners Service

Hydrological Forecast Models

ENC Distribution

Waterway Depth Information Provider

Dredging Operations

Tidal Information

Shipping Statistics and Shipping Fees

Shore-based Pilot Applications

Future external information source/sink

Other allied services

Ship Agents

Ship Register(s)

Regional Maritime Cooperation

(such as HELCOM, SafeSeaNet)

External Communication Provider

External Meteorological Services

und Wetter

Port State Control

Different Security Services

Customs

Inland Waterway Authoroties

Port Authorities

Land Surveyors

Visual Transmission

(mainly)

OP/

DP

SB

Configuration type for Traffic Information

Traffic Support and Traffic Control

HMI

Configuration Type for

Security Applications

HMI

Configuration Type for

Maritime Rescue Coordination (MRCC)

HMI

Configuration Type for Accident

Management and Pollution Combat

bekämpfung und Unfallmanagement

HMI

Configuration Type for Waterways

Maintenance Operations (e.g. Dredging)

HMI

Configuration Type for Editing of

Notices to Mariners

HMI

Configuration Type for

Lock Operation

HMI

Configuration Type for

Ice Reports / Ice Breaker Operations

HMI

Configuration Type for

Ship Data Consistency Analysis

HMI

Configuration Type for

Data Evaluation and Data Mining

HMI

Configuration Type for

Pilot Dispatching Center

Betrieb Lotsenwachstation

HMI

DGNSS Correction Service (DGN)

HMI

OP/DP

SB

Aviation Communication Service (AVI)

HMI

OP/DP

SB

Radar Service (RAD)

HMI

OP/DP

SB

AIS Service (AIS)

HMI

OP/DP

SB

Direction Finding Service (DFS)

HMI

OP/DP

SB

Medium Wave Broadcast Service (MWB)

HMI

OP/DP

SB

Short Wave GMDSS Service (SWG)

HMI

OP/DP

SB

GMDSS VHF DSC Service (DSC)

HMI

OP/DP

SB

Fixed Visual Aids Service (FXA)

HMI

OP/DP

SB

VHF Communication Service (VHF)

HMI

OP/DP

SB

Short Wave Communication

Service (SWC)

HMI

OP/DP

SB

CCTV Video Service (VID)

HMI

OP/DP

SB

Local Public Address Service (LPA)

HMI

OP/DP

SB

Gateway Service   (GWY) – individual configuration types (Gateway) for each external user (sources / sinks)

HMI

Environmental Sensor Service (ENS)

HMI

OP/DP

SB

- Radio Transmission -

Atmosphere

Waterway

Local Area Network(s)  within the Nodes

Local Area Network(s)  within the Nodes

Satellite

Radio Navigation GNSS

On-site

Infrastructure

(Energy, Housing,..)

(INF)

HMI

OP/

DP

Shore-based

Wide-Area

Network Service

(SBN)

HMI

OP/

DP

LRIT Data Center

SAR

General Shipping

-Merchant Marine

-  Fishing Vessels

-Leisure Crafts

-Inland Waterways Barges on Sea Fairways

Administration ships

-  multi purpose vessels

-buoy tenders

-ice breaker

-  SAR vessels

-   pilot vessels

-survey vessels

-  customs vessels

-  police vessels

-  Navy vessels

Floating Objects

-Lost Cargo

-Life boats / rafts

-  floating pollutants (e. g. oil)

Shipping

Passenger

Crew/Pilot

Application(s)

Radio link transceiver

PPU

Application Services

Value-added Data Processing Services

Data Collection and Data Transfer Services

User Interaction Service   (UIA)

LEGEND:

HMI	Human-Machine-Interface

M2M	Machine-Machine-Interface

OP/DP		Technical Operating Personnel (operates and maintains technical equipment; no licence to change structure and/or functionality) / 
Technical Development Personnel (plans, deploys and further develops technical equipment; licence to change structure and/or functionality)

NOTE: 	By default, external users can request a HMI (Configuration Type to be provided by the User Interaction Service) and/or a dedicated Gateway (Machine-to-Machine-Interface; configuration type to be provided by Gateway Service), depending on individual operational requirements.

Shore-based
Operators / Users

HMI

Acoustic

Transmission

Service Couplers in Local Area Network(s) within the Nodes

Physical status

Indicators

External/other network users

requested by every service 
(request + access encapsulated by service)

requested by every service 
(request + access encapsulated by service)

OP/

DP

## SB

Co-located third party services

All M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

M2M

Other / Future Configuration Types

may be added as required

Information Providers / Information Sinks /
Allied Services

Off-Shore Structures

Floating Visual Aids Service (FLA)

HMI

OP/DP

SB

--- introduce a description of the individual technical e-Navigation services in a short manner (few sentences each)

--- refer to the User Interaction Service and the Gateway Service: introduce cautionary note, that the figure above does not want to be normative in regard to the operational services supported in each and every admin by the system layout; rather the important points are:

- specific task of interfacing / interaction between machine and human (=> UIS) or specific task of interfacing a system to another “external” system by gateway (=> Gateway Service)

- exploit commonalities in functions for configuration types, e. g. “traffic image display” will be required by several operational users, although the configuration settings of those different operational users may be different (e.g. tactital traffic display vs. strategic traffic display) => concept of Configuration Type

- Compare the following chapters explaining in more detail the options for partial setup and/or configuration by different national members

- Consequences and Options

  - Consequential requirements for the common shore-based e-Navigation system and its components

- Reliability/continuity: common shore-based e-Nav system is embedded in national, regional and global topologies of systems, which rely on the contribution of the systems of the National Members => chain of information/data flow must not be disrupted by failures

=> drawing maybe

- latency: timely delivery of the information/data ….

  - The usefulness of the common shore-based eNAV system architecture as a reference framework

- What would be the consequences out of the above layout?

  - <provide a section on how the shore-based system architecture helps map the user requirements to concrete technical solutions, i. e. forward-pointer to the “eNav service concept” (refer to rec 210 “generic service model” etc.)

  - <User guidance on the use of those templates>

  - <Points to Recommendations xxx regarding applications and their mapping to actual services>

  - Options for different administration’s systems setup

- What would be the options to fit the generic system layout to individual administration’s setups?

  - Tie to IMO’s requirements of “extensibility” and “scalability”: Not all components identified in the common shore-based e-Navigation system architecture need to be provided/ deployed by every single National Member’s system. Hence, the common shore-based e-Navigation system architecture is scalable. Also, there may be additional individual technical e-Navigation service not indicated in the above introduction a National Member may wish to set up. Hence, the common shore-based e-Navigation system architecture is extensible. It should be noted, that scalability and extensibility need to comply with the principles of the common shore-based e-Navigation system architecture. This scalability and extensibility can be applied both for the range of individual technical e-Navigation services provided as well as the functionality level of an individual technical e-Navigation service.

  - Options to swap between HMI (User Interaction Service) vs. M2M (Gateway Service) => Example should suffice

  - What if there is a split responsibility for the above services in one country? How can the system concept accommodate this situation?  => not every service/ configuration type described above needed to be implemented by every national member; the point of the description rather is to have a common reference for discussion technical solutions.

- Introduction to case studies as examples how to use the common shore-based e-Navigaton system layout

--- introduction text to the Annexes as case studies as solution:

- Case study : Example for Application Specific AIS messaging, e.g. met data broadcast => Annex 1

- Case study: e-ANSI mapped to system layout => Annex 2

- Case study: GNSS anomaly detection via AIS and handling => Annex 3

- References

[1] 	e-Nav 101 – IALA Recommendation on “The e-Navigation architecture – the shore-based perspective”

[2]	IMO/MSC-85 approved Annexes 12 / 13 of IMO/NAV54 report (NAV54/25) entitled “Draft Strategy for the Development and Implementation of e-Navigation” and “Draft Framework for the Implementation Process for the e-Navigation Strategy“

[3]	<IMO’s formal invitation to IALA for contribution; maybe from MSC85>

[4]	“IALA Recommendation e-NAV-101 on The e-Navigation Architecture - the Shore-based Perspective”

- eNav 110ff - e-Nav system requirement analysis and e-Nav applications descriptions

- maybe relevant international standards regarding the system architecture paradigm(s)

<introduce references once the document is finalized>

- Glossary

- <extract key words from the figures (see below) and explain them briefly, if needed>

< add as soon as there are new terms defined>

Single Window: “A system that allows traders to lodge information with a single body to fulfill all import or export related regulatory requirements.” (UN/CEFACT definition; compare also UN/CEFACT Recommendation No. 33)

- Abbreviations

--- introduce service names in alphabetical order

<introduce abbreviations once the document is finalized>

- Appendix 1: Example for Application Specific AIS messaging as applied to common shore-based e-Navigation system architecture

- Appendix 2: the e-ANSI concept applied to common shore-based e-Navigation system architecture

= Electronic Aid-to-Navigation information service (= operational service designator)

- Appendix 3: the GNSS anomaly detection via AIS and handling applied to common shore-based e-Navigation system architecture

“Regional” in the context of this Recommendation is seen from a global perspective, i.e. a region could be a continent or a substantial part of a continent. E. g. the Baltic Sea as a whole is a region in this regard.

According to the IALA Constitution a National Members is an authority of any country, or any part of that country, which is legally responsible for the provision, maintenance or operation of marine aids to navigation within that country, or any part of that country (hereinafter referred to as National Member).

This implies that user requirements are captured and documented.

It should be noted, that by convention the individual technical e-Navigation services (i.e. the instances in the object-oriented terminology) are capitalised (“Service”) as opposed to technical e-Navigation services (using small letters) in a generic sense (i.e. the class descriptions in object-oriented terminology).

It should also be noted, that there is no such thing as a “super-service”, which would manage or control all services. Such a “super-service” is unnecessary when adhering properly to the object-oriented paradigm and would also constitute single point of failure of the e-Navigation system architecture. (This does not contradict a centralised control of the technical operation of the shore-based e-Navigation system or its centralised maintenance, however.)