| From: e-Navigation Committee |  |
| --- | --- |
| To: PAP18 | 13 March 2009 |

The need for an holistic view concerning e-Navigation developments

# Background

While considering various input papers, the AIS Tehcnical Working Group (WG3) expressed a growing concern regarding the lack of a holistic approach in particular between the shoreside and shipside developments in the existing e-Navigation process.  Without a holistic approach, we are at risk of creating gaps in service that will leave e-NAV squarely in the theoretical realm, rather than the practical realm, unless we make a paradigm shift.

# Examples of gaps

While the following examples are diverse the gaps are consistent and not technology dependant.  These gaps are:

- Symbology

- Consistent data representation (units, timezones, etc.)

- The requirement to provide the information to the mariner

- Requirements for ship-borne equipment to display services

- Training

- Timeliness in ship side / shore side developments

## Electronic AtoNs (Virtual AIS AtoNs)

Considering e-NAV6/19/28 Development of virtual AtoN systems, a number of issues were raised.

New technology exists in the form of the capability of AIS to provide virtual Aids to Navigation. Other means of providing electronic Aids to Navigation could be suggested, however known gaps prevent this new tool from being mature and useful for Aids to Navigation authorities, and more importantly for shipping.

Potential roles of virtual AtoNs have been suggested:

- Emergency wreck marking

- Marking narrow lanes without creating obstructions

- Marking suggested waypoints without creating obstructions

While the technological means to provide an electronic AtoN service exists, experience demonstrates that very few ships will ever notice the existence of an AIS AtoN.  The potential role of such a service is largely unexplored, as are the associated risks involved.  Display requirements are missing, and display symbology needs to be developed.  Also there has been no consideration of the training requirement for mariners and no recognition by navigators that these AIS AtoNs are in place.  Because of these gaps, the ‘tool’ is present in the box, but using the tool is ineffective.  The AtoN Authority can broadcast the virtual AtoN service, but without closing these gaps the service will not be of value to shipping in terms of improved safety and efficiency of navigation.

Examples of the utilization of AIS AtoN’s are described in Annex 1.

## AIS Binary messages

AIS Binary Messages have the capability to provide a multitude of services.  IMO has defined a list of international binary messages that have the potential to provide valuable services, both shore side and ship side, however the gaps prevent coherent delivery and usage of such services.

## MET/ HYDRO

IMO has defined the binary application message, symbology for an AtoN exists, requirement for the display of AIS targets on radar displays exists, and IALA recommendations regarding AIS AtoNs exist.  What doesn’t exist is the requirement for capabilities to display the met/hydro binary information onboard.

## AIS DGNSS

From the beginning AIS has been blamed for incorrect positioning information, however the AIS is required to broadcast the GNSS position of the ships navigation system, which in most instances is less accurate, than the (more modern) internal GNSS receiver in the AIS mobile station.  Furthermore, if the AIS receive DGNSS corrections via message 17, it will broadcast a corrected position, however this is not the position being used to navigate the ship.  There is no requirement in the IMO performance standard for the AIS to provide DGNSS corrections to the ships navigation system.

# Proposed solution

- Suggest that the IALA eNAV committee consider the whole picture, and not allow terms of reference restrictions prevent us from addressing gaps.

- Request that IALA national members submit input to IMO requesting the following:

  - Transition planning should allow for the inclusion of e-Navigation in performance standards.  Amend Regulation 15 of SOLAS chapter V, to require an e-Navigation section whenever a performance standard is defined or revised.  The e-NAV section should, as a minimum, reference e-NAV guidelines, addressing both ship-side and shore-side perspectives.

  - e-NAV guidelines to be developed and maintained by CIRM and IALA jointly with a view to integrate shore side services into shipborne equipment.

# Benefits

Benefits of creating an efficient coordination process for filling the gaps could be

Commercial advantages

- Potential for safer polar shipping, saving time and fuel

- VTS auto reporting on passagelines, savings in VTS operations

- Provision of virtual waypoint for efficient route planning

- Improvement of port logistics through early notice of arrival

- Protection of the environment

Safety advantages

- Corrected positions

- Wreck marking

- Availability of relevant Met/Hydro information

# Annex 1 – Examples of AIS AtoN utilization

## IALA Wreck Marking Buoy with AIS

Several IALA member competent authorities have equipped themselves with AIS AtoN systems to be used on wreck marking buoys. An example use of this AIS AtoN would be as follows:

A new danger or wreck is marked firstly with virtual AIS AtoN(s) transmitted from an existing AIS network (where available) and secondly with a buoy complying with the IALA recommendation, exhibiting blue and yellow daymark and light. The buoy is also equipped with AIS AtoN which broadcasts AIS Type 21 Message. In addition, the AIS AtoN unit also broadcasts three other AIS Type 21 Messages, as Virtual AIS AtoN messages. The four AIS Type 21 Messages define a box over the wreck position via north, south, east, and west cardinal marks. A vessel with suitable display apparatus on the bridge will be alerted to the new danger by the four cardinal marks. (The AIS AtoN may also broadcast a safety related message.)

If a vessel approaching the new danger is not able to process and correctly display the four AIS Type 21 Messages, it will only have the visual signal from the buoy, and possibly an AIS safety related message, to alert it to the new danger.  If the visibility is poor, the bridge watchkeeper may not be aware of the visual signal.  The vessel may thus strike the new danger.

## AtoN provision in the Arctic environment

There is a growing realization, that the opening of Arctic waters to shipping transit may be imminent, as a result of climate change.  This may create an opportunity for shipping to take the shorter route between the Pacific and the Atlantic oceans across the arctic, saving both time and fuel. However providing Aids to Navigation in such regions, is difficult and expensive.

The ability to supplement traditional physical AtoN technology with electronic (virtual) AtoNs may prove to be a feasible option for providing more cost effective and dynamic AtoN services, and thus enable safer and more efficient navigation in the area.

However, with the current gaps in display requirements, symbology, and recognition by mariners, providing such a service will have little or no effect.