Collection of Regional Applications for AIS Binary Messages

IALA eNAV Committee – AIS Technical WG

11 March 2009

Submission Form [Test run example]

| # Registrant (i.e., Maritime Administration) | A N Other |
| --- | --- |
| Name of Application (keep short) | Monitoring of AtoN |
| Type of Binary Message (e.g., msg 6, 8) | 6 |
| DAC and FI Used | DAC = 250 FI = 0 |
| Used by (list organisations known to be using this binary message) | Commissioners of Irish Lights |
| Additional Information (include typical applications, e.g., AtoN monitoring) | AtoN monitoring of light power systems |
| How portrayed (e.g., text only, graphical on ECDIS/ECS, etc.) | Graphical on ECS (ICAN Aldebaran II & Horizion) |
| Number of Slots | 1 |
| Reporting Rate | 3 minutes |
| Date - first used - status | July 2007 In use |
| Technical Point-of- Contact | Osamu Toyama e-mail: o-toyama@zenilite.co.jp |

| Parameter Name | # bits | Description |
| --- | --- | --- |
| Sub-application ID | 16 | Default: 1 |
| Voltage data | 12 | Lantern supply voltage data Max 409.6V |
| Current data | 10 | Lantern drain current data Max 102.3A |
| Power supply type | 1 | AC or DC 0: AC 1: DC |
| Light Status | 1 | Light on or Light off 0: light off 1: light on |
| Battery Status | 1 | Good or low voltage 0: good 1: low voltage |
| Off-position status | 1 | Off position or on position O: on position 1: off position |
| Spare | 6 | For future use should be 0 |