e-NAV Committee	e-NAV6/9/3

6th Session	9-13 March 9-13

Agenda item 9

U.S. Coast Guard

Update in Using AIS to Notify Competent Authorities

of Local GNSS Anomalies

Introduction

- At eNAV5 the US Coast Guard proposed that guidance be developed for competent authorities on using AIS responses to monitor local GNSS anomalies in their areas of responsibility.  eNAV5 noted the possibility of using AIS reception to monitor local anomalies in GNSS, and incorporated within a revision of R-129 GNSS Vulnerability and Mitigation Measures. Concern was expressed regarding the possible transmission of estimated (dead reckoning) position via AIS; this was considered to be a flawed concept as such information is potentially hazardously misleading.  Other WG’s were invited to review and discuss this input document, and it was recommended to keep this issue on the work program.

Discussion

- Since eNAV5 the USCG was able to implement a means for real time display of possible indications of GNSS anomalies with received AIS position reports.  Vessel reports were flagged as possible GNSS anomalies in cases where synchronization had recently been shifted from “UTC direct” (sync state becomes non zero) within a specified time, where the time stamp was greater than 59, or when reported position defaulted to 91/181 Latitude/Longitude when GNSS signal was lost.  This display of possible GNSS anomalies is still experimental and needs to be refined.  Most displayed “GNSS anomaly” indications reflected a loss of UTC direct sync state while retaining normal heading stamp data (possibly indicating that the AIS integral GNSS antenna is disconnected).  However some ships displaying a GNSS anomaly showed a normal UTC direct but inoperative position sensor (an unexplained failed GNSS with no backup) – See Figure 1, while others showed both a loss of UTC direct and position sensor (possibly a true GNSS anomaly) – See Figure 2.  No ships were found to have a position sensor operating in the dead reckoning mode (time stamp = 62).  At the time this report was written, AIS responses from 237 vessels off the US coast displayed a GNSSS anomaly as described above.  None of them had a time stamp of 61 (manually entered position) or 62 (dead reckoning).

Figure 1 - UTC direct retained, position sensor inoperative

Figure 2 - Loss of both UTC direct and position sensor

- Both ITU-R Recommendation M.1371-3 and IEC 61993-2, which define in detail operation of AIS, especially Class A AIS, are currently being updated by their responsible working groups.  Consequently an opportunity exists to amend these standards in such a way as to improve the usefulness of AIS reports from ships in immediately notifying maritime navigation authorities of the existence and location of GNSS anomalies.  For example, the IEC TC80 working group responsible for revising 61993-2 has added testing of each GNSS data interface sentence interconnecting the external position fixing device to the AIS, a test lacking in the current certification standard.

Recommendation

- In developing guidance for competent authorities on using AIS responses to monitor local GNSS anomalies, as well as liaison to IEC and ITU, the following issues might be considered:

  - What time delay after a vessel position report sync state transitions from UTC direct (sync state = 0) to another value should the report be flagged as a potential GNSS anomaly, noting the many AIS installations in which the integral GNSS receiver antenna is permanently disconnected?

  - What scenario would cause an AIS to transmit a heading of stamp of 60 or 63 AND a UTC Direct sync state = 0?

  - Should the definition of time stamp data values >59 be reviewed?  For example, should better use be made of values 61 (position manually entered) and 62 (dead reckoning)?  What is the precise difference between the default value 60 (time stamp is not available) and the value 63 (positioning system is inoperative), and how are those values determined?  Does a position report of 91/181 Latitude/Longitude duplicate a time stamp value of 60 and/or 63?

  - Class B/CS units cease transmitting in the absence of a GNSS signal unless interrogated by a base station.  Can responses from these units be used in any practical way to identify GNSS anomalies?  If so, how?

  - What value do single bit GNSS RAIM and position accuracy flags provide maritime navigation authorities now (See Table 1 below)?  Can use be made of these two single-bit flags in helping to identify GNSS anomalies?  Should the definition of these flags be revised or clarified in any way to make them more useful to maritime navigation authorities?

  - Class A AIS units are required to accept the following IEC 61162-1/NMEA 0183 GNSS sentences (IEC 61993-2 §7.6.2.3):  GNS, GLL, GBS (RAIM indicator), and DTM (reference datum).  Does value exist in requiring GSV (GNSS satellites in view) & GGA (GPS fix data) be included?   GSV in particular includes signal-to-noise ratio (SNR) values of 0 to 99 in units of dB.  Should an international application identifier be established for binary broadcast messages transmitting this information?

  - Should the AIS Base Station and AtoN Station, which also uses GNSS receivers, also provide maritime authorities expanded GNSS data interfaces sentences such as GSV?

  - Should a new GNSS data interface sentence be developed, designed to provide as complete GNSS anomaly information as is possible?

  - What recommendations, if any, should be given ITU and IEC TC80 in their maintenance of ITU-R Rec. M.1371-3 and IEC 61993-2?

  - The working group currently revising IEC 61993-2 is considering the addition of a test to compare reported positions between the external position fixing device and the AIS integral GNSS receiver, and to display an alarm to the operator if a difference greater than 100 meters for 5 minutes is detected.  Is there value in maritime navigation authorities being made aware of such an alarm if it were to occur unexpectedly?

5.	eNAV members are invited to share their experiences in using AIS position reports to identify and locate GNSS anomalies at the next session of this Committee, and at other fora as appropriate.

Table 1 (from Rec. ITU-R M.1371-3)

Determination of position accuracy information

| Accuracy status from RAIM (for 95% of position fixes)(1) | RAIM flag | Differential correction status(2) | Resulting value of PA flag |
| --- | --- | --- | --- |
| No RAIM process available | 0 | Uncorrected | 0 = low (>10 m) |
| EXPECTED RAIM error is < 10m | 1 |  | 1 = high (<10 m) |
| EXPECTED RAIM error is > 10m | 1 |  | 0 = low (>10 m) |
| No RAIM process available | 0 | Corrected | 1 = high (<10 m) |
| EXPECTED RAIM error is < 10m | 1 |  | 1 = high (<10 m) |
| EXPECTED RAIM error is > 10m | 1 |  | 0 = low (>10 m) |
| (1) The connected GNSS receiver indicates the availability of a RAIM process by a valid GBS sentence of IEC 61162-1; in this case the RAIM-flag should be set to “1”. The threshold for evaluation of the RAIM information is 10 m. The RAIM expected error is calculated based on the GBS parameters “expected error in latitude” and “expected error in longitude” using the following formula: EXPECTED RAIM error= expected error in latitude2+expected error in longitude2 (2) The quality indicator in the position sentences of IEC 61162-1 received from the connected GNSS receiver indicates the correction status. |  |  |  |

The time currently specified is two weeks.  That time may have to be significantly reduced.

Time stamp = UTC second when the report was generated by the electronic position fixing system (EPFS) (0-59, or 60 if time stamp is not available, which should also be the default value, or 61 if positioning system is in manual input mode, or 62 if electronic position fixing system operates in estimated (dead reckoning) mode, or 63 if the positioning system is inoperative) (from ITU-R Rec. M.1371-3).

Approximately 1900 UTC 25 March 2009.