e-Navigation Committee

6th Session									9 March 2009

This report from the General Lighthouse Authorities’ Research and Radionavigation Directorate investigates the current loading of the AIS network and considers the additional loading which would occur due to the provision of differential corrections. It is provided to the Committee for information.

Summary

The provision of integrity for GNSS services is at present limited; though it can be provided by the IALA radiobeacon differential GPS (DGPS) service using a separate DGPS receiver, it may also be supplied by the AIS datalink (Message 17).

This study considers the additional load imposed by the provision of differential corrections over AIS using the defined message format of Message 17.

The busy sea area off North Foreland was chosen to collect data in order to establish whether there is sufficient channel capacity available. Analysis showed the current network load to be less than 18%.

The investigation proposed a worst-case scenario where three sets of GNSS differential corrections (representing GPS, GLONASS and Galileo), were broadcast over AIS from three separate base stations with overlapping coverage.

It showed that three sets of corrections from three locations (effectively nine sets of corrections) would only increase the loading by an additional 3.6% and would not significantly affect the user’s level of service.

Contents

1	Introduction	6

2	Integrity	6

3	Network Load	6

3.1	Sorting the AIS Messages	8

3.2	Estimating the Current Network Load	9

3.3	Network Load with GNSS or eLoran Corrections	11

4	Discussion	13

5	Conclusions	14

6	Recommendations	14

List of Figures

Figure 1 – Current proportion of message types………………………………………………8

Figure 2 – Current occupied bandwidth – average over a 3 day period…………………….9

Figure 3 – Vessel position reports over 72 hour period for channels A and B…………….10

Figure 4 – Correlation between ship position and base station reports……………………11

Figure 5 – Predicted bandwidth loading when three sets of differential corrections (Message 17) are provided by three separate base stations……………………………………………12

Figure 6 – Message proportions when providing three independent DGNSS corrections provided by three separate base stations…………………………………………………….13

List of Tables

Table 1 - AIS Message Numbers, with ASCII identifier and message description	8

Table 2 - Extra timeslots required per minute and equivalent network load when providing correction for up to three GNSS/eLoran services	11

Reference Documents

RD1	     IEC 61993-2, page 108

RD2  	     ITU-R M.1371-3

RD3	     IALA Recommendation A-124 Annex 16 DGNSS Broadcast from an AIS Service

# Introduction

This report investigates the Automatic Identification Systems (AIS) ability to provide position integrity and the consequences for network load.

Though AIS relies on a Global Navigation Satellite System (GNSS) for position and for time synchronisation, currently available GNSS services (GPS and GLONASS) do not provide integrity checks. Therefore it is normal to provide an AIS unit with differential corrections (including integrity) from an external source such as the vessel’s differential GPS (DGPS) receiver; however, this is neither mandatory nor part of the AIS standard [RD1]. An alternative method of providing differential corrections is to use the defined message structure of the AIS network [RD2] – GNSS Broadcast Binary Message (Message 17).  This has implications on network loading and potential consequences on the user’s level of service.

Providing DGNSS corrections over AIS is designed to give the user exactly the same service as provided by the IALA radiobeacons, the only difference is the data is transported via the AIS network.

In order to find the current network load, a three-day sample of AIS traffic was collected from North Foreland Lighthouse in Kent – AIS coverage from this location included the busy Dover Straits, the Thames Estuary and part of the southern North Sea.

Firstly the sampled data was analysed to determine the current network load. Secondly the extra network load created by providing GNSS corrections was calculated based on the scenario of three separate sets of GNSS corrections broadcast from three base stations with overlapping coverage.

Figures representing the current network load and the extra network load are presented to help assess the merits of providing differential corrections over the AIS datalink.

# Integrity

The Automatic Identification System (AIS) relies on GNSS for position and for timing, necessary for network synchronisation. Current GNSS services do not provide integrity checks on the position data; therefore for AIS to have position integrity, it requires differentially corrected data. This is currently provided by IALA radio beacons around the UK coastline. These enhance the standard GPS service by providing greater accuracy and integrity checks to the received data. In the future AIS may provide a backup to this service. AIS can provide differential corrections through Message 17, but further study and analysis are required to determine the implications for network loading and the consequences for the user’s level of service.

# Network Load

In order to evaluate the current AIS network load a sample of data was required.

Initially it was envisaged that the MCA AIS data feed would provide the data; however, this data is heavily filtered making it unsuitable for network load analysis. The alternative was to collect data from a busy area over a period of several days. North Foreland lighthouse was identified as an ideal location; because it is close to the Dover Straits, which provide a busy AIS environment and also has AIS equipment installed.

Data was collected over 3 days from the North Foreland AIS base station (currently used as an AIS AtoN) from 30 May – 2 June 2008. This produced a large file (~180Mbyte), which comprised 3.4 million time stamped AIS sentences. Processing the data was initially difficult, with both Notepad™ and Wordpad™ refusing the task; Textpad (Helios Software Solutions) however did load the file, though very slowly.  The first few lines of the collected data are shown below:

12:47:39 !AIVDM,1,1,,A,D02:o:hkDLfq6DM6EIHu6D0,2*6B

12:47:39 !AIVDM,1,1,,A,13aRV2002vP74VVMfFV2sRG:05Jt,0*0A

12:47:39 !AIVDM,1,1,,A,33:kc>0tBP0:0R4MCjKchaK80000,0*16

12:47:39 !AIVDM,1,1,,B,13UVEP502<08KghMGeu836M:0`Bw,0*43

12:47:39 !AIVDM,1,1,,B,13M@@QP03@07WF`M;p9;u9e<0@FT,0*2D

12:47:39 !AIVDM,1,1,,A,13aP1v002b0=1H`MawF2m2E>2@FV,0*7F

The first column is the timestamp, then a space before the message identifier (!AIVDM), moving along to the right shows a column with the AIS channel ‘A’ or ‘B’. The next field after the channel column is the start of the encapsulated binary data, with the first character identifying the message number. Note that the first row has the message identifier ‘D’, this corresponds to AIS Message Number 20 (see Table 1). The following row message identifiers show 1,3,1,1,1 and give an indication of the dominance of Message 1.  For more details of the AIS message structure see [RD2].

| AIS Message Number, Identifier & Description |  |  |
| --- | --- | --- |
| Number | Identifier | Description |
| 1,2,3 | 1,2,3, | Position Reports – Output periodically from mobile station |
| 4 | 4 | Base Station Reports – UTC time, date position |
| 5 | 5 | Ship static and voyage related binary data – class A only |
| 6 | 6 | Addressed Binary Message – between 1 and 5 slots |
| 7 | 7 | Binary Acknowledge – ack. to mess. 6 |
| 8 | 8 | Binary Broadcast Message – between 1 and 5 slots |
| 9 | 9 | Standard SAR aircraft position report |
| 10 | : | UTC and date enquiry |
| 11 | ; | UTC and date response – response to mess. 10 |
| 12 | < | Addressed Safety Related Message. |
| 13 | = | Safety Related Acknowledge – ack. to mess. 12. |
| 14 | > | Safety Related Broadcast Message. |
| 15 | ? | Interrogation – used for interrogations over link except for UTC and date. Class A mobile can be interrogated for mess. 3 & 5. Class B mobile can be interrogated for mess. 18 & 19. |
| 16 | @ | Assigned mode command – Time slot (TS) assignment transmitted by base station when operating as a controlling entity. |
| 17 | A | GNSS Broadcast Binary Message – Transmitted by base station to convey DGNSS data. |
| 18 | B | Standard Class B position Report – instead of message 1,2,3. |
| 19 | C | Extended Class B position Report - Transmitted after change in ship related data (as opposed to voyage or dynamic data). |
| 20 | D | Data Link Management Message – Used by base stations to pre-announce and reserve TS allocation. |
| 21 | E | AtoN report – Used by station mounted on an AtoN, transmission typically set broadcast every 3 –10 minutes. |
| 22 | F | Channel Management – Broadcast by base station to command the VHF data link parameters for a geographical area. |
| 23 | G | Group Assignment Command |
| 24 | H | Class B static data report |

Table 1 - AIS Message Numbers, with ASCII identifier and message description

## Sorting the AIS Messages

In order to process the data it was necessary to sort the messages into categories based on message numbers. This was achieved with a software application written in C++. The program used the data file collected from North Foreland (NF30May.txt) to sort the data into 24 new files based on the message numbers. The quantity of messages held in each file was used to form a proportional message plot shown in Fig 1. Note that not all messages (1-24) occurred; therefore for clarity Fig 1 omits unused messages.

Figure 1 - Current proportion of message types

Note from Figure 1 that the vast majority of messages are scheduled ship position reports (Message 1), which is in line with expectations.

## Estimating the Current Network Load

Estimating the network load is necessary to establish if there is spare capacity available, which may be used to provide extra services such as GNSS or eLoran corrections.

Network load is the proportion of used to available bandwidth.

As AIS is a two channel Time Division Multiple Access (TDMA) system the network load can be calculated as follows:

Network Load (%)=Timeslots UsedTimeslots Availablex100

Where the Timeslots Available is the product of the data collection period (minutes) and the available timeslots per minute. The Timeslots Used is the product of number of messages collected and the message duration (in timeslots). On examination it was found that all messages used only one timeslot with the exception of Message 5, which used two.

Figure 2 - Current occupied bandwidth – average over a 3 day period

Fig 2 shows the proportion of occupied to available capacity. The Occupied Bandwidth equates to Network Load, which is 17.8%; the remainder is shown as Spare Capacity.

It must be remembered that the Occupied Bandwidth displayed is averaged over the three-days of data collection and is the sum of traffic on both AIS channels ‘A’ and ‘B’. It is possible that over time there are peaks and troughs in shipping densities that may depend on factors such as tides/weather etc. It is also possible that the channel loading is unbalanced. These scenarios are investigated with a C++ application designed to capture the frequency of occurrence of a Message 1 for each channel over time. The results are shown in Fig 2.

Fig 3 shows that the channel loading for A and B is well balanced. It also shows an apparent random distribution of the vessel position reports, though there appears to be a higher level of activity during the second afternoon, peaking at around 1500 hrs. The peak in position reports could be due to either of the following:

Figure 3 - Vessel position reports over 72 hour period for channels A and B

- Higher densities of shipping within the nominal coverage area

- An increased coverage area because of an electromagnetic anomaly such as tropospheric ducting. This may cause an extension to the normal radio horizon as the wave front is forced to follow the curvature of the Earth, consequently increasing the number of targets visible to the local AIS network.

To determine the most likely explanation, a scaled version of the base station reports (Message 4) was plotted with the vessel position reports (see Fig 4). If (a) were true, Message 4 should appear flat, due to the constant number of base stations in range and the periodic nature of this type of message. If (b) were true there should be correlation between the lines, as more base stations as well as vessels come into range.

Some correlation appears to exist between the vessel and base station reports, though it is by no means conclusive.

Figure 4 - Correlation between ship position and base station reports

## Network Load with GNSS or eLoran Corrections

Providing differential corrections for GNSS and/or eLoran via the AIS network has the potential to enhance the accuracy and integrity of the AIS system. It may also provide a valuable back-up to navigation systems currently reliant on the IALA radiobeacon differential GPS (DGPS) service. The benefits of providing differential corrections over AIS have to be weighed in terms of the channel occupancy, which is examined in this section.

Network loading is measured in terms of timeslots. Finding the increased network load required a calculation to determine the number of extra timeslots needed to provide the correction data.

Reference [RD3] explains the timeslots and intervals required to provide differential corrections using Message 17. The results are summarised in Table 2 below for GNSS systems, though eLoran corrections may be presented in the same way as a GNSS service.

| Number of GNSS Systems (e.g. GPS Galileo, GLONASS etc.) | Slots required for a full set of differential corrections | Slots per frame for slot reservations and integrity warnings | Total number of slots (per minute) | Increased Network Load |
| --- | --- | --- | --- | --- |
| 1 | 12 | 18 | 30 | 0.67% |
| 2 | 24 | 18 | 42 | 0.93% |
| 3 | 36 | 18 | 54 | 1.2% |

Table 2 - Extra timeslots required per minute and equivalent network load when providing correction for up to three GNSS/eLoran services

A worst-case scenario would be that several base stations in an area are all producing three sets of correction data, in this case the network load is increased linearly with the number of correction sources. Therefore three base stations each providing three sets of correction data would give a network load of 3.6%. The bandwidth share is shown in Fig 5.

Figure 5 - Predicted bandwidth loading when three sets of differential corrections (Message 17) are provided by three separate base stations

Fig 5 shows that even for the high level of differential corrections illustrated in the worst-case scenario above, the increase in loading is marginal and therefore the provision of this service should not adversely affect the level of service experienced by the user.

A proportional breakdown of the network load (with regard to message types), for the scenario described above is shown in Fig 6.

Figure 6 - Message proportions when providing three independent DGNSS corrections provided by three separate base stations

# Discussion

This study was conducted to investigate the network load implications of providing position integrity via the AIS network. Integrity checks are unavailable from current GNSS services. However, most AIS equipment is able to receive integrity checked differential corrections via an external connection to the vessel’s differential GPS. As an alternative or back-up, it is possible to provide the same service through the AIS network using Message 17, with a slight increase in network load.

The busy sea area chosen for this study, which included the Dover Straits, showed a network load of less than 18%. It also showed a good balance in traffic load between the AIS channels A and B), with some variation in traffic density over time).

Calculating the extra network load when providing differential correction using Message 17, showed that the addition of three sets of GNSS corrections representing GPS, GLONASS and Galileo would occupy only 1.2% of total capacity when broadcast from a single base station. This scaled up to 3.6% for a worst-case scenario envisaged when three base stations within the same coverage area, are all transmitting the three sets of corrections).

However, it must be remembered that the IALA radiobeacon differential service has some significant benefits over the AIS Message 17 counterpart, two of which are listed below:

- Range of the IALA radiobeacon is minimally 50M and effectively much more, as opposed to a nominal 25M range for AIS.

- The shorter range of AIS would make it necessary to provide communication links between DGPS reference stations and AIS base stations (with availability figures to consider). The alternative and more reliable solution is for each AIS base station to have a reference station and integrity monitor, though this may have significant cost implications.

It must be noted that this study is based purely on successfully decoded messages and cannot account for lost or corrupted messages. It is possible that corrupted messages form a significant portion of the apparently available capacity. Further investigations are required to identify the proportion of lost or corrupted messages and the reasons behind any losses, such as timeslot synchronisation poor signal-to-noise ratio etc. This would require the acquisition of special analysis software.

The low network load may be a result of using DGPS for precise timing. There could be consequences for network loading if all users had to revert to less precise timing (e.g. standard GPS time).  This may be worth investigating.

The use of Message 17 would give the mariner two sources of differential corrections; either could be used for navigation. The AIS receiver gives priority to the position information from an external GNSS receiver, if it is differentially corrected and to the internal receiver if it is not.

# Conclusions

- AIS channel loading is quite low (18%), even in a busy sea area.

- Providing differential corrections for up to three GNSS would only impose a small additional loading (1.2%).

- In an area covered by three different AIS Base Stations, this additional loading could be 3.6%.