International Association of Lighthouse Authorities

MARITIME BUOYAGE SYSTEM

HISTORICAL BACKGROUND

As recently as 1976 there were more than thirty different buoyage systems in use world wide, many of these systems having rules in complete conflict with one another. This resulted in a situation particularly confusing at night when a mariner could be unexpectedly confronted by a light, the meaning of which was not clear. Such confusion was especially dangerous when the unidentified light was marking a new and as yet uncharted danger, such as a recent wreck. This left the mariner in doubt as to this proper course of action leading him to make a wrong and perhaps disastrous decision.

There has always been disagreement over the way in which buoy lights should be used since they first appeared towards the end of the 19th century. In particular, some countries favoured using red lights to mark the port hand side of channels and others favoured them for marking the starboard hand.

Another major difference of opinion revolved around the principles to be applied when laying out marks to assist the mariner. Most countries adopted the principle of the Lateral system whereby marks indicate the port and starboard sides of the route to be followed according to some agreed direction. However, several countries also favoured using the principle of Cardinal marks whereby dangers are marked by one or more buoys or beacons laid out in the quadrants of the compass to indicate where the danger lies in relation to the mark, this system being particularly useful in the open sea where the Lateral buoyage direction may not be readily apparent.

Over the years, many attempts were made to solve these differences of opinion but without success.  The nearest approach to international agreement on a unified system of buoyage was reached at Geneva in 1936. Unfortunately, this Agreement, drawn up under the auspices of the League of Nations, was never ratified due to the outbreak of World War II. The Agreement proposed the use of either Cardinal marks or Lateral marks but separated them into two different systems. It also provided for the use of the colour red on port hand marks and largely reserved the colour green for wreck marking.

At the end of World War II many countries found their aids to navigation destroyed and the process of rehabilitation had to be undertaken urgently. In the absence of anything better, the Geneva rules were adopted with or without variation to suit local conditions and the equipment available. This led to wide and sometimes conflicting differences particularly in the crowded waters of North Western Europe.

Much of the North and South American continents and some countries of the Pacific continued to favour “red to starboard” and used only a Lateral system of buoyage.

The whole unsatisfactory situation was well known to IALA, and as long ago as 11965, the Association set up an international Technical Committee to examine the problem and to suggest a solution.

There were three basic problems facing the Committee:

- the need to retain existing equipment as far as possible to avoid undue expense

- how were the colours green and red to be used when marking channels?

- the need to combine the Lateral and Cardinal rules.

Attempts to bring complete unity had little success. Fresh impetus was given to the task of the Committee by a series of disastrous wrecks in the Dover Strait area in 1971. These wrecks, situated in one lane of a traffic separation scheme, defied all attempts to mark them in a way all could readily understand.

To meet the conflicting requirements, it was thought necessary as a first step to formulate two systems, one using the colour red to mark the port hand side of the channels and the other using the colour red to mark the starboard hand side of channels. These were called System A and System B respectively.

The rules for System A which included both cardinal and lateral marks were completed in 1976 and agreed by the Inter-Governmental Maritime Consultative Organization (IMCO). The introduction of the System began in 10977 and its use has gradually spread throughout Europe, Australia, New Zealand, Africa, the Gulf and some Asian Countries.

The rules for System B were completed in early 1980 and these were felt to be suitable for application in the countries of North, Central and South America, Japan, Korea and The Philippines.

The rules for the two Systems were so similar that the IALA Executive Committee felt able to combine the two sets of rules into one, known as “The IALA Maritime Buoyage System”. This single set of rules allows Lighthouse Authorities the choice of using red to port, or red to starboard, on a regional basis, the two regions being known as Region A and Region B.

To achieve this single set of rules and to meet the needs of Region B countries, it was proposed to make certain small additions to the agreed System A rules. These additions were of minor nature and did not make any significant change to the System A buoyage already in course of introduction.

At a Conference convened by IALA in November 1980 with the assistance of IMCO and the International Hydrographic Organization (IHO), the Lighthouse Authorities form 50 countries and the representatives of 9 International Organisations concerned with aids to navigation met, and agreed to adopt the rules of the new combined System. The boundaries of the buoyage regions were also decided and illustrated on the map annexed to the rules. The Conference also underlined the need for cooperation between neighbouring countries and with Hydrographic Services in the introduction of the new System.

Although the maritime buoyage system has served the maritime community well since its inception in the 1970s, after the IALA Conference 2006 in Shanghai, China, it was determined to review the system in light of changes in the shipping environment and the development of digital aids to navigation.

The results of a widespread consultation found the fundamental principles of the MBS should be retained. However, due to changes in shipping patterns, innovations and technological developments, some enhancements to the MBS are required.

Ideally, a unified marking arrangement would, in principle, be desirable for Regions A and B.  However, almost all view this change as impractical, costly, detrimental to safety and possibly unachievable.

The most significant changes in this revision are the inclusion of the New Danger Mark, description of other aids to navigation specifically excluded from the original MBS, and the integration of electronic marks via radio transmission.

Thus the IALA Maritime Buoyage System will continue to help the Mariner of any nationality to fix his position and avoid dangers without fear of ambiguity, now and for the years to come.

International Association of Lighthouse Authorities

MARITIME BUOYAGE SYSTEM

And Other Aids to Navigation

GENERAL PRINCIPLES OF THE SYSTEM

[PREAMBLE] The responsibility for safe navigation resides with the mariner, through the appropriate use of aids to navigation in conjunction with official nautical documents and prudent seamanship, including voyage planning as defined in IMO resolution 893.  This booklet provides guidance on the Maritime Buoyage System and other aids to navigation.

The IALA Aids to Navigation system is comprised of two components: The Maritime Buoyage System and other aids to navigation.  This is primarily a physical system, however all of the marks described may be provided by electronic (digital) means.

Within this system there are 7 types of marks which may be used in combination. The mariner can easily distinguish between these marks by readily identifiable characteristics.

Lateral marks differ between Buoyage Regions A and B, as described below, whereas the other six (6) types of marks are common to both regions.

LATERAL MARKS

Following the sense of a “conventional direction of buoyage”, lateral marks in Region A utilize red and green colours by day and night to denote the port and starboard sides of channels respectively. However, in Region B these colours are reversed with red to starboard and green to port.

A modified lateral mark may be used at the point where a channel divides to distinguish the preferred channel, that is to say the primary route or channel that is so designated by an Authority.

CARDINAL MARKS

Cardinal marks indicate that the deepest water in the area lies to the named side of the mark. This convention is necessary even though for example, a North mark may have navigable water not only to the North but also East and West of it. The mariner will know he is safe to the North, but shall consult his chart for further guidance.

Cardinal marks do not have a distinctive shape but are normally pillar or spar. They are always painted in yellow and black horizontal bands and their distinctive double cone top-marks are always black.

An aide-memoire to their colouring is provided by regarding the top-marks as pointers to the positions of the black band(s):

- Top-marks pointing upward: black band above yellow band

- Top-marks pointing downward: black band below yellow band

- Top-marks pointing away from each other: black bands above and below a yellow band

- Top-marks pointing towards each other: black band with yellow bands above and below.

Cardinal marks also have a special system of flashing white lights. The rhythms are basically all “very quick” (VQ) or “quick” (Q) flashing but broken into varying lengths of the flashing phase.  “Very quick flashing” is defined as a light flashing at a rate of either 120 or 100 flashes per minute, “quick flashing” is a light flashing at either 60 or 50 flashes per minute.

The characters used for Cardinal marks will be seen to be as follows:

- North: Continuous very quick flashing or quick flashing;

- East: Three “very quick” or “quick” flashes followed by darkness;

- South: Six “very quick” or “quick” flashes followed immediately by a long flash, then darkness;

- West: Nine “very quick” or “quick” flashes followed by darkness.

The concept of three, six, nine is easily remembered when one associates it with a clock face. The long flash, defined as a light appearance of not less than 2 seconds, is merely a device to ensure that three or nine “very quick” or “quick” flashes cannot be mistaken for six.

It will be observed that two other marks use white lights. Each has a distinctive light rhythm that cannot be confused with the very quick or quick flashing light of the Cardinal marks.

ISOLATED DANGER MARK

The Isolated Danger mark is placed on a danger of small area that has navigable water all around it. Because the extent of the danger and the safe passing distance cannot be specified by these guidelines for all circumstances in which this mark may be used, the mariner shall consult his chart for guidance. Distinctive double black spherical top-marks and Group flashing (2) white lights, serve to associate Isolated Danger marks with Cardinal marks.

SAFE WATER MARKS

The Safe Water mark has navigable water all around it, but does not mark a danger. Safe Water marks can be used, for example, as fairway, mid-channel or landfall marks.

Safe Water marks have an appearance quite different from danger marking buoys. They are spherical, or alternatively pillar or spar with red and white vertical stripes and a single red spherical top-mark. Their lights, if any, are white using isophase, occulting, one long flash or Morse “A” rhythms.

SPECIAL MARKS

Special marks are used to indicate a special area or feature whose nature may be apparent from reference to a chart or other nautical publication. They are not generally intended to mark channels or obstructions.

Special marks are yellow. They may carry a yellow “X” top-mark, and any light used is also yellow. To avoid the possibility of confusion between yellow and white in poor visibility, the yellow lights of Special marks do not have any of the rhythms used for white lights.

Their shape will not conflict with that of navigational marks, this means, for example, that a special buoy located on the port hand side of a channel may be cylindrical, but will not be conical. Special marks may be lettered or numbered, and may also include the use of a pictogram to indicate their purpose.

NEW DANGER MARKS

“New Dangers” are newly discovered hazards, natural or man made, that are not yet shown in nautical publications, and may be indicated by:

- Exactly duplicating the normal mark until the information is sufficiently promulgated; or

- Using the Emergency Wreck Marking Buoy;

The Emergency Wreck Marking Buoy has blue and yellow vertical stripes in equal number, with a vertical/perpendicular yellow cross top-mark, and display a blue and yellow alternating light.

New Dangers Marks may be removed when the appropriate Authority is satisfied that information concerning the “New Danger” has been sufficiently promulgated or the new danger has been resolved.

A New Danger mark may carry a Racon coded Morse “D” or other radio transmitting device such as automatic identification systems.

OTHER MARKS

Other marks include lighthouses, sector lights, leading lines, major floating aids, and marks of non-lateral significance. These marks are intended to aid navigation as information to mariners, generally without regards to channel limits or obstructions.

- Lighthouses are fixed aids to navigation that display different colours and/or rhythms over designated arcs.

- Sector lights display different colours and/or rhythms over designated arcs. The colour of the light provides directional information to the mariner.

- Leading lines allow ships to be guided with precision along a portion of a straight route using the alignment of lights (leading lights) or marks (leading marks).

- Major floating aids include lightships and large navigational buoys intended to mark approaches from off shore areas. A light vessel may also display a white riding light to signify a vessel at anchor.

- Aids of non-lateral significance include those other minor aids that are outside of well defined channels and otherwise do not indicate the port and starboard sides of the route to be followed.

International Association of Lighthouse Authorities

MARITIME BUOYAGE SYSTEM

RULES

- GENERAL

  - Scope

The Maritime Buoyage System and Other Aids to Navigation provides rules that apply to all fixed and floating marks serving to indicate:

    - The lateral limits of navigable channels.

    - Natural dangers and other obstructions such as wrecks.

    - Landfall, course to steer, and other areas or features of importance to the mariner.

    - New dangers.

  - Types of marks

The Maritime Buoyage System and Other Aids to Navigation provides seven types of marks that may be used in combination:

    - Lateral marks, used in conjunction with a “conventional direction of buoyage”, generally used for well defined channels.  These marks indicate the port and starboard sides of the route to be followed.  Where a channel divides, a modified lateral mark may be used to indicate the preferred route.  Lateral marks differ between Buoyage Regions A and B as described in MBS Sections 2 and 8.

    - Cardinal marks, used in conjunction with the mariner's compass, to indicate where the mariner may find navigable water.

    - Isolated Danger marks to indicate isolated dangers of limited size that have navigable water all around them.

    - Safe Water marks to indicate that there is navigable water all around their position, e.g. mid-channel marks.

    - Special marks not primarily intended to assist navigation but to indicate an area or feature referred to in nautical documents.

    - New Danger marks to indicate a newly discovered hazard not yet shown in nautical documents that poses a potential danger to passing vessels.

    - Other marks used to provide information to assist navigation.

  - Method of characterising marks

The significance of the mark depends upon one or more of the following features:

    - By night, colour and rhythm of light.

    - By day, colour, shape, top-mark, with or without light (including colour and rhythm).

    - By electronic (digital) symbol.

- LATERAL MARKS

  - Definition of “conventional direction of buoyage”

The “conventional direction of buoyage”, which must be indicated in appropriate nautical documents, may be either:

    - The general direction taken by the mariner when approaching a harbour, river, estuary or other waterway from seaward, or

    - The direction determined by the proper authority in consultation, where appropriate, with neighbouring countries.  In principle it should follow a clockwise direction around land masses.

  - Buoyage Regions

There are two international Buoyage Regions A and B where lateral marks differ. These buoyage regions are indicated in Section 8.

  - Description of Lateral Marks used in Region A

|  | 2.3.1. Port hand Marks | 2.3.2. Starboard hand Marks |
| --- | --- | --- |
| Colour | Red | Green |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single red cylinder (can) | Single green cone, point upward |
| Light (when fitted) |  |  |
| Colour | Red | Green |
| Rhythm | Any, other than that described in section 2.3.3. | Any, other than that described in section 2.3.3. |

    - At the point where a channel divides, when proceeding in the “conventional direction of buoyage”, a preferred channel may be indicated by a modified Port or Starboard lateral mark as follows:

|  | 2.3.3.1. Preferred channel to Starboard | 2.3.3.2. Preferred channel to Port |
| --- | --- | --- |
| Colour | Red with one broad green horizontal | Green with one broad red horizontal |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single red cylinder (can) | Single green cone, point upward |
| Light (when fitted) |  |  |
| Colour | Red | Green |
| Rhythm | Composite group flashing (2 + 1) | Composite group flashing (2 + 1) |

  - Description of Lateral Marks used in Region B

|  | 2.4.1. Port hand Marks | 2.4.2. Starboard hand Marks |
| --- | --- | --- |
| Colour | Green | Red |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single green cylinder (can) | Single red cone, point upward |
| Light (when fitted) |  |  |
| Colour | Green | Red |
| Rhythm | Any, other than that described in section 2.4.3. | Any, other than that described in section 2.4.3. |

    - At the point where a channel divides, when proceeding in the “conventional direction of buoyage”, a preferred channel may be indicated by a modified Port or Starboard lateral mark as follows:

|  | 2.4.3.1. Preferred channel to Starboard | 2.4.3.2. Preferred channel to Port |
| --- | --- | --- |
| Colour | Green with one broad red horizontal | Red with one broad green horizontal |
| Shape (Buoys) | Cylindrical (can), pillar or spar | Conical, pillar or spar |
| Topmark (if any) | Single green cylinder (can) | Single red cone, point upward |
| Light (when fitted) |  |  |
| Colour | Green | Red |
| Rhythm | Composite group flashing (2 + 1) | Composite group flashing (2 + 1) |

  - General Rules for Lateral Marks

    - Shapes

Where lateral marks do not rely upon cylindrical (can) or conical buoy shapes for identification they should, where practicable, carry the appropriate topmark.

    - Numbering or lettering

If marks at the sides of a channel are numbered or lettered, the numbering or lettering shall follow the “conventional direction of buoyage”.

    - If necessary, synchronization of lights may be utilized.

- CARDINAL MARKS

  - Definition of Cardinal quadrants and marks

    - The four quadrants (North, East, South and West) are bounded by the true bearings NW-NE, NE-SE, SE-SW, SW-NW, taken from the point of interest.

    - A Cardinal mark is named after the quadrant in which it is placed.

    - The name of a Cardinal mark indicates that it should be passed to the named side of the mark.

  - Use of Cardinal Marks

A Cardinal mark may be used, for example:

    - To indicate that the deepest water in that area is on the named side of the mark.

    - To indicate the safe side on which to pass a danger.

    - To draw attention to a feature in a channel such as a bend, a junction, a bifurcation or the end of a shoal.

  - Description of Cardinal Marks

|  | 3.3.1. North Cardinal Mark | 3.3.2. East Cardinal Mark |
| --- | --- | --- |
| Topmark (a) | 2 black cones, one above the other, points upward | 2 black cones, one above the other, base to base |
| Colour | Black above yellow | Black with a single broad horizontal yellow band |
| Shape (Buoys) | Pillar or spar | Pillar or spar |
| Light (when fitted) |  |  |
| Colour | White | White |
| Rhythm | VQ or Q | VQ(3) every 5s or Q(3) every 10s |

|  | 3.3.3. South Cardinal Mark | 3.3.4. West Cardinal Mark |
| --- | --- | --- |
| Topmark (a) | 2 black cones, one above the other, points downward | 2 black cones, one above the other, point to point |
| Colour | Yellow above black | Yellow with a single broad horizontal black band |
| Shape (Buoys) | Pillar or spar | Pillar or spar |
| Light (when fitted) |  |  |
| Colour | White | White |
| Rhythm | VQ(6) + Long flash every 10s or Q(6) + Long flash every 15s | VQ(9) every 10s or Q(9) every 15s |

Note: (a) The double cone top-mark is a very important feature of every Cardinal mark by day, and should be used wherever practicable and be as large as possible with a clear separation between the cones.

- ISOLATED DANGER MARKS

  - Definition of Isolated Danger Marks

An Isolated Danger mark is a mark erected on, or moored on or above, an isolated danger that has navigable water all around it.

  - Description of Isolated Danger Marks

| Top-mark (b) | 2 black spheres, one above the other |
| --- | --- |
| Colour | Black with one or more broad horizontal red bands |
| Shape (Buoys) | Optional, but not conflicting with lateral marks; pillar or spar preferred |
| Light (when fitted) |  |
| Colour | White |
| Rhythm | Group flashing (2) |

Note: (b) The double sphere topmark is a very important feature of every Isolated Danger mark by day, and should be used wherever practicable and be as large as possible with a clear separation between the spheres.

- SAFE WATER MARKS

  - Definition of Safe Water Marks

Safe Water marks serve to indicate that there is navigable water all round the mark; these include centre line marks and mid-channel marks. Such a mark may also be used as an alternative to a Cardinal or a Lateral mark to indicate a landfall.

  - Description of Safe Water Marks

| Colour | Red and white vertical stripes |
| --- | --- |
| Shape (Buoys) | Spherical; pillar or spar with spherical topmark |
| Topmark (if any) | Single red sphere |
| Light (when fitted) |  |
| Colour | White |
| Rhythm | Isophase, occulting, one long flash every 10s or Morse “A” |

- SPECIAL MARKS

  - Definition of Special Marks

Marks used to indicate a special area or feature whose nature may be apparent from reference to a chart or other nautical publication. They are not generally intended to mark channels or obstructions; examples of Special Marks are:

    - Ocean Data Acquisition Systems (ODAS) marks.

    - Traffic separation marks where use of conventional channel marking may cause confusion.

    - Spoil Ground marks.

    - Military exercise zone marks.

    - Cable or pipeline marks.

    - Recreation zone marks.

    - Offshore renewable energy installations

    - Aquaculture

  - Description of Special Marks

| Colour | Yellow |
| --- | --- |
| Shape (Buoys) | Optional, but not conflicting with navigational marks |
| Topmark (if any) | Single yellow “X” shape |
| Light (when fitted) |  |
| Colour | Yellow |
| Rhythm | Any, other than those described in sections 3, 4 or 5 |

The use of pictograms is authorized, as defined by a competent authority.

  - Additional Special Marks

Special marks other than those listed in paragraph 6.1 and described in paragraph 6.2 may be established by the responsible administration to meet exceptional circumstances. These additional marks shall not conflict with navigational marks and shall be promulgated in appropriate nautical documents and the International Association of Lighthouse Authorities notified as soon as practicable.

- NEW DANGER MARK

  - Definition of New Danger Mark

The New Danger mark is used for newly discovered hazards not yet indicated in nautical documents. “New Dangers” include naturally occurring obstructions such as sandbanks or rocks or man-made dangers such as wrecks.

  - Marking of New Dangers

    - “New Dangers” shall be marked in accordance with these rules by exactly duplicating the normal mark, or by using the Emergency Wreck Marking Buoy. If the appropriate Authority considers the danger to be especially grave at least one of the marks shall be duplicated as soon as practicable.

    - If using a Cardinal or Lateral lighted mark for this purpose a VQ or Q light character shall be used.

    - Any duplicate mark shall be identical to its partner in all respects.

    - A “New Danger” may be marked by a Racon, coded Morse “D” showing a signal length of 1 nautical mile on the radar display, or other radio transmitting device, such as automatic identification systems.

    - The new danger mark may be removed when the appropriate Authority is satisfied that information concerning the “New Danger” has been sufficiently promulgated or the danger otherwise resolved.

    - Emergency Wreck Marking Buoy is described below:

| Colour | Blue/Yellow vertical stripes in equal number dimensions（minimum 4 stripes and maximum 8 |
| --- | --- |
| Shape (Buoys) | Cylinder or spar |
| Topmark (if any) | Vertical/perpendicular Yellow cross |
| Light (when fitted) |  |
| Colour | Yellow/blue alternating |
| Rhythm | One second of blue light and one second of yellow light with 0.5 sec. of darkness between |

- OTHER MARKS

  - LEADING LINES

    - Definition of Leading Lines

A group of two or more marks or lights, in the same vertical plane such that the navigator can follow the leading line at the same bearing.

    - Description of Leading Lines

Leading Line structures can be any colour or shape that provide a distinctive mark that cannot be confused with adjacent structures. The IALA Guidelines for the design of leading lines allow for the provision of lights for use by day and/or night and daymarks.

| Colour | No colour significance. Competent authority determines colour in accordance the background dominant colour at the location. |
| --- | --- |
| Shape (Buoys) | No shape significance. Square geometrical figures are recommended. |
| Topmark (if any) | Not used |
| Light (when fitted) |  |
| Colour | Any colour. Competent authority determines colour in accordance with the background dominant colour at the location. |
| Rhythm | Isophase Synchronized |

  - SECTOR LIGHTS

    - Definition of Sector Lights

A sector light is a fixed aid to navigation that displays a light of different colours and/or rhythms over designated arcs.

    - Description of Sector Lights

A sector light may be used:

- To provide directional information in a fairway

- To indicate a turning point, a junction with other channels, a hazard or something else of importance for the navigator

| Colour |  |
| --- | --- |
| Shape (Buoys) | None, light only |
| Topmark (if any) |  |
| Light (when fitted) |  |
| Colour | If using to mark channel limits follow convention for IALA region indicated in Section 9 |
| Rhythm |  |

  - LIGHTHOUSES

    - Definition of Lighthouses

A lighthouse is a fixed AtoN that displays different colours and/or rhythms over designated arcs.

    - Description of Lighthouses

A lighthouse is a major AtoN that houses a long or medium range light for identification by night and may provide a day mark for identification by day. A sector light may be used:

| Colour | Any specifically utilized elsewhere in the MBS, to allow readily identifiable |
| --- | --- |
| Shape (Buoys) | Lighthouse structures can be any shape or colour, generally designed to provide a distinctive day mark that cannot be confused with an adjacent structures |
| Topmark (if any) |  |
| Light (when fitted) |  |
| Colour | White, Red, or Green |
| Rhythm | Any not used to allow light to be readily identifiable. |

  - MAJOR FLOATING AIDS

    - Definition of Major Floating Aids

Major floating aids include light ships and large navigational buoys.

    - Description of Major Floating Aids

Major floating aids are generally deployed at critical locations, intended to mark approaches from off shore areas, where shipping traffic concentrations are high. May carry a Racon or other radio transmission device or sound signal.

| Colour |  |
| --- | --- |
| Shape (Buoys) |  |
| Topmark (if any) |  |
| Light (when fitted) |  |
| Colour | White |
| Rhythm |  |

  - AIDS OF NON-LATERAL SIGNIFICANCE

    - Definition of Aids of Non-Lateral Significance

Minor aids outside of well defined channels and otherwise do not indicate the port and starboard sides of the route to be followed.

    - Description of Aids of Non-Lateral Significance

[TO BE DETERMINED]

| Colour |  |
| --- | --- |
| Shape (Buoys) |  |
| Topmark (if any) |  |
| Light (when fitted) |  |
| Colour | White |
| Rhythm |  |

- INTERNATIONAL BUOYAGE REGIONS A AND B

There are two international Buoyage Regions A and B where Lateral marks differ as described in Section 2. The current geographical divisions of these two Regions are shown on the following world map.