| Radiocommunication Study Groups |  |
| --- | --- |
| Source: Documents 5B/115, 5B/129 Subject: WRC-11 Agenda item 1.10 | Document 5B/TEMP/98-E |
|  | 6 November 2008 |
|  | English only |
| Working Party 5B |  |
| draft new report itu-r m.[sat-ais] |  |
| Improved satellite detection of AIS |  |

1	Introduction

e-NAV6/10/14

Satellite detection of AIS messages has been requested by administrations. In response, studies were made that were documented in Report ITU-R M.2084 (2006) “Satellite detection of automatic identification system messages.” The Report considered the operational and technical characteristics of the shipborne AIS and the requirements and limitations of reception of AIS messages from satellites. Further studies were proposed to deal with the technical limitations that hinder detection of AIS-equipped target ships in high-traffic areas (e.g. the Dover Straits, Singapore, North East United States, and the northern Gulf of Mexico) on the designated AIS VDL (VHF Data Link). The technical limitations specifically cited were:

1)	the length of the AIS message in the time slot (insufficient time buffer for the satellite detection range);

2)	the large number of messages in the satellite antenna footprint (excess reuse of the time slots in the VDL as detected by the satellite);

3)	the difficulty satellite AIS has in distinguishing signals between AIS messages and communications from terrestrial services within the satellite antenna footprint (coverage pattern).

This Report on further studies shows the need for a special AIS message that is shortened in length and can be transmitted on a special reporting schedule on two other frequencies when those frequencies are designated. Accordingly, a new Recommendation ITU-R M.1371 AIS message (Message 27) and new Appendix 18 frequencies for this service are proposed.

# 2	Solving the problem of overlapping messages (blurred reception)

The differences in propagation delay between vessels and the satellite causes the AIS messages to overlap. Thus, the slot composition must be adjusted to increase the time buffer so that the AIS transmissions can be received by the satellite in separate slots without a time overlap. Table 1 serves as the basis for the proposed new Message 27 slot composition shown in Table 5 based on an allowance for the “propagation time delay difference (bits)”. Tables 2-4 show the technical parameters, reporting intervals and data packet structure of the AIS messages that are currently approved.

TABLE 1

Satellite AIS reception technical parameters

| AIS satellite propagation calculations |  |  |  |
| --- | --- | --- | --- |
| Constants: |  |  |  |
| Speed of light (metres/sec) | 299 792 458 |  |  |
| AIS bit-time @ 9600 bps (milliseconds) | 0.1041667 |  |  |
| Nautical mile (km per nm) | 1.852 |  |  |
| Calculations: | Orbit #1 | Orbit #2 | Orbit #3 |
| Satellite orbital altitude (km/nm) | 600/324 | 948/512 | 1 000/540 |
| Slant range to horizon (km/nm) | 2 831/1 529 | 3 604/1 946 | 3 709/2 003 |
| Ground range to horizon (km/nm) | 2 664/1 438 | 3 281/1 772 | 3 359/1 814 |
| Difference in propagation distance (km/nm) | 2 231/1 205 | 2 656/1 434 | 2 709/1 463 |
| Propagation time delay difference (milliseconds) | 7.44 | 8.86 | 9.04 |
| Propagation time delay difference (bits) | 71 | 85 | 87 |
| Typical satellite visibility (minutes)(1) | 10.2 | 13.6 | 14.0 |
| (1) These satellite visibility periods are for a typical overhead pass of the satellite based on a circular, polar-orbiting satellite for the indicated satellite altitudes and a target ship located at 40º Latitude. Typical visibility periods will vary depending on a number of factors including satellite inclination angle and target ship latitude. |  |  |  |

TABLE 2

Overview of shipboard AIS technical parameters

| AIS parameters | Values |
| --- | --- |
| Frequencies | 161.975 and 162.025 MHz |
| Channel bandwidth | 25 kHz |
| Platforms | Class A ships, Class B ships, coast stations, navigation aids |
| Power | 12.5 W (Class A); 2 W (Class B) |
| Antenna type(1) | 1/2 λ dipole |
| Antenna gain(1) | 2 dBi with cosine-squared vertical elevation pattern; Minimum gain = –10 dBi |
| Cable loss(1) | 3 dB (estimated) |
| Receiver sensitivity | –107 dBm for 20% packet error rate (PER) (minimum) –109 dBm for 20% PER (typical) |
| Modulation | 9 600 bits per second GMSK |
| Multiple access mode | TDMA (self-organizing, random, fixed and incremental) |
| TDMA frame length | 1 min; 2 250 time-slots |
| TDMA slot length | 26.7 ms; 256 bits (see Table 4) |
| Message types | 26 types |
| Message length | 1 to 5 slots with 1 slot being the dominant type |
| Periodic message interval | 2 s to 6 min transmit intervals (see Table 3) |
| Required D/U protection ratio | 10 dB at PER = 20%(2) |
| (1) Typical parameters not defined in Recommendation ITU-R M.1371. (2) Parameter specified in IEC 61993-2. |  |

TABLE 3

Reporting intervals for AIS messages

| AIS platform | Reporting interval |
| --- | --- |
| Dynamic information: |  |
| Coast station | 3 1/3 to 10 s interval (10 s nominal) |
| Class A ship | 2 s to 3 min interval (approximately 7 s average) (see Table 4) |
| Class B ship | 5 s to 3 min interval (30 s nominal) |
| Search and rescue aircraft | 10 s interval |
| Aid to navigation | 3 min interval |
| Static and voyage information | 6 min interval |
| Safety and administrative messages | As required |
| Data message | As required |

TABLE 4

Default AIS data packet bit structure

| Power ramp up | 8 bits |  |
| --- | --- | --- |
| Training sequence | 24 bits | Necessary for synchronization |
| Start flag | 8 bits |  |
| Data field | 168 bits | Default length |
| Cyclic redundancy code | 16 bits | Necessary for error detection |
| End flag | 8 bits |  |
| Buffer | 24 bits (typically, the last 20-bit positions are empty) | Necessary to accommodate bit stuffing, propagation and repeater delays, and jitter |
| Total | 256 bits |  |

TABLE 5

Modified AIS packet bit structure for satellite reception

| Slot composition | Bits | Notes |
| --- | --- | --- |
| Ramp up | 8 | Standard |
| Training sequence | 24 | Standard |
| Start flag | 8 | Standard |
| Data field | 96 | Data field is 168 bits for other single-slot messages. This field is shortened by 72 bits to support the satellite AIS system buffer |
| CRC | 16 | Standard |
| End flag | 8 | Standard |
| Satellite AIS system buffer | 96 | Bit stuffing = 4 bits Synch jitter (mobile station) = 3 bits Synch jitter (mobile/satellite) = 1 bit Propagation time delay difference = 87 bits Spare = 1 bit |
| Total | 256 | Standard (NOTE – Only 160 bits are used in the 17 millisecond transmission) |

Thus the proposed data field for Message 27 is shown in Table 6.

TABLE 6

Proposed new data field for AIS satellite detection Message 27

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Message ID | 6 | Identifier for this message (similar to all other messages) |
| Repeat indicator | 2 | Repeat indicator value should be 3 |
| User ID | 30 | MMSI number |
| Position accuracy | 1 | As defined for Message 1 |
| RAIM flag | 1 | As defined for Message 1 |
| Navigational status | 4 | As defined for Message 1 |
| Longitude | 18 | Longitude in 1/10 min (180º, East = positive, West = negative |
| Latitude | 17 | Latitude in 1/10 min (90º, North = positive, South = negative |
| SOG | 6 | Knots (0-62); 63 = not available = default |
| COG | 9 | Degrees (0-359); 511 = not available = default |
| Status of current GNSS position | 1 | 0 = Position is the current GNSS position; 1 = Reported position is not the current GNSS position = default |
| Spare | 1 | Set to zero, to preserve byte boundaries |
| Total number of bits | 96 |  |

# 3	Solving the problem of the large number of messages in the satellite antenna footprint

The large number of messages in the satellite antenna footprint (excess reuse of the time slots in the VDL as detected by the satellite) is attributable to both the large number of ships and the reporting rate. Studies show that 100% of AIS Class A ships can be detected if:

1)	an appropriate reporting rate for the AIS Class A ships is selected;

2)	coastal ships within range of an AIS base station are eliminated; and

3)	the AIS Class B is eliminated from satellite reception.

## 3.1	Selecting a reporting interval for satellite reception of the AIS Class A

The proposed reporting interval for the proposed new Message 27 is 3 minutes in order to give the best detection probability at a typical observation time of 13.6 minutes as shown in Fig. 1. The reports from administrations on simulation results (for AIS Class A) also support this proposed reporting interval. Note that a 2-channel system (refer to § 4 for the proposed use of the two channels 75 and 76) doubles the loading capacity and provides interference protection.

FIGURE 1

Optimizing the reporting rate for AIS Class A ships

3-minute interval

6-minute interval

3-min; 2-channels

Assumptions:

1)	Uniform ship distribution

2)	Target ship located at 40 deg. latitude

3)	160 bit packet length (256 bits normal packet – 96 bits buffer for Message 27)

4)	Polar orbiting satellite with 13.6 minutes average satellite visibility period.

Conclusions:

1)	A 3-minute reporting interval can achieve 100% detection of AIS Class A ships

2)	AIS Class A already has an available 3-minute schedule (refer to Table 3)

3)	A 2-channel system increases loading capacity and provides interference protection.

## 3.2	Eliminating coastal ships within range of an AIS base station

An AIS base station transmits a special Base Station Message 4 that is received by all ships within radio range of that station. Since that AIS base station presumably receives AIS position reports from all ships within that range, it is not necessary for those ships to transmit the proposed AIS Satellite Message 27. Therefore, it is proposed that when a ship receives an AIS Base Station Message 4, the ship should reset the 3-minute message timer for the proposed Message 27. This provision will greatly improve the probability of detection by reducing the number of reports.

## 3.3	Eliminating the AIS Class B from the satellite reception

Reports from administrations on simulation results indicate that both AIS Class A transmissions at 12.5 Watts and Class B “CS” transmissions at 2 Watts have sufficient signal margins to support satellite AIS reception. However, the satellite footprint is too wide to accommodate the expected combined population of both classes of AIS. Furthermore, some administrations have stated the need, based on simulation results, to request that special frequencies be designated for transmitting the proposed new Message 27. Therefore, this proposal is to add the new proposed Message 27 to the AIS Class A only with a message priority level 4 and on the other frequencies. In that case, the AIS Class A equipment would not be required to receive on those frequencies and the reporting interval could be implemented with RATDMA based solely on the activity of AIS 1 and AIS 2 without the need to monitor the satellite-designated channel to further qualify candidate time slots for transmission.

# 4	Operating frequencies for satellite detection of AIS

Separate frequencies for satellite detection of AIS should be considered from within Appendix 18 because the tuning range of the shipborne AIS Class A is limited to these frequencies. Also, with respect to possible additional AIS frequencies, Report ITU-R M.2084 indicated that the interference environment resulting from the existing services in those bands must be taken into account in determining the feasibility of accommodating satellite AIS in any given band or channel, due to the large satellite antenna footprint that overlaps both land and sea. Examination of Appendix 18 indicates that only 3 frequencies (channels 16, 75 and 76) are exclusively dedicated to maritime use and restricted from terrestrial use on a global basis. Footnote n), referring to channels 75 and 76, currently limits power to 1 Watt for radio transmissions because they are adjacent channels to channel 16 (the distress and calling channel). However, transmission of Message 27 at 12.5 Watts only once every 3 minutes for 17 milliseconds, alternating between channels 75 and 76, would not interfere with voice communications on any of these channels 16, 75 and 76, and it would be detectable over a 1 Watt radio transmission. It should be noted that at least one administration uses channels 75 and 76 for low-power (1 Watt) maritime communications in-port.

## 4.1	Mathematical analysis of impact to channels 16, 75 and 76

Studies (the JSC Report) have shown that AIS transmissions do not significantly degrade voice communications on the adjacent channels due to the shortness of the duration of one AIS time slot compared to the time intervals used by human speech. The JSC Report used the metric of “voice articulation score” (VAS) to make this assessment, and it concluded that the VAS on the channel

adjacent to the AIS was 90-95% for the normal reporting interval of the shipborne AIS. In this particular case, the proposed new satellite AIS Message 27 is significantly shorter (only 17 milliseconds) than one AIS time slot (27 milliseconds), and the interval of the proposed transmission is significantly less (only once every 3 minutes) than the AIS reporting interval (2-10 seconds). It is unlikely that this short burst will open the squelch of shipborne VHF radios. The burst may possibly be detected during the reception of a voice call from a distant station, but the duration of the proposed burst will not spoil the detect-ability of human speech, based on the VAS metrics used in the JSC Report.

Since the coastal (covered by an AIS base station) SOLAS ships (AIS Class A) and the non-SOLAS ships (AIS Class B) are proposed to be eliminated, the effects of the SOLAS ships at sea can be assessed on their potential deterioration of the voice communications quality on the shared channels (75 and 76) and the adjacent channel (16). This analysis is based on the aggregate duty cycle of the potential radio interference from the total number of SOLAS ships in close proximity (ship-ship radio range) of each other.

For the adjacent channel 16, the required (IEC 61097-3) adjacent channel rejection ratio (70 dB) of the receiver reduces this to a co-site interference problem, or, at most, a problem of two ships passing in the shipping lanes. In this case, the deterioration of the communications on channel 16 is based on the duty cycle (DN) of the proposed Message 27 transmissions by two ships. DN = N(B/T), where N = number of adjacent ships (2), B = burst length (0.017 seconds) and T = time interval between scheduled transmissions (180 seconds). Thus, for the effect on channel 16, D2 = 2(0.017/180) = 0.000189 = 0.0189%, and from the VAS metrics in the JSC Report, this would be imperceptible to a human listener on the radio channel.

For the proposed two shared channels 75 and 76, the duty cycle (DN) on each channel from the proposed Message 27 (independent of voice communications on these channels) is DN = N(B/T), where N = number of ships (ships far offshore, excluding coastal ships) within ship-ship radio range of each other (this is an assumed distance of 5 NM, based on the 1 Watt power level setting for voice communications on these channels), B = burst length (0.017 seconds) and T = time interval between scheduled transmissions on each of the two channels (360 seconds). Within a 5 NM radius in the offshore shipping lanes, assuming 1 NM minimum separations, there could be at most 10 ships, and that would result in D10 = 10(0.017/360) = 0.000472 = 0.0472%, and this would result in a very high VAS (voice articulation score) using the metrics set out in the JSC Report.

Note that since the ships that are within range of an AIS base station (assumed to be 50 NM) are eliminated from this scenario, any coastal assignments of the channels 75 and 76 (assumed to be limited to 1-Watt power level) are not affected and thus are of no consequence to this analysis.

# 5	Summary

This Report addresses the technical limitations cited in Report ITU-R M.2084 that invited further studies. The possible solutions reached herein are:

1)	A special short AIS message (proposed Message 27, of only 96 bits) that is tailored for satellite reception would solve the problem of blurred reception.

2)	A special reporting interval (proposed 3 minutes) is needed for the satellite AIS message.

3)	Ships within range of an AIS base station should suppress transmission of this message.

4)	Separate operating frequencies in addition to AIS 1 and AIS 2 are needed that are not subject to terrestrial use.

5)	Frequencies should be considered only from Appendix 18 due to the limited tuning range of the shipborne AIS.

6)	Appendix 18 contains only 3 frequencies (channels 16, 75 and 76) that are exclusively dedicated to maritime use (channels 75 and 76 are proposed to be shared with this service). This proposal meets the intent of footnote n) to Appendix 18 for interference mitigation.

7)	Satellite detection of the shipborne AIS should be limited to the AIS Class A (SOLAS Class) because the AIS Class B population is too large to be included.

8)	Recommendation ITU-R M.1371-3 should be revised to add the proposed Message 27 along with its transmissions on the designated channels 75 and 76, and the AIS Class A equipment should be updated to add this message to facilitate improved satellite AIS detection.

Additional studies on AIS satellite detection are encouraged by administrations. Annex 1 provides an example.

Annex 1

Technical alternative for further consideration for satellite detection
of AIS messages

# 1	Solving the problem of overlapping messages (blurred reception)

The differences in propagation delay between vessels and the satellite causes the AIS messages to overlap.  Table 1 serves as the basis to evaluate the propagation time delay difference. Tables 2-4 show the technical parameters, reporting intervals and data packet structure of the AIS messages that are currently approved.

TABLE 1

Satellite AIS reception technical parameters

| AIS satellite propagation calculations |  |  |  |
| --- | --- | --- | --- |
| Constants: |  |  |  |
| Speed of light (metres/sec) | 299792458 |  |  |
| AIS bit-time @ 9600 bps (milliseconds) | 0.1041667 |  |  |
| Nautical mile (km per nm) | 1.852 |  |  |
| Calculations: | Orbit #1 | Orbit #2 | Orbit #3 |
| Satellite orbital altitude (km/nm) | 600/324 | 948/512 | 1000/540 |
| Slant range to horizon (km/nm) | 2831/1529 | 3604/1946 | 3709/2003 |
| Ground range to horizon (km/nm) | 2664/1438 | 3281/1772 | 3359/1814 |
| Difference in propagation distance (km/nm) | 2231/1205 | 2656/1434 | 2709/1463 |
| Propagation time delay difference (milliseconds) | 7.44 | 8.86 | 9.04 |
| Propagation time delay difference (bits) | 71 | 85 | 87 |
| Typical satellite visibility (minutes) | 10.2 | 13.6 | 14.0 |
| Effect of angle nadir-satellite-user (Altitude 1000km) : |  |  |  |
| Nadir-Satellite-User angle | 60° | 55° | 50° |
| Slant range to horizon (km/nm) | 3709/2003 | 2195/1185 | 1787/965 |
| Difference in propagation distance (km/nm) | 2709/1463 | 1195/645 | 787/425 |
| Propagation time delay difference (milliseconds) | 9.04 | 3.99 | 2.63 |
| Propagation time delay difference (bits) | 87 | 38 | 25 |

TABLE 2

Overview of shipboard AIS technical parameters

| AIS parameters | Values |
| --- | --- |
| Frequencies | 161.975 and 162.025 MHz |
| Channel bandwidth | 25 kHz |
| Platforms | Class A ships, Class B ships, coast stations, navigation aids |
| Power | 12.5 W (Class A); 2 W (Class B) |
| Antenna type(1) | 1/2 λ dipole |
| Antenna gain(1) | 2 dBi with cosine-squared vertical elevation pattern; Minimum gain = –10 dBi |
| Cable loss(1) | 3 dB (estimated) |
| Receiver sensitivity | –107 dBm for 20% packet error rate (PER) (minimum) –109 dBm for 20% PER (typical) |
| Modulation | 9 600 bits per second GMSK |
| Multiple access mode | TDMA (self-organizing, random, fixed and incremental) |
| TDMA frame length | 1 min; 2 250 time-slots |
| TDMA slot length | 26.7 ms; 256 bits (see Table 4) |
| Message types | 26 types |
| Message length | 1 to 5 slots with 1 slot being the dominant type |
| Periodic message interval | 2 s to 6 min transmit intervals (see Table 3) |
| Required D/U protection ratio | 10 dB at PER = 20%(2) |
| (1) Typical parameters not defined in Recommendation ITU-R M.1371. (2) Parameter specified in IEC 61993-2. |  |

TABLE 3

Reporting intervals for AIS messages

| AIS platform | Reporting interval |
| --- | --- |
| Dynamic information: |  |
| Coast station | 3 1/3 to 10 s interval (10 s nominal) |
| Class A ship | 2 s to 3 min interval (approximately 7 s average) (see Table 4) |
| Class B ship | 5 s to 3 min interval (30 s nominal) |
| Search and rescue aircraft | 10 s interval |
| Aid to navigation | 3 min interval |
| Static and voyage information | 6 min interval |
| Safety and administrative messages | As required |
| Data message | As required |

TABLE 4

Default AIS data packet bit structure

| Power ramp up | 8 bits |  |
| --- | --- | --- |
| Training sequence | 24 bits | Necessary for synchronization |
| Start flag | 8 bits |  |
| Data field | 168 bits | Default length |
| Cyclic redundancy code | 16 bits | Necessary for error detection |
| End flag | 8 bits |  |
| Buffer | 24 bits (typically, the last 20-bit positions are empty) | Necessary to accommodate bit stuffing, propagation and repeater delays, and jitter |
| Total | 256 bits |  |

The 32 last bits of the AIS message (end flag + spare bits) presented in Table 4 can be overlapped.

Considering a margin of 8 bits for jitters, it remains a margin of 24 bits for propagation delay difference. It means that two messages sent by ships with a nadir-satellite-ship angle under 49.5° will eventually collide if they are sent on the same TDMA slot, but they will never overlap one on the other if they are sent on different TDMA slots.

The interest of a relatively directive antenna is then twofold:

–	it eliminates the blurred reception;

–	it improves the probability of detection, the diminution of the coverage being more than compensated by the diminution of the instantaneous collision.

Even considering a large antenna, the overlapping case happens when a signal emitted from a ship with a nadir-satellite-ship angle over 49.5° is sent on a TDMA slot so that at least an other ship closed to nadir send is message on the following TDMA slot.

In this case, it is interesting to note that the Doppler shift and the power are different between the two signals, and that the beginning of a message (ramp up, training sequence, start flag) are common to all messages, allowing correlation. If only one message has been sent from nadir, it is possible by correlation to determine the phase, power and time of arrival of its training sequence, to remove it from the collision, and then to end the demodulation of the first received signal.

The overlapping probability is then relatively reduced.

In order not to reduce the data delivered in AIS messages, two solutions are possible:

–	reduce the on-satellite antenna beamwidth to keep the same message than for ground;

–	modify the order of the message to ensure the demodulation of the most interesting part, as described in Tables 5 and 6.

TABLE 5

Modified AIS packet bit structure for satellite reception

| Slot composition | Bits | Notes |
| --- | --- | --- |
| Ramp up | 8 | Standard |
| Training sequence | 24 | Standard |
| Start flag | 8 | Standard |
| Data field | 184 | See Table 6 |
| End flag | 8 | Standard. Propagation time delay difference = 8 bits |
| Buffer | 24 | Bit stuffing = 4 bitsSynch jitter (mobile station) = 3 bitsSynch jitter (mobile/satellite) = 1 bitPropagation time delay difference = 16 bits |
| Total | 256 | Standard |

Thus the proposed data field for Message 27 is shown in Table 6.

TABLE 6

Proposed new data field for AIS satellite detection Message 27

| Parameter | Number of bits | Description |
| --- | --- | --- |
| Core message | 96 | Message ID, MMSI number, Latitude, Longitude, Speed, Course, … |
| CRC 16 | 16 | CRC on core message |
| Information message | 72 | Destination, … |
| Total number of bits | 184 |  |

# 3	Solving the problem of the large number of messages in the satellite antenna footprint

The large number of messages in the satellite antenna footprint (excess re-use of the time slots in the VDL as detected by the satellite) is attributable to both the large number of ships and the reporting rate. Studies show that 100% of AIS Class A ships can be detected if:

1)	an appropriate reporting rate for the AIS Class A ships is selected; and

2)	the AIS Class B is eliminated from satellite reception.

## 3.1	Selecting a reporting interval for satellite reception of the AIS Class A

The proposed reporting interval for the proposed new Message 27 is 3 minutes in order to give the best detection probability at a typical observation time of 13.6 minutes as shown in Fig. 1. The reports from administrations on simulation results (for AIS Class A) also support this proposed reporting interval. Note that the use of two frequencies (refer to § 4 for the proposed use of the two channels 75 and 76) will double this capacity and also provide added protection from interference due to possible problems on a single channel.

FIGURE 1

Optimizing the reporting rate for AIS Class A ships

Assumptions:

1)	Uniform ship distribution

2)	Target ship located at 40 deg. latitude

3)	160 bit packet length (256 bits normal packet – 96 bits buffer for Message 27)

4)	Polar orbiting satellite with 13.6 minutes average satellite visibility period.

Conclusions:

1)	A 3-minute reporting interval can achieve 100% detection of AIS Class A ships

2)	AIS Class A already has an available 3-minute schedule (refer to Table 3).

## 3.2	Improving the multi-access

Within a circle with a diameter around 20 Nm, the communication is possible, which allows the SOTDMA access. But in the satellite field of view, there is currently no coordination at a global scale, so that the access between cells may be seen as random access.

An interesting solution would be to take benefit of the fact that the emitters know their position (thanks to their GNSS receiver), for instance by attributing to geographical zones a set of TDMA slots. A good shape for these zones would be alveolus, like in a beehive, with diameter 10 Nm, so that the attribution of the TDMA slots inside the zone could be ensured by the same protocol than the attribution of slots for classical SOTDMA. To improve the performance, it is possible to attribute to each alveolus a different number of slots and to dynamically modify this number of slots in relation with the traffic. Another improvement is to attribute the same slots to different zones in the world that can not be simultaneously seen by a satellite, or seen with a Doppler shift different enough to enable demodulation.

Considering a coverage of the constellation of 35% and a required reporting period of 3 hours, with a random access with a 3 minute reporting interval, a probability of detection of 95% is achieved with 13 500 ships in the footprint (without considering blurred reception). Without collision, to reach the same probability, 7 messages shall be sent each 3 hours ((1-35%)^7 < 5% ), so that a reporting interval of 1 500s (25 minutes) can be chosen. The number of ships which may be seen is then 1 500/0.0267=56 250 ships. Pessimistically considering that 25% of the slots are lost because of the constraint to attribute different slots in the satellite field of view and that again 25% of the slots are lost because all slots attributed to the alveolus are not used, there is still a capacity gain by a factor over 2.

This calculation shows that, even with pessimistic hypothesis, such method would enable a gain of capacity of a factor 2.

In case of use of a 3rd AIS frequency, it would be interesting to take benefits of the necessary retrofit of AIS equipment to propose such efficient protocol instead random access: this would be beneficial for the maritime end users, and would avoid a predictable mid-term saturation of this 3rd channel.

Thus, in addition to the decision of adding a third frequency, the decision to open a study group for the definition of an efficient multi-access protocol for space-based AIS should be taken.

## 3.3	Eliminating the AIS Class B from the satellite reception

Reports from administrations on simulation results indicate that both AIS Class A transmissions at 12.5 Watts and Class B “CS” transmissions at 2 Watts have sufficient signal margins to support satellite AIS reception. However, the satellite footprint is too wide to accommodate the expected combined population of both classes of AIS. Furthermore, some administrations have stated the need, based on simulation results, to request that special frequencies be designated for transmitting the proposed new Message 27. Therefore, this proposal is to add the new proposed Message 27 to the AIS Class A only with a message priority level 4 and on the other frequencies. In that case, the AIS Class A equipment would not be required to receive on those frequencies and the reporting interval could be implemented with RATDMA based solely on the activity of AIS 1 and AIS 2 without the need to monitor the satellite-designated channel to further qualify candidate time slots for transmission.

# 4	Operating frequencies for satellite detection of AIS

Separate frequencies for satellite detection of AIS should be selected from within Appendix 18 because the tuning range of the shipborne AIS Class A is limited to these frequencies. Also, with respect to possible additional AIS frequencies, Report ITU-R M.2084 indicated that the interference environment resulting from the existing services in those bands must be taken into account in determining the feasibility of accommodating satellite AIS in any given band or channel, due to the large satellite antenna footprint that overlaps both land and sea. Examination of Appendix 18 indicates that only 3 frequencies (channels 16, 75 and 76) are exclusively dedicated to maritime use and restricted from terrestrial use on a global basis. Footnote n), referring to channels 75 and 76, currently limits power to 1 Watt for radio transmissions because they are adjacent channels to channel 16 (the distress and calling channel). However, transmission of Message 27 at 12.5 Watts only once every 3 minutes for 17 milliseconds, alternating between channels 75 and 76, would not interfere with voice communications on any of these channels 16, 75 and 76, and it would be detectable over a 1 Watt radio transmission. It should be noted that at least one administration uses channels 75 and 76 for low-power (1 Watt) maritime communications in-port.

## 4.1	Mathematical analysis of impact to channels 16, 75 and 76

Studies (the JSC Report) have shown that AIS transmissions do not significantly degrade voice communications on the adjacent channels due to the shortness of the duration of one AIS time slot compared to the time intervals used by human speech. The JSC Report used the metric of “voice articulation score” (VAS) to make this assessment, and it concluded that the VAS on the channel adjacent to the AIS was 90-95% for the normal reporting interval of the shipborne AIS. In this particular case, the proposed new satellite AIS Message 27 is significantly shorter (only 17 milliseconds) than one AIS time slot (27 milliseconds), and the interval of the proposed transmission is significantly less (only once every 3 minutes) than the AIS reporting interval (2-10 seconds). It is unlikely that this short burst will open the squelch of shipborne VHF radios. The burst may possibly be detected during the reception of a voice call from a distant station, but the duration of the proposed burst will not spoil the detect-ability of human speech, based on the VAS metrics used in the JSC Report.

For SOLAS ships underway (not in port) it is also important to consider the communications quality based in the possible deterioration of the SINAD (signal, interference, noise and distortion).

The duty cycle (D) of the proposed Message 27 is: D = B/T, where B = burst length (0.017 seconds) and T = time interval (180 seconds).

Thus, D = 0.017/180 = 0.000094.

This equates to a SINAD value for radiocommunications of:

SINAD = 10 log (1/(DxN), where D = duty cycle of the Message 27, and N = the number of ships that are significantly closer than the two ships carrying on the voice communications.

Thus, SINAD = 40.2 dB for N = 1, and SINAD = 20.08 dB for N = 104.

The SINAD value used by IEC for receiver sensitivity testing is 20 dB. For SOLAS ships underway (not in port), it is not feasible to have enough ships (more than N = 104) in the near vicinity (within the ship-ship radio transmission range, approximately 20 nm) to cause the SINAD for a desired voice signal to be reduced below 20 dB.

# 5	Summary

This Report addresses the technical limitations cited in Report ITU-R M.2084 that invited further studies. The possible solutions reached herein are:

1)	A special AIS message (proposed Message 27,) that is tailored for satellite reception is a solution to solve the problem of blurred reception.

2)	When using a 3rd AIS frequency, a special reporting interval (proposed 3 minutes) allows the detection of Class A equipments by satellite with a good performance. In such case, it would be interesting to take benefits of the necessary retrofit of AIS ships equipment to

propose a more specific multi-access protocol instead of random access. This would be beneficial for the maritime end users, would avoid a predictable mid-term saturation of this 3rd channel, and would eventually allow the detection of Class B ships.

3)	Separate operating frequencies in addition to AIS 1 and AIS 2 are needed that are not subject to terrestrial use.

4)	Frequencies should be taken only from Appendix 18 due to the limited tuning range of the shipborne AIS.

5)	Appendix 18 contains only 3 frequencies (channels 16, 75 and 76) that are exclusively dedicated to maritime use (channels 75 and 76 should be designated to be shared with this service). Accordingly, footnote n) to Appendix 18 should be modified to facilitate this sharing arrangement.

6)	The inclusion of AIS Class B population in the population of ships that can be tracked by space-based AIS would depend on the performance achievable with the chosen multi-access algorithm.

7)	Recommendation ITU-R M.1371-3 should be revised to add the proposed Message 27 along with its transmissions on the designated channels 75 and 76, and at least the new AIS Class A equipments should be updated to add this message to facilitate improved satellite AIS detection.

8)	Accordingly with item 2, the decision to open a study group for the definition of an efficient multi-access protocol for space-based AIS, for the 3rd AIS potential frequency, should be taken.

______________

JSC Report (JSC-PR-04-007) “EMC Analysis of Universal Automatic Identification and Public Correspondence Systems in the Maritime VHF Band” primarily addresses the effects of AIS on the VPC service, but the information is useful in assessing this type of co-channel and adjacent channel interference. This document is freely available on the United States Coast Guard Navcen website: http://www.navcen.uscg.gov/enav/ais/JSC-PR-04-007.pdf.

These satellite visibility periods are for a typical overhead pass of the satellite based on a circular, polar-orbiting satellite for the indicated satellite altitudes and a target ship located at 40º Latitude. Typical visibility periods will vary depending on a number of factors including satellite inclination angle and target ship latitude.

JSC Report (JSC-PR-04-007) “EMC Analysis of Universal Automatic Identification and Public Correspondence Systems in the Maritime VHF Band” primarily addresses the effects of AIS on the VPC service, but the information is useful in assessing this type of co-channel and adjacent channel interference. This document is freely available on the United States Coast Guard Navcen website.