# PNT aspects of the AIS (Version 2)

Reference is made to the notion of a future AIS (Version 2), and the list of requirements for such an AIS (Version 2) given in this document (input document e-NAV6-10-8 refers).

There appear possible some improvements regarding the Position, Navigation and Timing (PNT) aspects of the AIS. While it is recognized that the present AIS (= Version 1) is not fully capable of supporting the following PNT aspects, a future AIS (Version 2) could be made capable of fully supporting PNT from the outset.

The following features should be included:

- Pseudo range measurement capabilities of the AIS, i.e. improvement of the AIS-immanent Time of Emission (TOE) capabilities for Time of Arrival (TOA) measurements in order to calculate pseudo ranges between participating AIS stations:

- Support for Ranging Mode (R-Mode) for mobiles: Mobile stations

- Mobile AIS stations calculate their position by using AIS transmissions from AIS base stations

- Mobile AIS stations calculate their position by using AIS transmissions from other AIS mobile stations

- Mobile AIS stations combine all available pseudo ranges, both from base stations and other mobiles

- Support for Inverse Ranging Mode (Inverse R-Mode) for shore stations: AIS shore stations use pseudo range measurements to validate the position report of an AIS mobile station.

- Improved integrity and accuracy tagging of the AIS position reports:

- Separate integrity tagging from accuracy tagging in a position report, because these data items are orthogonal: While a position report may be highly accurate, its integrity may be questionable; also, a fully reliable position report (integrity given) may not be highly accurate.

- Improve integrity information for the position given in the report: shipboard source(s) of integrity information used

- Improve accuracy information including more levels of accuracy

These improvements should be included in the list of requirements for AIS (Version 2).

The WG2 of the IALA e-Navigation Committee is invited to consider these improvements and to liaise with WG3 regarding incorporation of PNT aspects in the list of requirements for such an AIS (Version 2).

When drafting the PNT contributions to the IALA strategic master plan regarding e-Navigation (document e-NAV6-10-8 refers again), the WG2 is invited to consider the necessary task breakdown for the above PNT aspects developed by the German administration provided in their input document to e-NAV 4 Committee meeting (Shanghai, 2008-February; input document eNAV4-7-10a/b; refer in particular to part II (= eNAV4-7-10b) of that document).